## Performance results:

| ### SDSL Cell Pipe Line performance 2320Mbps – 24AWG |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length is changed on the CPE (one of the pipes) side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data rate=2320Mbps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AWG=24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 0 | 2 |  | 4 |  | 6 |  | 8 |  | 10 |  |  |
| Sync rate Down |  | 2320 | 2320 |  | 2320 |  | 2320 |  | 2320 |  | 2320 |  |  |
| (kbps) Up |  | 2320 | 2320 |  | 2320 |  | 2320 |  | 2320 |  | 2320 |  |  |
| FTP Down |  | 2052 | 2052 |  | 2052 |  | 2052 |  | 2052 |  | 2052 |  |  |
| (kbps) Up |  | 2052 | 2052 |  | 2052 |  | 2052 |  | 2052 |  | 2052 |  |  |
| CPE Attn |  | 0 | 3 |  | 5 |  | 8 |  | 10 |  | 12 |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 15 | 15 |  | 15 |  | 15 |  | 11 |  | 4 |  |  |
| CRC |  | 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |
| AWG=24 with noise impairment (T1.601 test w/ level @ -45dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 0 | 2 |  | 4 |  | 6 |  | 8 |  | 10 |  |  |
| Sync rate Down |  | 2320 | 2320 |  | 2320 |  | 2320 |  | 2320 |  | 2320 |  |  |
| (kbps) Up |  | 2320 | 2320 |  | 2320 |  | 2320 |  | 2320 |  | 2320 |  |  |
| FTP Down |  | 2052 | 2052 |  | 2052 |  | 2052 |  | 2052 |  | 2052 |  |  |
| (kbps) Up |  | 2052 | 2052 |  | 2052 |  | 2052 |  | 2052 |  | 2052 |  |  |
| CPE Attn |  | 0 | 3 |  | 5 |  | 8 |  | 10 |  | 12 |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 15 | 15 |  | 15 |  | 15 |  | 10 |  | 2 |  |  |
| CRC |  | 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |
| AWG=24 with noise impairment (dslnext w/ level @ -53.8 dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 0 | 2 |  | 4 |  | 6 |  | 8 |  | 10 |  |  |
| Sync rate Down |  | 2320 | 2320 |  | 2320 |  | 2320 |  | 2320 |  | 2320 |  |  |
| (kbps) Up |  | 2320 | 2320 |  | 2320 |  | 2320 |  | 2320 |  | 2320 |  |  |
| FTP Down |  | 2052 | 2052 |  | 2052 |  | 2052 |  | 2052 |  | 2052 |  |  |
| (kbps) Up |  | 2052 | 2052 |  | 2052 |  | 2052 |  | 2052 |  | 2052 |  |  |
| CPE Attn |  | 0 | 3 |  | 5 |  | 8 |  | 10 |  | 12 |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 15 | 15 |  | 15 |  | 15 |  | 12 |  | 3 |  |  |
| CRC |  | 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |
| AWG = 24AWG Bridge Tap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Note: Tap A @coe side and Tap B @cpe side) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 7.5 | 7.5 |  | 7.5 |  |  |  |  |  |  |  |  |
| Tap A (k-ft) |  | 1.5 | 1.5 |  | 0 |  |  |  |  |  |  |  |  |
| Tap B (k-ft) |  | 1.5 | 0 |  | 15 |  |  |  |  |  |  |  |  |
| Sync rate Down |  | 2320 | 2320 |  | 2320 |  |  |  |  |  |  |  |  |
| (kbps) Up |  | 2320 | 2320 |  | 2320 |  |  |  |  |  |  |  |  |
| FTP Down |  | 2052 | 2052 |  | 2052 |  |  |  |  |  |  |  |  |
| (kbps) Up |  | 2052 | 2052 |  | 2052 |  |  |  |  |  |  |  |  |
| CPE Attn |  | 11 | 10 |  | 10 |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 1 | 6 |  | 10 |  |  |  |  |  |  |  |  |
| CRC |  | 0 | 0 |  | 0 |  |  |  |  |  |  |  |  |
| ### SDSL Cell Pipe Line performance 2320Mbps – 26AWG |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length is changed on the CPE (one of the pipes) side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data rate=2320Mbps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AWG=26 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  | 2 |  | 4 |  | 6 |  | 8 |  |  |  |
| Sync rate Down | 2320 |  |  | 2320 |  | 2320 |  | 2320 |  | 2320 |  |  |  |
| (kbps) Up | 2320 |  |  | 2320 |  | 2320 |  | 2320 |  | 2320 |  |  |  |
| FTP Down | 2052 |  |  | 2052 |  | 2052 |  | 2052 |  | 2052 |  |  |  |
| (kbps) Up | 2052 |  |  | 2052 |  | 2052 |  | 2052 |  | 2052 |  |  |  |
| CPE Attn | 0 |  |  | 4 |  | 7 |  | 10 |  | 16 |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  | 15 |  | 15 |  | 10 |  | 3 |  |  |  |
| CRC | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  |  |  |
| AWG=26 with noise impairment (T1.601 test w/ level @ -45dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  | 2 |  | 4 |  | 6 |  |  |  |  |  |
| Sync rate Down | 2320 |  |  | 2320 |  | 2320 |  | 2320 |  |  |  |  |  |
| (kbps) Up | 2320 |  |  | 2320 |  | 2320 |  | 2320 |  |  |  |  |  |
| FTP Down | 2052 |  |  | 2052 |  | 2052 |  | 2052 |  |  |  |  |  |
| (kbps) Up | 2052 |  |  | 2052 |  | 2052 |  | 2052 |  |  |  |  |  |
| CPE Attn | 0 |  |  | 4 |  | 7 |  | 10 |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  | 15 |  | 15 |  | 11 |  |  |  |  |  |
| CRC | 0 |  |  | 0 |  | 0 |  | 0 |  |  |  |  |  |
| AWG=26 with noise impairment (dslnext w/level @ -53.8 dBM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  | 2 |  | 4 |  | 6 |  | 8 |  |  |  |
| Sync rate Down | 2320 |  |  | 2320 |  | 2320 |  | 2320 |  | 2320 |  |  |  |
| (kbps) Up | 2320 |  |  | 2320 |  | 2320 |  | 2320 |  | 2320 |  |  |  |
| FTP Down | 2052 |  |  | 2052 |  | 2052 |  | 2052 |  | 2052 |  |  |  |
| (kbps) Up | 2052 |  |  | 2052 |  | 2052 |  | 2052 |  | 2052 |  |  |  |
| CPE Attn | 0 |  |  | 4 |  | 7 |  | 10 |  | 16 |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  | 15 |  | 15 |  | 13 |  | 3 |  |  |  |
| CRC | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |  |  |  |
| AWG = 26 AWG Bridge Tap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Note: Tap A @coe side and Tap B @cpe side) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 5 |  |  | 5 |  | 5 |  |  |  |  |  |  |  |
| Tap A (k-ft) | 1.5 |  |  | 1.5 |  | 0 |  |  |  |  |  |  |  |
| Tap B (k-ft) | 1.5 |  |  | 0 |  | 15 |  |  |  |  |  |  |  |
| Sync rate Down | 2320 |  |  | 2320 |  | 2320 |  |  |  |  |  |  |  |
| (kbps) Up | 2320 |  |  | 2320 |  | 2320 |  |  |  |  |  |  |  |
| FTP Down | 2052 |  |  | 2052 |  | 2052 |  |  |  |  |  |  |  |
| (kbps) Up | 2052 |  |  | 2052 |  | 2052 |  |  |  |  |  |  |  |
| CPE Attn | 11 |  |  | 10 |  | 10 |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 7 |  |  | 15 |  | 11 |  |  |  |  |  |  |  |
| CRC | 0 |  |  | 0 |  | 0 |  |  |  |  |  |  |  |

| ### SDSL Cell Pipe Line performance 1552Mbps – 24AWG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SDSL Cell Pipe XAVi 20s to Stinger |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note :1. Stinger ver:7.11.3c10, XAVi sdsl pipe ver: 2.11B (May/26/2000) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Length is changed on the CPE (one of the pipes) side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data rate=1552Mbps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AWG=24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 0 |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  | 14 |  |
| Sync rate Down |  | 1552 |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |
| (kbps) Up |  | 1552 |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |
| FTP Down |  | 1368 |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |
| (kbps) Up |  | 1368 |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |
| CPE Attn |  | 0 |  | 3 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 13 |  |  | 18 |  |  | 21 |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 15 |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 9 |  |  | 3 |  |
| CRC |  | 0 |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |
| AWG=24 with noise impairment (T1.601 test w/ level @ -45dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 0 |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  |  |  |
| Sync rate Down |  | 1552 |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |  |  |
| (kbps) Up |  | 1552 |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |  |  |
| FTP Down |  | 1368 |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |  |  |
| (kbps) Up |  | 1368 |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |  |  |
| CPE Attn |  | 0 |  | 3 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 13 |  |  | 17 |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 15 |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 11 |  |  | 6 |  |  |  |  |
| CRC |  | 0 |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |
| AWG=24 with noise impairment (dslnext w/ level @ -53.8 dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 0 |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  |  |  |
| Sync rate Down |  | 1552 |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |  |  |
| (kbps) Up |  | 1552 |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |  |  |
| FTP Down |  | 1368 |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |  |  |
| (kbps) Up |  | 1368 |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |  |  |
| CPE Attn |  | 0 |  | 3 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 13 |  |  | 18 |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 15 |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 8 |  |  |  |  |
| CRC |  | 0 |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |
| AWG = 24AWG Bridge Tap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Note: Tap A @coe side and Tap B @cpe side) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 7.5 |  | 7.5 |  |  | 7.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap A (k-ft) |  | 1.5 |  | 1.5 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap B (k-ft) |  | 1.5 |  | 0 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down |  | 1552 |  | 1552 |  |  | 1552 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  | 1552 |  | 1552 |  |  | 1552 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down |  | 1368 |  | 1368 |  |  | 1368 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  | 1368 |  | 1368 |  |  | 1368 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn |  | 12 |  | 11 |  |  | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 12 |  | 15 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### SDSL Cell Pipe Line performance 1552Mbps – 26AWG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length is changed on the CPE (one of the pipes) side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data rate=1552Mbps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AWG=26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  |  |  |  |  |
| Sync rate Down | 1552 |  |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |  |  |  |  |
| (kbps) Up | 1552 |  |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |  |  |  |  |
| FTP Down | 1368 |  |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |  |  |  |  |
| (kbps) Up | 1368 |  |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |  |  |  |  |
| CPE Attn | 0 |  |  |  | 4 |  |  | 8 |  |  | 11 |  |  | 16 |  |  | 22 |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 11 |  |  | 5 |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |  |  |
| AWG=26 with noise impairment (T1.601 test w/ level @ -45dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  |  |  |  |  |
| Sync rate Down | 1552 |  |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | Failed |  |  |  |  |  |  |
| (kbps) Up | 1552 |  |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |  |  |  |  |  |  |  |
| FTP Down | 1368 |  |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |  |  |  |  |  |  |  |
| (kbps) Up | 1368 |  |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |  |  |  |  |  |  |  |
| CPE Attn | 0 |  |  |  | 4 |  |  | 8 |  |  | 11 |  |  | 16 |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 8 |  |  |  |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |
| AWG=26 with noise impairment (dslnext w/level @ -53.8 dBM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  |  |  |  |  |
| Sync rate Down | 1552 |  |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |  |  |  |  |
| (kbps) Up | 1552 |  |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |  |  |  |  |
| FTP Down | 1368 |  |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |  |  |  |  |
| (kbps) Up | 1368 |  |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |  |  |  |  |
| CPE Attn | 0 |  |  |  | 4 |  |  | 8 |  |  | 11 |  |  | 16 |  |  | 22 |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 11 |  |  | 4 |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |  |  |
| AWG = 26 AWG Bridge Tap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Note: Tap A @coe side and Tap B @cpe side) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 5 |  |  |  | 5 |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap A (k-ft) | 1.5 |  |  |  | 1.5 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap B (k-ft) | 1.5 |  |  |  | 0 |  |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down | 1552 |  |  |  | 1552 |  |  | 1552 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up | 1552 |  |  |  | 1552 |  |  | 1552 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down | 1368 |  |  |  | 1368 |  |  | 1368 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up | 1368 |  |  |  | 1368 |  |  | 1368 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn | 12 |  |  |  | 11 |  |  | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Loop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSA |  |  | 1 |  |  | 2 |  |  | 4 |  |  | 5 |  |  | 6 |  |  | 7 |  |  | 8 |  |  |
| Sync rate Down |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |
| (kbps) Up |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  | 1552 |  |  |
| FTP Down |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |
| (kbps) Up |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  | 1368 |  |  |
| CPE Attn |  |  | 17 |  |  | 15 |  |  | 17 |  |  | 18 |  |  | 19 |  |  | 16 |  |  | 18 |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 10 |  |  | 10 |  |  | 5 |  |  | 11 |  |  | 8 |  |  | 8 |  |  | 8 |  |  |
| CRC |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |

| ### SDSL Cell Pipe Line performance 1168Mbps – 24AWG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length is changed on the CPE (one of the pipes) side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data rate=1168Kbps |  |  |  |  |  |  |  |  |  |  |
| Test Results: |  |  |  |  |  |  |  |  |  |
| AWG=24 |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 15 |
| Sync rate Down | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 |
| (kbps) Up | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 |
| FTP Down | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 |
| (kbps) Up | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 |
| CPE Attn | 0 | 3 | 6 | 8 | 10 | 13 | 17 | 21 | 22 |
| Q |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |
| Q | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 10 | 7 |
| CRC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| AWG=24 with noise impairment (T1.601 test w/ level @ -45dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 |  |
| Sync rate Down | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 |  |
| (kbps) Up | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 |  |
| FTP Down | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 |  |
| (kbps) Up | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 |  |
| CPE Attn | 0 | 3 | 6 | 8 | 10 | 13 | 17 | 21 |  |
| Q |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |
| Q | 15 | 15 | 15 | 15 | 15 | 14 | 7 | 4 |  |
| CRC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| AWG=24 with noise impairment (dslnext w/ level @ -53.8 dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 15 |
| Sync rate Down | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 |
| (kbps) Up | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 | 1168 |
| FTP Down | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 |
| (kbps) Up | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 | 1032 |
| CPE Attn | 0 | 3 | 6 | 8 | 10 | 13 | 17 | 21 | 22 |
| Q |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |
| Q | 15 | 15 | 15 | 15 | 15 | 15 | 14 | 8 | 6 |
| CRC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| AWG = 24AWG Bridge Tap |  |  |  |  |  |  |  |  |  |  |  |
| (Note: Tap A @coe side and Tap B @cpe side) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 7.5 | 7.5 | 7.5 |  |  |  |  |  |  |
| Tap A (k-ft) | 1.5 | 1.5 | 0 |  |  |  |  |  |  |
| Tap B (k-ft) | 1.5 | 0 | 15 |  |  |  |  |  |  |
| Sync rate Down | 1168 | 1168 | 1168 |  |  |  |  |  |  |
| (kbps) Up | 1168 | 1168 | 1168 |  |  |  |  |  |  |
| FTP Down | 1032 | 1032 | 1032 |  |  |  |  |  |  |
| (kbps) Up | 1032 | 1032 | 1032 |  |  |  |  |  |  |
| CPE Attn | 12 | 11 | 11 |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |
| Q | 15 | 15 | 15 |  |  |  |  |  |  |
| CRC | 0 | 0 | 0 |  |  |  |  |  |  |

| ### SDSL Cell Pipe Line performance 1168Mbps – 26AWG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SDSL Cell Pipe XAVi 20s to Stinger |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note :1. Stinger ver:7.11.3c10, XAVi sdsl pipe ver: 2.11B (May/26/2000) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Length is changed on the CPE (one of the pipes) side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data rate=1168Kbps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AWG=26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  |  |  |  |  |
| Sync rate Down | 1168 |  |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  |  |  |  |  |
| (kbps) Up | 1168 |  |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  |  |  |  |  |
| FTP Down | 1032 |  |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  |  |  |  |  |
| (kbps) Up | 1032 |  |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  |  |  |  |  |
| CPE Attn | 0 |  |  |  | 4 |  |  | 8 |  |  | 11 |  |  | 16 |  |  | 21 |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 10 |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |  |  |
| AWG=26 with noise impairment (T1.601 test w/ level @ -45dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  |  |  |  |  |
| Sync rate Down | 1168 |  |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  |  |  |  |  |
| (kbps) Up | 1168 |  |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  |  |  |  |  |
| FTP Down | 1032 |  |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  |  |  |  |  |
| (kbps) Up | 1032 |  |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  |  |  |  |  |
| CPE Attn | 0 |  |  |  | 4 |  |  | 8 |  |  | 11 |  |  | 16 |  |  | 21 |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 9 |  |  | 3 |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |  |  |
| AWG=26 with noise impairment (dslnext w/level @ -53.8 dBM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  |  |  |  |  |
| Sync rate Down | 1168 |  |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  |  |  |  |  |
| (kbps) Up | 1168 |  |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  |  |  |  |  |
| FTP Down | 1032 |  |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  |  |  |  |  |
| (kbps) Up | 1032 |  |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  |  |  |  |  |
| CPE Attn | 0 |  |  |  | 4 |  |  | 8 |  |  | 11 |  |  | 16 |  |  | 21 |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 9 |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |  |  |
| AWG = 26 AWG Bridge Tap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Note: Tap A @coe side and Tap B @cpe side) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 5 |  |  |  | 5 |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap A (k-ft) | 1.5 |  |  |  | 1.5 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap B (k-ft) | 1.5 |  |  |  | 0 |  |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down | 1168 |  |  |  | 1168 |  |  | 1168 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up | 1168 |  |  |  | 1168 |  |  | 1168 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down | 1032 |  |  |  | 1032 |  |  | 1032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up | 1032 |  |  |  | 1032 |  |  | 1032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn | 12 |  |  |  | 11 |  |  | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Loop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSA |  |  | 1 |  |  | 2 |  |  | 4 |  |  | 5 |  |  | 6 |  |  | 7 |  | 8 |  |  |  |
| Sync rate Down |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  | 1168 |  |  |  |
| (kbps) Up |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  |  | 1168 |  | 1168 |  |  |  |
| FTP Down |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  | 1032 |  |  |  |
| (kbps) Up |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  |  | 1032 |  | 1032 |  |  |  |
| CPE Attn |  |  | 16 |  |  | 11 |  |  | 17 |  |  | 18 |  |  | 19 |  |  | 15 |  | 17 |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 13 |  |  | 15 |  |  | 9 |  |  | 15 |  |  | 13 |  |  | 14 |  | 15 |  |  |  |
| CRC |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  | 0 |  |  |  |
| EXT CSA |  |  | 9 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down |  |  | 1168 |  |  | 1168 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 1168 |  |  | 1168 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down |  |  | 1032 |  |  | 1032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 1032 |  |  | 1032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn |  |  | 21 |  |  | 23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 8 |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MID-CSA |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down |  |  | 1168 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 1168 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down |  |  | 1032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 1032 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn |  |  | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ### SDSL Cell Pipe Line performance 784Mbps – 24AWG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length is changed on the CPE (one of the pipes) side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data rate=784Kbps |  |  |  |  |  |  |  |  |  |  |
| Test Results: |  |  |  |  |  |  |  |  |  |
| AWG=24 |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 15 |
| Sync rate Down | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 |
| (kbps) Up | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 |
| FTP Down | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 |
| (kbps) Up | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 |
| CPE Attn | 0 | 3 | 5 | 8 | 10 | 13 | 15 | 19 | 21 |
| Q |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |
| Q | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| CRC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| AWG=24 with noise impairment (T1.601 test w/ level @ -45dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 15 |
| Sync rate Down | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 |
| (kbps) Up | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 |
| FTP Down | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 |
| (kbps) Up | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 |
| CPE Attn | 0 | 3 | 5 | 8 | 10 | 13 | 16 | 19 | 21 |
| Q |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |
| Q | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 10 | 7 |
| CRC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| AWG=24 with noise impairment (dslnext w/ level @ -53.8 dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 15 |
| Sync rate Down | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 |
| (kbps) Up | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 | 784 |
| FTP Down | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 |
| (kbps) Up | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 | 684 |
| CPE Attn | 0 | 3 | 5 | 8 | 10 | 13 | 16 | 19 | 21 |
| Q |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |
| Q | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 13 |
| CRC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| AWG = 24AWG Bridge Tap |  |  |  |  |  |  |  |  |  |  |  |
| (Note: Tap A @coe side and Tap B @cpe side) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 7.5 | 7.5 | 7.5 |  |  |  |  |  |  |
| Tap A (k-ft) | 1.5 | 1.5 | 0 |  |  |  |  |  |  |
| Tap B (k-ft) | 1.5 | 0 | 15 |  |  |  |  |  |  |
| Sync rate Down | 784 | 784 | 784 |  |  |  |  |  |  |
| (kbps) Up | 784 | 784 | 784 |  |  |  |  |  |  |
| FTP Down | 684 | 684 | 684 |  |  |  |  |  |  |
| (kbps) Up | 684 | 684 | 684 |  |  |  |  |  |  |
| CPE Attn | 12 | 11 | 11 |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |
| Q | 15 | 15 | 15 |  |  |  |  |  |  |
| CRC | 0 | 0 | 0 |  |  |  |  |  |  |

| ### SDSL Cell Pipe Line performance 784Mbps – 26AWG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length is changed on the CPE (one of the pipes) side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data rate=784Kbps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AWG=26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  |  |  |
| Sync rate Down | 784 |  |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  |  |  |
| (kbps) Up | 784 |  |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  |  |  |
| FTP Down | 684 |  |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  |  |  |
| (kbps) Up | 684 |  |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  |  |  |
| CPE Attn | 0 |  |  |  | 4 |  |  | 7 |  |  | 11 |  |  | 15 |  |  | 20 |  |  | 23 |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 11 |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |
| AWG=26 with noise impairment (T1.601 test w/ level @ -45dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 2 |  |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  |  |  |  |  |  |
| Sync rate Down | 784 |  |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | FAILED |  |  |  |  |  |  |  |
| (kbps) Up | 784 |  |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  |  |  |  |  |  |  |  |  |
| FTP Down | 684 |  |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up | 684 |  |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  |  |  |  |  |  |  |  |  |
| CPE Attn | 4 |  |  |  | 7 |  |  | 11 |  |  | 15 |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 8 |  |  |  |  |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |
| AWG=26 with noise impairment (dslnext w/level @ -53.8 dBM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 2 |  |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  |  |  |  |  |  |
| Sync rate Down | 784 |  |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  |  |  |  |  |  |
| (kbps) Up | 784 |  |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  |  |  |  |  |  |
| FTP Down | 684 |  |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  |  |  |  |  |  |
| (kbps) Up | 684 |  |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  |  |  |  |  |  |
| CPE Attn | 4 |  |  |  | 7 |  |  | 11 |  |  | 15 |  |  | 20 |  |  | 23 |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 8 |  |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |
| AWG = 26 AWG Bridge Tap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Note: Tap A @coe side and Tap B @cpe side) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 5 |  |  |  | 5 |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap A (k-ft) | 1.5 |  |  |  | 1.5 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap B (k-ft) | 1.5 |  |  |  | 0 |  |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down | 784 |  |  |  | 784 |  |  | 784 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up | 784 |  |  |  | 784 |  |  | 784 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down | 684 |  |  |  | 684 |  |  | 684 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up | 684 |  |  |  | 684 |  |  | 684 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn | 11 |  |  |  | 10 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Loop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSA |  |  | 1 |  |  | 2 |  |  | 4 |  |  | 5 |  |  | 6 |  |  | 7 |  |  | 8 |  |  |  |
| Sync rate Down |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  |  |
| (kbps) Up |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  | 784 |  |  |  |
| FTP Down |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  |  |
| (kbps) Up |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  | 684 |  |  |  |
| CPE Attn |  |  | 15 |  |  | 14 |  |  | 15 |  |  | 16 |  |  | 17 |  |  | 15 |  |  | 16 |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  |  |
| CRC |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |
| EXT CSA |  |  | 9 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down |  |  | 784 |  |  | 784 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 784 |  |  | 784 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down |  |  | 684 |  |  | 684 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 684 |  |  | 684 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn |  |  | 19 |  |  | 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 14 |  |  | 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MID-CSA |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down |  |  | 784 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 784 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down |  |  | 684 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 684 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn |  |  | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ANSI loop |  |  | 2 |  |  | 5 |  |  | 7 |  |  | 8 |  |  | 9 |  |  | 13 |  |  |  |  |  |  |
| Sync rate Down |  |  | FAILED |  |  | FAILED |  |  | FAILED |  |  | FAILED |  |  | FAILED |  |  | FAILED |  |  |  |  |  |  |
| (kbps) Up |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ### SDSL Cell Pipe Line performance 144Mbps – 24AWG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length is changed on the CPE (one of the pipes) side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data rate=144Kbps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AWG=24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 0 |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  | 14 | 15 |  |  |  |  |  |
| Sync rate Down |  | 144 |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 | 144 |  |  |  |  |  |
| (kbps) Up |  | 144 |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 | 144 |  |  |  |  |  |
| FTP Down |  | 120 |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 | 120 |  |  |  |  |  |
| (kbps) Up |  | 120 |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 | 120 |  |  |  |  |  |
| CPE Attn |  | 0 |  | 3 |  |  | 5 |  |  | 8 |  |  | 11 |  |  | 14 |  |  | 17 |  |  | 20 | 21 |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 15 |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 | 15 |  |  |  |  |  |
| CRC |  | 0 |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 | 0 |  |  |  |  |  |
| AWG=24 with noise impairment (T1.601 test w/ level @ -45dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 0 |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  | 14 | 15 |  |  |  |  |  |
| Sync rate Down |  | 144 |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 | 144 |  |  |  |  |  |
| (kbps) Up |  | 144 |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 | 144 |  |  |  |  |  |
| FTP Down |  | 120 |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 | 120 |  |  |  |  |  |
| (kbps) Up |  | 120 |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 | 120 |  |  |  |  |  |
| CPE Attn |  | 0 |  | 3 |  |  | 5 |  |  | 8 |  |  | 11 |  |  | 14 |  |  | 17 |  |  | 20 | 21 |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 15 |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 | 15 |  |  |  |  |  |
| CRC |  | 0 |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 | 0 |  |  |  |  |  |
| AWG=24 with noise impairment (dslnext w/ level @ -53.8 dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 0 |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  | 14 | 15 |  |  |  |  |  |
| Sync rate Down |  | 144 |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 | 144 |  |  |  |  |  |
| (kbps) Up |  | 144 |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 | 144 |  |  |  |  |  |
| FTP Down |  | 120 |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 | 120 |  |  |  |  |  |
| (kbps) Up |  | 120 |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 | 120 |  |  |  |  |  |
| CPE Attn |  | 0 |  | 3 |  |  | 5 |  |  | 8 |  |  | 11 |  |  | 14 |  |  | 17 |  |  | 20 | 21 |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 15 |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 | 15 |  |  |  |  |  |
| CRC |  | 0 |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 | 0 |  |  |  |  |  |
| AWG = 24AWG Bridge Tap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Note: Tap A @coe side and Tap B @cpe side) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) |  | 7.5 |  | 7.5 |  |  | 7.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap A (k-ft) |  | 1.5 |  | 1.5 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap B (k-ft) |  | 1.5 |  | 0 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down |  | 144 |  | 144 |  |  | 144 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  | 144 |  | 144 |  |  | 144 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down |  | 120 |  | 120 |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  | 120 |  | 120 |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn |  | 11 |  | 10 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  | 15 |  | 15 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  | 0 |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### SDSL Cell Pipe Line performance 144Mbps – 26AWG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length is changed on the CPE (one of the pipes) side |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data rate=144Kbps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Results: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AWG=26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  |  | 14 |  |  | 16 | 18 |
| Sync rate Down | 144 |  |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  |  | 144 |  |  | 144 | 144 |
| (kbps) Up | 144 |  |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  |  | 144 |  |  | 144 | 144 |
| FTP Down | 120 |  |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  |  | 120 |  |  | 120 | 120 |
| (kbps) Up | 120 |  |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  |  | 120 |  |  | 120 | 120 |
| CPE Attn | 0 |  |  |  | 4 |  |  | 8 |  |  | 12 |  |  | 16 |  |  | 20 |  |  | 23 |  |  |  | 26 |  |  | 29 | 31 |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  |  | 15 |  |  | 13 | 9 |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  | 0 |  |  | 0 | 0 |
| AWG=26 with noise impairment (T1.601 test w/ level @ -45dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  |  | 14 |  |  | 16 | 18 |
| Sync rate Down | 144 |  |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  |  | 144 |  |  | 144 | 144 |
| (kbps) Up | 144 |  |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  |  | 144 |  |  | 144 | 144 |
| FTP Down | 120 |  |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  |  | 120 |  |  | 120 | 120 |
| (kbps) Up | 120 |  |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  |  | 120 |  |  | 120 | 120 |
| CPE Attn | 0 |  |  |  | 4 |  |  | 8 |  |  | 12 |  |  | 16 |  |  | 20 |  |  | 23 |  |  |  | 26 |  |  | 29 | 31 |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  |  | 11 |  |  | 7 | 4 |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  | 0 |  |  | 0 | 0 |
| AWG=26 with noise impairment (dslnext w/level @ -53.8 dBM) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 0 |  |  |  | 2 |  |  | 4 |  |  | 6 |  |  | 8 |  |  | 10 |  |  | 12 |  |  |  | 14 |  |  | 16 | 18 |
| Sync rate Down | 144 |  |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  |  | 144 |  |  | 144 | 144 |
| (kbps) Up | 144 |  |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  |  | 144 |  |  | 144 | 144 |
| FTP Down | 120 |  |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  |  | 120 |  |  | 120 | 120 |
| (kbps) Up | 120 |  |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  |  | 120 |  |  | 120 | 120 |
| CPE Attn | 0 |  |  |  | 4 |  |  | 8 |  |  | 12 |  |  | 16 |  |  | 20 |  |  | 23 |  |  |  | 26 |  |  | 29 | 31 |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  |  | 13 |  |  | 9 | 6 |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  | 0 |  |  | 0 | 0 |
| AWG = 26 AWG Bridge Tap |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Note: Tap A @coe side and Tap B @cpe side) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length (k-ft) | 5 |  |  |  | 5 |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap A (k-ft) | 1.5 |  |  |  | 1.5 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tap B (k-ft) | 1.5 |  |  |  | 0 |  |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down | 144 |  |  |  | 144 |  |  | 144 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up | 144 |  |  |  | 144 |  |  | 144 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down | 120 |  |  |  | 120 |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up | 120 |  |  |  | 120 |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn | 10 |  |  |  | 10 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q | 15 |  |  |  | 15 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC | 0 |  |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Loop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSA |  |  | 1 |  |  | 2 |  |  | 4 |  |  | 5 |  |  | 6 |  |  | 7 |  |  | 8 |  |  |  |  |  |  |  |
| Sync rate Down |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  |  |  |  |  |  |
| FTP Down |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  |  |  |  |  |  |
| CPE Attn |  |  | 16 |  |  | 14 |  |  | 16 |  |  | 17 |  |  | 18 |  |  | 15 |  |  | 17 |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  |  |  |  |  |  |
| CRC |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |
| EXT CSA |  |  | 9 |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down |  |  | 144 |  |  | 144 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 144 |  |  | 144 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down |  |  | 120 |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 120 |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn |  |  | 19 |  |  | 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 15 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MID-CSA |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sync rate Down |  |  | 144 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 144 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FTP Down |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CPE Attn |  |  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ANSI loop |  |  | 1 |  |  | 2 |  |  | 5 |  |  | 7 |  |  | 8 |  |  | 9 |  |  | 13 |  |  |  |  |  |  |  |
| Sync rate Down |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  | 144 |  |  |  |  |  |  |  |
| FTP Down |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  |  |  |  |  |  |
| (kbps) Up |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  | 120 |  |  |  |  |  |  |  |
| CPE Attn |  |  | 30 |  |  | 28 |  |  | 17 |  |  | 25 |  |  | 25 |  |  | 23 |  |  | 23 |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COE Attn |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q |  |  | 10 |  |  | 13 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  | 15 |  |  |  |  |  |  |  |
| CRC |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |