### AlphaServer 1200 Pedestal—Rackmount
V2.5—3 March 1998
DIGITAL Systems and Options Catalog

Product Description 	AlphaServer 1200 is an Alpha microprocessor server, available in 400 MHz and 533 MHz versions with 4 MB ECC cache in a pedestal enclosure. Systems support two CPUs and are available with a choice of three popular operating systems:  DIGITAL UNIX, OpenVMS, and Microsoft Windows NT Server.  AlphaServer 1200 offers the following high-reliability features: heat sensor, fan failure and power supply sensors, and ECC memory. Integrated on system motherboard are Fast Narrow Single-Ended SCSI-2 controller, diskette controller, two serial ports, one parallel port, and keyboard and mouse controller.  Packaged systems also include memory, an UltraSCSI controller, 10/100 Ethernet adapter and 4.3 GB of UltraSCSI storage.

AlphaServer 1200 Pedestal System supports up to 10 internal storage devices including floppy diskette drive, CD-ROM, one additional 5.25-inch removable media device, and seven hot swap StorageWorks wide or narrow disk drives. The system supports up to 63 GB of internal storage with seven 9.0 GB disks.

AlphaServer 1200 system supports DIGITAL UltraSCSI StorageWorks components.  This wide SCSI bus means an increase in maximum transfer from 20 MB/sec to 40 MB/sec. UltraSCSI bus segments and bus expanders allow an UltraSCSI bus to run over much greater distances.

ServerWORKS Manager provides advanced server and network management capabilities and is supplied with all AlphaServer systems. Simple Network Management Protocol (SNMP) enables information to pass from the managed system to the console for DIGITAL UNIX, Windows NT, and OpenVMS systems. Using ServerWORKS, the system manager may build and view topological maps of the network. Detailed server information is viewable, including system, network, storage, and environmental information. Alarms may be set against critical system parameters and events. All AlphaServer systems are supplied with management tools to complement ServerWORKS Manager. For more information, see the StorageWorks Software section of this catalog.

DIGITAL believes the information in this publication is accurate as of its publication date; such information is subject to change without notice. DIGITAL is not responsible for any inadvertent errors.

DIGITAL conducts its business in a manner that conserves the environment and protects the safety and health of its employees, customers, and the community.

DIGITAL, the DIGITAL logo are trademarks of Digital Equipment Corporation.

Printed in USA. Copyright 1998 Digital Equipment Corporation. All rights reserved.

Step 1—AlphaServer 1200 Pedestal Systems

- Windows NT Packaged systems

- North American variants include Windows NT license, media (CD-ROM) kit North American English

- Selection of language specific Windows NT license, media (CD-ROM) kit is mandatory for all non-North American variants, see Step 11.

- Windows NT software will be factory installed (FIS) on hard disk drive

- DIGITAL UNIX and OpenVMS Packaged systems include factory installed (FIS) software on hard disk drive.

- Options ordered  that are factory installable, will be factory installed unless specified as spares.

- Minimum supported operating systems are:

- DIGITAL UNIX V3.2G

- OpenVMS V7.1-1H1

- Windows NT 4.0

Pedestal Systems include

- Alpha microprocessor 21164

- 400-MHz CPU with 4 MB ECC onboard cache, or

- 533-MHz CPU with 4 MB ECC onboard cache

- Pedestal enclosure with:

- 6 expansion slots
Five PCI slots and one PCI/EISA combination slot

- Integral Fast Narrow Single-Ended SCSI adapter for internal removable media devices (CD-ROM and tape) support only

- 16 DIMM memory slots support 8 memory options

- 10 storage slots
One diskette drive slot
One CD-ROM drive slot
One additional removable media slot
Seven StorageWorks wide and/or narrow hard drive slots

- Dual 450-Watt self-sensing, self-switching (120/240V, 60/50Hz) power supplies

- Two serial ports, support full duplex asynchronous modem control

- One bi-directional enhanced parallel port

- PS/2 style keyboard port and mouse port

- 1.44 MB diskette drive in dedicated slot

- PCI-based one-port UltraSCSI Single-Ended adapter—uses one PCI slot

- PCI-based Fast 100 Ethernet (Twisted Pair)—uses one PCI slot

- 600 MB CD-ROM—uses one removable media slot

- One 4.3 GB UltraSCSI  hard disk drive

- Memory—64 MB (Windows NT only), 128 MB or 256 MB (DIGITAL UNIX and OpenVMS)

- 3-button mouse

- 120V power cord.  Mandatory selection of country specific power cord for all non-North American variants, see Step 12.

- Windows NT and DIGITAL UNIX variants include a 101-key, PC style U.S. English keyboard.

- OpenVMS variants include a 108 key, VT style U.S. English keyboard.

- Select country specific keyboard for all non-U.S.

- English language documentation.  Certain language selections are available; see Step 11.

- Integrated Advanced Server Management features, including ServerWorks Manager kit

- Hardware Warranty: Three-year on-site*

- Software Warranty1

- 90-day SPD conformance with advisory telephone support for DIGITAL UNIX and OpenVMS

- Warranty on Microsoft's Windows NT Server 4.0 is conformance to the written material accompanying the software for a period of ninety days

- Windows NT Server 4.0 plus 10-client access license, media (CD-ROM) kit or2

- DIGITAL UNIX Unlimited User license, Server Extension license, Internet AlphaServer System Software kit, Netscape’s Enterprise Server 3.0, or

- OpenVMS base license with System Manager license, Enterprise Integrated Package V2.0

1.		Service upgrades are available; see Step 13, Hardware and Software Supplemental Services

2.	Windows NT language specific media kit is mandatory for non-North American variants; see Step 11.

Step 1—AlphaServer 1200 Pedestal Systems (continued)

Windows NT

- DIGITAL OEM license agreement for Windows NT mandates compliance with specific configuration rules:

- Windows NT Language Configuration

- North American variants include pre-installed Windows NT Server 4.0, plus 10-client access license/North American English media (CD-ROM) kit.

- For all non-North American variants, selection of language-specific Windows NT Server 4.0 license/media (CD-ROM) kit is mandatory, see Step 11.

- Selection of graphics option is mandatory for Windows NT Pedestal Packaged Systems

- Video monitor is required for system management; order separately if not available at customer site.

Windows NT Pedestal Packaged Systems

Note:	Windows NT Pedestal Packaged systems have four available PCI slots.  Installation of graphics option is mandatory and reduces available PCI slots to three.

| Order Number | O/S Media | CPU | Power Cord | Keyboard | Memory | Disk | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DN-54HHA-BA | Included1 | 5/400 | 120 V | Included | 64 MB | 4.3 GB | Mandatory | Required |
| DN-54HHA-BB | Mandatory | 5/400 | Mandatory | Mandatory | 64 MB | 4.3 GB | Mandatory | Required |
| DN-54HHA-DA | Included1 | 5/400 | 120 V | Included | 256 MB | 4.3 GB | Mandatory | Required |
| DN-54HHA-DB | Mandatory | 5/400 | Mandatory | Mandatory | 256 MB | 4.3 GB | Mandatory | Required |

| DN-54KHA-BA | Included1 | 5/533 | 120 V | Included | 64 MB | 4.3 GB | Mandatory | Required |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DN-54KHA-BB | Mandatory | 5/533 | Mandatory | Mandatory | 64 MB | 4.3 GB | Mandatory | Required |
| DN-54KHA-DA | Included1 | 5/533 | 120 V | Included | 256 MB | 4.3 GB | Mandatory | Required |
| DN-54KHA-DB | Mandatory | 5/533 | Mandatory | Mandatory | 256 MB | 4.3 GB | Mandatory | Required |

1.	Includes Windows NT Server license, media (CD-ROM) kit North American English.

Key:	Mandatory items must be on purchase order at initial order acceptance.

Required items are essential for full system operation.

Recommended items enhance system functionality.

DIGITAL UNIX

- Systems include pre-installed DIGITAL UNIX V4.0B, Base license, Unlimited User license, Server Extension license, and Internet Access Software license and Netscape’s Enterprise Server 3.0.

- Systems ordered in the Americas and AP include U.S. English keyboard and 120 V power cords.  Order country specific power cords and keyboard for all systems ordered in Europe from Step 12.

- Order media and documentation separately.

DIGITAL UNIX Pedestal Packaged Systems

Note:	DIGITAL UNIX Pedestal Packaged systems have four available PCI slots.   Installation of graphics option is recommended and reduces available PCI slots to three.

| Order Number | O/S | CPU | Memory | Disk | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- |
| DA-54HHA-CA | DIGITAL UNIX | 5/400 | 128 MB | 4.3 GB | Recommended | Recommended |
| DA-54HHA-DA | DIGITAL UNIX | 5/400 | 256 MB | 4.3 GB | Recommended | Recommended |

| DA-54KHA-CA | DIGITAL UNIX | 5/533 | 128 MB | 4.3 GB | Recommended | Recommended |
| --- | --- | --- | --- | --- | --- | --- |
| DA-54KHA-DA | DIGITAL UNIX | 5/533 | 256 MB | 4.3 GB | Recommended | Recommended |

Key:	Mandatory items must be on purchase order at initial order acceptance.

Required items are essential for full system operation.

Recommended items enhance system functionality.

Step 1—AlphaServer 1200 Pedestal Systems (continued)

OpenVMS

- Systems include pre-installed OpenVMS V7.1-1H1, Base license, and DIGITAL Enterprise Integration Package that  includes: OpenVMS Management Station, DECamds, Archive Backup System for OMT, Archive Backup Agent for NT, Datametrics ViewPoint Data Collector, Datametrics ViewPoint Management Console, eXcursion, ISG/Navigator ODBC & OLE DB Server, ISG/Navigator ODBC & OLE DB Client, Internet Product Suite, Pathworks V5.0 Lan Manager, Pathworks32, Team Links Mail Windows, MAPI Driver for All-In-1, DECwindows Motif, DECprint Supervisor, DEC TCP/IP, DECnet, OpenVMS Disk Services for Windows NT, Office Server for OpenVMS, Office Server Client Access license.

- Systems ordered in the Americas and AP include U.S. English keyboard and 120 V power cords.  Order country specific power cords and keyboard for all systems ordered in Europe from Step 12.

- Order media and documentation separately.

OpenVMS Pedestal Packaged Systems

Note:	OpenVMS Pedestal Packaged systems have four available PCI slots.  Installation of graphics option is recommended and reduces available PCI slots to three.

| Order Number | O/S | CPU | Memory | Disk | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- |
| DY-54HHA-CA | OpenVMS | 5/400 | 128 MB | 4.3 GB | Recommended | Recommended |
| DY-54HHA-DA | OpenVMS | 5/400 | 256 MB | 4.3 GB | Recommended | Recommended |

| DY-54KHA-CA | OpenVMS | 5/533 | 128 MB | 4.3 GB | Recommended | Recommended |
| --- | --- | --- | --- | --- | --- | --- |
| DY-54KHA-DA | OpenVMS | 5/533 | 256 MB | 4.3 GB | Recommended | Recommended |

Key:	Mandatory items must be on purchase order at initial order acceptance.

Required items are essential for full system operation.

Recommended items enhance system functionality.

Step 2¾Additional CPU Modules (SMP Upgrades)

- AlphaServer 1200 System supports one additional CPU

- Addition CPU (i.e., 5/400 ) and operating system must be of the same type in each system .

| KN307-BB | DIGITAL UNIX SMP upgrade includes one 5/400 CPU and DIGITAL UNIX SMP license |
| --- | --- |
| KN307-BC | OpenVMS SMP upgrade includes one 5/400 CPU and OpenVMS SMP license |
| KN307-BD | Windows NT SMP upgrade includes one 5/400 CPU; SMP license not required |

| KN307-DB | DIGITAL UNIX SMP upgrade includes one 5/533 CPU and DIGITAL UNIX SMP license |
| --- | --- |
| KN307-DC | OpenVMS SMP upgrade includes one 5/533 CPU and OpenVMS SMP license |
| KN307-DD | Windows NT SMP upgrade includes one 5/533 CPU; SMP license not required |

Step 3—Memory

- Windows NT Packaged systems include one 64 MB, or one 256 MB memory kit.

- DIGITAL UNIX and OpenVMS Packaged systems include, two 64 MB (128 MB systems), or one 256 MB memory kit.

- System supports up to eight memory kits (each kit includes industry-standard DIMMs and ECC support).

- System supports maximum of 2 GB memory.

- If 64 MB option is installed in memory bank 0 only one additional 64 MB option can be installed in bank 1.

- To expand a 64 MB or 128 MB systems beyond 128 MB, select 256 MB option and installed in Memory Bank 0.  System memory can then be expanded using either 64 MB or 256 MB memory kits.

| MS300-BA | –64 MB (–32 MB DIMMs) memory kit |
| --- | --- |
| MS300-DA | 256 MB (128 MB DIMMs) memory kit |

Step 4—Storage

Internal Storage

- Pedestal enclosure supports ten storage devices

- One dedicated diskette drive slot

- Two removable media slots

- Seven 3.5-inch internal hard disk drives

- Systems include

- One UltraSCSI Single-Ended adapter (KZPBA-CA) with cable to support up to seven internal hard disk drives.

- One CD-ROM drive and support for one additional removable device connected to integral Fast Narrow Single-Ended (FNSE) SCSI-2 controller.

Step 4a—Disk Drives

| DS-RZ28L-VA | 2.1 GB 7200 RPM 8-bit Narrow SCSI disk drive |
| --- | --- |
| DS-RZ29L-VA | 4.3 GB 7200 RPM 8-bit Narrow SCSI disk drive |
| DS-RZ40-VA | 9.0 GB 7200 RPM 8-bit Narrow SCSI disk drive |

| DS-RZ1BB-VW | 2.1 GB 7200 RPM 16-bit UltraSCSI disk drive |
| --- | --- |
| DS-RZ1CB-VW | 4.3 GB 7200 RPM 16-bit UltraSCSI disk drive |
| DS-RZ1DB-VW | 9.0 GB 7200 RPM 16-bit UltraSCSI disk drive |

Internal UltraSCSI Cable Kit

| H8520-AA | UltraSCSI bus splitter kit. Required when configuring internal storage shelf in split-bus mode. Includes SCSI bus extender card, SCSI data cables, terminator, power harness assembly, and installation guide. |
| --- | --- |

Step 4b—Removable Media

- Systems include CD-ROM drive in 5.25” drive connected to integral FNSE SCSI-2 controller.

- System supports one additional removable media device. Optional 5.25” half-height removable media slot in Pedestal systems is connected to integral FNSE SCSI-2 controller.

Internal Tape Drive and additional CD-ROM

| PBXRD-DB | 600 MB 5.25 inch half-height 12X CD-ROM drive (RRD46) Frost White |
| --- | --- |
| PBXTZ-AA | 2.0 GB 5.25-inch half-height QIC SCSI tape drive (TZK11) 217 Gray |
| TLZ09-LK | 8.0 GB 5.25-inch x 1.6-inch 4 mm DAT SCSI tape drive Frost White |
| TLZ10-LK | 12/24 GB 5.25-inch 4 mm DAT SCSI tape drive Frost White |

Tape Drives and additional CD-ROM for StorageWorks shelves

| TLZ09-VA | 8 GB 4 mm DAT 93 MB/min (compressed) 5.25-inch narrow SCSI tape drive in StorageWorks carrier |
| --- | --- |
| DS-TLZ10-VA | 12/24 GB 4 mm DAT narrow SCSI tape drive in StorageWorks carrier |
| TKZ9E-VA1 | 2/5/7/10/14 GB 8 mm Helical Scan 5.25-inch SCSI tape drive in StorageWorks carrier |
| TLZ9L-VA | 32-64 GB 4 mm DAT 5.25-inch SCSI tape drive loader in StorageWorks carrier |
| DS-TZ89N-VW | 35/70 GB 5.25 inch single-ended SCSI DLT in StorageWorks carrier |
| TZ88N-VA | 20/40 GB DLT 3 MB/s SCSI cartridge tape drive in 5.25-inch StorageWorks carrier |
| TZK11-VA | 2 GB QIC SCSI tape drive in StorageWorks carrier |
| DS-RRD46-VA | 600 MB 12X CD-ROM drive in StorageWorks carrier |

1.	Supported on DIGITAL UNIX and OpenVMS systems only.

Step 4b—Removable Media (continued)

Tabletop Tape Drives

| DS-TLZ10-DB2 | 12/24 GB 4 mm DAT Tabletop narrow single-ended SCSI drive |
| --- | --- |
| DS-TZ89N-TA2 | 35/70 GB DLT Tabletop narrow single-ended SCSI drive |

2.	Requires KZPAA-AA FNSE SCSI adapter, one BN21N-02 cable, and one BN19x-2E power cord.

Step 4c—UltraSCSI StorageWorks Shelves

UltraSCSI Configuration Rules

- UltraSCSI configurations require UltraSCSI components (controllers, adapters, shelves, disks, and cables).

- UltraSCSI adapters and RAID controllers support UltraSCSI disks at UltraSCSI speeds in UltraSCSI Top Gun Blue shelves (DS-BA356-xx).

- Fast-10 BA356-xx SCSI shelves may not be retrofitted. Replace Fast-10 SCSI shelf with DS-BA356-xx shelf when adding an UltraSCSI disk and when connecting to UltraSCSI adapter.

- UltraSCSI and Fast-10 drives can be mixed in DS-BA356 Top Gun Blue UltraSCSI shelf.  Drives negotiate maximum transfer speeds with UltraSCSI adapter /controller.

- KZPAC ordered with systems will be factory reset to operate in UltraSCSI mode.

- KZPAC controllers ship in Fast-10 mode, but can run at UltraSCSI speeds by running RCU utility.

UltraSCSI Connectors and Cables

- KZPBA-CA PCI one port UltraSCSI Single-Ended adapter, 68HD male connector, requires BN38C cables (2 to 20 m) to connect to UltraSCSI shelf.

- KZPBA-CB PCI one port UltraSCSI Differential adapter, 68HD male connector, requires BN38C cables (2 to 25 m) to connect to UltraSCSI shelf.

- KZPAC-AA/CA/CB RAID controller, VHDCI male connector, require BN37A cables (.3 to 20 m) to connect to UltraSCSI shelf.

BN38C VHDCI to 68HD Cables¾KZPBA UltraSCSI adapter to UltraSCSI shelf

| BN38C-02 | 2 m VHDCI male to 68HD male |
| --- | --- |
| BN38C-03 | 3 m VHDCI male to 68HD male |
| BN38C-05 | 5 m VHDCI male to 68HD male |
| BN38C-10 | 10 m VHDCI male to 68HD male |
| BN38C-20 | 20 m VHDCI male to 68HD male |

BN37A VHDCI to VHDCI Cables¾KZPAC RAID controller to UltraSCSI shelf

| BN37A-0C | 0.3 m VHDCI male to VHDCI male |
| --- | --- |
| BN37A-0E | 0.5 m VHDCI male to VHDCI male |
| BN37A-01 | 1.0 m VHDCI male to VHDCI male |
| BN37A-1E | 1.5 m VHDCI male to VHDCI male |
| BN37A-02 | 2.0 m VHDCI male to VHDCI male |
| BN37A-2E | 2.5 m VHDCI male to VHDCI male |
| BN37A-03 | 3.0 m VHDCI male to VHDCI male |
| BN37A-05 | 5.0 m VHDCI male to VHDCI male |
| BN37A-10 | 10.0 m VHDCI male to VHDCI male |
| BN37A-15 | 15.0 m VHDCI male to VHDCI male |
| BN37A-20 | 20.0 m VHDCI male to VHDCI male |
| BN37A-25 | 25.0 m VHDCI male to VHDCI male |

Step 4c—UltraSCSI StorageWorks Shelves (continued)

DS-BA356 StorageWorks UltraSCSI Shelf Kits

DS-BA36-xx UltraSCSI Top Gun Blue shelf provides support for UltraSCSI 2.1 GB and 4.3 GB disk drives.  Shelf is configured as single-bus or split-bus configuration and includes a personality module, and a 180W power supply.

- Each DS-BA356 shelf supports

- Seven 3.5-inch hard disk drives or

- One 5.25-inch device and four 3.5-inch hard disk drives, or

- Two 5.25-inch devices and one 3.5-inch hard disk drive

- Installation of redundant power supply reduces number of 3.5-inch devices by one

| DS-BA356-KF | UltraSCSI single-ended pedestal, single-bus personality module, 180W power supply |
| --- | --- |
| DS-BA356-KG | UltraSCSI single-ended pedestal, split-bus personality module, 180W power supply |
| DS-BA356-KH | UltraSCSI pedestal, differential personality module, 180W power supply |
| DS-BA356-RC | UltraSCSI single-ended RETMA rack, single-bus personality module, 180W power supply |
| DS-BA356-RD | UltraSCSI single-ended RETMA rack, split-bus personality module, 180W power supply |
| DS-BA356-JD | UltraSCSI single-ended METRIC rack, single-bus personality module, 180W power supply |
| DS-BA356-JE | UltraSCSI single-ended METRIC rack, split-bus personality module, 180W power supply |
| DS-BA35X-HH | 180W power supply |
| DS-BA35X-FA | UltraSCSI personality module, single bus |
| DS-BA35X-FB | UltraSCSI personality module, split bus (2 buses) |

Step 4d¾Storage Controllers

- Systems include KZPBA-CA PCI one-port UltraSCSI Single-Ended adapter.

- The following adapters and controllers can be configured in AlphaServer 1200 Systems.

- Charts indicate minimum level of operating system support.  Note: Minimum operating system level support for DIGITAL UNIX with MEMORY CHANNEL is V4.0B.

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPBA-CA | PCI one port UltraSCSI Single-Ended adapter, 68HD male connector | 4 | 4 | 4 |
| BN38C-02 | 2 m SCSI cable, required for each StorageWorks shelf SCSI port attached to KZPBA adapter. |  |  |  |

Step 4d¾Storage Controllers (continued)

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPBA-CB1 | PCI-based one port UltraSCSI Differential adapter, 68HD male connector | 3 | 4 | 4 |
| BN38C-02 | 2 m VHDCI male to HD 68-pin male SCSI cable, required for each StorageWorks shelf attached to KZPBA adapter. |  |  |  |

1.	DIGITAL UNIX and OpenVMS operating systems currently support the KZPBA-CB adapter via direct attachment in existing environments.  Multi-host shared SCSI bus configurations using the KZPBA-CB adapter are not supported on DIGITAL UNIX and OpenVMS systems at this time.

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPAA-AA | PCI one port FNSE SCSI controller | 2 | 2 | 2 |
| BN21N-02 | 2 m 8-bit to 16-bit SCSI cable |  |  |  |

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPAC-AA | PCI one-port FWSE SCSI RAID controller with 4 MB cache memory; includes RAID Array 230/Plus Subsystem software and documentation kit, VHDCI male connector; tape drives not supported | 31 | 41 | 41 |
| KZPSC-UB | Cache battery-backup for KZPAC controller |  |  |  |
| BN37A-02 | 02 m UltraSCSI cable, VHDCI male connector, required for each UltraSCSI StorageWorks shelf attached to the KZPAC controller |  |  |  |

1.	Windows NT systems have three available PCI slots after installation of mandatory graphics option.   Graphics option is not mandatory for DIGITAL UNIX and OpenVMS systems, these systems have four available PCI slots.

Step 4d¾Storage Controllers (continued)

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPAC-CA | PCI three-port RAID controller with 4 MB cache memory; includes RAID Array 230/Plus Subsystem Software and documentation kit; tape drives not supported. Order SCSI cable for each active port. Requires BN31K-0E or KZPAC-SB for third port connection. | 21 | 3 | 3 |
| KZPAC-CB | Same as KZPAC-CA with 8 MB cache memory | 21 | 3 | 3 |
| KZPSC-UB | Cache battery-backup for KZPAC controller |  |  |  |
| BN37A-1E | 1.5 m UltraSCSI cable, VHDCI male connector, required for each F20 StorageWorks shelf attached to KZPAC controller. Note: Manufacturing may substitute BN37A-02 cable for certain cabinet configurations. |  |  |  |
| BN37A-02 | 02 m UltraSCSI cable |  |  |  |
| BN31S-1E | 1.5 m SCSI cable, required for each F10 StorageWorks shelf attached to KZPAC controller. |  |  |  |
| BN31K-0E | SCSI cable/bulkhead assembly kit for KZPAC-CA/CB; required for connection to third port; uses on PCI bulkhead slot |  |  |  |
| KZPAC-SB | SCSI cable/bulkhead assembly kit with two ports for KZPAC-CA/CB, allows connection of 2 third-port outputs using one PCI bulkhead slot |  |  |  |

1.		KZPAC-SB two port SCSI cable/bulkhead assembly kit is required for third-port connection.  Windows NT systems have three available PCI slots after installation of mandatory graphics option.  Three remaining slots support two KZPAC three-port controllers with KZPAC-SB two port SCSI cable/bulkhead assembly kit.  Graphics option is not mandatory for DIGITAL UNIX and OpenVMS systems, these systems have four available PCI slots, and support three KZPAC three-port controllers.

Step 4d¾Storage Controllers (continued)

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPSA-BB | PCI one-port FWD SCSI controller; requires DWZZB-VW signal converter if connecting directly to StorageWorks shelf. | 31 | 41 | 41 |
| BN21K-02 BN21K-03 BN21K-05 BN21K-10 BN21K-15 BN21K-20 | 02 m 16-bit SCSI cable (internal) 03 m 16-bit SCSI cable (external) 05 m 16-bit SCSI cable (external) 10 m 16-bit SCSI cable (external) 15 m 16-bit SCSI cable (external) 20 m 16-bit SCSI cable (external) |  |  |  |

1.	Windows NT systems have three available PCI slots after installation of mandatory graphics option.  Graphics option is not mandatory for DIGITAL UNIX and OpenVMS systems, these systems have four available PCI options

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description |  |  | OpenVMS V7.1-1H1 |
| CIPCA-BA | PCI-CI adapter. Requires two PCI slots. Requires one of the following CI cables per adapter. |  |  | 2 |
| BNCIA-10 BNCIA-20 BNCIA-45 | 10 m CI cable 20 m CI cable 45 m CI cable |  |  |  |

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description |  |  | OpenVMS V7.1-1H1 |
| KFPSA-AA | PCI-DSSI controller. Connecting to StorageWorks shelf directly, requires HSDxx controller. |  |  | 4 |
| BC29S-06 BC29S-09 | Connection to HSD10 installed in BA36R Pedestal shelf Connection to HSD10 installed in BA36R Cabinet shelf |  |  |  |

Step 4d¾External Storage Controllers

SCSI controllers

- Controllers require KZPSA-BB Fast Wide Differential SCSI controller.

- HSZ50-Ax requires one QB-5CJAA-SA kit
HSZ52-Ax requires two QB-5CJAA-SA kits
HSZ52-AJ requires four QB-5CJAA-SA kits

| HSZ50-AF | StorageWorks RAID Array 450/HSZ50 32 MB SCSI controller, 6 SCSI channels, 36 SCSI-2 device connections, 32 LUN maximum, 32 MB cache module, single external cache battery system building block |
| --- | --- |
| HSZ50-AH | StorageWorks RAID Array 450/HSZ50 64 MB SCSI controller, 6 SCSI channels, 36 SCSI-2 device connections, 32 LUN maximum, 64 MB cache module, single external cache battery system building block |
| HSZ50-AJ | StorageWorks RAID Array 450/HSZ50 128 MB SCSI controller, 6 SCSI channels, 36 dual, 42 single SCSI-2 device connections, 32 LUN maximum, 128 MB cache module, single external cache battery system building block |

| HSZ52-AF | StorageWorks RAID Array 450/HSZ50 64 MB dual-SCSI controllers, 12 SCSI channels, 36 SCSI-2 device connections, 32 LUN maximum, two cache modules, one dual external cache battery system building block, two external cache batteries, two 2 m cables |
| --- | --- |
| HSZ52-AH | StorageWorks RAID Array 450/HSZ50 128 MB dual-SCSI controllers, 12 SCSI channels, 36 SCSI-2 device connections, 32 LUN maximum, two cache modules, one dual external cache battery system building block, two external cache batteries, two 2 m cables |

Step 4d¾External Storage Controllers (continued)

SCSI controllers

| HSZ52-AJ | StorageWorks RAID Array 450/HSZ50 256 MB dual-SCSI controllers, 12 SCSI channels, 36 SCSI-2 device connections, 32 LUN maximum, two cache modules, one dual external cache battery system building block, two external cache batteries, two 2 m cables |
| --- | --- |
| HSZ54-AJ | StorageWorks RAID Array 450/HSZ50 512 MB quad-SCSI controllers, 12 SCSI channels, 72 SCSI-2 device connections, 64 LUN maximum, four cache modules, two dual external cache battery system building blocks, four external cache batteries, four 2 m cables |

DSSI controllers

- Supported on OpenVMS systems only.

- Qualified for use only with KFPSA-AA DSSI adapter.

- QB-5C5AA-SA software kit required per external cache (i.e., 1 per HSD50, 2 per HSD52)

| HSD50-AF | StorageWorks DSSI Array controller, supports 7 SCSI-2 disks, tapes, optical devices |
| --- | --- |
| HSD50-AH | StorageWorks DSSI Array controller with 16 MB cache, supports 7 SCSI-2 disks, tapes, optical disks |
| HSD50-AJ | StorageWorks DSSI Array controller with 32 MB cache, supports 7 SCSI-2 disks, tapes, optical disks |
| HSD52-AF | StorageWorks DSSI Array controller; supports 18 SCSI-2 disks, tapes, optical devices, RAID 0, and includes base firmware and license |
| HSD52-AH | StorageWorks DSSI Array controller with 16 MB cache; supports 18 SCSI-2 disks, tapes, optical disks, RAID 0; includes base firmware and license |
| HSD52-AJ | StorageWorks DSSI Array controller with 32 MB cache; supports 18 SCSI-2 disks, tape, optical disks, RAID 0; includes base firmware and license |

CI Controllers

- Supported on OpenVMS systems only.

- Requires CIPCA-BA Storage controller and cables (Step 5d).

- QB-5C4AA-SA software kit required per external cache (i.e., 1 per HSJ50, 2 per HSJ52)

| HSJ50-AF | CI 6 channel Array controller 32 MB cache w/cache battery |
| --- | --- |
| HSJ50-AH | CI 6 channel Array controller 64 MB cache w/cache battery |
| HSJ50-AJ | CI 6 channel Array controller 128MB cache w/cache battery |
| HSJ52-AF | Dual CI 6 channel Array controller 64 MB cache w/cache battery |
| HSJ52-AH | Dual CI 6 channel Array controller 128 MB cache w/cache battery |
| HSJ52-AJ | Dual CI 6 channel Array controller 256MB cache w/cache battery |
| HSJ54-AJ | Quad CI 6 channel Array controller 512MB cache w/cache battery |
| SC008-AC/AD | Star Coupler Passive Hub device, supports eight CI ports |

Step 5—Graphics Adapters

- Selection of graphics option is mandatory for Windows NT Pedestal Packaged Systems.

- Raid Configuration Utility is accessible via the console port for DIGITAL UNIX and OpenVMS systems. Optional graphics capability is available.

- Select graphics adapter, monitor, and country specific keyboard for DIGITAL UNIX and OpenVMS systems if required.

- Graphics adapter must be installed in PCI slot 0, maximum one per system.

| PB2GA-JC | PCI based 1 MB DRAM graphics adapter 1024 x 768 |
| --- | --- |
| PB2GA-JD | PCI based 2 MB DRAM graphics adapter 1024 x 768 |

Step 6¾Networks and Communications

Systems include DE500-AA PCI 10/100-Mbit Ethernet controller—uses one PCI slot.

| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| --- | --- | --- | --- | --- |
| DE500-AA | PCI 10/100-Mbit Fast Ethernet controller (Twisted Pair) | 4 | 4 | 4 |
| BN25G-03 BN25G-04 BN25G-07 | 3 m (9.8 ft.) cable for 10BaseT Twisted Pair connection 4 m (13.1 ft.) cable for 10BaseT Twisted Pair connection 7 m (22.3 ft.) cable for 10BaseT Twisted Pair connection |  |  |  |

| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| --- | --- | --- | --- | --- |
| DE450-CA | PCI 10-Mbit Ethernet controller; AUI, 10BaseT, or 10Base2 | 31 | 41 | 41 |
| BNE4C-02 BNE4C-05 | 2 m cable for AUI connection, Ethernet/IEEE 802.3 devices 5 m cable for AUI connection |  |  |  |
| BN25G-03 BN25G-04 BN25G-07 | 3 m (9.8 ft.) cable for 10BaseT Twisted Pair connection 4 m (13.1 ft.) cable for 10BaseT Twisted Pair connection 7 m (22.3 ft.) cable for 10BaseT Twisted Pair connection |  |  |  |
| BC16M-06 BC16M-15 BC16M-30 | –6-ft cable for 10Base2 ThinWire connection 15-ft cable for 10Base2 ThinWire connection 30-ft cable for 10Base2 ThinWire connection |  |  |  |

| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| --- | --- | --- | --- | --- |
| DEFPA-AB | PCI to FDDI Adapter, single attachment station (SAS), multi-mode fiber (MMF), SC connector | 31 | 41 | 41 |
| DEFPA-DB | PCI to FDDI Adapter, DAS, MMF, SC connector | 31 | 41 | 41 |
| BN34D-03 BN34D-10 BN34B-03 BN34B-10 BN34B-20 BN34B-30 | –3 m (9.8 ft.) MIC-SC dual fiber optic cable 10 m MIC-SC dual fiber optic cable –3 m (9.8 ft.) SC to SC dual fiber optic cable 10 m SC to SC dual fiber optic cable 20 m SC to SC dual fiber optic cable 30 m SC to SC dual fiber optic cable |  |  |  |

| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| --- | --- | --- | --- | --- |
| DEFPA-UB | PCI to FDDI Adapter, SAS, TP-PMD, requires BN25H-03 | 31 | 41 | 41 |
| DEFPA-MB | PCI to FDDI Dual Adapter, DAS, TP-PMD, requires BN25H-03 | 31 | 41 | 41 |
| BN25H-03 | 3 m (9.8 ft.) FDDI crossover cable |  |  |  |

1.	Windows NT systems have three available PCI slots after installation of mandatory graphics option.  Graphics option is not mandatory for DIGITAL UNIX and OpenVMS systems, these systems have four available PCI options

| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B |  |
| --- | --- | --- | --- | --- |
| CXI01-AA | ISA-based asynchronous multiport MUX 16 port XEM | 1 | 1 |  |
| CXI01-AD | ISA-based asynchronous multiport MUX 16 port EPC | 1 | 1 |  |
| CXI01-AB CXI01-AC CXI01-AE CXI01-AF | ISA-based asynchronous multiport MUX expander XEM ISA-based asynchronous MUX cable converter (RJ45-DB25) ISA-based asynchronous multiport MUX expander EPC ISA-based asynchronous MUX adapter (RJ45-MJ11), 8 pack |  |  |  |

Step 7—Printers and Remote Console

Systems include two EIA-232 asynchronous serial ports with 9-pin D-subminiature connectors.

Select serial printers as required. A 9-pin to MMJ adapter (H8571-J) is required for each connection. A cable must be ordered unless otherwise provided.

Step 8¾MEMORY CHANNEL Controller

DIGITAL UNIX Systems

- Require DIGITAL UNIX V4.0B and System Console Firmware Revision 2.0.

- Each system node in a MEMORY CHANNEL cluster requires a software license.

- Servers in a compute-server array require a DIGITAL UNIX Driver for MEMORY CHANNEL License.

- Servers in a TruCluster high-availability environment require a license for TruCluster for DIGITAL UNIX.

OpenVMS Systems

- Require OpenVMS V7.1-1H1 and OpenVMS Cluster license

Configuring information

- Two PCI to MEMORY CHANNEL Controllers (CCMAA-BA) supported for redundancy/failover.

- For two-system nodes, order one CCMAA-BA per system and one BC12N-10 cable to directly connect adapters.

- For three or four system nodes order one CCMHA-AA (MEMORY CHANNEL Hub), and one CCMAA-BA and one BC12N-10 cable per system node.

- CCMHA-AA (MEMORY CHANNEL Hub) is configured with four CCMLA-AA Line Cards and supports up to four nodes. Expansion up to eight system nodes can be achieved by adding up to four additional CCMLA-AA Line Cards.

- Maximum two CCMAA-BA supported per system.

| - Nodes | - MEMORY CHANNELs | - MEMORY CHANNEL Hubs | - Line Cards |
| --- | --- | --- | --- |
| 2 | —2 CCMAA-BA |  |  |
| 3-4 | 3-4 CCMAA-BA | 1 CCMHA-AA |  |
| 5-8 | 5-8 CCMAA-BA | 1 CCMHA-AA | 1-4 CCMLA-AA |

| CCMAA-BA | PCI to MEMORY CHANNEL Controller ¾Single port MEMORY CHANNEL option |
| --- | --- |
| CCMHA-AA | 100 MB MEMORY CHANNEL Hub with 4 Line Cards |
| CCMLA-AA | MEMORY CHANNEL Line Card for use with MEMORY CHANNEL Hub (CCMHA-AA) |
| CCMRA-AA | Rackmount kit for mounting MEMORY CHANNEL hub |
| BC12N-10 | MEMORY CHANNEL cable |

| QB-3RLAE-AA | TruCluster Software for DIGITAL UNIX |
| --- | --- |
| QB-4ZCAE-AA | DIGITAL UNIX Driver for MEMORY CHANNEL license |
| QL-MUZAE-AA | OpenVMS Cluster license for Alpha systems |

Note:	For stand alone configurations, CCMHA-AA MEMORY CHANNEL Hub, includes BN19P-1K power cord for Canada, Japan, US operation. For other regions, order power cords from Step 12.

Step 9—Rackmount Kit

Pedestal systems can be installed in standard 19”cabinets with a Rackmount Kit.

- Rackmount Kit includes mounting shelf, slide rails, front dress bezel, interlock mechanism, and cable management bracket.   System slides out front of cabinet for full service access.

- H9A10 cabinet with two power controllers supports a maximum of two AlphaServer 1200 systems and two front-mounted StorageWorks shelves, or one AlphaServer 1200 system and five StorageWorks shelves.

- H9A15 cabinet with two power controllers supports a maximum of three AlphaServer 1200 systems and one front-mounted StorageWorks shelf, or one AlphaServer 1200 system and six StorageWorks shelves.

- Order pedestal system, desired options, and rackmount kit (2T-R1200-AB).

- For first system, order one of the following cabinets.

- H9A10-RL

- H9A10-RT

- H9A15-RA

- H9A15-RG

- Order two US or two Non-US power cords

- BN20Z-4E	4.5 m power cord US, or BN35S-4E 	4.5 m power cord non-US

Step 10—Monitors

- Windows NT systems require a graphics monitor to run all functions.

- EISA Configuration Utility (ECU) is accessible via the console port for DIGITAL UNIX and OpenVMS systems. Optional graphics capability is available. Select graphics adapter, monitor, and country specific keyboard for DIGITAL UNIX and OpenVMS systems if required.

- Graphics monitors other than those listed below can be used if compatible with graphics adapter included with system.

Note: Higher resolution graphics are available with optional EISA or PCI graphics adapters (see Step 5).

| SN-VRCX5-WA/W3/W4 | 15" (13.9" viewable image size) Corporate Series auto-scan color monitor, flat square CRT, 0.28 mm dot pitch, VGA to 1024 x 768 @ 85 Hz, TCO 92, MPRII, Energy Star, attached video cable. -WA = Northern Hemisphere with 120 V power cord, -W3 = Northern Hemisphere without power cord. Select country specific power cords for -W3 Northern Hemisphere, and -W4 Southern Hemisphere variants. |
| --- | --- |
| SN-VRTX7-WA/W3 SN-VRT17-W4 | 17" (16.0" viewable image size) Professional Series auto-scan color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 @ 75Hz, TCO 92, MPRII, Energy Star, HD15 male to male video cable. -WA = Northern Hemisphere with 120 V power cord, -W3 = Northern Hemisphere without power cord, SN-VRT17-W4 = Southern Hemisphere. Select country specific power cord for W3 and W4 variant. |
| SN-VRCX1-WA/ W3/W4 | 21" (19.7" viewable image size) professional series auto-scan color monitor, Diamondtron CRT, 0.28 mm aperture grill pitch, VGA to 1600 x 1200 @ 75 Hz, TCO 92, MPRII, Energy Star, HD15 male to male video cable. -WA = Northern Hemisphere with 120 V power cord. -W3 = Northern Hemisphere without power cord. W4 = Southern Hemisphere without power cord. Select country specific power cord for W3 and W4 variants. |

Step 11—Software

- North American variants of Windows NT Packaged systems include Windows NT license, media (CD-ROM) kits

- Selection of language specific Windows NT media kit is mandatory for non North American variants.

- Order documentation kit if required.

Windows NT Server plus 10-client access license, media (CD-ROM) kits

| QB-23CAA-SB | Windows NT Server license, media kit North American English |
| --- | --- |
| QB-23C8A-SB | Windows NT Server license, media kit International English |
| QB-23CPA-SB | Windows NT Server license, media kit French |
| QB-23CGA-SB | Windows NT Server license, media kit German |
| QB-23CSA-SB | Windows NT Server license, media kit Spanish |
| QB-23CUA-SB | Windows NT Server license, media kit Italian |
| QB-23CJA-SB | Windows NT Server license, media kit Japanese |
| QB-23C5A-SB | Windows NT Server license, media kit Thai |
| QB-23CMA-SB | Windows NT Server license, media kit Swedish |
| QB-23CTA-SB | Windows NT Server license, media kit Hebrew |
| QB-23CQA-SB | Windows NT Server license, media kit Arabic |
| QB-23CHA-SB | Windows NT Server license, media kit Dutch |
| QB-23CVA-SB | Windows NT Server license, media kit Brazilian/Portuguese |
| QB-23C4A-SB | Windows NT Server license, media kit Korean |
| QB-23C3A-SB | Windows NT Server license, media kit Taiwanese |
| QB-23C2A-SB | Windows NT Server license, media kit PRC Chinese |

Windows NT Server Optional software and documentation

| QA-23CAA-GZ | Windows NT Server documentation kit |
| --- | --- |
| QA-23C8A-GZ | Windows NT International English Server documentation kit |

Step 11—Software (continued)

DIGITAL UNIX

Software Processor Code = E

- DIGITAL UNIX Packaged systems include Traditional Unlimited User license.

- DIGITAL UNIX Packaged systems require operating system media and documentation for first system on site.

DIGITAL UNIX Media and Documentation—required for first system on site

| QA-MT4AA-H8 | DIGITAL UNIX media and on-line documentation on CD-ROM |
| --- | --- |
| QA-MT4AA-GZ | DIGITAL UNIX full hard copy documentation |

DIGITAL UNIX Layered Products CD-ROM

| QA-054AA-H8 | Layered products media and documentation for DIGITAL UNIX on CD-ROM |
| --- | --- |

DECnet for DIGITAL UNIX

| QL-MTJAE-AA | DECnet/OSI end-system license for DIGITAL UNIX |
| --- | --- |
| QL-MTKAE-AA | DECnet/OSI extended function license for DIGITAL UNIX |

DECsafe ASE license

| QB-05SAE-AA | DECSafe Available Server license and documentation for DIGITAL UNIX Alpha systems—media available on layered product CD-ROM |
| --- | --- |
| QL-05SAE-AA | DECSafe Available Server license for DIGITAL UNIX Alpha systems |

OpenVMS

Software Processor Code = E

OpenVMS Concurrent Use Licenses are not specific to a single system and can be moved between systems at user discretion. OpenVMS Concurrent Use Licenses can also be shared in a mixed OpenVMS VAX and OpenVMS Alpha Cluster.

OpenVMS Concurrent Use Licenses

| QL-MT3AA-3B | OpenVMS Concurrent Use 1-user license |
| --- | --- |
| QL-MT3AA-3C | OpenVMS Concurrent Use 2-user license |
| QL-MT3AA-3D | OpenVMS Concurrent Use 4-user license |
| QL-MT3AA-3E | OpenVMS Concurrent Use 8-user license |
| QL-MT3AA-3F | OpenVMS Concurrent Use 16-user license |
| QL-MT3AA-3G | OpenVMS Concurrent Use 32-user license |
| QL-MT3AA-3H | OpenVMS Concurrent Use 64-user license |
| QL-MT3AA-3J | OpenVMS Concurrent Use 128-user license |
| QL-MT3AA-3K | OpenVMS Concurrent Use 256-user license |
| QL-MT2AE-AA | OpenVMS Traditional unlimited user license |

OpenVMS Media and Documentation

| QA-MT1AA-H8 | OpenVMS media and on-line documentation CD-ROM |
| --- | --- |
| QA-001AA-GZ | OpenVMS hard copy documentation |

OpenVMS Layered Products CD-ROM

| QA-03XAA-H8* | Layered products media and documentation for OpenVMS on CD-ROM |
| --- | --- |

*	Includes DIGITAL Enterprise Integration Server for OpenVMS media and documentation

Step 11—Software (continued)

DIGITAL Enterprise Integration Package

| QA-5LVAA-H8 | DIGITAL Enterprise Integration Server for OpenVMS media and documentation |
| --- | --- |

DECnet for OpenVMS

| QL-MTGAE-AA | DECnet extended function license for OpenVMS |
| --- | --- |
| QL-MTHAE-AA | DECnet end-system to extended function upgrade license for OpenVMS |

DSSI Information

| EK-410AB-MG | DSSI VMScluster Installation Guide |
| --- | --- |
| EK-D4AXP-TS | DSSI VMScluster Troubleshooting Guide |

OpenVMS Cluster

| QL-MUZAE-AA | OpenVMS Cluster license for Alpha systems |
| --- | --- |

OpenVMS Volume Shadowing

| QL-2A1AE-AA | OpenVMS Volume Shadowing license |
| --- | --- |

Step 12—Keyboards and Power Cords

Systems ordered in the Americas and AP include U.S. English keyboard.  Order country specific keyboard for all systems ordered in Europe.

| Windows NT/DIGITAL UNIX keyboard | OpenVMS keyboard | Frost White |
| --- | --- | --- |
| LK47W-A2 |  | U.S. |
|  | LK46W-A2 | U.S./United Kingdom |
| LK47W-AB | LK46W-AB | Belgian |
|  | LK46W-AC | Canadian/French |
| LK47W-AD | LK46W-AD | Danish |
| LK47W-AE |  | United Kingdom |
|  | LK46W-AF | Finnish |
| LK47W-AG | LK46W-AG | German |
|  | LK46W-AH | Dutch |
| LK47W-AI | LK46W-AI | Italian |
| LK47W-AK | LK46W-AK | Swiss/Generic |
|  | LK46W-AL | Swiss/German |
|  | LK46W-AM | Swedish |
| LK47W-AN | LK46W-AN | Norwegian |
| LK47W-AP | LK46W-AP | French |
|  | LK46W-AQ | Canadian/English |
| LK47W-AS | LK46W-AS | Spanish |
| LK47W-AV | LK46W-AV | Portuguese |

Pedestal and Monitor Power Cords

Systems ordered in the Americas and AP include two BN26J-1K 120 V (North American, Japan) power cords.  Order two country specific power cords for all systems ordered in Europe.

| BN26J-1K | North American, Japan, 120 V, 75-inches long |
| --- | --- |
| BN19H-2E | Australia, New Zealand, 2.5m long |
| BN19C-2E | Central Europe, 2.5 m long |
| BN19A-2E | UK, Ireland, 2.5 m long |
| BN19E-2E | Switzerland, 2.5 m long |

Step 12—Keyboards and Power Cords (continued)

Pedestal and Monitor Power Cords

| BN19K-2E | Denmark, 2.5 m long |
| --- | --- |
| BN19Z-2E | Italy, 2.5 m long |
| BN19S-2E | Egypt, India, South Africa, 2.5 m long |
| BN18L-2E | Israel, 2.5 m long |

Step 13—Hardware and Software Supplemental Support Services

Hardware—Americas and Asia Pacific only

- Systems include three-year hardware warranty, on-site with 5 x 9, 24-hour response time.

- Option order as part of the Initial System Sale take on the Warranty of the enclosure

- Option added after the Initial System Sale take on the Warranty of the enclosure until

- the enclosure warranty ends.

- All option unique AlphaServer 1200 option have a minimum of 1 Year Return to DIGITAL Warranty.

- Select optional Hardware Supplemental Support Services if required.

Supplemental Service for System with 1 GB of memory or less.

| FM-TC4HR-36 | 5 x 9, 4-hour response time |
| --- | --- |
| FM-TC512-36 | 5 x 12, 4-hour response time |
| FM-TC616-36 | 6 x 16, 4-hour response time |
| FM-TC724-36 | 7 x 24, 4-hour response time |
| FM-TCXHW-60 | Years 1-5, next day, Onsite |
| FM-TC4HR-60 | Years 1-5, 5x9, 4-hour response time |
| FM-TC512-60 | Years 1-5, 5x12, 4-hour response time |
| FM-TC616-60 | Years 1-5, 6x16, 4-hour response time |
| FM-TC724-60 | Years 1-5, 7x24, 4-hour response time |

Supplemental Service for System with more then 1GB of memory.

| FM-TF4HR-36 | 5 x 9, 4-hour response time |
| --- | --- |
| FM-TF512-36 | 5 x 12, 4-hour response time |
| FM-TF616-36 | 6 x 16, 4-hour response time |
| FM-TF724-36 | 7 x 24, 4-hour response time |
| FM-TFXHW-60 | Years 1-5, next day, Onsite |
| FM-TF4HR-60 | Years 1-5, 5x9, 4-hour response time |
| FM-TF512-60 | Years 1-5, 5x12, 4-hour response time |
| FM-TF616-60 | Years 1-5, 6x16, 4-hour response time |
| FM-TF724-60 | Years 1-5, 7x24, 4-hour response time |

Installation Services

| FM-TC12X-IN | Hardware product installation for AlphaServer 1200 |
| --- | --- |
| FM-TCMEM-IN | Add-on memory installation for AlphaServer 1200 |
| FM-TCSMP-IN | Add-on CPU installation for AlphaServer 1200 |

Step 13—Hardware and Software Supplemental Support Services (continued)

Software—Americas and Asia Pacific only

- DIGITAL UNIX and OpenVMS Systems include 90-day Conformance to SPD and Telephone Advisory Support. Select optional Software Supplemental Support Services, if required.

- Software service upgrades for Windows NT include advisory and remedial software support for the time period indicated.

- Software service upgrades for DIGITAL UNIX include advisory and remedial software support with new version license rights for operating system for time period indicated.

- Software service upgrades for OpenVMS include advisory and remedial software support with new version license rights for operating system and Enterprise Integration Package for time period indicated.

- Software service upgrades for SMP Upgrades running DIGITAL UNIX or OpenVMS include new version license rights for DIGITAL UNIX or OpenVMS SMP Upgrades for the time period indicated. (One required for each SMP Upgrade).

| FM-WNT03-12 | 12-month Software Supplemental Support for Windows NT AlphaServer 1200 systems |
| --- | --- |
| FM-WNT03-36 | 36-month Software Supplemental Support for Windows NT AlphaServer 1200 systems |
| FM-WNT03-60 | 60-month Software Supplemental Support for Windows NT AlphaServer 1200 systems |
| FM-10UNS-12 | 12-month Software Supplemental Support for DIGITAL UNIX AlphaServer 1200 systems |
| FM-10UNS-36 | 36-month Software Supplemental Support for DIGITAL UNIX AlphaServer 1200 systems |
| FM-10UNS-60 | 60-month Software Supplemental Support for DIGITAL UNIX AlphaServer 1200 systems |
| FM-10VMS-12 | 12-month Software Supplemental Support for OpenVMS AlphaServer 1200 systems |
| FM-10VMS-36 | 36-month Software Supplemental Support for OpenVMS AlphaServer 1200 systems |
| FM-10VMS-60 | 60-month Software Supplemental Support for OpenVMS AlphaServer 1200 systems |

Step 13—Hardware and Software Supplemental Support Services (continued)

Software—Americas and Asia Pacific only

| FM-12SMP-12 | 12-month Software Supplemental Support for OpenVMS or DIGITAL UNIX AlphaServer 1200 SMP Upgrade |
| --- | --- |
| FM-12SMP-36 | 36-month Software Supplemental Support for OpenVMS or DIGITAL UNIX AlphaServer 1200 SMP Upgrade |
| FM-12SMP-60 | 60-month Software Supplemental Support for OpenVMS or DIGITAL UNIX AlphaServer 1200 SMP Upgrade |

Step 13a—Hardware and Software Supplemental Support Services (Europe only)

Europe does not have specific part numbers for Hardware and Software Supplemental Support Services. Prices are quoted based on world wide service reference pricing, contact MCS Sales in your country for information on Hardware and Software Supplemental Support Services.

AlphaServer 1200 System Diagram Front View

1.	StorageWorks Shelf for hard disk drives

2.	CD-ROM drive

3.	Optional Removable media drive

4.	Floppy diskette drive

AlphaServer 1200 System Diagram Side and Back View

–1.	CPU modules

–2.	Memory riser modules

–3.	Power inlet

–4.	SCSI ports

–5.	PCI/EISA bulkhead ports

–6.	Power supply

–7.	Parallel port

–8.	RCM modem port

–9.	COM2 port

10.	COM1 port

AlphaServer 1200 Pedestal System Specifications

| Shipping Dimension |  |  |  |  |
| --- | --- | --- | --- | --- |
| Height Width Depth Weight |  | 60 cm (23.8 in.) 43 cm (16.9 in) 65 cm (25.6 in) 43 kg (95 lb) typical 71 kg (156 lb) maximum |  |  |
| Installed Dimensions |  |  |  |  |
| Height Width Depth Weight |  | 46 cm (18.1 in.) 35.8 cm (14.1 in.) 58.6 cm (23.1 in.) 39 kg (86 lb) typical 51 kg (113 lb) maximum |  |  |
| Clearances | Operating |  |  | Service |
| Front Rear Sides | 75 cm (29.5 in.) 15 cm (6 in.) None |  |  | 75 cm (29.5 in.) 75 cm (29.5 in.) 75 cm (29.5 in.) |
| Environmental |  |  |  |  |
| Temperature | Operating* Rate of change Nonoperating Storage (60 days) Rate of change |  |  | 10-40° C (50-104° F) 11° C/hr (20° F/hr) -40-66° C (-40-151° F) -40-66° C (-40-151° F) 20° C/hr (36F/hr) |
| Relative humidity | Operating Nonoperating Storage (60 days) Rate of change |  |  | 10–90% 10–95% 10–95% 20%/hr |
| Maximum wet bulb temperature | Operating Storage (60 days) |  |  | 28° C (82° F) 46° C (115°F) |
| Minimum dew point temperature | Operating Storage (60 days) |  |  | 2° C (36° F) Not tested |
| Maximum heat dissipation Dual supply | Current 4097 Btu/hr |  |  |  |
| Air flow and quality | Intake location Exhaust location Particle size Concentration |  |  | Front Rear N/A N/A |
| Altitude | Operating† Nonoperating |  |  | 3048 m (10,000 ft) 12192 m (40,000 ft) |
| Mechanical shock | Operating Nonoperating |  |  | 10 G 10 ms 20 G peak 30 ms |
| Vibration | Operating Nonoperating |  |  | 10-500 Hz .1 G peak 1.03Grms 5-300Hz |
| Acoustics | Operating Idle |  |  | Average Declared 5.7 LwA, B 38 LpAm, dba 5.6 LwA, B 37 LpAm, dba |
| Electrical—Power Supplies are universal 120/240 Vac |  |  |  |  |
| Nominal ac voltage Operating Voltage range Power source phase Nominal frequency Frequency range Maximum inrush current RMS current at nominal – –voltage (steady state) –Dual power supply | 100-120 Vac 90-128 Vac Single 60 Hz 59-61 Hz 50 Amps 4.6 Amps each supply |  | 220-240 Vac 180-256 Vac Single 50 Hz 49-51 Hz 50 Amps 2.2 Amps each supply |  |
| Power cord | Type Length U.S. plug |  | IEC 320 C13 190 cm (75 in.) NEMA 5-15 |  |

*	Maximum operating temperature at Sea Level. Reduce by 1 C (1.8 F) for each 600 m (2000 ft) above Sea Level.

†	Higher altitudes are possible if maximum operating temperature is reduced (see Temperature, above); other restrictions may apply, such as maximum permissible altitude for hard drives.

AlphaServer 1200A Pedestal System Specifications (continued)

| Regulatory |  |
| --- | --- |
| Agency approvals | UL: Listed to UL1950 (2nd edition) CSA: Certified to CAN/CSA-C22.2 No. 950-M89 TUV: EN 60950 VDE 0805 GS marked ZH1/618 FCC: 15J Part 15 Class A CE: EN55022, EN50082 VCCI Class II ITE BCIQ: CISPR22, CNS13438 C-Tick: CISPR22, AS/NZS 3548 |
| Reviewed to | AS/NZ 3260:1993 Australian/ New Zealand Standard EN 60950/A2: 1993 European Norm IEC 950 (2nd edition) |