### DIGITAL Server 3100R and 3200R Rackmount Systems
V2.0—10 February 1988
DIGITAL Systems and Options Catalog

Product Description 	DIGITAL Server 3100R and 3200R are rackmount departmental servers supporting one or two Intel processors.  The 3100R features the Pentium Pro 
200 MHz processor, while the 3200R features Pentium II 266 MHz and 300 MHz processors.  The servers are packaged in a 19”-wide rackmount drawer form factor for mounting in a space-efficient DIGITAL Server Rackmount enclosure, or in any industry standard 19” rack.  Drawer height is 5U (8.75-inches)

DIGITAL Server 3100R and 3200R rackmount system supports up to eight storage devices including floppy diskette drive, CD-ROM, two additional 5.25-inch or 3.5-inch removable media devices, and four hot-swap drive bays.

DIGITAL Server 3100R and 3200R are targeted at the departmental server market requiring high availability solutions for file/print, BackOffice Enterprise and third party applications

ServerWORKS, Digital’s server and network management software, is included with the DIGITAL Server 3100R and 3200R, lowering total cost of ownership and making it easier to install, deploy and manage DIGITAL Server networks. ServerWORKS helps system administrators save valuable time through remote alarming, management and diagnosis.  Administrators can take preventive measures before a problem occurs, or conduct a remote diagnosis of the problem and dispatch assistance, equipped with the right components or upgrades on the first trip.

DIGITAL believes the information in this publication is accurate as of its publication date; such information is subject to change without notice. DIGITAL is not responsible for any inadvertent errors.

DIGITAL conducts its business in a manner that conserves the environment and protects the safety and health of its employees, customers, and the community.

DIGITAL, the DIGITAL logo are trademarks of Digital Equipment Corporation.

Printed in USA. Copyright 1998 Digital Equipment Corporation. All rights reserved.

Step 1¾DIGITAL Server 3100R and 3200R Rackmount System

DIGITAL Server 3100R and 3200R are available in two configurations

- Base System

- Includes one memory kit (64 MB)

- Disk drives and Windows NT operating system are not included

- Windows NT license and media kit is not available from DIGITAL for DIGITAL Server 3100R and 3200R systems, order directly from Microsoft, or from an authorized Microsoft reseller.

- Kernals—not available in North America

- Memory, disk drives, and Windows NT operating system are not included

- Order memory and disk drives separately

- Windows NT license and media kit is not available from DIGITAL for DIGITAL Server 3100R and 3200R systems, order directly from Microsoft, or from an authorized Microsoft reseller.

- Options ordered will be factory installed unless specified as spares.

DIGITAL Server 3100R and 3200R Rackmount Systems include

- DIGITAL Server 3100R with
Intel Pentium Pro 200 MHz CPU with 512 KB cache, or

- DIGITAL Server 3200R with 
Intel Pentium II 266 MHz or 300 MHz CPU, each with 512 KB cache

- Upgradeable to two SMP processors

- Rackmount drawer enclosure with:

- Seven expansion slots
Two PCI slots, four EISA slots, one PCI/EISA

- Eight storage bays
One diskette drive (included)
One CD-ROM drive (included)
Two additional 5.25-inch or 3.5-inch removable media bays
Four hot-swap SCA-2 disk drive bays

- Eight SIMM sockets (supports four SIMM memory kits)

- UltraSCSI adapter integrated on motherboard

- S3 Trio64 SVGA with 1 MB DRAM integrated on motherboard

- 10/100 Mbit Fast Ethernet integrated on motherboard

- 300-Watt switch-selectable (110/220V, 60/50Hz) power supply

- Two serial ports

- One bi-directional enhanced parallel port

- PS/2 style keyboard port and mouse port

- One video port

- AC Power input

- Two SCSI port knockouts on rear panel

- 1.44 MB diskette drive in dedicated slot

- 600 MB 12X CD-ROM

- Systems include slide rails and mounting hardware for mounting into 19” rack

- Cable management arm

- Integrated Advanced Server Management features, including ServerWorks Manager kit

- Hardware Warranty: Three-year on-site

DIGITAL Server 3105R Base System

| Order Number | O/S Media | CPU Speed | Country kit | Memory | Disk | UltraSCSI Adapter | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR-R2C5W-AA | Optional | 200 MHz | Optional | 64 MB | Optional | Included | Included | Optional |

DIGITAL Server 3200R Base Systems

| Order Number | O/S Media | CPU Speed | Country kit | Memory | Disk | UltraSCSI Adapter | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR-R2B5W-AA | Optional | 266 MHz | Optional | 64 MB | Optional | Included | Included | Optional |
| FR-R2B6W-AA | Optional | 300 MHz | Optional | 64 MB | Optional | Included | Included | Optional |

Step 1¾DIGITAL Server 3100R and 3200R Rackmount System (continued)

DIGITAL Server 3105R Kernel

| Order Number | O/S Media | CPU Speed | Country kit | Memory | Disk | UltraSCSI Adapter | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR-R2C5W-AX | Optional | 200 MHz | Optional | Optional | Optional | Included | Included | Optional |

DIGITAL Server 3200R Kernals

| Order Number | O/S Media | CPU Speed | Country kit | Memory | Disk | UltraSCSI Adapter | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR-R2B5W-AX | Optional | 266 MHz | Optional | Optional | Optional | Included | Included | Optional |
| FR-R2B6W-AX | Optional | 300 MHz | Optional | Optional | Optional | Included | Included | Optional |

Step 2¾Additional CPU Modules (SMP Upgrades)

- DIGITAL Server 3100R and 3200R Systems support up to one additional CPU for a maximum of two processors.

- Additional CPU must be of the same type as initial processor, e.g. Pentium Pro 200 MHz CPU 512 KB cache, or Pentium II 266 MHz or 300 MHz CPU, each with 512 KB cache.

| FR-PCB4U-AB | 200MHz Pentium Pro CPU chip upgrade, with 512 KB cache |
| --- | --- |
| FR-PCB4U-AD | 266MHz Pentium II CPU chip upgrade, with 512 KB cache |
| FR-PCB4U-AF | 300MHz Pentium II CPU chip upgrade, with 512 KB cache |

Step 3¾Memory

- DIGITAL Server 3100R and 320R Kernals do not included memory, order separately (optional).

Server 3100R Memory Expansion:

- DIGITAL Server 3100R Base Systems include 64 MB memory (2 x 32 MB SIMMs).

- DIGITAL Server 3100R systems support a total of four SIMM memory kits (eight sockets—four banks of two).

- Different capacity SIMMs are supported.  Memory kits must be installed in banks (2 SIMMs per bank), each bank must contain same capacity, type and speed, e.g. 2 x 16 MB SIMMs.

| FR-PC77M-BK | –32 MB (2 x–16 MB) EDO SIMM memory kit |
| --- | --- |
| FR-PC77M-BL | –64 MB (2 x–32 MB) EDO SIMM memory kit |
| FR-PC77M-BM | 128 MB (2 x–64 MB) FastPage SIMM memory kit |
| FR-PC77M-BN | 256 MB (2 x128 MB) FastPage SIMM memory kit |

Server 3200R Memory Expansion

- DIGITAL Server 3200R Base Systems include 64 MB memory (1 x 64 MB DIMM).

- DIGITAL Server 3200R systems support up to a maximum of four DIMM memory kits.

- Different capacity DIMMs are supported.  Supported DIMMs supplied by DIGITAL are 168-pin, 72-bit EDO/ECC DIMMs operating at 60ns

| FR-PCSMA-AC | –32 MB (1 x –32 MB) EDO DIMM memory kit |
| --- | --- |
| FR-PCSMA-AD | –64 MB (1 x –64 MB) EDO DIMM memory kit |
| FR-PCSMA-AE | 128 MB (1 x 128 MB) EDO DIMM memory kit |

Step 4¾Storage

- Systems include UltraSCSI adapter integrated on motherboard

- Rackmount enclosure supports eight storage devices

- One dedicated diskette drive bay

- One CDROM drive bay

- Two removable 5.25-inch or 3.5-inch drive bay

- Four hot-swap drive bays for StorageWorks SCA-2 drives

Step 4a—Internal Storage

- All systems include

- One SCSI CD-ROM drive

- One 1.44 MB diskette drive

SCA-2 SCSI Disk Drives

| FR-CDCBA-DA | 2.1 GB SCSI SCA-2 7200 RPM hard disk drive |
| --- | --- |
| FR-CECBA-DA | 4.3 GB SCSI SCA-2 7200 RPM hard disk drive |
| FR-CFCBA-DA | 9.0 GB SCSI SCA-2 7200 RPM hard disk drive |
| FR-CGCBA-DA | 18.0 GB SCSI SCA-2 7200 RPM hard disk drive |

Removable Media—Internal Tape Drive and additional CD-ROM

| FR-PCXCR-AR | 600 MB 5.25 inch half-height 12X CD-ROM drive |
| --- | --- |
| FR-PCXAT-AJ | 4/8 GB 5.25 inch half-height SCSI DDS3 DAT tape drive |
| FR-PCXAT-AK | 12/24 GB 5.25 inch half-height SCSI DDS3 DAT tape drive |
| FR-PCXLT-AB | 15/30 GB 5.25 inch half-height DLT tape drive |
| FR-PCXLT-AD | 35/70 GB 5.25 inch half-height DLT tape drive |

Step 4b¾Storage Controllers

- Systems include UltraSCSI adapter integrated on motherboard

- The following adapters and controllers can be configured in DIGITAL Server 3100R and 3200R Systems.

- RAID support for 0, 1, 5, 0+1)

| FR-PCTAZ-DE | One-port PCI Adaptec 2940 UW UltraSCSI controller, maximum three per system |
| --- | --- |
| FR-PCTAR-GA | One-port PCI UltraSCSI RAID controller, maximum three per system |
| FR-PCTAR-GB | Two-port PCI UltraSCSI RAID controller, maximum three per system |
| FR-PCTAR-GC | Three-port PCI UltraSCSI RAID controller, maximum three per system |
| FR-PCTAR-UB | PCI RAID cache battery backup |

SCSI Cables and Terminators

| FR-BN37A-02 | VHDCI male to VHDCI male, 2.0 meter |
| --- | --- |
| FR-BN37A-05 | VHDCI male to VHDCI male, 5.0 meter |
| FR-BN21H-02 | 50-pin to 50-pin external SCSI cable, 2.0 meter |
| FR-BN21K-02 | 68-pin to 68-pin external SCSI cable, 2.0 meter |
| FR-BN38C-02 | 68-pin HD to VHDCI external SCSI cable, 2.0 meter |
| FR-PCXAR-WF | Internal SCSI cable, 580mm, SCSI-3, 68-pin HDIDC to 68-pin HDIDC |
| FR-PCXAR-WG | Internal SCSI cable, 580mm, SCSI-3, 68-pin HDIDC to 68-pin panel mount |
| FR-PCXAR-WJ | SCSI terminator, 68-pin/68-pin single-ended active terminator |
| FR-BN31S-1E | 1.5 meter SCSI cable, connects between PCI RAID controller and BA356 rackmount shelf |

Step 4c—DIGITAL Cluster Kits for Windows NT

Cluster Kits

- Cluster Kits includes all necessary hardware and cluster software required to create a 2-node Windows NT cluster environment.

- Systems, disk drives, and Windows NT server licenses are not included.

| FR-CK356-UR | BA356R Cluster Kit—Rackmount version |
| --- | --- |

Step 4d—External Storage

Rackmount shelves

| FR-BA356-PD | UltraSCSI shelf for rackmounting, supports 7 UltraSCSI SBB disk drives |
| --- | --- |
| FR-BA35X-HK | 180 Watt universal ac input power supply; maximum two per FR-BA365-PD shelf |
| FR-SWXRA-Z1 | RAID Array 310 subsystem |

RAID Storage Building Blocks (SBBs) for BA356 and BA310 rackmount shelves

| FR-RZ1BB-VW | 2.1 GB UltraSCSI SSB, 7200 RPM |
| --- | --- |
| FR-RZ1CB-VW | 4.3 GB UltraSCSI SSB, 7200 RPM |
| FR-RZ1DB-VW | 9.0 GB UltraSCSI SSB, 7200 RPM |

Step 5¾Monitors

- Windows NT systems require a graphics monitor to run all functions.

- Monitor part numbers do not include power cord.  Order country specific power cord.

- Graphics monitors other than those listed above can be used if compatible with SVGA graphics included with system.

| FR-PCXCV-DW | 15” (13.9” viewable) Value monitor, Northern Hemisphere |
| --- | --- |
| FR-PCXCV-DX | 15” (13.9” viewable) Value monitor, Equatorial |
| FR-PCXBV-FZ | 17” (16.0” viewable) Corporate monitor, Northern Hemisphere |
| FR-PCXBV-FX | 17” (16.0” viewable) Corporate monitor, Southern Hemisphere |
| FR-PCXBV-FY | 17” (16.0” viewable) Corporate monitor, Equatorial |
| FR-PCXAV-CZ | 19” (18.0 viewable) Professional monitor, Northern Hemisphere |
| FR-PCXAV-CY | 19” (18.0 viewable) Professional monitor, Southern Hemisphere |
| FR-PCXAV-WZ | 21” (19.6 viewable) Professional monitor, Northern Hemisphere |
| FR-PCXAV-WY | 21” (19.6 viewable) Professional monitor, Southern Hemisphere |

Step 6¾Graphics Adapters

- Systems include S3 Trio64 SVGA with 1 MB DRAM embedded on the motherboard.  Maximum of one graphics subsystem supported per system.

- Select graphics adapter only if a spare PCI graphics card is required.

| FR-PCXAG-BD | PCI-based Matrox Millennium graphics adapter, 4 MB DRAM |
| --- | --- |
| FR-PCCAM-CC | 4 MB DRAM upgrade for Matrox Millennium graphics adapter |

Step 7¾Networks and Communications

- Systems include 10/100 Mbit Fast Ethernet integrated on motherboard

- Select networking cable for Ethernet controller

- BN25G-03/04/07 or BN24Q-xx for 10BaseT (Twisted Pair)

| FR-DE500-BA | PCI-based Fast Ethernet network interface card, maximum four per system |
| --- | --- |
| FR-DE450-CA | PCI-based DIGITAL Etherworks 32-bit network interface card (Twisted Pair, Thickwire, ThinWire), maximum four per system |
| FR-PCXDN-BB | PCI-based SMC 10Mbps Ethernet network interface card |
| FR-PCXDN-BC | PCI-based SMC 9432TX Fast Ethernet network interface card |
| FR-PCXTN-AD | PCI-based Token Ring card |
| FR-DEFPA-AB | PCI-based DEC FDDI controller, single attachment (SAS), maximum four per system |
| FR-DEFPA-DB | PCI-based DEC FDDI controller, dual attachment (DAS), maximum four per system |
| FR-DEFPA-UB | PCI-based DEC FDDI controller (UTP), maximum four per system |
| FR-PCXFA-AA | EISA-based, 56Kbps modem, maximum four per system |

Step 8¾Country Kits and Power Cords

- Country kits include a 105-key, PC-style keyboard and a two-button mouse.

- Selection of country is optional since multiple servers within a rackmount enclosure do not require separate keyboards and mice.

Country Kits

| FR-PCR2K-AA | North America |
| --- | --- |
| FR-PCR2K-AB | Belgium |
| FR-PCR2K-AE | United Kingdom |
| FR-PCR2K-AF | Finland |
| FR-PCR2K-AG | Germany |
| FR-PCR2K-AI | Italy |
| FR-PCR2K-AJ | Japan |
| FR-PCR2K-AK | Switzerland (French) |
| FR-PCR2K-AN | Norway |
| FR-PCR2K-AP | France |
| FR-PCR2K-AS | Spain |

Bundled Rackmount Power Cords—includes eight power cords

| FR-PCSRA-DS | North America, Japan |
| --- | --- |
| FR-PCSRA-DT | Norway |
| FR-PCSRA-DU | United Kingdom/Ireland |
| FR-PCSRA-DV | Central Europe |
| FR-PCSRA-DW | Israel |
| FR-PCSRA-DX | Switzerland |
| FR-PCSRA-DY | Australia |

Individual Rackmount Power Cord—includes one power cord

| FR-PCTXH-JA | North America, Japan |
| --- | --- |
| FR-BN19A-2E | United Kingdom/Ireland |
| FR-BN19C-2E | Central Europe |
| FR-BN19K-2E | Denmark |
| FR-BN24T-2E | Switzerland |

Step 9¾Operating System Support

DIGITAL Server 3100R and 3200R supports the following operating systems

- Microsoft Windows NT Server 4.0

- Microsoft Windows NT Server Enterprise Edition 4.0

- Novell IntranetWare (NetWare 4.11 included)

- SCO UNIXWare 2.1.2

- SCO OpenServer 5.0.4

- Banyan VINES 7.0

- IBM OS/2 Warp 4.0

Step 10¾Remote Server Management System

System Package

System Package includes hardware kit and software kit (see below).

| FR-PCDSM-AA | Remote Server Management Station System Package, includes Server Manager hardware kit, English user documentation, Server Management software kit, Americas |
| --- | --- |
| FR-PCDSM-AC | Remote Server Management Station System Package, includes Server Manager hardware kit, User documentation, Server Management software kit, Americas French/Spanish |
| FR-PCDSM-AE | Remote Server Management Station System Package, includes Server Manager hardware kit, English user documentation, Server Management software kit, United Kingdom/Ireland |
| FR-PCDSM-AG | Remote Server Management Station System Package, includes Server Manager hardware kit, User documentation, Server Management software kit, German/Dutch/Italian |
| FR-PCDSM-AK | Remote Server Management Station System Package, includes Server Manager hardware kit, User documentation, Server Management software kit, Swiss |
| FR-PCDSM-AP | Remote Server Management Station System Package, includes Server Manager hardware kit, User documentation, Server Management software kit, European French/Spanish |
| FR-PCDSM-AX | Remote Server Management Station System Package, includes Server Manager hardware kit, English user documentation, Server Management software kit, no power cord |
| FR-PCDSM-CM | Remote Server Management Station System Package, includes Server Manager hardware kit, User documentation, Server Management software kit, Kanji |

Hardware Kit

Hardware kit includes EISA serial interface board configuration diskette, keyboard jumper cable, data cable, power supply, power cord, and breakout box/cable.

| FR-PCDSC-AA | Remote Server Management Station Hardware Kit, Americas |
| --- | --- |
| FR-PCDSC-AC | Remote Server Management Station Hardware Kit, Americas French/Spanish |
| FR-PCDSC-AE | Remote Server Management Station Hardware Kit, United Kingdom/Ireland |
| FR-PCDSC-AG | Remote Server Management Station Hardware Kit, Central Europe power German/Dutch/Italian |
| FR-PCDSC-AK | Remote Server Management Station Hardware Kit, Swiss power French/German/Dutch/Italian |
| FR-PCDSC-AP | Remote Server Management Station Hardware Kit, Central Europe power, French/Spanish |
| FR-PCDSC-AX | Remote Server Management Station Hardware Kit, no power, English |
| FR-PCDSC-CM | Remote Server Management Station Hardware Kit, Kanji |

Software Kit

| QB-3BEE-SA | Remote Server Management Station Software Kit |
| --- | --- |

Step 11¾Uninterruptable Power Supplies for Rackmount

- DIGITAL Server 3100R and 3200R systems include a 300 Watt power supply.  No additional power supplies are supported.

- A rackmount UPS may be added within the rack.

| 4N-APACH-AF | APC Smart-UPS 2200 VA, 120V, 3U rackmount |
| --- | --- |
| 4N-APACH-AG | APC Smart-UPS 3000 VA, 120V, 3U rackmount |

DIGITAL Server 3100R and 3200R System Diagram Front View

DIGITAL Server 3100R and 3200R System Diagram Side and Back View

- A.	SCA-2 drive bay

B.	Speaker

C.	Cooling fan

D.	Main Logic Board

E.	Pentium Pro or Pentium II processor and memory module with retainer

F.	Ethernet daughter card

G.	Power Supply

H.	SCA-2 backplane

I.	Diskette drive

DIGITAL Server 3100R and 3200R Rackmount System Specifications

| Dimensions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Height Width Depth Weight |  | 220mm (8.6 in.) Standard EIA 310-D (RETMA) 638mm (25.1 in.) 28 kg (62 LB) typical |  |  |
| Environmental |  |  |  |  |
| Temperature | Operating* Storage (60 days) Rate of change |  |  | 10-35° C (50-95° F) -20-65° C (-4-149° F) 20° C/hr (36F/hr) |
| Relative humidity | Operating Storage (60 days) Rate of change |  |  | 20–80% 10–90% 20%/hr |
| Maximum wet bulb temperature | Operating Storage (60 days) |  |  | 35° C (95° F) 65° C (149°F) |
| Altitude | Operating Nonoperating |  |  | 2438 m (8,000 ft) 4876 m (16,000 ft) |
| Mechanical shock | Nonoperating |  |  | 30 G peak 25 ms |
| Electrical—Power Supplies are universal 120/240 Vac |  |  |  |  |
| Nominal ac voltage Operating Voltage range Nominal frequency Frequency range Maximum inrush current Rated input current | 100-120 Vac 90-135 Vac 60 Hz 47-63 Hz 50 Amps 9.5 Amps |  | 220-240 Vac 180-265 Vac 50 Hz 47-63 Hz 50 Amps 5 Amps |  |