### AlphaStation 600A 5/500 
V1.4—29 December 1997
DIGITAL Systems and Options Catalog

Product Description	The new AlphaStation 600A 5/500 high performance deskside Workstation series delivers premium performance and excellent expandability, providing access to thousands of applications running on DIGITAL UNIX, OpenVMS, and Windows NT Workstation systems.

The AlphaStation 600A uses the Alpha microprocessor 21164A

AlphaStation 600A 5/500 MHz CPU performance measures 14.8 SPECint95, and 15.0 SPECfp95 with 8 Mbytes cache.

The system is housed in a deskside enclosure with capacity for up to 1 Gbyte of ECC memory, 128-bit wide memory bus, nine option slots, and six storage bays (one dedicated Floppy diskette drive bay, two removable media bays, and three hard disk drive bays).  The system supports a wide variety of industry-standard peripherals and PCI and EISA options.  Other standard features include Twisted Pair Ethernet, stereo-quality audio, and an array of external ports for serial/parallel communications.  This combination of standard features allows multiple in-box configurations without the need for additional tabletop options or expansion boxes.

AlphaStation systems come with the best hardware warranty in the industry—a three-year warranty; 1 year on site, 2 years return to factory.  DIGITAL’s Trade-In ‘97 program provides a cost-effective upgrade path for users of older workstations.

Digital believes the information in this publication is accurate as of its publication date; such information is subject to change without notice. Digital is not responsible for any inadvertent errors.

Digital conducts its business in a manner that conserves the environment and protects the safety and health of its employees, customers, and the community.

Digital, the DIGITAL logo are trademarks of Digital Equipment Corporation.

Printed in USA. Copyright 1997 Digital Equipment Corporation. All rights reserved.

Systems

- DIGITAL UNIX and OpenVMS systems include factory-installed software (FIS).

- Options ordered will be factory installed unless specified as spares.

AlphaStation 600A 5/500 systems include

- Alpha microprocessor 21164A 500-MHz CPU with 
8 MB cache

- Deskside enclosure which includes:

- Nine expansion slots
—Seven 32-bit PCI
—Two EISA

- Four memory options (sixteen SIMM memory slots) supports up to 1 GB memory

- Integral fast wide SCSI-2 controller

- Six storage slots: 
One dedicated diskette drive slot
Two removable media slots (CD-ROM included in one slot)
Three 3.5-inch hard disk drive slots for 1- and 1.6-inch (16-bit) wide ready drives

- 400-Watt power supply

- One serial port, support full modem control

- One parallel port

- Keyboard port and mouse port

- PCI-based Ethernet (DE500) 10 or 100 MB/Sec

- 1.44 Mbyte diskette drive

- 600 Mbyte CD-ROM drive

- Audio, headset, and microphone

- 3-button mouse

- Hardware documentation

- Hardware Warranty

- Software Warranty: 90-day SPD conformance with advisory telephone support

- DIGITAL UNIX V4.0A 2-user base license
DIGITAL Open3D license or

- OpenVMS V7.1 base license plus concurrent use license
DIGITAL Open3D license
Multimedia Services license
DIGITAL NAS Client 150 license or

- Windows NT Workstation 4.0 media kit, 
 DIGITAL Light & Sound license and CD-ROM

Ordering menus that follows include

AlphaStation 600A 5/500 Base Configurations.

Menus are streamlined for ease of ordering.  To configure a customized system, use Base Configuration menus.

System options are in a separate section.

Warranty Statement

AlphaStations are sold with specific warranty response times, hours of coverage, warranty duration, and a specific manner by which the warranty service will be delivered.  DIGITAL also makes available extended coverage offerings to uplift or extend the service coverage and/or response time:  See Supplemental Services in AlphaStation Options section.

### AlphaStation 600A 5/500 Base Configurations

Y=in base configuration; M=mandatory option; O=option

| AlphaStation 600A 5/500 | Resources Used |  | DIGITAL UNIX | Open VMS | Windows NT |
| --- | --- | --- | --- | --- | --- |
| 120V/240V |  |  | PB65A-AA | PB65A-BA | PB65A-CA |
| CPU Alpha microprocessor | 21164A 500-MHz | Y | Y | Y | Y |
| Floppy Disk Drive 1.44 MB | Dedicated bay | Y | Y | Y | Y |
| CD-ROM 600 MB 12X | 5.25” SCSI bay | Y | Y | Y | Y |
| Headset, microphone | 1 EISA slot | Y | Y | Y | Y |
| Mouse |  | Y | Y | Y | Y |
| Ethernet (DE500) | 1 32-bit PCI slot | Y | Y | Y | Y |
| Cache (onboard) |  | Y | 8 MB | 8 MB | 8 MB |
| Operating system |  | Y | Y | Y | Y |
| User documentation—English |  | Y | Y | Y | Y |

| Remaining available resources |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I/O slots PCI 32-bit wide |  |  | 6 | 6 | 6 |
| EISA |  |  | 1 | 1 | 1 |
| Memory options (a) |  |  | 4 | 4 | 4 |
| SCSI bays Internal 5.25” |  |  | 1 | 1 | 1 |
| Internal 3.50” |  |  | 3 | 3 | 3 |
| Note: 3.5-inch hard disk drive supported in 5.25-inch CD-ROM bay, brackets are included. Brackets support 3.5 x 1-inch disk drive only |  |  |  |  |  |
| 1. Country Options | Select |  | 1 | 1 | 1 |
| PS/2 style keyboard (b) VMS / VT style keyboard |  | M | LK47W-xx LK46W-xx | LK47W-xx LK46W-xx | LK47W-xx LK46W-xx |
| Power Cord (c) |  | M | BNxxx-xx | BNxxx-xx | BNxxx-xx |
| Note: LK47W-AA and LK46W-AA, Include English keyboards and U.S. power cord. Power cord does not need to be ordered separately. |  |  |  |  |  |
| 2. Memory Required: 1 Optional Available: 2, 3, or 4 | Select |  | Minimum Required 64 MB 1, 2, 3 or 4 | Minimum Required 64 MB 1, 2, 3, or 4 | Minimum Required 32 MB 1, 2,3 or 4 |
| –32 MB | 1 option | M | MSP01-CE | MSP01-CE | MSP01-CE |
| –64 MB | 1 option | M | MSP01-CF | MSP01-CF | MSP01-CF |
| 128 MB | 1 option | M | MSP01-CG | MSP01-CG | MSP01-CG |
| 256 MB | 1 option | M | MSP01-CH | MSP01-CH | MSP01-CH |
| Note: To reach 1 GB maximum memory order MSP01-CH options |  |  |  |  |  |
| 3. Wide hard drive | Select |  | 1, 2 or 3 | 1, 2 or 3 | 1, 2 or 3 |
| 2.1 GB drive 7200 RPM (d) | 1” bay 1 SCSI | M | PBXRW-JC | PBXRW-JC | PBXRW-JC |
| 4.3 GB drive 7200 RPM (d) | 1.6” bay 1 SCSI | M | PBXRW-NB | PBXRW-NB | PBXRW-NB |
| 9.1 GB drive 7200 RPM (d) | 1.6” bay 1 SCSI | M | PBXRZ-SA | PBXRZ-SA | PBXRZ-SA |
| 4. Monitors | Select |  | 1 | 1 | 1 |
| 17” Color Monitor NH/SH (e) |  | M | SN-VRTX7-WA/W3 SN-VRT17-W4 | SN-VRTX7-WA/W3 SN-VRT17-W4 | SN-VRTX7-WA/W3 SN-VRT17-W4 |
| 21” Color Monitor NH/SH (e) |  | M | SN-VRCX1-WA/W3/W4 | SN-VRCX1-WA/W3/W4 | SN-VRCX1-WA/W3/W4 |
| 24” Color monitor (e) Northern Hemisphere Only Use PBXGI-AC graphics |  | M | SN-PCXAV-AZ |  | SN-PCXAV-AZ |

(a)	Sixteen SIMM slots support four memory options (4 SIMMs per option).  Memory options can be mixed.

(b)	Select country-specific keyboard from AlphaStation Options section.

(c)	Select country-specific power cord from AlphaStation Options section.

(d)		Selection of one hard disk drive is mandatory for Base systems.  Internal SCSI uses a wide 68-pin connector.  To add a narrow drive, order PBXKP-BA (wide to narrow adapter).  Order one adapter for each additional drive.

(e)	Monitors include video cable and 120V power cord, order country specific power cord for 240V use from AlphaStation Options section.

### AlphaStation 600A 5/500 Base Configurations (continued)

Y=in base configuration; M=mandatory option; O=option

| AlphaStation 600A 5/500 | Resources Used |  | DIGITAL UNIX | Open VMS | Windows NT |
| --- | --- | --- | --- | --- | --- |
| 120V/240V | 21164A 500-MHz |  | PB65A-AA | PB65A-BA | PB65A-CA |

| 5. Graphics | Select |  | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- |
| PowerStorm 3D30 8-plane (f) | 1 PCI slot | M | PBXGB-AA | PBXGB-AA | PBXGB-AA |
| PowerStorm 4D20 24-plane (f) | 1 PCI slot | M | PBXGB-CA | PBXGB-CA | PBXGB-CA |
| PowerStorm 4D50T (f) | 2 PCI slots | M | PBXGI-AB |  | PBXGI-AB |
| PowerStorm 4D60T* (f) | 2 PCI slots | M | PBXGI-AC |  | PBXGI-AC |
| * Includes 16 MB Texture Memory. Upgradeable to 32 MB. |  |  |  |  |  |
| 5a. PowerStorm Texture Memory Upgrades—supported on PowerStorm 4D40T, 4D50T, and 4D60T graphics options only |  |  |  |  |  |
| –4 MB Texture memory module |  | O | PBXGI-GA |  | PBXGI-GA |
| 16 MB Texture memory module |  | O | PBXGI-GB |  | PBXGI-GB |
| 32 MB Texture memory module |  | O | PBXGI-GC |  | PBXGI-GC |
| 6. Removable media | Select |  | 0 or 1 | 0 or 1 | 0 or 1 |
| 8 GB 4 mm DAT TLZ09 | 5.25” SCSI bay | O | PBXTL-DA | PBXTL-DA | PBXTL-DA |
| 2.3 GB QIC TZK20 | 5.25” SCSI bay | O | PBXTZ-CA | PBXTZ-CA | PBXTZ-CA |
| 7. SCSI Storage Controller |  |  |  |  |  |
| PCI Fast SCSI-2 controller* | 1 PCI slot | O | KZPAA-AA | KZPAA-AA | KZPAA-AA |
| * Maximum two Fast SCSI-2 controllers per system |  |  |  |  |  |
| 8. Communication Options |  |  |  |  |  |
| PCI High-performance Ethernet | 1 PCI slot | O | DE435-AA | DE435-AA | DE435-AA |
| PCI FDDI controller (fiber) | 1 PCI slot | O | DEPFA-AA/BA/CA | DEFPA-AB | DEFPA-AB |
| 9. Software Media and Documentation kits |  |  |  |  |  |
| DIGITAL UNIX |  | O | QA-MT4AA-H8 |  |  |
| OpenVMS |  | O |  | QA-MT1AA-H8 |  |
| AlphaStation 600A User Documentation |  | O | EK-AL655-UI* | EK-AL655-UI* | EK-AL655-UI* |
| AlphaStation 600A Installation Guide |  | O | EK-AS800-IN* | EK-AS800-IN* | EK-AS800-IN* |
| * English User Documentation and Installation Guide are included with Advantage and Base Configurations. |  |  |  |  |  |
| 10. 500 MHz CPU Upgrade with 8 MB Onboard Cache (g) |  |  |  |  |  |
| AlphaStation 600 5/266 or 5/333 to AlphaStation 600A 5/500 MHz CPU upgrade |  | O | PB62U-CA | PB62U-CA | PB62U-CA |
| Note: Installation is not included with CPU upgrade, order FM-WSDSD-IN separately |  |  |  |  |  |
| 11. Additional Options: See AlphaStation Options for a comprehensive list of qualified options (h) |  |  |  |  |  |

(f)	PowerStorm Graphics options require 2 PCI slots.  Graphics options require DIGITAL Open3D media, included in factory installed software on DIGITAL UNIX and OpenVMS systems.  Windows NT graphics options include Graphics Support Services Software license, media and documentation.  3D multiscreen hardware not accelerated for Windows NT

Graphics configuration rules for DIGITAL UNIX systems:

- Up to 3 PowerStorm 3D30 or 4D20 graphics options and 3 monitors are supported

- Up to 3 PowerStorm 4D50T or 4D60T graphics options and 3 monitors are supported

- PowerStorm graphics options must be homogeneous (Three 3D30, 4D20, 4D50T or 4D60T)

Graphics configuration rules for OpenVMS systems:

- Up to 3 PowerStorm 3D30 or 4D20 graphics options and 3 monitors are supported

- Graphics options must be homogeneous (Three 3D30 or  4D20)

Graphics configuration rules for Windows NT systems:

- 1 PowerStorm 3D30, 4D20, 4D50T or 4D60T graphics option and 1 monitor is supported

(g)	The upgrade board has sixteen memory slots vs thirty two in the original product.  Although the original memory SIMMS will work, users with large memory may find that they cannot use all the SIMMS on the upgrade board.  Upgrade customers may wish to upgrade to the higher density SIMMS offered with the 600A 5/500 product.  These are MSP01-CE/CF/CG/CH.  32, 64, 128 and 256 MB options respectively.

(h)	For PCI or EISA cards, check the availability of slots (6 PCI, and 1 EISA available).

System Diagram

Specifications

| PCI -32 bit | 132 MB/second |
| --- | --- |
| Fast Wide SCSI-2 bus | 20 MB/s transfer rate |
| Ethernet | 10 or 100 MBit/s Twisted Pair standard |
| Power Requirements |  |
| Line voltage | 120 V/240 V |
| Voltage tolerance | 90-128 V/190-256 V |
| Frequency single phase | 50 Hz/ 60 Hz |
| Frequency tolerance | 47-63 Hz |
| Maximum running current | 8.0A/4.0A A.C. |
| Typical power consumption | 165 W A.C. |
| Maximum power consumption | 960 VA |
| Typical power factor | 0.6 |
| Operating Environment |  |
| Operating temperature | 10° to 40° C (50° to 104° F) |
| Operating humidity —Maximum wet bulb | 20% to 80% relative humidity 40° C (104° F) |
| Storage temperature | -20° C to 65° C (-4° F to 149° F) |
| Storage humidity —Maximum wet bulb | 10% to 90% relative humidity 65° C (149° F) |
| Maximum altitude —Operating —Nonoperating | 2,438 m (8,000 ft) maximum 4,876 m (16,000 ft) maximum |
| Nonoperating shock | 30G, 25 ms halfsine |
| Physical Characteristics |  |
| Height | 469.9 mm (18.5 inches) |
| Width | 241.3 mm (9.5 inches) |
| Depth | 660.4 mm (26.0 inches) |
| Weight | 31.8 kg (70 lb) |
| Regulatory |  |
| Agency Approval | AlphaStation 600A 5/500 is FCC Class B AlphaStation 600 5/266 to AlphaStation 600A 5/500 upgrade is FCC Class B |