### DIGITAL Server 7300 Pedestal-Rackmount
V2.6¾22 May 1998
DIGITAL Systems and Options Catalog

Product Description	The DIGITAL Server 7300 family of servers support 5/533 MHz and 5/400 MHz processing power, each with 4 MB of on-board cache, and a 600 MHz CPU with 8 MB of cache.  CPU board upgrades are available to upgrade 5/400 MHz servers to 5/533 or 5/600 MHz servers.  SMP upgrades are also available for the 5/400 MHz, 5/533 and 5/600 MHz CPU servers.  DIGITAL Server 7300 enhancements include integrated 100 Mbit Network adapter and support for UltraSCSI adapters, shelves, and disks, all color-coded in frost white.

Systems include a dual Ultra SCSI embedded adapter and UltraSCSI cables for very high density connections VHDCI.  Configuration information is available for Pedestal, Cabinet, and System Drawer to support the implementation of UltraSCSI technology.

Enhanced operating system software is Windows NT V4.0 with UltraSCSI backward compatibility.

ServerWORKS Manager provides proactive server and network management capabilities and is bundled with all DIGITAL Server systems.  Simple Network Management Protocol (SNMP) enables information to pass from the managed system to the ServerWORKS management console for Windows NT systems.  ServerWORKS is feature rich and uses an autodiscovery process to automatically discover all servers and network devices on the LAN and build a Topology Map so the administrator can see how the systems are connected together.  Detailed server information is available by drilling down into the server to retrieve critical data that is displayed on an easy to read system browser.  System, network, storage, and environmental information are retrievable.  Alarms can be set on critical system parameters and events to notify the administrator of a problem before it occurs enabling proactive management (instead of reactive management) of systems. All DIGITAL Server systems are supplied with management tools to complement ServerWORKS Manager.

DIGITAL believes the information in this publication is accurate as of its publication date; such information is subject to change without notice. Digital is not responsible for any inadvertent errors.

DIGITAL conducts its business in a manner that conserves the environment and protects the safety and health of its employees, customers, and the community.

DIGITAL, and  the DIGITAL logo are trademarks of Digital Equipment Corporation.

Printed in USA. Copyright 1998 Digital Equipment Corporation. All rights reserved.

Step 1—DIGITAL Server 7300 Pedestal and Cabinet Systems

All System configurations consist of One System Drawer installed in a Pedestal or Cabinet enclosure

- System Drawer supports up to four CPUs and up to four memory options

- DIGITAL Server 7300 System Drawer includes

- DIGITAL Server 7300 System Drawer with 
Alpha microprocessor 21164
5/400 MHz CPU, 4 MB secondary cache, or
5/533 MHz CPU, 4 MB secondary cache, or
5/600 MHz CPU, 8 MB secondary cache

- Eight PCI slots (six available)

- Integral (internal-only SCSI bus) to support removable media (CD-ROMs and tapes)

- Integral CD-ROM drive (standard)

- 1.44 MB diskette drive (standard)

- Two EIA-232 asynchronous serial ports,
9-pin D-subminiature connectors

- One parallel port, 25-pin D-subminiature connectors

- Keyboard and mouse ports

- 450 W auto sensing power supply

- Memory (included memory uses one memory slot)

- PCI card with two UltraSCSI ports, one 10/100 Ethernet port, and two BN37A-02 SCSI cables—uses one PCI slot

- S3 TRIO 1 MB RAM graphics adapter—uses one PCI slot

- Selection of Country Kit for all systems is mandatory

- Integral remote system console

- Operating system license

- Customer documentation

- Hardware warranty1

- Software warranty1

- 1.	Service upgrades are available, see Step 11.

- Step 1a—DIGITAL Server 7300 Pedestal and Cabinet Systems

- Digital’s OEM license agreement for Windows NT mandates compliance with specific configuration rules:

- Windows NT Language Configuration

- All Windows NT Packaged Systems include factory-installed software (FIS) on 4.3 GB hard disk drive.

- Selection of language-specific Windows NT Server 4.0 license/media (CD-ROM) kit is mandatory 
when ordering a Windows NT Packaged System, 
see Step 10.

- Windows NT is not available from Digital for 
DIGITAL Server 7300 Memory Packages Systems, order directly from Microsoft, or from an authorized Microsoft reseller.

- Software Warranty

- Warranty on Microsoft's Windows NT Server 4.0 is conformance to the written material accompanying the software for a period of ninety days.

- Video monitor is required for system management; order separately if not available at customer site.

- Step 1a—DIGITAL Server 7300 Pedestal Systems

- All Pedestal Systems consists of:

- A System Drawer installed in a Pedestal enclosure with one UltraSCSI shelf.

- Pedestal systems support a maximum of one system drawer and up to three UltraSCSI shelves.

- Pedestal Windows NT Packaged Systems

- Selection of language-specific Windows NT Server 4.0 license/media (CD-ROM) kit is mandatory when ordering a Pedestal Windows NT Packaged System, see Step 10.  Country Kit is also mandatory, see Step 8.

| - Worldwide |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| - FR-K9F2W-AA | - Mandatory | - 400 MHz | - Mandatory | - 128 MB | - 4.3 GB | - Included | - Included | - Required |
| - FR-K9F2W-AB | - Mandatory | - 400 MHz | - Mandatory | - 2 GB | - 4.3 GB | - Included | - Included | - Required |
| - FR-K9F5W-AA | - Mandatory | - 533 MHz | - Mandatory | - 128 MB | - 4.3 GB | - Included | - Included | - Required |
| - FR-K9F5W-AB | - Mandatory | - 533 MHz | - Mandatory | - 2 GB | - 4.3 GB | - Included | - Included | - Required |
| - FR-K9FAW-AA* | - Mandatory | - 600 MHz | - Mandatory | - 1 GB | - 4.3 GB | - Included | - Included | - Required |

*	FR-K9FAW-AA includes 50 ns memory

- Step 1a—DIGITAL Server 7300 Pedestal Systems (continued)

- Pedestal Windows NT Packaged Systems

- Selection of language-specific Windows NT Server 4.0 license/media (CD-ROM) kit is mandatory when ordering a Pedestal Windows NT Packaged System, see Step 10.  Country Kit is also mandatory, see Step 8.

| - North America |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| - FR-K9F2A-AA | - Mandatory | - 400 MHz | - Mandatory | - 128 MB | - 4.3 GB | - Included | - Included | - Required |
| - FR-K9F2A-AB | - Mandatory | - 400 MHz | - Mandatory | - 2 GB | - 4.3 MB | - Included | - Included | - Required |
| - FR-K9F5A-AA | - Mandatory | - 533 MHz | - Mandatory | - 128 MB | - 4.3 GB | - Included | - Included | - Required |
| - FR-K9F5A-AB | - Mandatory | - 533 MHz | - Mandatory | - 2 GB | - 4.3 GB | - Included | - Included | - Required |
| - FR-K9FAA-AA* | - Mandatory | - 600 MHz | - Mandatory | - 1 GB | - 4.3 GB | - Included | - Included | - Required |

*	FR-K9FAA-AA includes 50 ns memory

- Pedestal Memory Packaged Systems

- Windows NT is not available from DIGITAL for Pedestal Memory Packaged Systems, order directly from Microsoft, or from an authorized Microsoft reseller.  Country Kit is mandatory, see Step 8.

| - Worldwide |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| - FR-K9F2W-WB | - Required1 | - 400 MHz | - Mandatory | - 128 MB | - Required | - Included | - Included | - Required |
| - FR-K9F2W-WC | - Required1 | - 400 MHz | - Mandatory | - 2 GB | - Required | - Included | - Included | - Required |
| - FR-K9F5W-WB | - Required1 | - 533 MHz | - Mandatory | - 128 MB | - Required | - Included | - Included | - Required |
| - FR-K9F5W-WC | - Required1 | - 533 MHz | - Mandatory | - 2 GB | - Required | - Included | - Included | - Required |

| - North America |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| - FR-K9F2A-WB | - Required1 | - 400 MHz | - Mandatory | - 128 MB | - Required | - Included | - Included | - Required |
| - FR-K9F2A-WC | - Required1 | - 400 MHz | - Mandatory | - 2 GB | - Required | - Included | - Included | - Required |
| - FR-K9F5A-WB | - Required1 | - 533 MHz | - Mandatory | - 128 MB | - Required | - Included | - Included | - Required |
| - FR-K9F5A-WC | - Required1 | - 533 MHz | - Mandatory | - 2 GB | - Required | - Included | - Included | - Required |

- 1.	Windows NT Server 4.0 license and media (CD-ROM) kit is not available from DIGITAL for these systems.  Order directly from Microsoft, or from an authorized Microsoft reseller.

- Key:	Mandatory items must be on purchase order at initial order acceptance.
Required items are essential for full system operation.

- Step 1b—DIGITAL Server 7300 Cabinet Systems

- All Cabinet Systems consists of:

- A System Drawer installed in a Cabinet enclosure with one UltraSCSI shelf.

- Cabinet systems support up to three system drawers and up to eight UltraSCSI shelves.

- Cabinet Windows NT Packaged Systems-

- Selection of language-specific Windows NT Server 4.0 license/media (CD-ROM) kit is mandatory when ordering a Cabinet Windows NT Packaged System, see Step 10.  Country Kit is also mandatory, see Step 8.

| - Worldwide |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| - FR-R9F2W-AA | - Mandatory | - 400 MHz | - Mandatory | - 128 MB | - 4.3 GB | - Included | - Included | - Required |
| - FR-R9F2W-AB | - Mandatory | - 400 MHz | - Mandatory | - 2 GB | - 4.3 GB | - Included | - Included | - Required |

- Step 1b—DIGITAL Server 7300 Cabinet Systems (continued)

- Cabinet Windows NT Packaged Systems

| - Worldwide |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| - FR-R9F5W-AA | - Mandatory | - 533 MHz | - Mandatory | - 128 MB | - 4.3 GB | - Included | - Included | - Required |
| - FR-R9F5W-AB | - Mandatory | - 533 MHz | - Mandatory | - 2 GB | - 4.3 GB | - Included | - Included | - Required |
| - FR-R9FAW-AA * | - Mandatory | - 600 MHz | - Mandatory | - 1 GB | - 4.3 GB | - Included | - Included | - Required |

| - North America |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| - FR-R9F2A-AA | - Mandatory | - 400 MHz | - Mandatory | - 128 MB | - 4.3 GB | - Included | - Included | - Required |
| - FR-R9F2A-AB | - Mandatory | - 400 MHz | - Mandatory | - 2 GB | - 4.3 MB | - Included | - Included | - Required |
| - FR-R9F5A-AA | - Mandatory | - 533 MHz | - Mandatory | - 128 MB | - 4.3 GB | - Included | - Included | - Required |
| - FR-R9F5A-AB | - Mandatory | - 533 MHz | - Mandatory | - 2 GB | - 4.3 GB | - Included | - Included | - Required |
| - FR-R9FAA-AA* | - Mandatory | - 600 MHz | - Mandatory | - 1 GB | - 4.3 GB | - Included | - Included | - Required |

*	FR-R9FAW-AA and FR-R9FAA-AA include 50 ns memory

- Cabinet Memory Packaged Systems

- Windows NT is not available from DIGITAL for Cabinet Memory Packaged Systems, order directly from Microsoft, or from an authorized Microsoft reseller.  Country Kit is mandatory, see Step 8.

| - Worldwide |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| - FR-R9F2W-WB | - Required1 | - 400 MHz | - Mandatory | - 128 MB | - Required | - Included | - Included | - Required |
| - FR-R9F2W-WC | - Required1 | - 400 MHz | - Mandatory | - 2 GB | - Required | - Included | - Included | - Required |
| - FR-R9F5W-WB | - Required1 | - 533 MHz | - Mandatory | - 128 MB | - Required | - Included | - Included | - Required |
| - FR-R9F5W-WC | - Required1 | - 533 MHz | - Mandatory | - 2 GB | - Required | - Included | - Included | - Required |

| - North America |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| - FR-R9F2A-WB | - Required1 | - 400 MHz | - Mandatory | - 128 MB | - Required | - Included | - Included | - Required |
| - FR-R9F2A-WC | - Required1 | - 400 MHz | - Mandatory | - 2 GB | - Required | - Included | - Included | - Required |
| - FR-R9F5A-WB | - Required1 | - 533 MHz | - Mandatory | - 128 MB | - Required | - Included | - Included | - Required |
| - FR-R9F5A-WC | - Required1 | - 533 MHz | - Mandatory | - 2 GB | - Required | - Included | - Included | - Required |

- 1.	Windows NT Server 4.0 license and media (CD-ROM) kit is not available from DIGITAL for these systems.  Order directly from Microsoft, or from an authorized Microsoft reseller.

- Key:	Mandatory items must be on purchase order at initial order acceptance.
Required items are essential for full system operation.

- Step 1c—System Drawers

- Cabinet systems support up to two additional System Drawers and up to eight UltraSCSI shelves.

- Additional System Drawers are orderable with and without Windows NT operating system.

- To add System Drawer(s) with Windows NT operating system the following options are mandatory:
System Drawer(s), Windows NT Server 4.0 license/media (CD-ROM) kit(s), 4.3 GB hard disk drive(s) with Factory Installed Software (FIS) FR-CECBA-NT.

- Note:  4.3 GB hard disk drive with Windows NT factory installed (FR-CECBA-NT) is only available when ordered with a system drawer and a Windows NT Server 4.0 license/media kit.

- System Drawer includes mounting kit with 6-foot extender cables for mouse and keyboard, one 10-foot video cable, and appropriate power cords.

- For assistance in configuring a Cabinet System with four System Drawers, contact DIGITAL CustomSystems at 
1-800-DIGITAL.

- Systems Drawers for Cabinet Enclosure with Windows NT Operating System

| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - FR-PD03A-A9 | - Mandatory1 | - 400 MHz | - Required | - Required | - Mandatory | - Included | - Included | - Required |
| - FR-PD03A-B9 | - Mandatory1 | - 533 MHz | - Required | - Required | - Mandatory | - Included | - Included | - Required |
| - FR-PD03A-C9 | - Mandatory1 | - 600 MHz | - Required | - Required | - Mandatory | - Included | - Included | - Required |

- 1.	Selection of language-specific Windows NT Server 4.0 license/media (CD-ROM) kit is mandatory.

- Systems Drawers for Cabinet Enclosure without Windows NT Operating System

| - Order Number | - O/S Media | - CPU Speed | - Country kit | - Memory | - Disk | - Ethernet | - Graphics | - Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - FR-PD03A-A9 | - Required2 | - 400 MHz | - Required | - Required | - Required | - Included | - Included | - Required |
| - FR-PD03A-B9 | - Required2 | - 533 MHz | - Required | - Required | - Required | - Included | - Included | - Required |
| - FR-PD03A-C9 | - Required2 | - 600 MHz | - Required | - Required | - Required | - Included | - Included | - Required |

- 2.	Windows NT Server 4.0 license and media (CD-ROM) kit is not available from DIGITAL for these systems.  Order directly from Microsoft, or from an authorized Microsoft reseller.

- Key:	Mandatory items must be on purchase order at initial order acceptance.
Required items are essential for full system operation.

- Step 2—Additional CPU Modules (SMP Upgrades)

- DIGITAL Server 7300 System Drawer supports three additional CPUs

- CPUs (i.e., 5/533 4 MB) must be of the same type in each System Drawer.

| - FR-PD03U-AA | - Windows NT SMP upgrade includes one 5/400 (4 MB) CPU; SMP license not required |
| --- | --- |
| - FR-PD03U-BA | - Windows NT SMP upgrade includes one 5/533 (4 MB) CPU; SMP license not required |
| - FR-PD03U-CA | - Windows NT SMP upgrade includes one 5/600 (8 MB) CPU; SMP license not required |

- Step 3—Additional Power Supply

- Power supply included with System Drawer provides support for two processors, memory, and I/O adapters.

| - FR-H7291-AA | - 450 Watt power supply |
| --- | --- |

| - DIGITAL Server 7300 |  |
| --- | --- |
| - 1st 450 W power supply (included) | - Supports up to 2 CPUs and 8 active PCI slots |
| - 2nd 450 W power supply | - Supports 3rd or 4th CPU or provides N+1 for up to 2 CPUs and 8 active PCI slots |
| - 3rd 450 W power supply | - Provides N+1 for up to 4 CPUs |

- Step 4—Memory

- Windows NT Server 4.0 supports maximum memory of 4 GB.
Note: Windows NT Server supports up to 8 GB memory with Oracle 8 NT.

- DIGITAL Server 7300 System Drawer supports up to four memory options

- Note:	Field installation of memory requires largest memory option to be placed in Memory Slot 0.

| - FR-ACSMA-AA | - 128 MB memory option (60 ns) |
| --- | --- |
| - FR-ACSMA-AB | - 512 MB memory option (60 ns) |
| - FR-ACSMA-AC | - 1 GB memory option (60 ns) |
| - FR-ACSMA-AD | - 2 GB memory option (60 ns) |
| - FR-ACSMA-AE | - 1 GB High Performance memory Option (50 ns) |
| - FR-ACSMA-AF | - 2 GB High Performance memory Option (50 ns) |

- Step 5—Storage

- System Drawer includes PCI card with two UltraSCSI ports, one Ethernet port, and two BN37A-02 SCSI cables.

- Pedestal and Cabinet Windows NT Packaged systems include UltraSCSI shelf and one 4.3 GB UltraSCSI disk drive.

- Pedestal and Cabinet Memory Packaged systems include UltraSCSI shelf.

- Disk Drives

| - FR-CDCBA-CA | - 2.1 GB UltraSCSI SCA-2 7200 RPM disk drive, frost white |
| --- | --- |
| - FR-CECBA-CA | - 4.3 GB UltraSCSI SCA-2 7200 RPM disk drive, frost white |
| - FR-CECCA-CA | - 4.3 GB UltraSCSI SCA-2 10,000 RPM disk drive, frost white |
| - FR-CFCBA-CA | - 9.1 GB UltraSCSI SCA-2 7200 RPM disk drive, frost white |
| - FR-CGCBA-CA | - 18 GB UltraSCSI SCA-2 7200 RPM disk drive, frost white |

- Removable Media

- All System Drawers include CD-ROM drive in 5.25-inch removable media slot connected to integral FNSE SCSI-2 controller.

- System Drawer installed in Pedestal Enclosure supports one additional removable media device. Optional 5.25-inch half-height removable media slot in Pedestal systems is connected to integral FNSE SCSI-2 controller.

- Tape Drives and additional CD-ROM for Pedestal System Drawer only

| - FR-PCXCR-AR | - 600 MB 12x speed CD-ROM 5.25-inch drive, frost white |
| --- | --- |
| - FR-PCXAT-AJ | - 4/8 GB 4 mm DAT 93 MB/min (compressed) 5.25-inch SCSI tape drive |
| - FR-PCXAT-AK | - 12/24 GB 4 mm DAT 3.25-inch SCSI tape drive |
| - PBXTZ-AA | - 2 GB QIC tape drive (internal) |
| - TLZ10-LK | - 8 GB 4 mm DAT 93 MB/min (compressed) 5.25-inch SCSI tape drive, frost white |

- Replacement Floppy Drive

| - FR-PCXRJ-AD | - 1.44 MB 3.5” floppy drive, frost white |
| --- | --- |

- Tape Drives and additional CD-ROM for BA356 shelves

| - FR-PCXLT-AD | - 35/70 GB 5.25 inch single-bus SCSI DLT drive |
| --- | --- |
| - FR-PCXAT-EA | - 25/50 GB 5.25-inch SCSI AIT tape drive |
| - FR-PCXCR-AR | - 600 MB 12x speed CD-ROM 5.25-inch drive, frost white |

- Step 5a—UltraSCSI Shelves

- Pedestal and Cabinet Windows NT Packaged Systems include UltraSCSI shelf and one 4.3 GB UltraSCSI disk drive.

- Pedestal and Cabinet Memory Packaged Systems include UltraSCSI shelf.

- Pedestal Enclosure supports up to a maximum of three UltraSCSI shelves.

- Cabinet Enclosure supports up to a maximum of eight UltraSCSI shelves.

- Additional UltraSCSI shelves require controller(s) and cables.

- UltraSCSI Configuration Rules

- UltraSCSI configurations require UltraSCSI components (adapters/shelves/disks/cables).

- UltraSCSI adapters and RAID controllers support UltraSCSI disks at UltraSCSI speeds in UltraSCSI shelves (FR-BA356-xx) shelves.

- UltraSCSI drives and Fast-10 devices can be mixed in BA356 UltraSCSI shelf. Drives negotiate maximum transfer speeds with UltraSCSI adapter/controller.

BA356 UltraSCSI Shelf Kit

UltraSCSI Shelf kits include 16-bit UltraSCSI shelf and 16-bit UltraSCSI personality module with Bus Extender chip, 
180-Watt power supply and mounting hardware.  Power cord must be ordered separately, BN20Z-4E for North America and BN35S-4E for Europe.

- Each shelf supports:

- Seven 3.5-inch hard disk drives or

- One 5.25-inch device and four 3.5-inch hard disk drives or

- Two 5.25-inch devices and one 3.5-inch hard disk drive

- Installation of redundant power supply reduces number of 3.5-inch devices by one.

| - FR-BA356-PD | - UltraSCSI shelf, supports 7 UltraSCSI SBB disk drives, requires power cord, BN20Z-4E for North America and BN35S-4E for Europe |
| --- | --- |
| - FR-BA35X-HK | - 180-Watt universal ac input redundant power supply, power cord included in shelf kits |

- Step 5b¾SCSI Adapters, Controllers, Connectors, and Cables

- Systems include FR-PCTBZ-AA PCI card with two UltraSCSI ports, one 10/100 Mbit Ethernet port, and two BN37A-02 SCSI cables.

- The following UltraSCSI controllers can be configured on DIGITAL Server 7300 System Drawers running Windows NT 4.0 operating system.

| - Order Number | - Description | - Maximum # Supported Controllers |
| --- | --- | --- |
| - FR-PCTBZ-AA | - Two-Port PCI UltraSCSI Single-Ended 68-pin VHDCI, and one Channel 10/100Mb Ethernet adapter. Note: Must be placed in PCI bus 0 to access onboard configuration utilities. | - 4 |
| - FR-BN37A-02 | - 2 m cable, VHDCI male to VHDCI male |  |
| - FR-BN37A-05 | - 5 m cable, VHDCI male to VHDCI male |  |

| - Order Number | - Description | - Maximum # Supported Controllers |
| --- | --- | --- |
| - FR-KZPAC-AA | - PCI one-port UltraSCSI RAID controller with 4 MB cache memory; includes RAID Array 230/plus Subsystem software and documentation kit; tape drives not supported. When used in UltraSCSI shelf, operates in UltraSCSI mode. | - 4 |
| - FR-PCTAR-UB | - Cache battery-backup for KZPAC controller |  |
| - FR-BN37A-02 | - 2 m cable, VHDCI male to VHDCI male |  |
| - FR-BN37A-05 | - 5 m cable, VHDCI male to VHDCI male |  |

- Step 5b¾SCSI Adapters, Controllers, Connectors, and Cables (continued)

| - Order Number | - Description | - Maximum # Supported Controllers |
| --- | --- | --- |
| - FR-KZPAC-CA | - PCI three-port UltraSCSI RAID controller with 4 MB cache memory; includes RAID Array 230/plus Subsystem Software and documentation kit; tape drives not supported. | - 4 |
| - FR-KZPAC-CB | - Same as above with 8 MB cache memory | - 4 |
| - MS100-BB | - 8 MB EDRAM SIMM, upgrades KZPAC-CA to -CB |  |
| - FR-PCTAR-UB | - Cache battery-backup for KZPAC controller |  |
| - BN31K-0E | - SCSI cable/bulkhead assembly kit for connection of third port on FR-KZPAC-CA/CB to PCI/EISA bulkhead slot. Uses one PCI/EISA bulkhead slot. |  |
| - FR-KZPAC-SB | - SCSI bulkhead assembly kit with two ports, allows connection of two third-port outputs on FR-KZPAC controllers to a single bulkhead PCI/EISA slot. Uses one PCI/EISA bulkhead slot. |  |
| - FR-BN37A-02 | - 2 m cable, VHDCI male to VHDCI male |  |
| - FR-BN37A-05 | - 5 m cable, VHDCI male to VHDCI male |  |

| - Order Number | - Description | - Maximum # Supported Controllers |
| --- | --- | --- |
| - FR-PCTAZ-DF | - PCI one port UltraSCSI Single-ended adapter (68-pin HD) | - 4 |
| - FR-PCTAZ-EC | - PCI two port UltraSCSI Single-Ended adapter (68-pin HD | - 4 |
| - FR-PCTAZ-FA | - PCI one-port Adaptec 2944UW differential SCSI adapter |  |
| - BN21N-02 | - 2 m cable, 8-bit to 16-bit SCSI, 68-pin HD to 50-pin HD |  |
| - KZPBA-CB | - PCI one-port UltraSCSI Differential adapter, supports external tapes only |  |
| - FR-BN38C-02/05 | - 2 meter/5 meter 68-HD male to VHDCI male SCSI cable connects from KZPBA-CB to BA356-PA/PD storage |  |

- Step 5c—Windows NT Cluster Kits

- The RA310 Cluster Kit (FR-CK310-RF) includes DIGITAL Clusters for Windows NT.   Kit also supports Microsoft Cluster Server, requires Windows NT Server Enterprise Edition for each server, order separately.

- The BA356 and RA7000 Cluster Kits (FR-CK356-xx and FR-CK700-xx) support both DIGITAL Clusters for Windows NT, requires Windows NT Server 4.0 for each server, and one copy of the DCNT software (QB-53V9A-SA). Microsoft Cluster Server requires Windows NT Server Enterprise for each server, order separately.

- Systems, disk drives, and Windows NT Server licenses are not included.

| - FR-CK310-RF | - RA310 Cluster Kit |
| --- | --- |

| - FR-CK356-UP | - BA356U Cluster Kit – pedestal |
| --- | --- |
| - FR-CK356-UR | - BA356U Cluster Kit – rackmount |

| - FR-CK700-UP | - RA7000 Cluster Kit – pedestal |
| --- | --- |
| - FR-CK700-UR | - RA7000 Cluster Kit – rackmount |

- Step 5d—External Storage

| - FR-BA356-PA | - UltraSCSI standalone pedestal enclosure with 180 W power supply, supports 7 UltraSCSI SBB disk drives, requires power cord. |
| --- | --- |
| - FR-BA356-PD | - UltraSCSI rackmount enclosure with 180 W power supply, supports 7 UltraSCSI SBB disk drives, requires power cord. |
| - FR-BA35X-HK | - 180 W universal AC input power supply. FR-BA365-PA/-PD include power supply, one additional 180 W power supply is supported. Disk drives supported are reduced by one if second power supply is installed. |

- Step 5d—External Storage (continued)

- RAID Array 310

| - FR-SWXRA-Z1 | - RAID Array 310 Pedestal Cabinet Storage Subsystem |
| --- | --- |
| - FR-SWXRA-Z2 | - RAID Array 310 Expansion Kit, requires FR-SWXRA-Z1 |
| - FR-SWXRA-03 | - RAID Array 310 replacement controller |
| - SWDKT-MA | - RAID Array 310 platform kit for Windows NT Alpha |

- RAID Array 450

| - DS-SWXRA-W0 | - RAID Array 450 Subsystem, office enclosure |
| --- | --- |
| - DS-SWXRA-W4 | - RAID Array 450 Subsystem, rackmount enclosure |
| - HSZ50-AF | - RAID Array 450 HSZ50 controller with 32 MB cache |
| - HSSIM-AA | - 32 MB cache module for HSZ50 controller |
| - SWXBP-SD | - RA450 150 W Shelf Power Supply |
| - HS35X-BA | - Single cache battery in one SBB |
| - HS35X-BB | - Dual cache batteries in one SBB |
| - SWXBF-AA | - Dual speed fan kit |
| - SWXBE-AA | - Environmental Monitor Unit (EMU) |

- RAID Array 7000

| - DS-SWXRA-HA | - RAID Array 7000 with dual HSZ70 controllers |
| --- | --- |
| - DS-SWXRA-HC | - RAID Array 7000 with single HSZ70 controller |
| - DS-HSZ70-AH | - RAID 7000 controller with 64 MB cache |
| - DS-BA35X-HH | - 180 W power supply |
| - DS-BA35X-HE | - Power Control Unit |
| - DS-BA35X-BC | - Single replacement external cache battery |
| - DS-BA35X-BD | - Dual replacement external cache batteries |
| - DS-BA35X-EB | - Environmental Monitoring Unit (EMU) |
| - DS-BA35X-EC | - Power Verification and Addressing (PVA) |
| - DS-BA35X-MK | - Dual-speed Fan Kit |
| - BNF01-10 | - 10-meter Cable - host connection cable |

- RAID Array 10000

| - DS-SWXES-AA/AB | - ESA 10000 high capacity/general base unit |
| --- | --- |
| - DS-SWXES-BA/BB | - ESA 10000 high bandwidth base unit |
| - DS-SWXES-CA/CB | - ESA 10000 dual expansion base unit |
| - DS-SWXES-DA/DB | - ESA 10000 single expansion, dual controllers base unit |
| - DS-HSSIM-AB | - 64 MB cache upgrade for HSZ70 |
| - DS-BA35X-HH | - 180 W power supply, three supported for each BA370 cabinet |
| - DS-BA35X-HE | - AC input box for BA370 |
| - DS-SW6XP-AA/AB | - SW600 power distribution unit, one supported for each SW600 |

- SCSI Cables

| - FR-BN21K-02 | - External SCSI cable, 2 meter SCSI-2, 68-pin HD to 68 HD right angle |
| --- | --- |
| - FR-PCXAR-WK | - External SCSI cable, 2 meter SCSI-2, 68-pin HD to 50 pin HD straight |
| - FR-PCXAR-WL | - External SCSI cable, 2 meter SCSI-2, 68-pin HD to 50 pin LD |
| - BN21W-xx | - Y cable |

- Step 5d—External Storage (continued)

- Disk Drives

| - FR-CDCBA-CA | - 2.1 GB UltraSCSI SCA-2 7200 RPM disk drive, frost white |
| --- | --- |
| - FR-CECBA-CA | - 4.3 GB UltraSCSI SCA-2 7200 RPM disk drive, frost white |
| - FR-CECCA-CA | - 4.3 GB UltraSCSI SCA-2 10,000 RPM disk drive, frost white |
| - FR-CFCBA-CA | - 9.1 GB UltraSCSI SCA-2 7200 RPM disk drive, frost white |
| - FR-CGCBA-CA | - 18 GB UltraSCSI SCA-2 7200 RPM disk drive, frost white |

| - DS-RZ1BB-VW | - 2.1 GB UltraSCSI 7200 RPM hard disk drive (blue SBB) |
| --- | --- |
| - DS-RZ1CB-VW | - 4.3 GB UltraSCSI 7200 RPM hard disk drive (blue SBB) |
| - DS-RZ1DB-VW | - 9.0 GB UltraSCSI 7200 RPM hard disk drive (blue SBB) |
| - DS-RZ1CD-VW | - 4.3 GB UltraSCSI 10,000 RPM hard disk drive (blue SBB) |

- Tape Drives

| - TLZ09-VA | - 8 GB 4 mm DAT 93 MB/min (compressed) 5.25-inch SCSI tape drive in StorageWorks carrier |  |
| --- | --- | --- |
| - DS-TLZ10-VA | - 12/24 GB 4 mm DAT 3.25-inch SCSI tape drive in StorageWorks carrier |  |
| - TZK11-VA | - 2 GB QIC tape drive in StorageWorks carrier |  |
| - DS-RRD46-VA | - 600 MB 12X CD-ROM drive in StorageWorks carrier |  |
| - TLZ9L-VA | - 96 GB 4mm DAT 1.2 MB/s loader in StorageWorks carrier |  |
| - DS-TZ89N-VW | - 35/70GB 5.25" SE DLT tape drive (SBB) |  |

- Tabletop and Rackmount

| - DS-TL891-NE | - 350/700GB 10-slot library - rackmount |
| --- | --- |
| - DS-TZ89N-TA | - 35/70GB SE DLT tape - tabletop |
| - TZ887-NT | - 5.25" 140/280GB 7-cartridge DLT loader - tabletop |
| - DS-TL891-NT | - 350/700GB 10-slot library - tabletop |
| - DS-TLZ10-DB | - 12/24GB 4mm DAT- tabletop |
| - TZK11-DA | - 2GB QIC tape drive - tabletop |
| - DS-TZS20-DB | - 25/50GB AIT 8mm tape drive- tabletop |
| - TZ88N-TA | - 20/40GB DLT - tabletop |

- Step 6—Monitors

- Systems include PCI-based graphics, which use one PCI slot.

- Windows NT systems require a graphics monitor to run all functions.  Order monitor unless available on site.

- Selection of a country specific power cord is mandatory for all monitors.

- Two 17-inch monitors are available, FR-PCXAV-Yx and FR-PCXBV-Fx. FR-PCXAV-Yx has optimal resolution of 1280 x 1024 @ 75Hz, suitable for 3D graphics and high resolution environments, a detachable 3-meter video cable is provided.  FR-PCXBV-Fx has optimal resolution of 1024 x 768 @ 85Hz, suitable for general productivity and high 2D graphics environments, a 3-meter video cable is attached.

| - FR-PCXBV-EZ | - 15" (13.9" viewable image size) high resolution color monitor, Corporate Series, flat-square invar CRT with 0.28 mm dot pitch, auto-scanning VGA to 1024 x 768 @ 65 Hz, refresh rates. TCO 92, MPRII, Energy Star compliant. With attached 3-meter video cable. Select country specific power cords for -EZ (Northern Hemisphere) variants. |
| --- | --- |

| - FR-PCXAV-YZ | - 17” (16.0” viewable image size) high resolution color monitor, Professional Series, Trinitron CRT with 0.25 mm aperture grill pitch, VGA to 1280 x 1024 @ 65 Hz, TCO 92, MPRII, Energy Star compliant, HD15 mail-to-male video cable included. Select country specific power cord for -YZ (Northern Hemisphere) variants. |
| --- | --- |

- Step 6—Monitors (continued)

| - FR-PCXBV-FZ | - 17” (16.0” viewable image size) high resolution color monitor, Corporate Series, flat-square invar CRT with 0.28 mm dot pitch, auto scanning VGA to 1024 x 768 @ 65 Hz refresh rates. TCO 92, MPRII, Energy Star compliant. With attached 3-meter video cable. Select country specific power cord for –FZ (Northern Hemisphere) variants. |
| --- | --- |

| - FR-PCXAV-WZ | - 21" (19.7" viewable image size) high resolution color monitor, Professional Series, Trinitron CRT with 0.25 mm aperture grill pitch, VGA to 1280 x 1024 @ 65 Hz, TCO 92, MPRII, Energy Star compliant, HD15 mail-to-male video cable included. Select country specific power cord for -WZ (Northern Hemisphere) variants. |
| --- | --- |

- Monitor Power Cords

- Selection of a country specific power cord is mandatory for all monitors.

| - FR-PCTXH-JA | - North American, Japan, 120 V, 90-inches long |
| --- | --- |
| - BN19H-2E | - Australia, New Zealand, 2.5 meters long |
| - BN19C-2E | - Central Europe, 2.5 meters long |
| - BN19A-2E | - UK, Ireland, 2.5 meters long |
| - BN19E-2E | - Switzerland, 2.5 meters long |
| - BN19K-2E | - Denmark, 2.5 meters long |
| - BN19Z-2E | - Italy, 2.5 meters long |
| - BN18L-2E | - Israel, 2.5 meters long |

- Step 6a—Graphics

- Systems include PCI-based S3Trio64 SVGA with 1 MB DRAM graphics adapter—uses one PCI slot.  Selection

- Select graphics adapter only if a spare PCI graphics card is required.

| - FR-PCXAG-BD | - PCI-based Matrox Millennium II graphics adapter, 4 MB DRAM |
| --- | --- |
| - FR-PCCAM-CC | - 4 MB DRAM memory upgrade for FR-PCXAG-BD |
| - FR-PB2GA-JC | - PCI-based S3Trio64 SVGA with 1MB DRAM graphics adapter |

- Step 7—Networks and Communications

- Systems include PCI card with two UltraSCSI ports, one Ethernet port, and two BN37A-02 SCSI cables.

| - FR-DE500-BA | - PCI 10/100-Mbit Ethernet controller (Twisted Pair), maximum four per system |
| --- | --- |
| - FR-DE450-CA | - PCI 10-Mbit Ethernet controller; AUI, 10BaseT, or 10Base2, maximum four per system |
| - FR-DEFPA-AB | - PCI to FDDI Adapter, single attachment station (SAS), multi-mode fiber (MMF), SC connector, maximum four per system |
| - FR-DEFPA-DB | - PCI to FDDI Adapter, DAS, MMF, SC connector, maximum four per system |
| - FR-DEFPA-UB | - PCI to FDDI Adapter, SAS, TP-PMD, requires BN25H-03, maximum four per system |
| - PBXDP-AA | - PCI-based 2-port synchronous controller |
| - PBXDP-AB | - PCI-based 4-port synchronous controller |
| - PBXDP-AC | - PCI-based 8-port synchronous controller |
| - CXI01-AA | - ISA-based asynchronous multiport MUX 16 port XEM |
| - CXI01-AB | - Asynchronous multiport expander (Xem) |
| - CXI01-AC | - Asynchronous mux cable converter (RJ45-DB25) |
| - CXI01-AD | - ISA-based asynchronous multiport MUX 16 port EPC |
| - CXI01-AE | - Asynchronous multiport MUX expander (EPC) |
| - CXI01-AF | - Asynchronous MUX RJ45-MMJ11 adapter (8pk) |

- Modems listed are for U.S. and Canada only

| - FR-PCXFA-AA* | - 56 Kbps internal ISA modem |
| --- | --- |
| - FR-PCXFA-AB* | - 56 Kbps external modem |

- Step 8—Country Kits and Power Cords

Selection of country is mandatory.  Country kits include a 105-key, PC-style keyboard, three button mouse, and two 2.5 meter power cords.

| - FR-PCK8W-AA | - North American country kit | - FR-PCK8W-AS | - Spanish country kit |
| --- | --- | --- | --- |
| - FR-PCK8W-AB | - Belgium country kit | - FR-PCK8W-AT | - Hebrew (Israel) country kit |
| - FR-PCK8W-AC | - French Canadian country kit | - FR-PCK8W-AV | - Portuguese country kit |
| - FR-PCK8W-AD | - Danish country kit | - FR-PCK8W-BH | - Greek country kit |
| - FR-PCK8W-AE | - UK/Ireland country kit | - FR-PCK8W-BQ | - Hungarian country kit |
| - FR-PCK8W-AF | - Finnish (SUOMI) country kit | - FR-PCK8W-BR | - Arabic country kit |
| - FR-PCK8W-AG | - German country kit | - FR-PCK8W-BT | - Russian (Cyrillic) country kit |
| - FR-PCK8W-AI | - Italian country kit | - FR-PCK8W-BU | - Turkish country kit |
| - FR-PCK8W-AK | - Swiss country kit | - FR-PCK8W-BY | - Japanese Kangi country kit |
| - FR-PCK8W-AM | - Swedish country kit | - FR-PCK8W-CT | - Latin America Spanish country kit |
| - FR-PCK8W-AN | - Norwegian country kit | - FR-PCK8W-CZ | - Czechoslovakian country kit |
| - FR-PCK8W-AP | - French country kit | - FR-PCK8W-EC | - European country kit |
| - FR-PCK8W-AQ | - Isobilingual Canadian country kit |  |  |

- Rackmount Power Cords

- Country kits include two 2.5 meter power cords. If additional power cords are selected, all power cords ship with system.

- For Rackmount systems, selection of a 4.5 meter power cord is recommended for fully configured cabinets.

| BN20Q-4E | 15-ft (4.5meter) 240V, North American, 50 Hz power cord, connects system to power distribution unit located inside cabinet |
| --- | --- |
| BN20Z-4E | 15-ft (4.5meter) 120V, 60 Hz power cord, connects system to power distribution unit located inside cabinet |
| 2T-00365-17 | 15-foot (4.5meter) 240V, IEC 320 power cord |

Step 9—Terminals and Printers

Systems include two EIA-232 asynchronous serial ports with 9-pin D-subminiature connectors, and two 9-pin to MMJ adapters (H8571-J).

Step 10—Software

Windows NT

Selection of language-specific Windows NT Server 4.0 license/media (CD-ROM) kit is mandatory when ordering a DIGITAL Server 7300 Windows NT Package System.

Windows NT is not available from Digital for DIGITAL Server 7300 Memory Packaged Systems and System Building Blocks, order directly from Microsoft.

Windows NT Server plus 10-client access license, media kits (CD-ROM)

Select for DIGITAL Server 7300 Windows NT Packaged Systems only

| QB-23CAA-SB | Windows NT Server license, media kit North American English |
| --- | --- |
| QB-23C8A-SB | Windows NT Server license, media kit International English |
| QB-23CPA-SB | Windows NT Server license, media kit French |
| QB-23CGA-SB | Windows NT Server license, media kit German |
| QB-23CSA-SB | Windows NT Server license, media kit Spanish |
| QB-23CUA-SB | Windows NT Server license, media kit Italian |
| QB-23CJA-SB | Windows NT Server license, media kit Japanese |
| QB-23CMA-SB | Windows NT Server license, media kit Swedish |
| QB-23CHA-SB | Windows NT Server license, media kit Dutch |
| QB-23CVA-SB | Windows NT Server license, media kit Brazilian/Portuguese |
| QB-23C4A-SB | Windows NT Server license, media kit Korean |
| QB-23C3A-SB | Windows NT Server license, media kit Taiwanese |
| QB-23C2A-SB | Windows NT Server license, media kit PRC Chinese |

Step 11—Hardware and Software Warranty and Supplemental Services

Hardware—Americas and Asia Pacific only

Warranty for DIGITAL Server 7300 includes only those options installed in the CPU system drawer for a period of three-year hardware warranty, on-site with 5 x 9, 24 hour response time.

DIGITAL Server 7300 options within the Pedestal or Cabinet that are external to the CPU System Drawer are covered by their own warranty attributes, and are not covered under the warranty attributes of the CPU System Drawer.

Note:	Suppplemental Services for DIGITAL Server 7300 Pedestal Systems brings all options installed in Pedestal up to the CPU System Drawer attribute level.

Supplemental Services for DIGITAL Server 7300 Pedestal Systems with less than 1 GB Memory

| FM-7E4HR-36 | 5 x 9, 4-hour response time 36 months |
| --- | --- |
| FM-7E724-36 | 7 x 24, 4-hour response time 36 months |

Supplemental Services for DIGITAL Server 7300 Pedestal Systems with 1 to 2 GB Memory

| FM-7F4HR-36 | 5 x 9, 4-hour response time 36 months |
| --- | --- |
| FM-7E724-36 | 7 x 24, 4-hour response time 36 months |

Supplemental Services for DIGITAL Server 7300 Pedestal Systems with more than 2 GB Memory

| FM-7G4HR-36 | 5 x 9, 4-hour response time 36 months |
| --- | --- |
| FM-7G724-36 | 7 x 24, 4-hour response time 36 months |

Note:	Suppplemental Services for DIGITAL Server 7300 Pedestal Systems brings all options installed in Pedestal up to the CPU System Drawer attribute level.

Step 11—Hardware and Software Warranty and Supplemental Services (continued)

Pedestal Installation Services

| FM-7BINS-IN | CPU System Drawer installation |
| --- | --- |
| FM-7EINS-IN | Pedestal installation |
| FM-73CPU-IN | Add on CPU |
| FM-73MEM-IN | Add on Memory |

Supplemental Services for Digital 7300 CPU System Drawer with less than 1 GB Memory

| FM-7B4HR-36 | 5 x 9, 4-hour response time 36 months |
| --- | --- |
| FM-7B724-36 | 7 x 24, 4-hour response time 36 months |

Supplemental Services for Digital 7300 CPU System Drawer with 1 to 2 GB Memory

| FM-7C4HR-36 | 5 x 9, 4-hour response time 36 months |
| --- | --- |
| FM-7C724-36 | 7 x 24, 4-hour response time 36 months |

Supplemental Services for Digital 7300 CPU System Drawer with more than 2 GB Memory

| FM-7C4HR-36 | 5 x 9, 4-hour response time 36 months |
| --- | --- |
| FM-7C724-36 | 7 x 24, 4-hour response time 36 months |

Software—Americas and Asia Pacific Only

Bronze Software Support for Tier 2 (9 hours a day, 5 days a week, two hour callback)

Provides Bronze software support for Tier 2 Windows NT server and layered products as identified on Supported Products List 1.  Telephone and electronic communications provide access to DIGITAL Windows NT technical expertise 9 hours a day, 5 days a week for time period indicated.  Includes two hour callback and product support provided by specialists.

| FM-NMB52-12 | 1 year, 9 hours a day, 5 days a week, two hour callback Bronze Software Support for Tier 2 Windows NT Server and Layered Products |
| --- | --- |
| FM-NMB52-36 | 3 year, 9 hours a day, 5 days a week, two hour callback Bronze Software Support for Tier 2 Windows NT Server and Layered Products |
| FM-NMB52-60 | 5 year, 9 hours a day, 5 days a week, two hour callback Bronze Software Support for Tier 2 Windows NT Server and Layered Products |

Extension for Bronze Software Support for Tier 2 (9 hours a day, 5 days a week, two hour callback)

Extends Bronze software support for Tier 2 Windows NT Hardware platforms.  Support is extended for all supported products from the main system to each additional system.  Telephone and electronic communications provide access to Digital Windows NT technical expertise 9 hours a day, 5 days a week for time period indicated.  Main System Support is a prerequisite, see Bronze Software Support for Tier 2 above.

| FM-NDB52-12 | Extends 1 year Bronze Software Support (FM-NMB52-12) for 1 year |
| --- | --- |
| FM-NDB52-36 | Extends 3 year Bronze Software Support (FM-NMB52-36) for 3 years |
| FM-NDB52-60 | Extends 5 year Bronze Software Support (FM-NMB52-60) for 5 years |

Bronze Software Support for Tier 2 (24 hours a day, 7 days a week, two hour callback)

Provides Bronze Software Support for Tier 2 Windows NT server and layered products as identified on Supported Products List 1.  Telephone and electronic communications provide access to Digital Windows NT technical expertise 24 hours a day, 7 days a week for time period indicated.  Includes two hour callback and product support provided by specialists.

| FM-NMB72-12 | 1 year 24 hours a day, 7 days a week, two hour callback Bronze Software Support for Windows NT Server and Layered Products |
| --- | --- |
| FM-NMB72-36 | 3 year 24 hours a day, 7 days a week, two hour callback Bronze Software Support for Windows NT Server and Layered Products |
| FM-NMB72-60 | 5 year 24 hours a day, 7 days a week, two hour callback Bronze Software Support for Windows NT Server and Layered Products |

Step 11—Hardware and Software Warranty and Supplemental Services

Silver Software Support for Tier 2 (24 hours a day, 7 days a week, high priority product support)

Provides Silver Software Support for Tier 2 Windows NT server and layered products as identified on Supported Products List 1.  Telephone and electronic communications provide:  access to Digital Windows NT technical expertise 24 hours a day, 7 days a week; technical account management; higher queue priority; product support provided by specialists; one hour call back during business hours, two hours other times for time period indicated.

| FM-NMS72-12 | 1 year 24 hours a day, 7 days a week, high priority product support Silver Software Support for Windows NT Server and layered products |
| --- | --- |
| FM-NMS72-36 | 3 year 24 hours a day, 7 days a week, high priority product support Silver Software Support for Windows NT Server and layered products |
| FM-NMS72-60 | 5 year 24 hours a day, 7 days a week, high priority product support Silver Software Support for Windows NT Server and layered products |

Windows NT Incident Software Support

| FM-NTICB-12 | Provides 1 year of 10 incidents Bronze Software Support for Windows NT Server and layered products as identified on the Supported Products List 1. This allows up to 10 problems (incidents) to be submitted to Customer Support Center. An incident is considered "used" upon closure of specific support problem regardless of the number of telephone calls involved. |
| --- | --- |
| FM-NTICS-12 | Provides 1 year of 75 incidents Silver Software Support for Windows NT Server and layered products as identified on the Supported Products List 1. This allows up to 75 problems (incidents) to be submitted to Customer Support Center. An incident is considered "used" upon closure of specific support problem regardless of the number of telephone calls involved. |

Installation Service

| FM-SMWNT-IN | Provides up to two days of Windows NT start-up activities including installation and configuration of basic functionality of Windows NT Server software, configuration includes default network protocols, up to 10 existing networked clients, and 1 local printer; and a brief on-site orientation. |
| --- | --- |

Step 11a—Hardware and Software Supplemental Support Services (Europe only)

Europe does not have specific part numbers for Hardware and Software Supplemental Support Services. Prices are quoted based on worldwide service reference pricing, contact MCS Sales in your country for information on Hardware and Software Supplemental Support Services.

Pedestal System Diagram

Cabinet System Diagram

Specifications

| Specifications | Pedestal System |  | Cabinet System |  | System Drawer |  |
| --- | --- | --- | --- | --- | --- | --- |
| External Dimensions |  |  |  |  |  |  |
| Height Width Depth Weight | 75 cm (29.5 in) 49 cm (19.3 in) 90 cm (35.4 in) 113.6 kg (250 lb) |  | 170 cm (67.0 in) 60 cm (23.6 in) Rack 90 cm (35.4 in) 350.9 kg (772 lb) |  | 30 cm (11.8 in) 45 cm (17.7 in) 69 cm (27.2 in) 45.5 kg (100 lb) |  |
| Clearances | Operating/ | Service | Operating/ | Service | Operating/ | Service |
| Front Rear Sides | 20 cm (7.9 in) 20 cm (7.9 in) 20 cm (7.9 in) | 61 cm (24.1 in) 61 cm (24.1 in) 61 cm (24.1 in.) | 60 cm (23.6 in) 60 cm (23.6 in) 20 cm (7.9 in) | 100 cm (39.4 in) 100 cm (39.4 in) 61 cm (24.0 in.) | 20 cm (7.9 in) 20 cm (7.9 in) 0 | 20 cm (7.9 in) 20 cm (7.9 in) 0 |
| Environmental |  |  |  |  |  |  |
| Temperature (min/maximum) Operating1 Nonoperating Storage (60 days) Rate of Change | 10°C - 40°C (50°F - 104°F) -40°C - 66°C (-40F °- 150.8°F) -40°C - 66°C (-40F °- 150.8°F) 11°C - 19.8°C/hr (20°F -6°F/hr) |  | 10°C - 40°C (50°F - 104°F) -40°C - 66°C (-40F °- 150.8°F) -40°C - 66°C (-40F °- 150.8°F) 11°C - 19.8°C/hr (20°F - 36°F/hr) |  | 10°C - 40°C (50°F - 104°F) -40°C - 66°C (-40°F - 150.8°F) -40°C - 66°C (-40°F - 150.8°F) 11°C - 19.8°C/hr (20°F-36°F/hr) |  |
| Relative Humidity Operating Nonoperating Storage (60 days) | 20%-90% noncondensing 10%-95% 10%-95% |  | 20%-90% noncondensing 10%-95% 10%-95% |  | 20%90% noncondensing 10%-95% 10%-95% |  |

Specifications (continued)

| Environmental (continued) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7300 Max Heat Dissipation |  | 18002 W, 6143 BTU/hr |  |  |  | 4800 W, 16382 BTU/hr |  |  |  | 1100 W, 3754 BTU/hr |  |
| Airflow Quantity Intake Exhaust |  | 200 CFM Rear Front |  |  |  | 300 CFM3 Rear Front and Top |  |  |  | 200 CFM Rear Front |  |
| Altitude Operating Nonoperating |  | 3,050 m (10,000 ft) 12,200 m (40,000 ft) |  |  |  | 3,050 m (10,000 ft) 12,200 m (40,000 ft) |  |  |  | 3,050 m (10,000 ft) 12,200 m (40,000 ft) |  |
| Mechanical Shock Operating |  | 1.5 G, 10+/-3 ms |  |  |  | 5 G, 10+/-3 ms |  |  |  | 10 G, 10+/-3 ms |  |
| Vibration (min/maximum) Operating |  | 5-10 Hz 10-500 Hz |  | 0.02 in-da 0.1 G |  | 5-10 Hz 10-500 Hz |  | 0.02in-da 0.1 G |  | 5-10 Hz 10-500 Hz | 0.02in-da 0.1 G |
| Acoustics |  | 6.2 LNPEc (Bels) |  |  |  | 6.7 LNPEc (Bels) |  |  |  | 5.8 LNPEc (Bels) |  |
| Electrical (min/maximum) |  | US |  | Europe |  | US |  | Europe |  |  |  |
| Voltage Phase Nominal Frequency | 100-120 V Single 50 Hz -60 Hz |  | 200-240 V Single 50 Hz -60 Hz |  | 100-120 V Single 50 Hz -60 Hz |  | 200-240 V Single 50 Hz -60 Hz |  | 100-120/200-240 V Single 50 Hz -60 Hz |  |  |
| Maximum rated current | 12 A (multiple cord set) |  | 10 A (multiple cord set) |  | 24 + 24 A dual controllers |  | 16 + 16 A dual controllers |  | 8.4 A/4.2 A (single cord set) 8 A/4 A (multiple cord set) |  |  |
| DIGITAL Server 7300 typical power | 720 W |  | 920 W |  | Config-specific |  | Config-specific |  | 650 W |  |  |
| Power Cord Length Plug |  | 450 cm (177 in) NEMA 5-15P |  | 200-250 cm Country specific |  | 450 cm (177 in) NEMA L5-30P |  | 450 cm IEC309, 32A |  | 250 cm (98.4 in) IEC 320 |  |
| Agency Approvals |  | CAN/CSA-C22.2 No. 950-M95 UL 1950, Ed.3 TUV EN 60 950/ A3 GS IEC 950 2nd Ed. 3rd amend FCC part 15.B Class A CE |  |  |  | CAN/CSA-C22.2 No. 950-M95 UL 1950, Ed.3 TUV EN 60 950/ A3 GS IEC 950 2nd Ed. 3rd amend FCC part 15.B Class A CE |  |  |  | CAN/CSA-C22.2 No. 950-M95 UL 1950, Ed.3 TUV EN 60 950/ A3 GS IEC 950 2nd Ed. 3rd amend FCC part 15.B Class A CE |  |

1.	Maximum operating temperature at sea level. Reduce by 1C (1.8F) for every 600 m (2000 ft) above sea level.

2.	Maximum power consumption for two line cords.  Cabinet fans only.