- Diskette drive

- Fan

- PCI riser card

- Memory boards

- Dual Ethernet ports

- Keyboard and mouse ports

- CPU and fan

- Power supply

- Disks

- CD-ROM drive

At A Glance

- AlphaServer DS10 systems are ideal for ISP/Internet applications, High Performance Technical Computing, software development, and replicated site applications

- Alpha processor module with 21264 466-MHz CPU with 2-MB onboard ECC cache

- Onboard IDE controller for internal storage

- Onboard dual 10/100 BaseT Ethernet ports

- Four PCI expansion slots, three 64-bit PCI slots, one 32-bit PCI slot

- 2-GB maximum memory capacity

- 1.44-MB diskette drive

- 600-MB CD-ROM drive

- 5.25 x 1.6 removable media bay or 3.5 x 1 hard disk drive bay, two 3.5 x 1 hard disk drive bays

- Two serial ports, one serial port with full-duplex asynchronous modem control, one RMC dedicated serial port with full- duplex asynchronous modem control

- Bi-directional enhanced parallel port

- PS/2-style keyboard port and mouse port

- 300W (120/240V, 60/50 Hz) power supply

- 3-button mouse

- U.S./English keyboard with North American orders

- U.S.  power cord with North American orders

- U.S. English documentation

- Optional rackmount kit must be ordered separately

- Tru64 UNIX or OpenVMS Factory Installed Software

- 90-day software product warranty

- Protected by Compaq Services, including three-year, on-site hardware with next business day response

| Processor |  |
| --- | --- |
| Alpha 21264 466-MHz processor |  |
| Cache Memory |  |
| 2-MB onboard cache |  |
| Upgradability |  |
| Supports upgrades of memory and hard drives Models DS10 Tru64 UNIX Servers 466 MHz CPU DA-PB10A-AB (128-MB memory) DA-PB10A-AC (256-MB memory) DA-PB10A-AD (512-MB memory) DA-PB10A-AE (1-GB memory) DS10 OpenVMS Servers 466 MHz CPU DY-PB10A-AB (128-MB memory) DY-PB10A-AC (256-MB memory) DY-PB10A-AD (512-MB memory) DY-PB10A-AE (1-GB memory) |  |
| Standard | One CPU |
| Memory |  |
| Standard | 128 MB, 256 MB, 512 MB, or 1 GB (one memory kit) |
| Maximum | 2 GB (two memory kits, kits include two industry-standard DIMMs with ECC support) |
| Network Controller |  |
| Onboard dual 10/100 BaseT Ethernet ports |  |
| Expansion |  |
| Storage bays | Five (Diskette drive bay, CD-ROM drive bay, 5.25 x 1.6 removable media bay or 3.5 x 1 hard disk drive bay, two 3.5 x 1 hard disk drive bays) |
| PCI slots | Four slots (three 64-bit, one 32-bit) |
| Storage |  |
| Diskette Drive | 1.44-MB diskette drive |
| CD-ROM | 5.25 half height CD-ROM drive |
| Hard Drives | 10-GB IDE disk drive or 9.1-GB and 18.2-GB UltraSCSI disk drives available |
| Storage Controller |  |
| Onboard IDE controller for internal storage, PCI SCSI controller optional |  |
| Interfaces |  |
| Parallel | One bi-directional enhanced port |
| Serial | Two ports, one serial port with full-duplex asynchronous modem control and one RMS dedicated serial port with full-duplex asynchronous modem control |
| Keyboard | One PS/2-style port |
| Mouse | One port |
| Power Supplies |  |
| 300W (120/240V, 60/50 Hz ) |  |
| Manageability |  |
| Software features for advanced server and network management, including ServerWORKS™ Manager ServerWORKS workgroup administrator ServerWORKS Manager application SNMP agents to manage SCO UNIX, Novell NetWare®, and IBM OS/2 Intelligent Server Management |  |
| OS Support |  |
| Tru64 UNIX systems include pre-installed software, base license, unlimited user license, server extension license, and Open Source Internet Solutions. OpenVMS systems include pre-installed software, base license, and Enterprise Integration Server License Package V3.0A. |  |
| Service and Support |  |
| Protected by Compaq Services, including a three-year, on-site hardware warranty with next business day response. Software warranty is a 90-day telephone advisory. Training, consulting, network integration, software support, comprehensive system maintenance and guaranteed uptime services are also available for customers requiring higher levels of service and support. |  |

| System requirements: |  |  |
| --- | --- | --- |
| - Tru64 UNIX V4.0F or OpenVMS V7. 2-1 |  |  |
| - Mandatory selection of IDE or SCSI system disk drive |  |  |
| - SCSI adapter and SCSI cable kit required for internal SCSI disks and tape |  |  |
| - Country-specific keyboard and power cord for all non-North American orders |  |  |
| - Software media and documentation for first system on site |  |  |
| Note: For system integration of any options with 6-3 part numbers, contact CustomSystems. |  |  |
| Mandatory Options |  |  |
| Maximum three internal disk drives |  |  |
| Systems support 3.5 x 1 internal drives only |  |  |
| IDE Disk Drive |  |  |
| 10-GB 5400 rpm IDE drive | 3X-RE56A-AA |  |
| SCSI Disk Drives |  |  |
| 9.1-GB 7,200 rpm UltraSCSI drive | DS-RZ2DA-WA |  |
| 9.1-GB 10,000 rpm UltraSCSI drive | DS-RZ1DD-WA |  |
| 18-GB 7,200 rpm UltraSCSI drive | DS-RZ2EA-WA |  |
| SCSI Adapters |  |  |
| PCI 1-port UltraSCSI single-ended adapter |  | KZPBA-CA |
| PCI 2-port UltraSCSI single-ended adapter and 10/100 Mbit Ethernet combo adapter |  | KZPCM-DA |
| PCI 1-channel Wide Ultra-2 (LVD) SCSI adapter |  | SN-KZPCA-AA |
| PCI 1-port UltraSCSI RAID controller |  | KZPAC-AA |
| PCI 3-port UltraSCSI RAID controller |  | KZPAC-CA/CB |
| SCSI Cable Kit |  |  |
| SCSI cable kit (includes SCSI cable and SCSI LED cable) | 3X-BN50A-03 |  |
| Memory |  |  |
| Maximum 2-GB memory |  |  |
| Systems include one memory kit; maximum 2 memory kits per systemEach kit includes two industry-standard DIMMs with ECC support |  |  |
| 128-MB memory kit, two 200-pin, 100 MHz, 64-MB DIMMs | 3X-MS310-BA |  |
| 256-MB memory kit, two 200-pin, 100 MHz, 128-MB DIMMs | 3X-MS310-CA |  |
| 512-MB memory kit, two 200-pin, 100 MHz, 256-MB DIMMs | 3X-MS310-DA |  |
| 1 GB memory kit, two 200-pin, 100 MHz, 512-MB DIMMs | 3X-MS310-EA |  |

| Internal Storage |  |
| --- | --- |
| Systems include 600-MB 32X CD-ROM drive and 1.44-MB diskette drive in dedicated drive slots. |  |
| Additional internal storage supported: Three 1 hard disk drive bays One removable 5.25 half-height removable media bay; note that one 3.5 x 1 hard disk drive is supported in removable media bay |  |
| SCSI adapter and SCSI cable kit required for SCSI disks or tapes |  |
| See “Mandatory Options” for supported disk drives and SCSI cable kit |  |
| Removable Media Devices |  |
| TLZ10 DAT 4 mm Internal tape drive (requires SN-PBXKP-BA wide-to-narrow SCSI adapter) | TLZ10-LB |
| Wide-to-narrow SCSI adapter | SN-PBXKP-BA |
| External Storage |  |
| Tabletop DAT Tape Drives |  |
| 4 mm DAT SCSI (SDT 9000) tabletop drive | DS-TLZ10-DB |
| 20/40-GB SCSI (DLT 4000E) tabletop drive | TZ88N-TA |
| 32/64-GB tabletop DAT loader | TLZ9L-DB |
| 5, 7, 10, 14-GB 8 mm tabletop, single-ended SCSI | TKZ9F-TA |
| 32/64-GB DAT loader in carrier | TLZ9L-VA |
| Hard Disk Drives Supported in StorageWorks Shelf |  |
| 9.1-GB 7,200 rpm UltraSCSI disk drive | DS-RZ1DA-VW |
| 18.2-GB 7,200 rpm UltraSCSI disk drive | DS-RZ1EA-VW |
| 36.4-GB 7,200 rpm UltraSCSI disk drive | DS-RZ1FB-VW |
| 9.1-GB 10,000 rpm UltraSCSI disk drive | DS-RZ1DD-VW |
| 18.2-GB 10,000 rpm UltraSCSI disk drive | DS-RZ1ED-VW |
| Controllers/Adapters |  |
| PCI 1-port UltraSCSI Single-ended adapter (uses one PCI slot); includes internal 68-pin and 50-pin HD connectors and external 68-pin HD connector maximum two per system | KZPBA-CA |
| PCI 1-port UltraSCSI Differential adapter (uses one PCI slot); includes external 68-pin HD connector maximum two per system | KZPBA-CB |
| PCI 2-port UltraSCSI Single-ended adapter and 10/100 Mb Ethernet combo adapter (uses 1PCI slot)  maximum two per system | KZPCM-DA |
| External Storage (continued) |  |
| Controllers/Adapters (continued) |  |
| 68-pin HD male-to-VHDCI male UltraSCSI cable; required for each external UltraSCSI shelf attached to KZPBA-CA/CB adapter | BN38C-xx |
| PCI 1-port UltraSCSI RAID controller with 4-MB cache memory, includes RAID Array 230/Plus Subsystem software and documentation kit and VHDCI female connector; tape drives not supported, maximum two per system18.2-GB or higher disk drives not supported on KZPAC-xx RAID controllers | KZPAC-AA |
| PCI 3-port UltraSCSI RAID controller with 4-MB cache memory, includes RAID Array 230/Plus Subsystem Software and documentation kit; tape drives not supported; SCSI cable required for each active port; requires BN31K-0E for external third-port connection, maximum two per system18.2-GB or higher disk drives not supported on KZPAC-xx RAID controllers | KZPAC-CA |
| Same as KZPAC-CA with 8-MB cache memory | KZPAC-CB |
| Cache battery backup for KZPAC controller | KZPSC-UB |
| SCSI cable/bulkhead assembly kit with KZPAC-CA/CB, allows external connection of one third port | BN31K-0E |
| SCSI cable/bulkhead assembly kit with two ports for KZPAC-CA/CB, allows external connection of two third ports | KZPAC-SB |
| Graphics Option |  |
| ELSA Gloria Synergy graphics with 8-MB SGRAM 1024 x 768, 2D/3D graphics accelerator; Tru64 UNIX V4.0F 2-head support, OpenVMS V7.2-1 3-head support | SN-PBXGK-BB |
| PowerStorm 350 graphics maximum three per Tru64 UNIX systems only | SN-PBXGD-AE |

| Monitors |  |  |  |  |
| --- | --- | --- | --- | --- |
| Graphics monitors other than those listed below can be used if compatible with SVGA graphics ordered with system. |  |  |  |  |
| Selection of video extension cable and country-specific power cord is mandatory for all monitors. |  |  |  |  |
| 15 Corporate Series auto-scan color monitor, VGA to 1024 x 768 @ 85 Hz; Northern Hemisphere without power cord |  |  | 3R-VRQV5-24 |  |
| Same as above except Southern Hemisphere without power cord |  |  | 3R-VRQV5-11 |  |
| 17 (16 viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 at 75 Hz, TCO 95, MPR-II, Energy Star, attached 1.8-m video cable; Northern Hemisphere without power cord |  |  | 3R-VRQP7-24 |  |
| Same as above except Southern Hemisphere without power cord |  |  | 3R-VRQP7-23 |  |
| 21 (19.6 viewable image size) auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1600 x 1200 at 75 Hz NI, TCO 95, Energy Star, includes 1.8-m video cable; Northern Hemisphere without power cord |  | 3R-VRQP1-24 |  |  |
| Same as above except Southern Hemisphere without power cord |  | 3R-VRQP1-23 |  |  |
| Video Extension Cable |  |  |  |  |
| 1.8-m video extension cable; order one for each monitor | BN39C-02 |  |  |  |
| Monitor Power Cords |  |  |  |  |
| North America | BN26J-1K |  |  |  |
| Japan, 2.5 m, Dentori approved | 3X-BN46F-02 |  |  |  |
| Australia, New Zealand, 2.5 m | BN19H-2E |  |  |  |
| Central Europe, 2.5 m | BN19C-2E |  |  |  |
| UK, Ireland, 2.5 m | BN19A-2E |  |  |  |
| Switzerland, 2.5 m | BN19E-2E |  |  |  |
| Denmark, 2.5 m | BN19K-2E |  |  |  |
| Italy, 2.5 m | BN19M-2E |  |  |  |
| Egypt, India, South Africa, 2.5 m | BN19S-2E |  |  |  |
| Israel, 2.5 m | BN18L-2E |  |  |  |
| Keyboards |  |  |  |  |
| Systems ordered in North America include U.S./English keyboard; selection of a country specific keyboard is mandatory for all non-North American orders |  |  |  |  |
| Tru64 UNIX U.S./English keyboard | SN-LKQ47-AA |  |  |  |
| Tru64 UNIX Belgian keyboard | SN-LKQ47-AB |  |  |  |
| Tru64 UNIX Canadian/French keyboard | SN-LKQ47-AC |  |  |  |
| Tru64 UNIX Danish keyboard | SN-LKQ47-AD |  |  |  |
| Tru64 UNIX UK keyboard | SN-LKQ47-AE |  |  |  |
| Tru64 UNIX Finnish keyboard | SN-LKQ47-AF |  |  |  |
| Tru64 UNIX German keyboard | SN-LKQ47-AG |  |  |  |
| Tru64 UNIX Dutch keyboard | SN-LKQ47-AH |  |  |  |
| Tru64 UNIX Italian keyboard | SN-LKQ47-AI |  |  |  |
| Tru64 UNIX Swiss/French keyboard | SN-LKQ47-AK |  |  |  |
| Tru64 UNIX Swedish keyboard | SN-LKQ47-AM |  |  |  |
| Tru64 UNIX Norwegian keyboard | SN-LKQ47-AN |  |  |  |
| Tru64 UNIX French keyboard | SN-LKQ47-AP |  |  |  |
| Tru64 UNIX Latin-American keyboard | SN-LKQ47-AR |  |  |  |
| Tru64 UNIX Spanish keyboard | SN-LKQ47-AS |  |  |  |
| Tru64 UNIX Hebrew keyboard | SN-LKQ47-AT |  |  |  |
| Tru64 UNIX Portuguese keyboard | SN-LKQ47-AV |  |  |  |
| Tru64 UNIX BHCSY keyboard | SN-LKQ47-AX |  |  |  |
| Tru64 UNIX International keyboard | SN-LKQ47-BA |  |  |  |
| Tru64 UNIX Greek keyboard | SN-LKQ47-BH |  |  |  |
| Tru64 UNIX Taiwanese keyboard | SN-LKQ47-BI |  |  |  |
| Tru64 UNIX Korean keyboard | SN-LKQ47-BK |  |  |  |
| Tru64 UNIX Polish keyboard | SN-LKQ47-BP |  |  |  |
| Tru64 UNIX Hungarian keyboard | SN-LKQ47-BQ |  |  |  |
| Tru64 UNIX Arabic keyboard | SN-LKQ47-BR |  |  |  |
| Tru64 UNIX Cyrillic keyboard | SN-LKQ47-BT |  |  |  |
| Tru64 UNIX Turkish keyboard | SN-LKQ47-BU |  |  |  |
| Tru64 UNIX Czech keyboard | SN-LKQ47-BV |  |  |  |
| Tru64 UNIX Thai keyboard | SN-LKQ47-CB |  |  |  |
| Tru64 UNIX S. Chinese keyboard | SN-LKQ47-CV |  |  |  |
| Tru64 UNIX Slovak keyboard | SN-LKQ47-CZ |  |  |  |
| OpenVMS U.S./English keyboard | LK461-A2 |  |  |  |
| OpenVMS Belgian keyboard | LK461-AB |  |  |  |
| OpenVMS Canadian/French keyboard | LK461-AC |  |  |  |
| OpenVMS Danish keyboard | LK461-AD |  |  |  |
| OpenVMS Finnish keyboard | LK461-AF |  |  |  |
| OpenVMS German keyboard | LK461-AG |  |  |  |
| OpenVMS Dutch keyboard | LK461-AH |  |  |  |
| OpenVMS Italian keyboard | LK461-AI |  |  |  |
| OpenVMS Swiss/French keyboard | LK461-AK |  |  |  |
| OpenVMS Swiss/German keyboard | LK461-AL |  |  |  |
| OpenVMS Swedish keyboard | LK461-AM |  |  |  |
| OpenVMS Norwegian keyboard | LK461-AN |  |  |  |
| Keyboards (continued) |  |  |  |  |
| OpenVMS French keyboard | LK461-AP |  |  |  |
| OpenVMS Canadian/English keyboard | LK461-AQ |  |  |  |
| OpenVMS Spanish keyboard | LK461-AS |  |  |  |
| OpenVMS Hebrew keyboard | LK461-AT |  |  |  |
| OpenVMS Portuguese keyboard | LK461-AV |  |  |  |
| OpenVMS Greek keyboard | LK461-BH |  |  |  |
| OpenVMS Romanian keyboard | LK461-BL |  |  |  |
| OpenVMS Polish keyboard | LK461-BP |  |  |  |
| OpenVMS Hungarian keyboard | LK461-BQ |  |  |  |
| OpenVMS Cyrillic keyboard | LK461-BT |  |  |  |
| OpenVMS Turkish keyboard | LK461-BU |  |  |  |
| OpenVMS Czech keyboard | LK461-BV |  |  |  |
| OpenVMS Turkish-French keyboard | LK461-BW |  |  |  |
| OpenVMS Yugoslavian keyboard | LK461-BY |  |  |  |
| OpenVMS Slovak keyboard | LK461-CZ |  |  |  |
| Power Cords |  |  |  |  |
| Pedestal |  |  |  |  |
| Systems ordered in North America include BN26J-1K 120V North American power cord; selection of country-specific power cord is mandatory for all non- North American orders. |  |  |  |  |
| North America, 2.5 m | 3X-BN46K-2E |  |  |  |
| Japan, 2.5 m, Dentori approved | 3X-BN46G-02 |  |  |  |
| Australia, New Zealand, 2.5 m | BN19H-2E |  |  |  |
| Central Europe, 2.5 m | BN19C-2E |  |  |  |
| UK, Ireland, 2.5 m | BN19A-2E |  |  |  |
| Switzerland, 2.5 m | BN19E-2E |  |  |  |
| Denmark, 2.5 m | BN19K-2E |  |  |  |
| Italy, 2.5 m | BN19M-2E |  |  |  |
| Egypt, India, South Africa, 2.5 m | BN19S-2E |  |  |  |
| Israel, 2.5 m | BN18L-2E |  |  |  |
| Rackmount |  |  |  |  |
| North America, 4.5 m | BN20Z-4E |  |  |  |
| Non-North America, 4.5 m | 3X-BN46L-4E |  |  |  |
| Japan, Dentori approved, 4.5 m | 3X-BN46D-4E |  |  |  |
| M-Series Cabinet Enclosures and Accessories |  |  |  |  |
| M-Series Rackmount Kit |  |  |  |  |
| Rackmount kit must be ordered separately. |  |  |  |  |
| DS10 M-Series rackmount kit |  |  |  | 3X-PBX01-AB |
| M-Series Cabinet Enclosure |  |  |  |  |
| Cabinet enclosures, top gun blue, are available in three sizes: 43 cabinet enclosure (H9A11-xx) 20U or 35 of usable vertical space 67 cabinet enclosure (H9A10-xx) 31U or 54.2 of usable vertical space 79 cabinet enclosure (H9A15-xx) 38U or 66.5" of usable vertical space |  |  |  |  |

- M-Series Cabinet Configuration Chart for AlphaServer DS10

| Cabinet | Mounting Kit | Cabinet Height / Space | System | Shelves | Ballast Kit |
| --- | --- | --- | --- | --- | --- |
| H9A11-MC/MD | Optional, must be ordered separately 3X-PBX01-AB | 35 / 20U | 1 2 3 4 or 5 6 | 6 6 4 2 0 | 1 1 0 0 0 |
| H9A10-MC/MD | Optional, must be ordered separately 3X-PBX01-AB | 54.2 / 33U | 1 2 3, or 4 5 6 7 8, 9, or 10 | 6 6 6 6 4 2 0 | 2 2 1 1 0 0 0 |
| H9A15-MC/MD | Optional, must be ordered separately 3X-PBX01-AB | 66.5 / 40U | 1 2 3 4, or 5 6 7 or 8 9 or 10 11 or 12 | N/A 6 6 6 6 4 2 0 | N/A 2 2 1 0 0 0 0 |

| M-Series Cabinet Enclosures and Accessories (continued) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| M-series Cabinet Enclosure Options |  |  |  |  |  |  |
| M-series Cabinet Enclosures require the mandatory selection of the following items: |  |  |  |  |  |  |
| - H9A11, H9A10, or H9A15 Cabinet, 120V or 240V |  |  |  |  |  |  |
| - Cabinet front door based on cabinet height (optional on H9A11 cabinet) |  |  |  |  |  |  |
| - Factory Integration |  |  |  |  |  |  |
| - Ballast kits as required |  |  |  |  |  |  |
| Cabinet Enclosures |  |  |  |  |  |  |
| 120V 24A controller with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific; includes rear door, H7600-AA power distribution unit, trim kit, and front filler bezel kit |  |  |  |  | H9A11-MC |  |
| 240V 16A controller with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe; includes rear door, H7600-DB power distribution unit, trim kit, and front filler bezel kit |  |  |  |  | H9A11-MD |  |
| 120V dual-24A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific; includes rear door, top fan tray, and two H7600-AA power distribution units |  |  |  |  | H9A10-MC |  |
| 240V dual-16 A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe; includes rear door, top fan tray, and 2 H7600-DB power distribution units |  |  |  |  | H9A10-MD |  |
| 120V dual-24A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific; includes rear door, top fan tray, and two H7600-AA power distribution units |  |  |  |  | H9A15-MC |  |
| 240V dual-16A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe; includes rear door, top fan tray, and two H7600-DB power distribution units |  |  |  |  | H9A15-MD |  |
| Cabinet Front Door |  |  |  |  |  |  |
| Top gun blue front door for H9A10-MC/MD Cabinet Enclosure |  |  |  |  | H9C10-TF |  |
| Top gun blue front door for H9A15-MC/MD Cabinet Enclosure |  |  |  |  | H9C15-TF |  |
| Top gun blue front door for H9A11-MC/MD Cabinet Enclosure: selection of front door for H9A11 is optional, cabinet includes front trim kit and filler bezel |  |  |  |  | H9C11-EB |  |
| Shelves supported in M-series Cabinets with front doors (H9A10/M9A15) |  |  |  |  |  |  |
| StorageWorks UltraSCSI single bus, single-ended shelf with 180 W power supply and RETMA mounting kit |  |  |  | BA36R-RC |  |  |
| M-Series Cabinet Enclosures and Accessories (continued) |  |  |  |  |  |  |
| Shelves supported in M-series Cabinets with front doors (H9A10/M9A15) (continued) |  |  |  |  |  |  |
| StorageWorks UltraSCSI split bus, single-ended shelf with 180 W power supply and RETMA mounting kit |  |  | BA36R-RD |  |  |  |
| StorageWorks controller shelf with RETMA mounting kit and NEMA power for H9A10-MC, H9A15-MC cabinets (Americas, Asia Pacific); order BN30B-02 power cord for H9A10-MD, H9A15-MD cabinets (Europe) |  |  | BA35R-MR |  |  |  |
| 180W power supply |  |  | CK-BA35X-HH |  |  |  |
| Shelves supported in M-series Cabinets with trim kit (H9A11) |  |  |  |  |  |  |
| StorageWorks UltraSCSI single bus, single-ended shelf with 180 W power supply and RETMA mounting kit, blue bezel |  |  | BA35C-AA |  |  |  |
| StorageWorks UltraSCSI split bus, single-ended shelf with 180 W power supply and RETMA mounting kit, blue bezel |  |  | BA35C-BA |  |  |  |
| BA370 Rackmounted Enclosure/Rear Cabinet Extender |  |  |  |  |  |  |
| BA370 front-mount rackmount enclosure includes RETMA rackmount kit; first BA370 enclosure must be front mounted in cabinet, 20U height |  |  | 2T-BA370-AS |  |  |  |
| BA370 rear-mount rackmount enclosure includes RETMA rackmount kit; second BA370 enclosure must be rear mounted in cabinet, 20U height Note that Rear Cabinet Extender, H9C10-TE or H9C15-TE, is mandatory for second BA370 |  |  | 2T-BA370-AR |  |  |  |
| Rear cabinet extender, top gun blue, for H9A10-MC/MD cabinets |  |  | H9C10-TE |  |  |  |
| Rear cabinet extender, top gun blue, for H9A15-MC/MD cabinets |  |  | H9C15-TE |  |  |  |
| Factory Integration |  |  |  |  |  |  |
| Factory Integration of systems and storage devices assembled and configured in cabinet enclosure for H9A10/15 cabinet enclosures |  |  | YS-ASCAA-AA |  |  |  |
| Factory Integration of systems and storage devices assembled and configured in cabinet enclosure for H9A11 cabinet enclosures |  |  | YS-ASCAA-AB |  |  |  |
| M-Series Cabinet Enclosures and Accessories (continued) |  |  |  |  |  |  |
| Ballast Kit |  |  |  |  |  |  |
| Ballast is dead weight added to a cabinet at specific locations for specific configurations to meet safety regulations for installation, operation, and service; see Cabinet Configuration Chart for ballast kit requirements |  |  |  |  |  |  |
| Two 35-40 lb steel plates |  |  | H9C10-MC |  |  |  |
| For more information on M-series Cabinet Enclosures: www.digital.com/alphaserver/products/m-series.html |  |  |  |  |  |  |
| Rackmount System Management Station for M-series Cabinets |  |  |  |  |  |  |
| M-Series cabinets accommodate Rackmount System Management Station components. For configuration and ordering information for console switches, flat panel display, and rackmounted keyboard drawer refer to: http://www.compaq.com/products/options/index-rack.html |  |  |  |  |  |  |
| Rackmount System Management Station is currently installed onsite only (not factory installed.) Contact CustomSystems for integration of Rackmount System Management Station components into M-Series cabinets. |  |  |  |  |  |  |
| For each Rackmount System Management Station component (control switch, flat panel display, keyboard drawer), order one rear cabinet extension bracket (2T-QH915-03) |  |  |  |  |  |  |
| Compaq CustomSystems Support |  |  |  |  |  |  |
| Cabinets containing systems and storage must be assembled by qualified assemblers: Compaq Manufacturing, Compaq CustomSystems Group, or an authorized Compaq Certified Integration Partner. |  |  |  |  |  |  |
| For custom integration of systems, options, and additional Compaq and third-party devices, or for other cabinets, doors, and trim kits of a different color, contact Compaq CustomSystems at 1-800-AT-COMPAQ (1-800-282-6672), or contact your local authorized Compaq Certified Integration Partner. |  |  |  |  |  |  |
| Network Options |  |  |  |  |  |  |
| Systems include dual embedded 10/100 BaseT Ethernet ports (BN25G-xx cable must be ordered separately) |  |  |  |  |  |  |
| Additional Ethernet ports can be added using available PCI slots |  |  |  |  |  |  |
| PCI 10/100-Mb 1-port Fast Ethernet controller (Twisted Pair); maximum two per system |  |  |  |  |  | DE500-BA |
| Network Options (continued) |  |  |  |  |  |  |
| PCI 10/100-Mb 4-Port Fast Ethernet controller (Twisted Pair); maximum two per system |  |  | DE504-BA |  |  |  |
| Cable for 10BaseT Twisted Pair connection |  |  | BN25G-xx |  |  |  |
| PCI 10/100-Mb Fast Ethernet controller (MMF) |  |  | DE500-FA |  |  |  |
| PCI 1-port 10/100-Mb Ethernet adapter (Twisted Pair): maximum two per Tru64 UNIX systems |  |  | 3X-DE600-AA |  |  |  |
| PCI 2-port 10/100-Mb Ethernet adapter (Twisted Pair): maximum one per Tru64 UNIX systems |  |  | 3X-DE602-AA |  |  |  |
| SC-SC Dual fibre optic cable, mm, pp xx = length in meters |  |  | BN34B-xx |  |  |  |
| PCI-to-GbE SX adapter; maximum two per system |  |  | DEGPA-SA |  |  |  |
| PCI 4-port asynchronous controller with cable; maximum two PBXDA-xx per system |  |  | PBXDA-AA |  |  |  |
| PCI 8-port asynchronous controller with cable; maximum two PBXDA-xx per system |  |  | PBXDA-AB |  |  |  |
| PCI 16-port asynchronous controller with cable; maximum two PBXDA-xx per system |  |  | PBXDA-AC |  |  |  |
| Software |  |  |  |  |  |  |
| Media and documentation required for first system on site. |  |  |  |  |  |  |
| Software processor code = E |  |  |  |  |  |  |
| Tru64 UNIX |  |  |  |  |  |  |
| Tru64 UNIX systems include Tru64 UNIX Unlimited User license, Server Extension license, Open Source Internet Solutions |  |  |  |  |  |  |
| Tru64 UNIX Media & Documentation kit |  | QA-MT4AA-H8 |  |  |  |  |
| Tru64 UNIX Full Documentation kit |  | QA-MT4AA-GZ |  |  |  |  |
| TruCluster Available Server License |  | QL-05SAE-AA |  |  |  |  |
| Layered products media and documentation for Tru64 UNIX on CD-ROM |  | QA-054AA-H8 |  |  |  |  |
| DECnet/OSI end-system license for Tru64 UNIX |  | QL-MTJAE-AA |  |  |  |  |
| DECnet/OSI extended function license for Tru64 UNIX |  | QL-MTKAE-AA |  |  |  |  |
| Open3D for Tru64 UNIX on CD-ROM, includes graphics drivers |  | QA-0AFAA-H8 |  |  |  |  |
| OpenVMS |  |  |  |  |  |  |
| OpenVMS systems include OpenVMS base license with system manager license and Compaq Enterprise Integration Server License Package for OpenVMS Revision V3.0A. |  |  |  |  |  |  |
| Software (continued) |  |  |  |  |  |  |
| OpenVMS (continued) |  |  |  |  |  |  |
| Compaq Enterprise Integration Package includes licenses for TCP/IP Services for OpenVMS, DECwindows Motif for OpenVMS Alpha, DECprint Supervisor for OpenVMS Alpha Plus, DECprint Supervisor for OpenVMS Open, DECnet-Plus for OpenVMS Alpha End System, Archive/Backup System for OpenVMS Management Tools, Archive/Backup Agent for Windows NT, OpenVMS Disk Services for Windows NT, Office Server for OpenVMS, PATHWORKS 32, PATHWORKS V6 for OpenVMS – Advanced Server, and DIGITAL Office Server Client Access License |  |  |  |  |  |  |
| OpenVMS V7.1-2 media and online documentation on CD-ROM | QA-MT1AR-H8 |  |  |  |  |  |
| OpenVMS V7.2-1 media and online documentation on CD-ROM | QA-MT1AT-H8 |  |  |  |  |  |
| OpenVMS media and online documentation on CD-ROM | QA-MT1AA-H8 |  |  |  |  |  |
| OpenVMS hard copy documentation | QA-001AA-GZ |  |  |  |  |  |
| Layered products media and documentation for OpenVMS on CD-ROM | QA-03XAA-H8 |  |  |  |  |  |
| Open3D for OpenVMS on CD-ROM, includes graphics drivers | QA-0ADAA-H8 |  |  |  |  |  |
| Concurrent Use 1-user license | QL-MT3AA-3B |  |  |  |  |  |
| Concurrent Use 2-user license | QL-MT3AA-3C |  |  |  |  |  |
| Concurrent Use 4-user license | QL-MT3AA-3D |  |  |  |  |  |
| Concurrent Use 8-user license | QL-MT3AA-3E |  |  |  |  |  |
| Concurrent Use 16-user license | QL-MT3AA-3F |  |  |  |  |  |
| Concurrent Use 32-user license | QL-MT3AA-3G |  |  |  |  |  |
| Concurrent Use 64-user license | QL-MT3AA-3H |  |  |  |  |  |
| Concurrent Use 128-user license | QL-MT3AA-3J |  |  |  |  |  |
| Concurrent Use 256-user license | QL-MT3AA-3K |  |  |  |  |  |
| Traditional Unlimited User license | QL-MT2AE-AA |  |  |  |  |  |
| Hardware and Software Supplemental Services |  |  |  |  |  |  |
| Hardware Supplemental Services |  |  |  |  |  |  |
| Systems include three-year, onsite hardware warranty with next business day response. |  |  |  |  |  |  |
| Select optional Hardware Supplemental Services if required |  |  |  |  |  |  |
| 3-year, 5 x 9, 4-hour response time | FM-MK4HR-36 |  |  |  |  |  |
| 5-year, 5 x 9, 4-hour response time | FM-MK4HR-60 |  |  |  |  |  |
| 3-year, 5 x 12, 4-hour response time | FM-MK512-36 |  |  |  |  |  |
| 5-year, 5 x 12, 4-hour response time | FM-MK512-60 |  |  |  |  |  |
| 3-year, 6 x 16, 4-hour response time | FM-MK616-36 |  |  |  |  |  |
| 5-year, 6 x 16, 4-hour response time | FM-MK616-60 |  |  |  |  |  |
| 3-year, 7 x 24, 4-hour response time | FM-MK724-36 |  |  |  |  |  |
| 5-year, 7 x 24, 4-hour response time | FM-MK724-60 |  |  |  |  |  |
| Years 4-5, 5 x 9, next day response time | FM-MKXHW-60 |  |  |  |  |  |
| Hardware and Software Supplemental Services (continued) |  |  |  |  |  |  |
| Software Supplemental Services |  |  |  |  |  |  |
| 1-year, 5 x 9 Bronze Software Supplemental Support, Tru64 UNIX | FM-D10U9-12 |  |  |  |  |  |
| 3-year, 5 x 9 Bronze Software Supplemental Support, Tru64 UNIX | FM-D10U9-36 |  |  |  |  |  |
| 5-year, 5 x 9 Bronze Software Supplemental Support, Tru54 UNIX | FM-D10U9-60 |  |  |  |  |  |
| 1-year, 7 x 24 Bronze Software Supplemental Support, Tru64 UNIX | FM-D10US-12 |  |  |  |  |  |
| 3-year, 7 x 24 Bronze Software Supplemental Support, Tru64 UNIX | FM-D10US-36 |  |  |  |  |  |
| 5-year, 7 x 24 Bronze Software Supplemental Support, Tru64 UNIX | FM-D10US-60 |  |  |  |  |  |
| 1-year, Bronze Node Software Supplemental Support, Tru64 UNIX | FM-D10UN-12 |  |  |  |  |  |
| 3-year, Bronze Node Software Supplemental Support, Tru64 UNIX | FM-D10UN-36 |  |  |  |  |  |
| 5-year, Bronze Node Software Supplemental Support, Tru64 UNIX | FM-D10UN-60 |  |  |  |  |  |
| 1-year, 5 x 9Bronze Software Supplemental Support, OpenVMS | FM-D10V9-12 |  |  |  |  |  |
| 3-year, 5 x 9Bronze Software Supplemental Support, OpenVMS | FM-D10V9-36 |  |  |  |  |  |
| 5-year, 5 x 9Bronze Software Supplemental Support, OpenVMS | FM-D10V9-60 |  |  |  |  |  |
| 1-year, 7 x 24 Bronze Software Supplemental Support, OpenVMS | FM-D10VS-12 |  |  |  |  |  |
| 3-year, 7 x 24 Bronze Software Supplemental Support, OpenVMS | FM-D10VS-36 |  |  |  |  |  |
| 5-year, 7 x 24 Bronze Software Supplemental Support, OpenVMS | FM-D10VS-60 |  |  |  |  |  |
| 1-year, Bronze Node Software Supplemental Support, OpenVMS | FM-D10VN-12 |  |  |  |  |  |
| 3-year, Bronze Node Software Supplemental Support, OpenVMS | FM-D10VN-36 |  |  |  |  |  |
| 5-year, Bronze Node Software Supplemental Support, OpenVMS | FM-D10VN-60 |  |  |  |  |  |

| System Unit |  | Physical Characteristics |  |  |
| --- | --- | --- | --- | --- |
|  |  | Dimensions (HxWxD) Weight (maximum) | 5.1 x 17.6 x 19 in/13 x 45 x 48 cm 36 lbs/16.3 kg |  |
|  |  | Environmental | Operating | Non-Operating |
|  |  | Temperature1 Humidity (non-condensing) Altitude (unpressurized) | 50 to 104F/10 to 40C 20% to 80% 10,000 ft / 3,048 m2 | –40 to 151F/–40 to 66C 10% to 95% 40,000 ft/12,192 m |
|  |  | Power Requirements |  |  |
|  |  | Nominal voltage range | 100 to 120 Vac/220 Vac |  |
|  |  | Operating voltage range | 90 to 128 Vac/180 to 265 Vac |  |
|  |  | Power source phase | Single/Single |  |
|  |  | Nominal frequency | 60 Hz/50 Hz |  |
|  |  | Frequency range | 59 to 61 Hz/49-51 Hz |  |
|  |  | Maximum inrush current | 11 amps/22 amps |  |
|  |  | Single power supply (single cordset) | 12 amps/5 amps 7.4 amps each supply/3.7 amps each supply |  |

- Maximum operating temperature at sea level. Reduce by 1.8F (1C) for each 2,000 ft (660 m) above sea level.

- Higher altitudes are possible if maximum operating temperature is reduced.