Revision 4.0

March  13, 2000

- Fan

- CPU and fan

- Power supply

- Memory boards

- Diskette/CD-ROM drive

- Fans

- Disk

- PCI riser card

- Dual Ethernet ports

- Keyboard and mouse ports

At A Glance

- AlphaServer DS10L Systems include:

- 1U Alpha processor module with 21264 466-MHz CPU with 2-MB onboard ECC cache

- Onboard IDE controller for internal storage

- Onboard dual 10/100 BaseT Ethernet ports

- One full-length PCI expansion slot

- 1-GB maximum memory capacity

- Optional combo 1.44-MB diskette drive/CD-ROM drive, or 3.5 x 1 hard disk drive bay

- Optional rackmount kit for single or quad configurations

- Two serial ports, one with full-duplex asynchronous modem control, one RMC dedicated serial port with full-duplex asynchronous modem control

- Bi-directional enhanced parallel port

- Dual USB ports (future support)

- PS/2-style keyboard port and mouse port

- 150W (120/240V, 60/50 Hz) power supply

- Integral remote system management console

- U.S. English documentation

- Tru64 UNIX or OpenVMS Factory Installed Software with 90-day software product warranty

- Protected by Compaq Services, including one-year, on-site hardware with next business day response

| Processor |  |
| --- | --- |
| Alpha 21264 processor 466 MHz |  |
| Cache Memory |  |
| 2-MB onboard cache |  |
| Upgradability |  |
| Supports upgrades of memory and hard drives |  |
| Standard | One CPU |
| Memory (Two DIMM Slots) |  |
| Standard | 128 MB, 256 MB, 512 MB, or 1 GB (one memory kit) |
| Maximum | 1 GB (One memory kit) |
| Network Controller |  |
| Onboard dual 10/100 BaseT Ethernet ports |  |
| Expansion |  |
| PCI slots | One full length slot |
| Storage Controller |  |
| Onboard IDE controller for internal storage, PCI SCSI controller optional |  |
| Storage |  |
| Diskette Drive/CD-ROM | Optional |
| Hard Drives | 10-GB IDE disk drive or 9.1-GB and 18.2-GB UltraSCSI disk drives available |
| Expansion Ports |  |
| Parallel | One bi-directional enhanced port |
| Serial | One serial port: Selectable as either one full duplex asynchronous modem control serial port or one RMC dedicated asynchronous modem control serial port |
| Pointing Device (Mouse) | One port |
| Keyboard | One PS/2-style port |
| USB | Two (future support) |
| Form Factor |  |
| 1 U Rackmount or Desktop (1.75” h x 17.5” w x 20.5” d), 20 lbs. |  |
| Manageability |  |
| Compaq Insight Manager |  |
| Power Supply |  |
| 150-Watt Power Supply |  |
| OS Support |  |
| AlphaServer Tru64 UNIX systems include pre-installed software, base license, unlimited user license, server extension license, Open Source Internet Solutions, and Netscape Enterprise Server 3.0 AlphaServer OpenVMS systems include pre-installed software, base license, TCP/IP Services for OpenVMS, DECWindows Motif for OpenVMS Alpha, DECnet Plus for OpenVMS End System, and Pathworks 32 Linux-Ready systems ship without system software. Red Hat 6.0 or SuSe 6.1 Linux may be purchased separately from a Linux distributor. Minimum Compaq OS support: Tru64 UNIX V4.0F, OpenVMS V7.1-2 or V.2-1. |  |
| Service and Support |  |
| Compaq Services provides a wide range of service offerings including a one-year limited warranty fully supported by a worldwide network of resellers and service providers; lifetime toll-free 7 x 24 hardware technical phone support. |  |

| Step 1  Select system |  |  |  |  |
| --- | --- | --- | --- | --- |
| AlphaServer DS10L systems require the mandatory selection of the following items: - System disk drive - If SCSI disk is selected, adapter and cable kit are required - If IDE disk is selected, cable kit is required - Power cord if used as desktop system - Rackmount systems require rackmount kit and cable management kit - Software media and documentation for first system onsite Notes: AlphaServer DS10L systems do not include keyboard, mouse, power cords, rackmount kits, CD-ROM, or floppy drives; order as required. Rackmount system minimum for factory integration is four AlphaServer DS10L minimum OS support  Tru64 UNIX V4.0F, OpenVMS V7.2-1 For system integration of options with 6-3 part numbers, contact CustomSystems (1-800-AT-COMPAQ) |  |  |  |  |
| OS | CPU | Memory | Order No. |  |
| Tru64 UNIX | 466 MHz | 128 MB | DA-71AAA-CA |  |
| Tru64 UNIX | 466 MHz | 256 MB | DA-71AAA-DA |  |
| Tru64 UNIX | 466 MHz | 512 MB | DA-71AAA-EA |  |
| Tru64 UNIX | 466 MHz | 1 GB | DA-71AAA-FA |  |
| OpenVMS | 466 MHz | 128 MB | DY-71AAA-CA |  |
| OpenVMS | 466 MHz | 256 MB | DY-71AAA-DA |  |
| OpenVMS | 466 MHz | 512 MB | DY-71AAA-EA |  |
| OpenVMS | 466 MHz | 1 GB | DY-71AAA-FA |  |
| Linux-Ready | 466 MHz | 128 MB | DJ-71AAA-CA |  |
| Linux-Ready | 466 MHz | 256 MB | DJ-71AAA-DA |  |
| Linux-Ready | 466 MHz | 512 MB | DJ-71AAA-EA |  |
| Linux-Ready | 466 MHz | 1 GB | DJ-71AAA-FA |  |
| Step 2  Select mandatory options |  |  |  |  |
| Step 2a  System Disk Drive  Mandatory |  |  |  |  |
| - Maximum two internal disk drives. The second drive is installed in the media bay. Select either a CD-ROM/floppy drive or second internal disk – not both – or leave the media bay empty. |  |  |  |  |
| - Internal support for 3.5 x 1 drives only |  |  |  |  |
| - For multiple system configurations that share a system disk external to system enclosure, contact Compaq CustomSystems. |  |  |  |  |
| IDE Disk Drive |  |  |  |  |
| 10-GB 5400 rpm IDE drive |  |  |  | 3X-RE56A-AA |
| 30-GB 7200 rpm IDE drive |  |  |  | 3X-RE57A-BA |
| System Disk Cable Configuration Rules |  |  |  |  |
| 2 IDE hard disks 2 3X-BN50A-04  One drop IDE cable |  |  |  |  |
| 1 IDE hard disk, 1 SCSI hard disk 1 3X-BN50A-05  One drop SCSI cable, LED activity cable, and 68-50-pin adapter 1 3X-BN50A-04  One drop IDE cable |  |  |  |  |
| 1 IDE hard disk, Floppy/CD combo 1 3X-PBXRY-AA  Floppy/CD combo with one drop IDE cable and floppy cable 1 3X-BN50A-04  17-04803-02 one drop IDE cable |  |  |  |  |

| 1 SCSI hard disk, Floppy/CD combo 1 3X-BN50A-05  One drop SCSI cable, ED activity cable, and 68-50-pin adapter 1 3X-PBXRY-AA  Floppy/CD combo with one drop IDE cable and floppy cable |  |  |  |
| --- | --- | --- | --- |
| 2 SCSI hard disks 1 3X-BN50A-02  Two drop SCSI cable, LED activity cable |  |  |  |
| SCSI Disk Drives |  |  |  |
| 9.1-GB 7200 rpm Ultra2 SCSI drive | DS-RZ2DA-WA |  |  |
| 9.1-GB 10000 rpm Ultra2 SCSI drive | DS-RZ1DD-WA |  |  |
| 18.2-GB 7200 rpm Ultra2 SCSI drive | DS-RZ2EA-WA |  |  |
| 18.2-GB 10000 rpm Ultra2 SCSI drive | DS-RZ2EF-WA |  |  |
| Step 2b  SCSI Adapter  (SCSI disks require SCSI adapter) |  |  |  |
| - SCSI adapter and SCSI cable kit required for internal SCSI disks and tape |  |  |  |
| PCI 1-port UltraSCSI single-ended adapter |  |  | KZPBA-CA |
| PCI 1-port UltraSCSI differential adapter |  |  | KZPBA-CB |
| PCI 2-port UltraSCSI single-ended adapter and 10/100 Mbit Ethernet combo adapter (Tru64 UNIX and OpenVMS only) |  |  | KZPCM-DA |
| PCI 1-channel Wide Ultra-2 (LVD) SCSI adapter |  |  | SN-KZPCA-AA |
| PCI 1-port UltraSCSI RAID controller (Tru64 UNIX and OpenVMS only) |  |  | KZPAC-AA |
| SCSI Cable Kit (Refer to “System Disk Cable Configuration Rules”) |  |  |  |
| Step 2c  Power Cords -- Mandatory for desktop systems |  |  |  |
| - For rackmount systems refer to Step 5, PDU/Fan/Cable Management Kits |  |  |  |
| Desktop Power Cords |  |  |  |
| North American 120V, 75 |  |  | BN26J-1K |
| Australia, New Zealand, 2.5m |  |  | BN19H-2E |
| Central Europe, 2.5m |  |  | BN19C-2E |
| UK, Ireland, 2.5m |  |  | BN19A-2E |
| Switzerland, 2.5m |  |  | BN19E-2E |
| Denmark, 2.5m |  |  | BN19K-2E |
| Italy, 2.5m |  |  | BN19M-2E |
| Egypt, India, South Africa, 2.5m |  |  | BN19S-2E |
| Israel, 2.5m |  |  | BN18L-2E |
| Japan, 2.5m |  |  | 3X-BN46F-02 |
| Step 3  Memory  Optional |  |  |  |
| - Maximum 1-GB memory |  |  |  |
| - Systems include one memory kit; maximum one memory kit per system  each kit includes two industry-standard DIMMs with ECC support |  |  |  |
| - To increase memory, order new memory and replace resident memory with the new DIMMS |  |  |  |
| 128-MB memory kit, two 200-pin, 100 MHz, 64-MB DIMMs |  | 3X-MS310-BA |  |
| 256-MB memory kit, two 200-pin, 100 MHz, 128-MB DIMMs |  | 3X-MS310-CA |  |
| 512-MB memory kit, two 200-pin, 100 MHz, 256-MB DIMMs |  | 3X-MS310-DA |  |
| 1-GB memory kit, two 200-pin, 100 MHz, 512-MB DIMMs |  | 3X-MS310-EA |  |

| Step 4  Internal Storage  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - System includes one bay in system enclosure; optional bay may be used as a media bay or drive bay: - One 1 hard disk drive bay (refer to Step 2a) or - One 3.5 x 1 hard disk drive supported in removable media bay |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - SCSI adapter and SCSI cable kit required for SCSI disks or tapes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.44-MB diskette drive/600-MB 24X CD-ROM combo |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-PBXRY-AA |  |  |  |  |  |
| 20/40-GB DAT tape drive, top gun blue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-SD20X-LB |  |  |  |  |  |
| 35/70-GB AIT tape drive, top gun blue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-SZ35X-LB |  |  |  |  |  |
| 50/100-GB AIT tape drive, top gun blue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-SZ50X-LB |  |  |  |  |  |
| Note: Refer to “System Disk Cable Configuration Rules” in Step 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step 5  M-series Cabinet Enclosures  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - M-Series cabinet enclosures support up to 40 DS10L systems and/or up to three StorageWorks shelves. M-series enclosures require selection of the following items: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - H9A11, H9A10, or H9A15 Cabinet, 120V or 240V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Minimum four AlphaServer DS10L systems for rack configuration; minimum five systems for H9A15 cabinet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Rack kit for each system (or quad rack for every four systems) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - PDU/Fan/Cable management kit according to the configuration chart |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Cabinet front door based on cabinet height (optional for H9A11 cabinet) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Factory integration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Ballast kit according to the configuration chart |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Power Cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Up to six BA356 shelves supported; back-to-back mounting not allowed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AlphaServer DS10L Rackmount Configuration Rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Minimum number of systems orderable in a rack is four for H9A11 or H9A10 cabinets; minimum of five for H9A15 cabinet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Power cords are only available in cable management kits as follows: The first eight systems in an H9A11 cabinet or the first 16 systems in an H9A10 or H9A15cabinet require a fan/cable management kit. For every 1-8 additional systems, a PDU/fan/cable management kit is required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: For more information on M-Series Cabinet Enclosures, see www.digital.com/alphaserver/products/m-series/index.html |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cabinet | Rack Kits 3X-PBX01-BC quad |  | Rack kit 3X-PBX01-BB single |  |  |  | Systems 1U |  | Shelves @ 4U |  |  | Fan Kit 3X-H7210-BA/BB/BC |  |  |  |  |  | PDU Kit 3X-H7210-AA/AB/AC |  |  |  |  |  |  |  |  |  | Ballast H9C10-MC |
| H9A11, H9A10, H9A15 | 1 for each set of 4 systems |  | 1 for each additional system |  |  |  | Per rack |  | Per rack |  |  | Used with existing PDU |  |  |  |  |  | Used when PDU is required |  |  |  |  |  |  |  |  |  |  |
| H9A11-MC/MD | 1 or 2 |  | 0 to 3 |  |  |  | 4-8 |  | 3 |  |  | 1 |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  | 0 |
|  | 2 or 3 |  | 0 to 3 |  |  |  | 9-12 |  | 2 |  |  | 1 |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 0 |
|  | 3 or 4 |  | 0 to 3 |  |  |  | 13-16 |  | 1 |  |  | 1 |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 0 |
|  | 4 or 5 |  | 0 to 3 |  |  |  | 17-20 |  | 0 |  |  | 1 |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 0 |
| H9A10-MD/MC | 1 or 2 |  | 0 to 3 |  |  |  | 4-8 |  | 6 |  |  | 1 |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  | 21 |
|  | 2 or 3 |  | 0 to 3 |  |  |  | 9-12 |  | 5 |  |  | 2 |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  | 0 |
|  | 3 or 4 |  | 0 to 3 |  |  |  | 13-16 |  | 4 |  |  | 2 |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  | 0 |
|  | 4 or 5 |  | 0 to 3 |  |  |  | 17-20 |  | 3 |  |  | 2 |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 0 |
|  | 5 or 6 |  | 0 to 3 |  |  |  | 21-24 |  | 2 |  |  | 2 |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 0 |
|  | 6 or 7 |  | 0 to 3 |  |  |  | 24-28 |  | 1 |  |  | 2 |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 0 |
|  | 7 or 8 |  | 0 to 3 |  |  |  | 29-32 |  | 0 |  |  | 2 |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 0 |
| H9A15-MC/MD | 1 to 2 |  | 0 to 3 |  |  |  | 5-8 |  | 6 |  |  | 1 |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  | 2 |
|  | 2 or 3 |  | 0 to 3 |  |  |  | 9-12 |  | 5 |  |  | 2 |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  | 1 |
|  | 3 or 4 |  | 0 to 3 |  |  |  | 13-16 |  | 4 |  |  | 2 |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  | 0 |
|  | 4 or 5 |  | 0 to 3 |  |  |  | 17-20 |  | 3 |  |  | 2 |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 0 |
|  | 5 or 6 |  | 0 to 3 |  |  |  | 21-24 |  | 3 |  |  | 2 |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 0 |
|  | 6 or 7 |  | 0 to 3 |  |  |  | 25-28 |  | 3 |  |  | 2 |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 0 |
|  | 7 or 8 |  | 0 to 3 |  |  |  | 29-32 |  | 2 |  |  | 2 |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 0 |
|  | 8 or 9 |  | 0 to 3 |  |  |  | 33-36 |  | 1 |  |  | 2 |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  | 0 |
|  | 9 or 10 |  | 0 to 3 |  |  |  | 37-40 |  | 0 |  |  | 2 |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  | 0 |
| - With 4, 5, and 6 systems, two ballast kits are required, with 7 and 8 systems, one ballast kit is required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cabinet Configuration Chart for AlphaServer DS10L with StorageWorks RA3000 and Disk Shelves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 12U of vertical cabinet space is reserved for one RA3000 controller shelf and up to four disk shelves mounted in top of cabinet, front to back; an additional 2U of vertical space is required for RA3000 controller shelf UPS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - RAID Array 3000 and UPS each require a separate PDU (H7600-AA/DB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Disk shelves can be a combination of RA3000 DS-SWXRA-GN disk shelves and StorageWorks BA36R-RC/RD disk shelves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - RA3000 controller shelf supports a maximum of four DS-SWXRA-GN disk shelves; third and fourth DS-SWXRA-GN disk shelf ordered will be installed in rear of M-series cabinet; rear mounted shelves require that a 2-m SCSI cable (BN37A-02) be ordered separately. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Systems are supported in remaining vertical space. The table below considers space requirements for the RA3000, systems, PDU kits, and UPS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 1.75 or 1U space is required between each system unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cabinet |  | Mounting Kit |  |  |  | Cabinet Height / Space |  |  |  | Systems |  |  |  |  |  |  |  |  | RA3000 + Shelves |  |  |  |  |  |  |  |  |  |
| H9A11-MX |  | 3X-PBX01-BB |  |  |  | 35" / 20U |  |  |  | 4-6 |  |  |  |  |  |  |  |  | 1 + 2 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 7-8 |  |  |  |  |  |  |  |  | 1 + 1 (basic) |  |  |  |  |  |  |  |  |  |
| H9A10-MC/MD |  | 3X-PBX01-BB |  |  |  | 54.2 / 32U |  |  |  | 4-8 |  |  |  |  |  |  |  |  | 1 + 4 (max) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 9-14 |  |  |  |  |  |  |  |  | 1 + 2 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 15-23 |  |  |  |  |  |  |  |  | 1 + 1 (basic) |  |  |  |  |  |  |  |  |  |
| H9A15-MC/MD |  | 3X-PBX01-BB |  |  |  | 66.5 / 40U |  |  |  | 5-16 |  |  |  |  |  |  |  |  | 1 + 4 (max) |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 17-22 |  |  |  |  |  |  |  |  | 1 + 3 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 23-24 |  |  |  |  |  |  |  |  | 1 + 2 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 25-30 |  |  |  |  |  |  |  |  | 1 + 1 (basic) |  |  |  |  |  |  |  |  |  |
| Cabinet Configuration Chart for AlphaServer DS10Lwith StorageWorks BA370 and Disk Shelves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 20U of vertical cabinet space is reserved for one front-mounted BA370 rackmount. Note that top fan (1U) is removed from cabinet when BA370 rackmount enclosure is installed. Additional PDU is required to support the BA370. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Systems and StorageWorks shelves are supported in remaining vertical space; StorageWorks shelves are not supported in rear of cabinet behind a front-mounted BA370 rackmount enclosure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 1.75 or 1U space is required between each system unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cabinet | Rack Kit |  |  |  | Cabinet Height/Space |  |  | Systems |  |  |  |  |  | BA370 |  |  |  |  |  |  |  |  | Shelves |  |  |  |  |  |
| H9A10-MC/MD | 3X-PBX01-BB |  |  |  | 56” / 32U |  |  | 13 |  |  |  |  |  | 1 (front) |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |
| H9A15-MC/MD | 3X-PBX01-BB |  |  |  | 66.5” / 40U |  |  | 20 |  |  |  |  |  | 1 (front) |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |
| Note: Installing one BA370 and one system in H9A15 cabinet requires the mandatory selection of a ballast kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| H9A11 Cabinet |  |  |  | H9A10 Cabinet |  |  |  |  |  |  |  |  | H9A15 Cabinet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M-series Cabinets |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 43" cabinet enclosure with 35" or 20U of usable vertical space; 120V 24A controller with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific; includes rear door, H7600-AA power distribution unit, trim kit, and front filler bezel kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9A11-MC |  |
| 43" cabinet enclosure with 35" or 20U of usable vertical space; 240V 16A controller with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe; includes rear door, H7600-DB power distribution unit, trim kit, and front filler bezel kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9A11-MD |  |
| 67" cabinet enclosure with 54.2" or 33U of usable vertical space; 120V dual-24A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific; includes rear door, top fan tray, and two H7600-AA power distribution units |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9A10-MC |  |
| 67" cabinet enclosure with 54.2" or 33U of usable vertical space; 240V dual-16 A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe; includes rear door, top fan tray, and two H7600-DB power distribution units |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9A10-MD |  |
| 79" cabinet enclosure with 66.5" or 40U of usable vertical space; 120V dual-24A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific; includes rear door, top fan tray, and two H7600-AA power distribution units |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9A15-MC |  |
| 79" cabinet enclosure with 66.5" or 40U of usable vertical space; 240V dual-16A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe; includes rear door, top fan tray, and two H7600-DB power distribution units |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9A15-MD |  |
| Cabinet Rackmount Kits – Refer to table above |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Single rack kit for AlphaServer DS10L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-PBX01-BB |  |  |  |
| Quad rack kit for AlphaServer DS10L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-PBX01-BC |  |  |  |
| PDU/Fan/Cable Management Kits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Kits include PDU, fans, shelf, and cables to serve eight AlphaServer DS10L systems. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Power Distribution Unit/Fan/Cable Management Kit (North America) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-H7210-AA |  |  |  |
| Power Distribution Unit/Fan/Cable Management Kit (Europe) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-H7210-AB |  |  |  |
| Power Distribution Unit/Fan/Cable Management Kit (Japan) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-H7210-AC |  |  |  |
| Fan/Cable Management Kits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Kits include fans, shelf, and cables to serve eight AlphaServer DS10L systems; for use with PDU that ships with cabinet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fan/Cable Management Kit (North America) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-H7210-BA |  |  |  |
| Fan/Cable Management Kit (Europe) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-H7210-BB |  |  |  |
| Fan/Cable Management Kit (Japan) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-H7210-BC |  |  |  |
| Shelves supported in M-series Cabinets with front doors (H9A10, H9A15) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| StorageWorks UltraSCSI single bus, single-ended shelf with 180W power supply and RETMA mounting kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BA36R-RC |  |  |  |
| StorageWorks UltraSCSI split bus, single-ended shelf with 180W power supply and RETMA mounting kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BA36R-RD |  |  |  |
| 180W power supply for redundant power in a BA36R (includes power cord) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CK-BA35X-HH |  |  |  |
| Shelves supported in M-series Cabinets with trim kit (H9A11) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| StorageWorks UltraSCSI single bus, single-ended shelf with 180W power supply and RETMA mounting kit, blue bezel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BA35C-AA |  |  |  |
| StorageWorks UltraSCSI split bus, single-ended shelf with 180W power supply and RETMA mounting kit, blue bezel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BA35C-BA |  |  |  |
| Cabinet Front Door  Required selection of door based on cabinet height |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Front door, top gun blue, for H9A11-MC/MD cabinet enclosure: selection of front door for H9A11 is optional, cabinet includes front trim kit and filler bezel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9C11-EB |  |  |  |
| Front door, top gun blue, for H9A10-MC/MD cabinet enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9C10-TF |  |  |  |
| Front door, top gun blue, for H9A15-MC/MD cabinet enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9C15-TF |  |  |  |
| Factory Integration  Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Factory Integration of systems and storage devices assembled and configured in cabinet enclosure in H9A10 or H9A15 cabinet enclosures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YS-ASCAA-AA |  |  |  |
| Factory Integration of systems and storage devices assembled and configured in cabinet enclosure for H9A11 cabinet enclosures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YS-ASCAA-AB |  |  |  |
| Ballast Kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Ballast is added to a cabinet at particular locations for specific configurations to meet safety regulations for installation, operation, and service; see Cabinet Configuration Charts in Step 5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Two 35-40 lb. steel plates |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9C10-MC |  |  |  |
| Compaq CustomSystems Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Cabinets containing systems and storage must be assembled by qualified assemblers: Compaq Manufacturing, Compaq CustomSystems, or an authorized Compaq Certified Integration Partner. - For custom integration of systems, options, cabinet expansion offerings, and additional Compaq and third party devices, or for other cabinets, doors, and trim kits of a different color, contact Compaq CustomSystems at 1-800-AT COMPAQ, or your local authorized Compaq Certified Integration Partner. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step 6  External Storage  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UltraSCSI Configuration Rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - UltraSCSI configurations require UltraSCSI components (controllers, adapters, shelves, disks, and cables) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - UltraSCSI adapters and RAID controllers support UltraSCSI disks at UltraSCSI speeds in UltraSCSI top gun blue shelves (DS-BA356-xx) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UltraSCSI StorageWorks Pedestals |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - UltraSCSI top gun blue pedestals provide support for UltraSCSI disk drives; pedestal is available in single-bus or split-bus configurations and includes a personality module and 180W power supply |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - UltraSCSI shelf supports: seven 3.5 hard disk drives, or one 5.25 device and four 3.5 hard disk drives, or two 5.25 devices and one 3.5 hard disk drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Installation of redundant power supply reduces number of 3.5 drive slots by one |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UltraSCSI single-ended pedestal, single-bus personality module, 180W power supply |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-BA356-KF |  |  |  |  |
| UltraSCSI single-ended pedestal, split-bus personality module, 180W power supply |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-BA356-KG |  |  |  |  |
| UltraSCSI pedestal, single-channel differential personality module, 180W power supply |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-BA356-KH |  |  |  |  |
| 180W power supply, or redundant power, includes 120V North American and 240V IEC power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CK-BA35X-HH |  |  |  |  |
| Tabletop Tape Drives |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Tape drives require single-ended UltraSCSI adapter and country-specific power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12/24-GB 4-mm DAT narrow single-ended tabletop SCSI tape, includes 120V North American power cord, requires BN31W-xx SCSI cable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-TLZ10-DB |  |  |  |  |
| 20/40-GB SCSI (DLT 4000E) tabletop drive (Tru64 UNIX and OpenVMS only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TZ88N-TA |  |  |  |  |
| 50/100-GB US AIT tabletop tape drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 157767-001 |  |  |  |  |
| 50/100-GB International AIT tabletop tape drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 157767-B31 |  |  |  |  |
| Storage Controllers/Adapters for External StorageWorks Shelves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Storage controllers require Tru64 UNIX V4.0F or OpenVMS V7.1-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Maximum # Supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Tru64 UNIX |  |  |  |  | OpenVMS |  |  |  | Linux |  |  |  |  |  |  |  |  |
| PCI Fibre Channel adapter (KGPSA-BC), (uses one slot) |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  | 1 |  |  |  | - |  |  |  |  |  | 380574-001 |  |  |
| Fibre Channel cable (BNGBX-xx) xx=02, 05, 15, 30, 50 meters x = 1, 2, 3, 4, 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 234457-B2x |  |  |
| PCI 2-port UltraSCSI single-ended adapter and 10/100 Mb Ethernet combo adapter (uses one PCI slot); includes internal 68-pin and 50-pin HD connectors and two external 68-pin VHDCI SCSI connectors, requires BN37A-xx cable to connect to single-ended UltraSCSI shelf |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  | 1 |  |  |  |  | - |  |  |  |  | KZPCM-DA |  |  |
| VHDCI male-to-VHDCI male UltraSCSI cable, xx = 02, 2E [2.5], 03, 05, 10, 20 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN37A-xx |  |  |
| PCI 1-port UltraSCSI single-ended adapter, 68-pin HD connector, requires BN38C cable (2 to 20 m) to connect to single-ended UltraSCSI shelf |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  | 3X-KZPCA-AA |  |  |
| 68-pin HD male-to-VHDCI male UltraSCSI cable; xx = 02, 03, 05, 10, 20 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN38C-xx |  |  |
| PCI 1-port UltraSCSI single-ended adapter (uses one PCI slot); includes internal 68-pin and 50-pin HD connectors and external 68-pin HD connector, requires BN38C-xx cable (2 to 20 m) to connect to single-ended UltraSCSI shelf |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  | KZPBA-CA |  |  |
| PCI 1-port UltraSCSI differential adapter (uses one PCI slot); includes external 68-pin HD connector, requires BN38C-xx cable (2 to 25 m) to connect adapter to differential UltraSCSI shelf or RAID controller |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  | KZPBA-CB |  |  |
| 68-pin HD male-to-VHDCI male UltraSCSI cable; xx = 02, 03, 05, 10, 20, 25 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN38C-xx |  |  |
| PCI 1-port UltraSCSI RAID controller with 4-MB cache memory (uses one PCI slot), includes RAID Array 230/Plus Subsystem software and documentation kit; includes VHDCI female connector; tape drives not supported; 18.2-GB disk drives not supported, requires BN37A-xx cable (.3 to 20 m) to connect to single-ended UltraSCSI shelf |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | - |  |  |  |  |  |  | KZPAC-AA |  |
| PCI 3-port UltraSCSI RAID controller with 4-MB cache memory (uses one PCI slot), includes RAID Array 230/Plus Subsystem software and documentation kit; includes VHDCI female connector; tape drives not supported; 18.2-GB disk drives not supported, requires BN37A-xx cable (.3 to 20 m) to connect to single-ended UltraSCSI shelf |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | - |  |  |  |  |  |  | KZPAC-CA |  |
| Same as KZPAC-CA with 8-MB cache memory |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  | 2 |  |  | - |  |  |  |  |  |  | KZPAC-CB |  |
| Cache battery backup for KZPAC controller |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | KZPSC-UB |  |
| Factory configured RAID of KZPAC with StorageWorks shelves Note: Factory configured RAID sets for disks in single-bus StorageWorks shelves connected to KZPAC require completion of questionnaire located at http://www.digital.com/alphaserver/fcr |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YS-ASDRD-BA |  |
| VHDCI male-to-VHDCI male UltraSCSI cable; xx = 02, 2E [2.5], 03, 05, 10, 20, 25 m Note: Manufacturing may substitute BN37A-02 cable for specific cabinet configurations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN37A-xx |  |
| SCSI cable/bulkhead assembly kit, supports one third-port connection from KZPAC-CA/CB, allows external connection of one third port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN31K-0E |  |
| SCSI cable/bulkhead assembly kit, supports two third-port connections from KZPAC-CA/CB, allows external connection of two third ports |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | KZPAC-SB |  |
| BA370 Rackmount Enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - M-series cabinets (H9A10 and H9A15) allow one BA370 enclosure to be installed in front cabinet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Host adapter, RAID Array controller, platform specific Solutions Software Kit, and disk drives must be ordered separately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BA370 front mount rackmount enclosure with five 180W power supplies, includes RETMA rackmount kit; first BA370 enclosure must be front mounted in cabinet, 20U height |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2T-BA370-AS |  |  |  |
| Factory configured RAID of rackmounted BA370/HSZ80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YS-ASCRD-AA |  |  |  |
| Note: Factory configured RAID sets for disks in rackmounted BA370 StorageWorks shelf controlled by one or two HSZ70s require completion of questionnaire located at http://www.digital.com/alphaserver/fcr |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Removable Media Devices for External StorageWorks Shelves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12/24-GB 4 mm DAT narrow SCSI tape drive in StorageWorks carrier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-TLZ10-VA |  |  |  |
| 35/70-GB AIT tape drive in StorageWorks carrier (Tru64 UNIX and OpenVMS only) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-AIT35-VW |  |  |  |
| 600-MB 32X CD-ROM drive in StorageWorks carrier |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RRD47-VA |  |  |  |
| StorageWorks Shelf  Requires selection of UltraSCSI adapter and SCSI cable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| StorageWorks UltraSCSI single-bus shelf kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BA36R-RC |  |  |  |
| StorageWorks UltraSCSI split-bus shelf kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BA36R-RD |  |  |  |
| External Storage Disk Drive (Supported in StorageWorks Shelf) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Maximum six (10,000 rpm) or seven (7,200 rpm) drives supported in StorageWorks shelf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - One power supply supports a maximum of six 10,000 rpm disks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1-GB 7200 RPM Ultra2 SCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1DA-VW |  |  |  |  |  |  |
| 18.2-GB 7200 RPM Ultra2 SCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1EA-VW |  |  |  |  |  |  |
| 36.4-GB 7200 RPM Ultra2 SCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1FB-VW |  |  |  |  |  |  |
| 9.1-GB 10000 RPM Ultra2 SCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1DD-VW |  |  |  |  |  |  |
| 18.2-GB 10000 RPM Ultra2 SCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1ED-VW |  |  |  |  |  |  |
| 36.4-GB 10000 rpm Ultra2 SCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1FC-VW |  |  |  |  |  |  |
| Step 7 UltraSCSI RAID Array Controllers and UltraSCSI RAID Packaged Solutions  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAID Array 3000 Rackmount Controller Shelf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Selection of RA3000 Solutions Software Kit is required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Supported SCSI adapter: KZPBA-CB PCI 1-port UltraSCSI differential adapter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RA3000 rackmount controller shelf, blue, with one controller, 110V Includes: UltraSCSI BA356 controller shelf, one HSZ22 2-channel controller with 16-MB cache, integrated Environmental Monitoring Unit (EMU), two 180W power supplies, two dual-port host differential UltraSCSI I/O SBBs, dual-channel UltraSCSI single-ended personality modules, one 120V 1000VA rackmount UPS, 5-m host SCSI cable (BN37A-05), BN38E-0B adapter, UPS serial cable, PC connection serial cable (DB-9), two 68-pin HD tri-link adapters (H8861-AA), two North American power cords, and RETMA mounting bracket (disk shelves and disks not included) Requires: Solutions Software Kit for platform, host adapter, RA3000 disk shelf, and disks Options: Second HSZ22 controller, up to four additional RA3000 disk shelves, cache memory upgrade |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-SWXRA-GH |  |  |  |  |  |
| RA3000 rackmount controller shelf, blue, with one controller, 230V Includes: UltraSCSI BA356 controller shelf, one HSZ22 2-channel controller with 16-MB cache, integrated Environmental Monitoring Unit (EMU), two 180W power supplies, two dual-port host differential UltraSCSI I/O SBBs, dual-channel UltraSCSI single-ended personality modules, one 230V 1000VA rackmount UPS, 5-m host SCSI cable (BN37A-05), BN38E-0B adapter, UPS serial cable, PC connection serial cable (DB-9), two 68-pin HD tri-link adapters (H8861-AA), RETMA mounting bracket (disk shelves and disks not included) Requires: Solutions Software Kit for platform, two country-specific power cords, host adapter, RA3000 disk shelf, and disks Options: Second HSZ22 controller, up to four additional RA3000 disk shelves, cache memory upgrade |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-SWXRA-GK |  |  |  |  |  |
| RAID Array 3000 Rackmount Disk Shelf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Select one to four disk shelves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Third and fourth DS-SWXRA-GN disk shelf ordered will be installed in rear of M-series cabinet; rear mounted shelves require that a 2-m BN37A-02 SCSI cable be ordered |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RA3000 rackmount disk shelf, blue, includes UltraSCSI BA356 disk shelf, two 180 W power supplies, single-ended UltraSCSI personality card, one half meter SCSI cable (BN37A-0E), two North American power cords, and RETMA mounting bracket (disks not included); order two country-specific power cords for non-North American use |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-SWXRA-GN |  |  |  |  |  |
| HSZ22 Controller |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Note that second HSZ22 controller ordered is field installed only, not factory installed; requires BN37A-0E SCSI jumper cable be ordered separately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Second HSZ22 controller with 16-MB cache and two 16-MB SIMMs for mirror cache, one BN37A-05 5-m UltraSCSI cable, one BN38E-0B cable adapter  field installable only; note that second HSZ22 controller does not require additional Solution Software Kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-HSZ22-AA |  |  |  |  |  |
| Second HSZ22 controller with 32-MB cache and two 32-MB SIMMs for mirror cache, one BN37A-05 5-m UltraSCSI cable, one BN38E-0B cable adapter  field installed only; note that second HSZ22 controller does not require additional Solution Software Kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-HSZ22-AB |  |  |  |  |  |
| RA3000 Solutions Software Kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - StorageWorks Command Console (SWCC) software |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Platform specific software |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Installation scripts and installation documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RA3000 Solutions Software Kit for Tru64 UNIX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QB-5TWAB-SA |  |  |  |  |  |
| RA3000 Solutions Software Kit for OpenVMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QB-5TWAC-SA |  |  |  |  |  |
| HSG80/HSZ80 External Controllers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fibre Channel controller (DS-HSG80-BJ), includes 2-MB cache expandable to 128 MB; requires 128697-B21 HSG80 ACS V8.4F software kit or 128698-B21 V8.4P software kit for each HSG80 ordered |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 380672-B21 |  |  |  |  |  |
| Fibre Channel controller (DS-HSG80-BH), includes 64-MB cache expandable to 128 MB; requires 128697-B21 HSG80 ACS V8.4F software kit or 128698-B21 V8.4P software kit for each HSG80 ordered |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 380671-B21 |  |  |  |  |  |
| UltraSCSI controller (DS-HSZ80-AH), includes 64-MB cache expandable to 128 MB; requires 400569-001 for Tru64 UNIX or 400571-001 for OpenVMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 400564-B21 |  |  |  |  |  |
| UltraSCSI controller (DS-HSZ80-AJ), includes 256-MB cache expandable to 512 MB; requires 400569-001 for Tru64 UNIX or 400571-001 for OpenVMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 400565-B21 |  |  |  |  |  |
| ESA 10000 , RAID Array 7000, RAID Array 8000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Enterprise Storage Array 10000 and RAID Array 7000 (HSZ70) product set are supported on AlphaServer DS10L systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Supported SCSI adapter for HSZ80/HSZ70  KZPBA-CB PCI 1-port UltraSCSI differential adapter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Supported SCSI adapter for HSG80  KGPSA-BC PCI 1-port Fiber-Channel adapter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step 8  Graphics Adapters  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - RAID Configuration Utility is accessible via console port for Tru64 UNIX and OpenVMS systems; optional graphics capability available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Select graphics adapter, monitor, and country-specific keyboard if required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Maximum # Supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Tru64 UNIX |  |  |  |  |  | OpenVMS |  |  | Linux |  |  |  |  |  |  |  |  |
| ELSA Gloria synergy graphics with 8-MB SGRAM 1600 x 1200, 2D/3D graphics accelerator |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  | SN-PBXGK-BB |  |  |
| Step 9 Monitors Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Graphics monitors other than those listed can be used if compatible with SVGA graphics ordered with system |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Selection of video cable and country-specific power cord is mandatory for all monitors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - AlphaServer DS10L has been designed for desktop use and will support a 21” monitor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 Corporate Series auto-scan color monitor, VGA to 1024 x 768 @ 85 Hz; Northern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQV5-24 |  |  |  |  |  |  |
| Same as above except Southern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQV5-11 |  |  |  |  |  |  |
| 17 (16.0 viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 @75 Hz, TCO 99, MPR-II, Energy Star, attached 1.8-m video cable; Northern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQP7-24 |  |  |  |  |  |  |
| Same as above except Southern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQP7-23 |  |  |  |  |  |  |
| 19 (16 viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 at 75 Hz, TCO 95, MPR-II, Energy Star, attached 1.8-m video cable; Northern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQP9-24 |  |  |  |  |  |  |
| Same as above except Southern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQP9-23 |  |  |  |  |  |  |
| 21 (19.6 viewable image size) auto-scanning color monitor, Trinitron CRT, 0.28 mm aperture grill pitch, VGA to 1600 x 1200 @85 Hz NI, TCO 95, Energy Star, includes 1.8-m video cable; Northern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQP1-24 |  |  |  |  |  |  |
| Same as above except Southern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQP1-23 |  |  |  |  |  |  |
| Video Cable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.8-m video cable — mandatory for each monitor ordered |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN39C-02 |  |  |  |  |  |  |
| Monitor Power Cords |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| North American, 120V, 75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN26J-1K |  |  |  |  |  |  |
| Australia, New Zealand, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19H-2E |  |  |  |  |  |  |
| Central Europe, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19C-2E |  |  |  |  |  |  |
| UK, Ireland, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19A-2E |  |  |  |  |  |  |
| UK, Ireland, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19A-2E |  |  |  |  |  |  |
| Switzerland, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19E-2E |  |  |  |  |  |  |
| Denmark, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19K-2E |  |  |  |  |  |  |
| Italy, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19M-2E |  |  |  |  |  |  |
| Egypt, India, South Africa, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19S-2E |  |  |  |  |  |  |
| Israel, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN18L-2E |  |  |  |  |  |  |
| Japan, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-BN46F-02 |  |  |  |  |  |  |
| Step 10  Networks and Communications  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Systems include dual embedded 10/100 BaseT Ethernet ports (BN25G-xx cable must be ordered separately) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Additional Ethernet ports can be added using available PCI slots |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Maximum # Supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Tru64 UNIX |  |  |  |  |  | OpenVMS |  |  | Linux |  |  |  |  |  |  |  |  |
| PCI 1-port 10/100-Mb Ethernet adapter (Twisted Pair) |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  | 3X-DE600-AA |  |
| PCI 2-port 10/100-Mb Ethernet adapter (Twisted Pair) |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  | 3X-DE602-AA |  |
| SC-SC Dual fibre optic cable, mm, pp xx = length in meters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN34B-xx |  |
| PCI 2-port 10/100 Mbit, Fast Ethernet adapter, UTP |  |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  | 3X-DE602-FA |  |
| PCI 10/100 Mbit 4-port Ethernet adapter, UTP |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  | DE504-BA |  |
| Cable for 10BaseT twisted pair connection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN25G-xx |  |
| PCI Gigabit Ethernet adapter, SC connectors, MMF, no boot support |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | 1 |  |  |  |  |  |  | DEGPA-SA |  |
| SC-to-SC dual fiber optic cable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN34B-xx |  |
| PCI FDDI adapter, single attachment station (SAS), multi-mode fiber (MMF), SC connector |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  | 3X-DEFPA-AC |  |
| PCI FDDI adapter, DAS, MMF, SC connector |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  | 3X-DEFPA-DC |  |
| MIC-SC dual fiber optic cable mm, pp xx = length in meters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN34D-xx |  |
| SC-to-SC dual fiber optic cable mm, pp xx = length in meters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN34B-xx |  |
| PCI FDDI adapter, SAS, TP-PMD, requires BN25H-03 cable |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  | 3X-DEFPA-UC |  |
| PCI FDDI dual adapter, DAS, TP-PMD, requires BN25H-03 cable |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | - |  |  | - |  |  |  |  |  |  | 3X-DEFPA-MC |  |
| 3m (9.8") FDDI cross-over cable mm, pp xx = length in meters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN25H-03 |  |
| PCI 4-port asynchronous controller; maximum two PBXDA-xx controllers per system |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | - |  |  |  |  |  |  | PBXDA-AA |  |
| PCI 8-port asynchronous controller, cable included; maximum two PBXDA-xx controllers per system |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | - |  |  |  |  |  |  | PBXDA-AB |  |
| PCI 16-port asynchronous controller, cable included; maximum two PBXDA-xx controllers per system |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  | - |  |  |  |  |  |  | PBXDA-AC |  |
| Step 11 Keyboards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Keyboards are not included with system and must be ordered separately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tru64 UNIX or Linux |  |  |  |  |  |  | OpenVMS |  |  |  |  |
| U.S./English keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AA |  |  |  |  |  |  | LK461-A2 |  |  |  |  |
| Belgian keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AB |  |  |  |  |  |  | LK461-AB |  |  |  |  |
| Canadian/French keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AC |  |  |  |  |  |  | LK461-AC |  |  |  |  |
| Danish keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AD |  |  |  |  |  |  | LK461-AD |  |  |  |  |
| UK keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AE |  |  |  |  |  |  | - |  |  |  |  |
| Finnish keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AF |  |  |  |  |  |  | LK461-AF |  |  |  |  |
| German keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AG |  |  |  |  |  |  | LK461-AG |  |  |  |  |
| Dutch keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AH |  |  |  |  |  |  | LK461-AH |  |  |  |  |
| Italian keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AI |  |  |  |  |  |  | LK461-AI |  |  |  |  |
| Swiss/French keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AK |  |  |  |  |  |  | LK461-AK |  |  |  |  |
| Swiss/German keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  | LK461-AL |  |  |  |  |
| Swedish keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AM |  |  |  |  |  |  | LK461-AM |  |  |  |  |
| Norwegian keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AN |  |  |  |  |  |  | LK461-AN |  |  |  |  |
| French keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AP |  |  |  |  |  |  | LK461-AP |  |  |  |  |
| Canadian/English keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  | LK461-AQ |  |  |  |  |
| Latin-American keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AR |  |  |  |  |  |  | - |  |  |  |  |
| Spanish keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AS |  |  |  |  |  |  | LK461-AS |  |  |  |  |
| Hebrew keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AT |  |  |  |  |  |  | LK461-AT |  |  |  |  |
| Portuguese keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AV |  |  |  |  |  |  | LK461-AV |  |  |  |  |
| BHCSY keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-AX |  |  |  |  |  |  | - |  |  |  |  |
| International keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-BA |  |  |  |  |  |  | - |  |  |  |  |
| Greek keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-BH |  |  |  |  |  |  | LK461-BH |  |  |  |  |
| Taiwanese keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-BI |  |  |  |  |  |  | - |  |  |  |  |
| Korean keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-BK |  |  |  |  |  |  | - |  |  |  |  |
| Romanian keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  | LK461-BL |  |  |  |  |
| Polish keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-BP |  |  |  |  |  |  | LK461-BP |  |  |  |  |
| Hungarian keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-BQ |  |  |  |  |  |  | LK461-BQ |  |  |  |  |
| Arabic keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-BR |  |  |  |  |  |  | - |  |  |  |  |
| Cyrillic keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-BT |  |  |  |  |  |  | LK461-BT |  |  |  |  |
| Turkish keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-BU |  |  |  |  |  |  | LK461-BU |  |  |  |  |
| Czech keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-BV |  |  |  |  |  |  | LK461-BV |  |  |  |  |
| Turkish/French keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  | LK461-BW |  |  |  |  |
| Yugoslavian keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  | LK461-BY |  |  |  |  |
| Thai keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-CB |  |  |  |  |  |  | - |  |  |  |  |
| S. Chinese keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-CV |  |  |  |  |  |  | - |  |  |  |  |
| Slovak keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-LKQ47-CZ |  |  |  |  |  |  | LK461-CZ |  |  |  |  |
| Keyboard or Mouse Extension Cable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 ft keyboard or mouse extension cable; to extend both keyboard and mouse order two cables |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 17-04016-01 |  |  |  |  |  |
| Mouse |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standard Compaq opal mouse |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SN-PBQWS-WA |  |  |  |  |  |
| Step 12  Console Terminal  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Any terminal can be used as a console device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Third-party console devices require an EIA-232 25-pin DSUB connector |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VT510 video terminal; requires H857-x cable, either male or female |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | VT510-xx |  |  |  |
| Console cable, female connector |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H857-A |  |  |  |
| Console cable, male connector |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H857-D |  |  |  |
| Console cable, male connector (IBM compatible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H897-E |  |  |  |
| Step 13  Software  Required selection of media and documentation for first system on site |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Software Processor Code = E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tru64 UNIX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Tru64 UNIX systems include Tru64 UNIX Traditional Unlimited User license, Server Extension, Internet Access Software kit, and Netscape's Enterprise Server 3.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tru64 UNIX media and online documentation on CD-ROM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QA-MT4AA-H8 |  |  |  |  |  |
| Tru64 UNIX full hard copy documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QA-MT4AA-GZ |  |  |  |  |  |
| StorageWorks Software Package, includes licenses for Logical Storage Manager and AdvFS Utilities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QB-5RXAE-AA |  |  |  |  |  |
| TruCluster Available Server License |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-05SAE-AA |  |  |  |  |  |
| Advanced Server for Tru64 UNIX, 25 client concurrent use license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-5U29M-3D |  |  |  |  |  |
| Advanced Server for Tru64 UNIX, 50 client concurrent use license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-5U29M-3E |  |  |  |  |  |
| Advanced Server for Tru64 UNIX, 100 client concurrent use license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-5U29M-3F |  |  |  |  |  |
| Advanced Server for Tru64 UNIX, 250 client concurrent use license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-5U29M-3G |  |  |  |  |  |
| Advanced Server for Tru64 UNIX, 500 client concurrent use license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-5U29M-3H |  |  |  |  |  |
| Layered products media and documentation for Tru64 UNIX on CD-ROM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QA-054AA-H8 |  |  |  |  |  |
| DECnet/OSI end-system license for Tru64 UNIX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MTJAE-AA |  |  |  |  |  |
| DECnet/OSI extended function license for Tru64 UNIX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MTKAE-AA |  |  |  |  |  |
| OpenVMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - OpenVMS systems include OpenVMS base license, TCP/IP Services for OpenVMS, DECWindows Motif for OpenVMS Alpha, DECnet Plus for OpenVMS End System, and Pathworks 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - OpenVMS Concurrent Use Licenses are not specific to a single system and can be moved between systems at user discretion; OpenVMS Concurrent Use Licenses can also be shared in a mixed OpenVMS VAX and OpenVMS Alpha Cluster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Concurrent Use 1-user license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MT3AA-3B |  |  |  |  |  |
| Concurrent Use 2-user license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MT3AA-3C |  |  |  |  |  |
| Concurrent Use 4-user license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MT3AA-3D |  |  |  |  |  |
| Concurrent Use 8-user license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MT3AA-3E |  |  |  |  |  |
| Concurrent Use 16-user license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MT3AA-3F |  |  |  |  |  |
| Concurrent Use 32-user license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MT3AA-3G |  |  |  |  |  |
| Concurrent Use 64-user license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MT3AA-3H |  |  |  |  |  |
| Concurrent Use 128-user license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MT3AA-3J |  |  |  |  |  |
| Concurrent Use 256-user license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MT3AA-3K |  |  |  |  |  |
| Traditional unlimited user license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MT2AQ-AA |  |  |  |  |  |
| OpenVMS media and documentation on CD-ROM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QA-MT1AA-H8 |  |  |  |  |  |
| OpenVMS full hard copy documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QA-001AA-GZ |  |  |  |  |  |
| Layered products media and documentation for OpenVMS on CD-ROM, includes Enterprise Integration Server for OpenVMS media and documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QA-03XAA-H8 |  |  |  |  |  |
| DIGITAL Enterprise Integration Server for OpenVMS media and documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QA-5LVAA-H8 |  |  |  |  |  |
| DECnet extended function license for OpenVMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MTGAE -AA |  |  |  |  |  |
| DECnet end-system to extended function upgrade license for OpenVMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MTHAE-AA |  |  |  |  |  |
| OpenVMS Cluster license for Alpha systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-MUZAE-AA |  |  |  |  |  |
| OpenVMS Volume Shadowing license |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QL-2A1AE-AA |  |  |  |  |  |
| Note: Linux-Ready systems ship without system software; Red Hat 6.0 or SuSe 6.1 Linux may be purchased separately from a Linux distributor. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step 14  Hardware and Software Supplemental Support Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Systems include 1-year hardware warranty, on-site, with 5 x 9, 24-hour response time; select optional Hardware Supplemental Support Services if required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Options ordered as part of "Initial System Sale" take on warranty of the enclosure; options added after "Initial System Sale" take on warranty of enclosure until enclosure warranty ends |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - All unique AlphaServer DS10L options have a minimum of 1-year Return-to-Compaq warranty |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hardware Supplemental Support  Americas and Asia Pacific only |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3-year, 5 x 9, 4-hour response time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-MK4HR-36 |  |  |  |  |  |
| 5-year, 5 x 9, 4-hour response time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-MK4HR-60 |  |  |  |  |  |
| 3-year, 5 x 12, 4-hour response time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-MK512-36 |  |  |  |  |  |
| 5-year, 5 x 12, 4-hour response time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-MK512-60 |  |  |  |  |  |
| 3-year, 6 x 16, 4-hour response time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-MK616-36 |  |  |  |  |  |
| 5-year, 6 x 16, 4-hour response time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-MK616-60 |  |  |  |  |  |
| 3-year, 7 x 24, 4-hour response time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-MK724-36 |  |  |  |  |  |
| 5-year, 7 x 24, 4-hour response time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-MK724-60 |  |  |  |  |  |
| 5-year, 7 x 24, 4-hour response time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-MK724-60 |  |  |  |  |  |
| Years 4-5, 5 x 9, next day response time |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-MKXHW-60 |  |  |  |  |  |
| Installation Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hardware product installation for AlphaServer DS10L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-TC12X-IN |  |  |  |  |  |
| Add-on memory installation for AlphaServer DS10L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-TCMEM-IN |  |  |  |  |  |
| Add-on CPU installation for AlphaServer DS10L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-TCSMP-IN |  |  |  |  |  |
| Software Supplemental Support  Americas and Asia Pacific only |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Systems include 90-day Conformance to SPD and Telephone Advisory Support; select optional Software Supplemental Support Services if required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Software service upgrades for Tru64 UNIX include advisory and remedial software support with new version license rights for operating system for time period indicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Software service upgrades for OpenVMS include advisory and remedial software support with new version license rights for operating system and Enterprise Integration Package for time period indicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Software service upgrades for SMP upgrades running Tru64 UNIX or OpenVMS include new version license rights for Tru64 UNIX or OpenVMS SMP Upgrades for time period indicated; one required for each SMP upgrade |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Software Supplemental Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tru64 UNIX |  |  |  |  |  |  |  | OpenVMS |  |  |  |  |  |
| 1-year, 5 x 9 Bronze Software Supplemental Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-D10U9-12 |  |  |  |  |  |  |  | FM-D10V9-12 |  |  |  |  |  |
| 3-year, 5 x 9 Bronze Software Supplemental Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-D10U9-36 |  |  |  |  |  |  |  | FM-D10V9-36 |  |  |  |  |  |
| 5-year, 5 x 9 Bronze Software Supplemental Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-D10U9-60 |  |  |  |  |  |  |  | FM-D10V9-60 |  |  |  |  |  |
| 1-year, 7 x 24 Bronze Software Supplemental Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-D10US-12 |  |  |  |  |  |  |  | FM-D10VS-12 |  |  |  |  |  |
| 3-year, 7 x 24 Bronze Software Supplemental Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-D10US-36 |  |  |  |  |  |  |  | FM-D10VS-36 |  |  |  |  |  |
| 5-year, 7 x 24 Bronze Software Supplemental Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-D10US-60 |  |  |  |  |  |  |  | FM-D10VS-60 |  |  |  |  |  |
| 1-year, Bronze Node Software Supplemental Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-D10UN-12 |  |  |  |  |  |  |  | FM-D10VN-12 |  |  |  |  |  |
| 3-year, Bronze Node Software Supplemental Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-D10UN-36 |  |  |  |  |  |  |  | FM-D10VN-36 |  |  |  |  |  |
| 5-year, Bronze Node Software Supplemental Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FM-D10UN-60 |  |  |  |  |  |  |  | FM-D10VN-60 |  |  |  |  |  |

| Physical Characteristics |  | Dimensions (WxDxH) | 17.5 x 20.5 x 1.7 in / 44.5 x 52 x 4.3 cm |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Shipping Dimensions | 23.6 x 21.7 x 10.1 in / 60 x 55 x 26 cm |  |  |  |
|  |  | Weight | 21 lbs / 9.4 kg |  |  |  |
| Cabinets |  |  | H9A10 M-Series |  | H9A15 M-Series |  |
|  |  | Dimensions (HxWxD) | 67 x 23.6 x 43.27 in/ 170 x 60 x 110 cm |  | 79 x 23.6 x 35.4 in 200 x 60 x 90 cm |  |
|  |  | Shipping Dimensions | 73 x 36 x 48 in / 185.5 x 91.5 x 122 cm |  | 85 x 36 x 48 in / 216 x 91.5 x 122 cm |  |
|  |  | Weight | Configuration dependent |  | 1000 lbs / 450 kg |  |
|  |  | Shipping Weight | Configuration dependent, maximum payload 1,000 lbs |  | 1056 lbs / 550 kg (normal) 1,408 lbs / 640 kg (maximum) |  |
| System Unit |  | Environmental |  |  |  |  |
|  |  | Temperature |  |  |  |  |
|  |  | Operating (Tower, Rackmount)1 | 50 to 95F / 10 to 35C |  |  |  |
|  |  | Non-operating | 41 to 122 F / 5 to 50C |  |  |  |
|  |  | Storage (60 days) | –40 to –151F / –40 to –66C |  |  |  |
|  |  | Rate of change | 20F/hr / 11C/hr |  |  |  |
|  |  | Relative Humidity |  |  |  |  |
|  |  | Operating | 10% to 90% RH |  |  |  |
|  |  | Non-operating | 10% to 95% |  |  |  |
|  |  | Storage (60 days) | 10% to 95% |  |  |  |
|  |  | Maximum Wet Bulb Temperature |  |  |  |  |
|  |  | Operating | 82F / 28C |  |  |  |
|  |  | Non-operating | 90F / 32C |  |  |  |
|  |  | Storage (60 days) | 115F / 46C |  |  |  |
|  |  | Heat Dissipation |  |  |  |  |
|  |  | H9A10/H9A15 Cabinets | Configuration dependent |  |  |  |
|  |  | Air Flow and Quality |  |  |  |  |
|  |  | Intake location | Front |  |  |  |
|  |  | Exhaust location | Rear pedestal and rack |  |  |  |
|  |  | Particle size | N/A |  |  |  |
|  |  | Concentration | N/A |  |  |  |
|  |  | Altitude |  |  |  |  |
|  |  | Operating 2 | 10,000 f/3,037m |  |  |  |
|  |  | Non-operating | 40,000 ft/12,192m |  |  |  |
|  |  | Acoustics (Declared values per ISO 9296 and ISO 7779) |  |  |  |  |
|  |  | Acoustics 3 | LWAd3B | LnAm5dBA (bystander position) |  |  |
|  |  | Idle | 6.0 | 46 |  |  |
|  |  | Operating | 6.1 | 47 |  |  |
|  |  | Vibration |  |  |  |  |
|  |  | Operating | 5-500 Hz at .1 G peak (limited by 0.02”/0.5mm double amplitude) |  |  |  |
|  |  | Non-operating | 1.03 Grms 5-300 Hz |  |  |  |
|  |  | Shock |  |  |  |  |
|  |  | Operating | 10 G 10ms half-sine |  |  |  |
| System Unit (continued) |  | Electrical (Power supplies are universal, PFC, auto ranging, 100/240 Vac) |  |  |  |  |
|  |  | Nominal Voltage (Vac) | 100 | 120 |  | 200-240 |
|  |  | Voltage Range (Vac) (temporary condition) | 90-100 | 110-128 |  | 180-250 |
|  |  | Power Source Phase | Single | Single |  | Single |
|  |  | Nominal Frequency (Hz) | 50/60 | 50/60 |  | 50/60 |
|  |  | Frequency Range (Hz) | 49 to 51/59 to 61 | 49 to 51/59 to 61 |  | 49 to 51/59 to 61 |
|  |  | RMS Current (maximum steady state) |  |  |  |  |
|  |  | Single power cord | 2.3A | 1.9A |  | 1.1A |
|  |  | Maximum VA | 230 | 229 |  | 220 |
|  |  | M-series Cabinet (configuration dependent) |  |  |  |  |
|  |  | Nominal voltage (Vac) | 100 | 120 |  | 220-240 |
|  |  | Each power cord | 24A | 24A |  | 16A |
|  |  | Power Cords |  |  |  |  |
|  |  | Pedestal | 1 (75 in / 190 cm) | IEC 320 C13 to NEMA 5-15 (N. America) or IEC 320 C13 to country-specific |  |  |
|  |  | Rackmount | 1 (14 ft 10 in / 452 cm) | IEC 320 C13 to NEMA 5-15 (N. America) or IEC 320 C13 to IEC 320 C14 (other countries) |  |  |
|  |  | Cabinet | 2 (10 ft 10 in / 330 cm) | 120V non-removable NEMA L5-30P or 200-240V non-removable IEC 309 |  |  |
|  |  | Regulatory |  |  |  |  |
|  |  | Agency Approvals | UL: Listed to UL1950 (3rd edition) |  |  |  |
|  |  |  | CUL Listed: CAN/CSA-C22.2 No. 950-M95 |  |  |  |
|  |  |  | TUV: EN 60950/A11: 1997 GS marked |  |  |  |
|  |  |  | FCC: Part 15.B Class A |  |  |  |
|  |  |  | CE: EN55022, EN50082 |  |  |  |
|  |  |  | VCCI Class II ITE |  |  |  |
|  |  |  | BMSI: CISPR22, CNS13438 |  |  |  |
|  |  |  | C-Tick: CISPR22, AS/NZS 3548 |  |  |  |
|  |  | Reviewed to | AS/NZ 3260:1993 Australian/New Zealand Standard |  |  |  |
|  |  |  | EN 60950/A11: 1997 European Norm |  |  |  |
|  |  |  | IEC950 (2nd edition, 4th amend) |  |  |  |

- Maximum operating temperature at sea level; reduce by 1.8F (1C) for each 600m (2,000 ft) above sea level.

- Higher altitudes are possible if maximum operating temperature is reduced (see Temperature); other restrictions may apply such as maximum permissible altitude for hard drives.

- Current values for specific configurations are available from Compaq, note that 1 B = 10 dBA.