- Operator Control Panel

- Removable media bay

- Combination CD-ROM/floppy drive

- Hard disk drives

- Power supplies

- Ultra2 SCSI adapter

- System motherboard

- CPU modules

- Server Management Module

At A Glance

- AlphaServer DS20E systems include:

- Alpha 21264 processor, 500 MHz or 667 MHz (dual processor capability)

- 4-MB level 2 writeback cache per 500 MHz processor or 8-MB L2 DDR cache per 667 MHz processor

- ECC-protected memory bus and cache

- 256-MB, 512-MB, or 1-GB  SDRAM memory expandable to 4 GB

- Five PCI and one shared PCI/ISA slot (six total slots)

- Four hot swap hard disk drive bays

- Internal mass storage capacity up to 72.8 GB

- Integrated environmental server management module

- Two 375-Watt hot pluggable power supplies with optional third for N+1 redundancy

- Combo 24X CD-ROM Drive and 1.44-MB floppy disk drive

- Protected by Compaq Services, including a three-year on-site limited warranty

July 19, 1999

2nd  Review – 5/17/99

Due Back –  5/21/99Revision 2.4

March 28, 2000

Edited By:

| Processor |  |  |
| --- | --- | --- |
| Alpha 21264 500 MHz or 667 MHz processor |  |  |
| Cache Memory |  |  |
| 4-MB level 2 writeback cache per 500 MHz processor 8-MB level 2 Dual Data Rate cache per 667 MHz processor |  |  |
| Upgradability |  |  |
| System upgradable to dual processing |  |  |
| Memory |  |  |
| Standard | 256-MB, 512-MB, or 1-GB ECC SDRAM memory standard |  |
| Maximum | 4 GB |  |
| Network Controller |  |  |
| Choice of Ethernet controllers |  |  |
| Expansion |  |  |
| I/O (Total) | 6 |  |
| PCI | 5 |  |
| Shared PCI/ISA | 1 |  |
| Storage Controller |  |  |
| Choice of storage controllers |  |  |
| Storage |  |  |
| Diskette Drive |  | 1.44 MB |
| CD-ROM |  | 24X Max IDE CD-ROM Drive |
| Internal Hard Drives |  | Choice of 9.1 GB, or 18.2 GB |
| Maximum Internal Storage |  | 72 GB |
| Expansion Ports |  |  |
| Parallel |  | 1 |
| Serial |  | 2 |
| Pointing Device (Mouse) |  | 1 |
| Keyboard |  | 1 |
| SCSI knockouts on rear of chassis |  | 4 |
| USB |  | 2 (future support) |
| Form Factor |  |  |
| Rackmount or Tower |  |  |
| Manageability |  |  |
| Compaq Insight Manager |  |  |
| Hot plug redundant power supply support; hot plug fan support |  |  |
| Power Supply |  |  |
| Two 375-Watt Hot Plug Power Supplies, standard; optional third N+1 for redundancy |  |  |
| OS Support |  |  |
| Tru64 UNIX AlphaServers include pre-installed software, base license, unlimited user license, server extension license, Open Source Internet Solutions, and Netscape Enterprise Server 3.0. Tru64 UNIX AlphaStations include a base 2-user and Compaq Open3D license. OpenVMS AlphaServer include pre-installed software, base license, and Enterprise Integration Server License Package V3.0A. OpenVMS AlphaStations include an OpenVMS base license, concurrent use 1-user license, and Compaq Open3D license. Minimum OS support: Tru64 UNIX V4.0F, OpenVMS V7.1-2 or V7.2-1 |  |  |
| Service and Support |  |  |
| Compaq Services provides a wide range of service offerings including a three-year limited warranty2 fully supported by a worldwide network of resellers and service providers; lifetime toll-free 7 x 24 hardware technical phone support. |  |  |

| Step 1  Select system |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AlphaServer DS20E systems require the mandatory selection of the following items: - Pedestal/tower or rackmount bezel kit - Disk drive cage - Ultra2 SCSI adapter - System disk drive - Ethernet adapter - Power cord - Software media and documentation for first system onsite Notes: DS20E minimum OS support Tru64 UNIX V4.0F, OpenVMS V7.1-2 or V7.2-1 Keyboard not included For system integration of options with 6-3 part numbers, contact CustomSystems |  |  |  |  |  |  |  |
| AlphaServers |  |  |  |  |  |  |  |
| OS |  | CPU | Memory / Cache | Order No. |  |  |  |
| Tru64 UNIX |  | 500 MHz | 256 MB / 4MB | DA-56NRA-DA |  |  |  |
| Tru64 UNIX |  | 500 MHz | 512 MB / 4MB | DA-56NRA-EA |  |  |  |
| Tru64 UNIX |  | 500 MHz | 1 GB / 4MB | DA-56NRA-FA |  |  |  |
| Tru64 UNIX |  | 667 MHz | 256 MB / 8MB | DA-56PAA-DA |  |  |  |
| Tru64 UNIX |  | 667 MHz | 512 MB / 8MB | DA-56PAA-EA |  |  |  |
| Tru64 UNIX |  | 667 MHz | 1 GB / 8MB | DA-56PAA-FA |  |  |  |
| OpenVMS |  | 500 MHz | 256 MB / 4MB | DY-56NRA-DA |  |  |  |
| OpenVMS |  | 500 MHz | 512 MB / 4MB | DY-56NRA-EA |  |  |  |
| OpenVMS |  | 500 MHz | 1 GB / 4MB | DY-56NRA-FA |  |  |  |
| OpenVMS |  | 667 MHz | 256 MB / 8MB | DY-56PAA-DA |  |  |  |
| OpenVMS |  | 667 MHz | 512 MB / 8MB | DY-56PAA-EA |  |  |  |
| OpenVMS |  | 667 MHz | 1 GB / 8MB | DY-56PAA-FA |  |  |  |
| AlphaStations |  |  |  |  |  |  |  |
| OS |  | CPU | Memory / Cache | Order No. |  |  |  |
| Tru64 UNIX |  | 500 MHz | 256 MB / 4MB | DA-56NRA-DW |  |  |  |
| Tru64 UNIX |  | 500 MHz | 512 MB / 4MB | DA-56NRA-EW |  |  |  |
| Tru64 UNIX |  | 500 MHz | 1 GB / 4MB | DA-56NRA-FW |  |  |  |
| Tru64 UNIX |  | 667 MHz | 256 MB / 8MB | DA-56PAA-DW |  |  |  |
| Tru64 UNIX |  | 667 MHz | 512 MB / 8MB | DA-56PAA-EW |  |  |  |
| Tru64 UNIX |  | 667 MHz | 1 GB / 8MB | DA-56PAA-FW |  |  |  |
| OpenVMS |  | 500 MHz | 256 MB / 4MB | DY-56NRA-DW |  |  |  |
| OpenVMS |  | 500 MHz | 512 MB / 4MB | DY-56NRA-EW |  |  |  |
| OpenVMS |  | 500 MHz | 1 GB / 4MB | DY-56NRA-FW |  |  |  |
| OpenVMS |  | 667 MHz | 256 MB / 8MB | DY-56PAA-DW |  |  |  |
| OpenVMS |  | 667 MHz | 512 MB / 8MB | DY-56PAA-EW |  |  |  |
| OpenVMS |  | 667 MHz | 1 GB / 8MB | DY-56PAA-FW |  |  |  |
| Step 2  Select mandatory options |  |  |  |  |  |  |  |
| Step 2a  Bezel Kits  Mandatory |  |  |  |  |  |  |  |
| - Pedestal/Tower kit or Rackmount kit is mandatory for each system |  |  |  |  |  |  |  |
| Pedestal/Tower bezel kit for 500 MHz systems |  |  |  |  | 3X-BA56P-PA |  |  |
| Rackmount bezel kit (M-Series Cabinets) for 500 MHz systems |  |  |  |  | 3X-BA56R-RA |  |  |
| Pedestal/Tower bezel kit for 667 MHz systems |  |  |  |  | 3X-BA56P-PB |  |  |
| Rackmount bezel kit (M-Series Cabinets) for 667 MHz systems |  |  |  |  | 3X-BA56R-RC |  |  |
| Step 2b  Disk Drive Cage  Mandatory |  |  |  |  |  |  |  |
| - Systems support one disk drive cage; disk drive cage supports four 3.5” hard disk drives |  |  |  |  |  |  |  |
| - Each drive cage includes Ultra Universal drive height converter (4-pack) to install 1.0” Universal drives. |  |  |  |  |  |  |  |
| 4-bay disk drive cage |  |  |  |  | 3X-BA611-4D |  |  |
| Step 2c  Ultra2 SCSI Adapter  Mandatory |  |  |  |  |  |  |  |
| - Selection of Ultra2 SCSI adapter to connect to internal disk drive cage is mandatory; SCSI cable included with system |  |  |  |  |  |  |  |
| Single channel LVD adapter, maximum 4 per system |  |  |  |  | 3X-KZPCA-AA |  |  |
| Step 2d System Disk Drive (Supported in Disk Drive Cage)  Mandatory |  |  |  |  |  |  |  |
| - Selection of one disk drive is mandatory |  |  |  |  |  |  |  |
| 9.1-GB 7200 RPM 1.0 Universal Ultra2 SCSI disk drive |  |  |  |  | 3R-A0525-AA |  |  |
| 9.1-GB 10000 RPM 1.0 Universal Ultra2 SCSI disk drive |  |  |  |  | 3R-A0526-AA |  |  |
| 18.2-GB 7200 RPM 1.0 Universal Ultra2 SCSI disk drive |  |  |  |  | 3R-A0527-AA |  |  |
| 18.2-GB 10000 RPM 1.0 Universal Ultra2 SCSI disk drive |  |  |  |  | 3R-A0561-AA |  |  |
|  | Ultra Universal drive height converter, 4-pack (included in disk drive cage). Customers with earlier 4-slot drive cages (3X-BA611-4D) need to order separately when installing 1” Universal drives |  |  |  |  | 3R-A0528-AA |  |
| Step 2e  Ethernet Adapters  Mandatory |  |  |  |  |  |  |  |
| - Selection of one Ethernet adapter is mandatory (Ethernet adapter is not required if KZPCM adapter is selected) |  |  |  |  |  |  |  |
| - See Step 12 for additional networking options |  |  |  |  |  |  |  |
| PCI 1-port 10/100 Mbit Fast Ethernet adapter, UTP, maximum four per system |  |  |  |  |  | 3X-DE600-AA |  |
| PCI 2-port 10/100 Mbit, Fast Ethernet adapter, UTP, maximum two per system |  |  |  |  |  | 3X-DE602-AA |  |
| Multi-mode fiber (MMF) add-on daughter card for 3X-DE602-AA |  |  |  |  |  | 3X-DE602-FA |  |
| PCI 10/100 Mbit 4-port Ethernet adapter, UTP, maximum one per system |  |  |  |  |  | DE504-BA |  |
| Cable for 10BaseT twisted pair connection |  |  |  |  |  | BN25G-xx |  |

| Step 2f  Power Cords  Mandatory |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Selection of power cord is mandatory for all systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pedestal Power Cords |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| North American 120V, 75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN26J-1K |  |  |  |
| Australia, New Zealand, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19H-2E |  |  |  |
| Central Europe, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19C-2E |  |  |  |
| UK, Ireland, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19A-2E |  |  |  |
| Switzerland, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19E-2E |  |  |  |
| Denmark, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19K-2E |  |  |  |
| Italy, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19M-2E |  |  |  |
| Egypt, India, South Africa, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN19S-2E |  |  |  |
| Israel, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN18L-2E |  |  |  |
| Japan, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-BN46F-02 |  |  |  |
| Rackmount Power Cords |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| US/Canada |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN20Z-4E |  |  |  |
| Non-US |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN35S-4E |  |  |  |
| Japan, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-BN46D-4E |  |  |  |
| Step 3  Additional CPU Modules  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - One 6/500-MHz or 6/667-MHz CPU included, one additional CPU supported. Second CPU must be the same speed. A 500 MHz system can be upgraded to 667 MHz by swapping out the 500 MHz CPU(s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SMP upgrade includes one 6/500-CPU with 4-MB cache and Tru64 UNIX SMP license |  |  |  |  |  |  |  |  |  |  |  |  |  |  | KN310-BB |  |  |  |
| SMP upgrade includes one 6/500-CPU with 4-MB cache and OpenVMS SMP license |  |  |  |  |  |  |  |  |  |  |  |  |  |  | KN310-BC |  |  |  |
| SMP upgrade includes one 6/667-CPU with 8-MB cache and Tru64 UNIX SMP license |  |  |  |  |  |  |  |  |  |  |  |  |  |  | KN311-BB |  |  |  |
| SMP upgrade includes one 6/667-CPU with 8-MB cache and OpenVMS SMP license |  |  |  |  |  |  |  |  |  |  |  |  |  |  | KN311-BC |  |  |  |
| Step 4  Memory  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - System includes one memory option; up to three additional memory options can be added |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 256-MB memory option (4 x 64 MB DIMMs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MS340-CA |  |  |  |
| 512-MB memory option (4 x 128 MB DIMMs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MS340-DA |  |  |  |
| 1-GB memory option (4 x 256 MB DIMMs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | MS340-EA |  |  |  |
| Step 5  Additional Power Supplies  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Two 375 watt power supplies included with system; optional third power supply provides N+1 redundancy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DS20E redundant power supply |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FR-H7910-AA |  |  |  |
| Step 6 Internal Removable Storage  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - System supports three internal removable media devices: One combo 24X CD-ROM drive and 1.44 MB disk drive One additional removable media slot provides support for one 5.25 half-height device; Requires KZPBA-CA or KZPCM-DA UltraSCSI adapter, or 3X-KZPCA-AA Ultra2 (LVD) adapter, and SCSI cable (3X-BC56C-2F). For optimum performance it is recommended that Ultra2 capable devices be connected to an Ultra2 adapter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 600-MB 32X CD-ROM drive (RRD47) 5.25 half-height, top gun blue |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RRD47-VD |  |  |  |
| 12/24-GB 4 mm DAT SCSI tape drive (TLZ10) 5.25 half-height, top gun blue |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TLZ10-LB |  |  |  |
|  | 5.25” half-height 20/40-GB 4 mm DAT Ultra 2 (LVD) SCSI tape drive, top gun blue |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-SD20X-LB |  |  |
|  | 5.25” half-height 35/70-GB 8 mm AIT Wide Ultra SCSI tape drive, top gun Blue |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-SZ35X-LB |  |  |
|  | 5.25” half-height 50/100-GB 8 mm AIT Ultra 2 (LVD) SCSI tape drive, top gun blue |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-SZ50X-LB |  |  |
| SCSI cable with one connector and active terminator to connect optional PCI adapter to 5.25 device |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-BC56C-2F |  |  |  |
| Step 7  M-series Cabinets  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M-series enclosures require selection of the following items: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - H9A10 or H9A15 cabinet, 120V or 240V |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Rack kit for each system |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - H9A10 or H9A15 rear cabinet extender |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Cabinet front door based on cabinet height |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Factory integration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Ballast kit if one or two systems are ordered for the H9A15 cabinet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Power Cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: For more information on M-Series Cabinet Enclosures, see www.digital.com/alphaserver/products/m-series/index.html |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cabinet Configuration Chart for AlphaServer DS20E (5U) with StorageWorks Shelves (4U) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cabinet |  | Rack Kit |  |  | Cabinet Height/Space |  | Systems |  |  | Shelves EV6 EV67 |  |  | Ballast Kit |  |  |  |  |  |
| H9A10-MC/MD |  | 3X-BA56R-RA / RC |  |  | 56” / 32U |  | 1 |  |  | 6 6 |  |  | - |  |  |  |  |  |
|  |  |  |  |  |  |  | 2 |  |  | 6 6 |  |  | - |  |  |  |  |  |
|  |  |  |  |  |  |  | 3 |  |  | 6 6 |  |  | - |  |  |  |  |  |
|  |  |  |  |  |  |  | 4 |  |  | 4 6 |  |  | - |  |  |  |  |  |
|  |  |  |  |  |  |  | 5 |  |  | 0 2 |  |  | - |  |  |  |  |  |
|  |  |  |  |  |  |  | 6 (EV67 667 MHz only) |  |  | 0 0 |  |  |  |  |  |  |  |  |
| H9A15-MC/MD |  | 3X-BA56R-RA / RC |  |  | 66.5” / 39U |  | 1 |  |  | 6 6 |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  | 2 |  |  | 6 6 |  |  | 1 |  |  |  |  |  |
|  |  |  |  |  |  |  | 3 |  |  | 6 6 |  |  | - |  |  |  |  |  |
|  |  |  |  |  |  |  | 4 |  |  | 6 6 |  |  | - |  |  |  |  |  |
|  |  |  |  |  |  |  | 5 |  |  | 4 6 |  |  | - |  |  |  |  |  |
|  |  |  |  |  |  |  | 6 |  |  | 0 4 |  |  | - |  |  |  |  |  |
| Note: 1.75” or 1U space is required between each system unit for EV6 500 MHz systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cabinet Configuration Chart for AlphaServer DS20E (5U) with StorageWorks RA3000 and Disk Shelves (12U) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 12U of vertical cabinet space is reserved for one RA3000 controller shelf and up to five disk shelves mounted in top of cabinet, front to back; an additional 2U of vertical space is required for RA3000 controller shelf UPS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Disk shelves can be a combination of RA3000 DS-SWXRA-GN disk shelves and StorageWorks BA36R-RC/RD disk shelves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - RA3000 controller shelf supports a maximum of four DS-SWXRA-GN disk shelves; third and fourth DS-SWXRA-GN disk shelf ordered will be installed in rear of M-series cabinet; rear mounted shelves require that a 2-m SCSI cable (BN37A-02) be ordered separately. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Systems are supported in remaining vertical space. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 1.75 or 1U space is required between each system unit for EV6 500 MHz systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cabinet |  | Mounting Kit |  | Cabinet Height / Space |  |  | Systems | RA3000 + Shelves |  |  |  |  |  |  | Ballast Kit |  |  |  |
| H9A10-MC/MD |  | 3X-BA56R-RA / RC |  | 54.2 / 32U |  |  | 1 | 1 + 5 |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  | 2 | 1 + 5 |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  | 3 | 1 + 5 |  |  |  |  |  |  | - |  |  |  |
| H9A15-MC/MD |  | 3X-BA56R-RA / RC |  | 66.5 / 39U |  |  | 1 | 1 + 5 |  |  |  |  |  |  | 1 |  |  |  |
|  |  |  |  |  |  |  | 2 | 1 + 5 |  |  |  |  |  |  | 1 |  |  |  |
|  |  |  |  |  |  |  | 3 | 1 + 5 |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  | 4 | 1 + 5 |  |  |  |  |  |  | - |  |  |  |
| Cabinet Configuration Chart for AlphaServer DS20E (5U) with StorageWorks BA370 (20U) and Disk Shelves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 20U of vertical cabinet space is reserved for one front-mounted BA370 rackmount enclosure, or one front-mounted and one rear-mounted BA370 enclosure in top of cabinet, front to back. Note that top fan (1U) is removed from cabinet when BA370 rackmount enclosure is installed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Systems and StorageWorks shelves are supported in remaining vertical space; StorageWorks shelves are not supported in rear of cabinet behind a front-mounted BA370 rackmount enclosure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - 1.75 or 1U space is required between each system unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cabinet |  | Rack Kit | Cabinet Height/Space |  |  | Systems |  | BA370 |  |  |  |  |  |  | Shelves |  |  |  |
| H9A10-MC/MD |  | 3X-BA56R-RA / RC |  |  |  | 1 |  | 1 (front) or |  |  |  |  |  |  | 2 |  |  |  |
|  |  |  |  |  |  | 2 |  | 2 (front & back) |  |  |  |  |  |  | 0 |  |  |  |
| H9A15-MC/MD |  | 3X-BA56R-RA / RC | 66.5” / 40U |  |  | 1 |  | 1 (front) or |  |  |  |  |  |  | 6 |  |  |  |
|  |  |  |  |  |  | 2 |  | 2 (front & back) |  |  |  |  |  |  | 4 |  |  |  |
|  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  | 0 |  |  |  |
| Note: Installing one BA370 and one system in an H9A15 cabinet requires the mandatory selection of a ballast kit; a ballast kit is not required if two BA370s and one system are installed in H9A15 cabinet. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M-series Cabinets |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 67" cabinet with 56" or 32U of usable vertical space, 120V dual-24A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific; includes rear door and top fan tray |  |  |  |  |  |  |  |  |  |  |  |  |  | H9A10-MC |  |  |  |  |
| 67" cabinet with 56" or 32U of usable vertical space, 240V dual-16A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe; includes rear door and top fan tray |  |  |  |  |  |  |  |  |  |  |  |  |  | H9A10-MD |  |  |  |  |
| 79" cabinet with 66.5" or 39U of usable vertical space, 120V dual-24A controller, each with 10 NEMA 5-15R outlets, input power plug NEMA L5-30P, Americas, Asia Pacific; includes rear door and top fan tray |  |  |  |  |  |  |  |  |  |  |  |  |  | H9A15-MC |  |  |  |  |
| 79" cabinet with 66.5" or 39U of usable vertical space, 240V dual-16A controller, each with 12 IEC C13 outlets, input power plug IEC 309, 16A, Europe; includes rear door and top fan tray |  |  |  |  |  |  |  |  |  |  |  |  |  | H9A15-MD |  |  |  |  |
| Rear Cabinet Extender  Required selection of one cabinet extender for each H9A10 or H9A15 cabinet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rear cabinet extender, top gun blue, for H9A10-MC/MD cabinets |  |  |  |  |  |  |  |  |  |  |  |  |  | H9C10-TE |  |  |  |  |
| Rear cabinet extender, top gun blue, for H9A15-MC/MD cabinets |  |  |  |  |  |  |  |  |  |  |  |  |  | H9C15-TE |  |  |  |  |
| Cabinet Front Door  Required selection of door based on cabinet height |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Front door, top gun blue, for H9A10-MC/MD cabinet enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  | H9C10-TF |  |  |  |  |
| Front door, top gun blue, for H9A15-MC/MD cabinet enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  | H9C15-TF |  |  |  |  |
| Factory Integration  Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Factory Integration of systems and storage devices assembled and configured in cabinet enclosure in predefined locations |  |  |  |  |  |  |  |  |  |  |  |  |  | YS-ASCAA-AA |  |  |  |  |
| Ballast Kit  Required only if one system is ordered for the H9A15 cabinet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Ballast is added to a cabinet at particular locations for specific configurations to meet safety regulations for installation, operation, and service; see Cabinet Configuration Chart |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Two 35-40 lb. Steel plates |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H9C10-MC |  |  |  |
| Rackmount Power Cord  Required for each system |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| US/Canada |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN20Z-4E |  |  |  |
| Non-US |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN35S-4E |  |  |  |
| Japan, 2.5m |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3X-BN46D-4E |  |  |  |
| Shelves Supported in M-series Cabinets  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UltraSCSI single-ended RETMA rack, single-bus personality module, 180W power supply |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BA36R-RC |  |  |  |
| UltraSCSI single-ended RETMA rack, split-bus personality module, 180W power supply |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BA36R-RD |  |  |  |
| 180W power supply, includes 120V North American and 240V IEC power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CK-BA35X-HH |  |  |  |
| Compaq CustomSystems Support |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Cabinets containing systems and storage must be assembled by qualified assemblers: Compaq Manufacturing, Compaq CustomSystems, or an authorized Compaq Certified Integration Partner. - For custom integration of systems, options, cabinet expansion offerings, and additional Compaq and third party devices, or for other cabinets, doors, and trim kits of a different color, contact Compaq CustomSystems at 1-800-AT COMPAQ, or your local authorized Compaq Certified Integration Partner. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step 8 External Storage  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UltraSCSI Configuration Rules |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - UltraSCSI configurations require UltraSCSI components (controllers, adapters, shelves, disks, and cables) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - UltraSCSI adapters and RAID controllers support UltraSCSI disks at UltraSCSI speeds in UltraSCSI top gun blue shelves (DS-BA356-xx) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UltraSCSI StorageWorks Pedestals |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - UltraSCSI top gun blue pedestals provide support for UltraSCSI disk drives; pedestal is available in single-bus or split-bus configurations and includes a personality module and 180W power supply |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - UltraSCSI shelf supports: seven 3.5 hard disk drives, or one 5.25 device and four 3.5 hard disk drives, or two 5.25 devices and one 3.5 hard disk drive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Installation of redundant power supply reduces number of 3.5 drive slots by one |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UltraSCSI single-ended pedestal, single-bus personality module, 180W power supply |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-BA356-KF |  |  |  |  |
| UltraSCSI single-ended pedestal, split-bus personality module, 180W power supply |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-BA356-KG |  |  |  |  |
| UltraSCSI pedestal, single-channel differential personality module, 180W power supply |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-BA356-KH |  |  |  |  |
| 180W power supply, includes 120V North American and 240V IEC power cord |  |  |  |  |  |  |  |  |  |  |  |  |  | CK-BA35X-HH |  |  |  |  |
| Tabletop Tape Drives |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Tape drives require UltraSCSI adapter and country-specific power cord |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12/24-GB 4-mm DAT narrow single-ended tabletop SCSI tape, includes 120V North American power cord, requires BN31W-xx SCSI cable |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-TLZ10-DB |  |  |  |  |
| 35/70-GB DLT single-ended tabletop tape drive, requires BN21K-xx SCSI cable |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-TZ89N-TA |  |  |  |  |
| 50/100-GB US AIT tabletop tape drive (includes SCSI cable and power cord) |  |  |  |  |  |  |  |  |  |  |  |  |  | 157767-001 |  |  |  |  |
| 50/100-GB International AIT tabletop tape drive (includes SCSI cable and power cord) |  |  |  |  |  |  |  |  |  |  |  |  |  | 157767-B31 |  |  |  |  |
| 50/100-GB Japan AIT tabletop tape drive (includes SCSI cable and power cord) |  |  |  |  |  |  |  |  |  |  |  |  |  | 157767-291 |  |  |  |  |
| Storage Controllers/Adapters for External StorageWorks Shelves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Storage controllers require Tru64 UNIX V4.0F or OpenVMS V7.1-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: The following controllers are only supported with StorageWorks shelves (not supported with the 3X-BA611-4D disk drive cage) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Maximum # Supported |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Tru64 UNIX |  |  |  | OpenVMS |  |  |  |  |  |  |
| PCI Fibre Channel adapter (KGPSA-BC), (uses one slot) |  |  |  |  |  |  |  | 4 |  |  |  | 4 |  |  |  |  | 380574-001 |  |
| Fibre Channel cable (BNGBX-xx) xx=02, 05, 15, 30, 50 meters x=1, 2, 3, 4, 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 234457-B2x |  |
| PCI 2-port UltraSCSI single-ended adapter and 10/100 Mb Ethernet combo adapter (uses one PCI slot); includes internal 68-pin and 50-pin HD connectors and two external 68-pin VHDCI SCSI connectors, requires BN37A-xx cable to connect to single-ended UltraSCSI shelf |  |  |  |  |  |  |  | 4 |  |  |  | 4 |  |  |  |  | KZPCM-DA |  |
| VHDCI male-to-VHDCI male UltraSCSI cable, xx = 02, 2E [2.5], 03, 05, 10, 20 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN37A-xx |  |
| PCI 1-port UltraSCSI single-ended adapter (uses one PCI slot); includes internal 68-pin and 50-pin HD connectors and external 68-pin HD connector, requires BN38C-xx cable (2 to 20 m) to connect to single-ended UltraSCSI shelf |  |  |  |  |  |  |  |  | 4 |  |  | 4 |  |  |  |  | KZPBA-CA |  |
| PCI 1-port UltraSCSI differential adapter (uses one PCI slot); includes external 68-pin HD connector, requires BN38C-xx cable (2 to 25 m) to connect adapter to differential UltraSCSI shelf or RAID controller |  |  |  |  |  |  |  |  | 4 |  |  | 4 |  |  |  |  | KZPBA-CB |  |
| 68-pin HD male-to-VHDCI male UltraSCSI cable; xx = 02, 03, 05, 10, 20, 25 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN38C-xx |  |
| PCI 1-port UltraSCSI RAID controller with 4-MB cache memory (uses one PCI slot), includes RAID Array 230/Plus Subsystem software and documentation kit; includes VHDCI female connector; tape drives not supported; 18.2-GB disk drives not supported, requires BN37A-xx cable (.3 to 20 m) to connect to single-ended UltraSCSI shelf |  |  |  |  |  |  |  |  | 4 |  | 4 |  |  |  |  | KZPAC-AA |  |  |
| PCI 3-port UltraSCSI RAID controller with 4-MB cache memory (uses one PCI slot), includes RAID Array 230/Plus Subsystem Software and documentation kit; tape drives not supported; 18.2-GB disk drives not supported; SCSI cable required for each active port; requires BN37X-xx cable (.3 to 20 m) to connect to single-ended UltraSCSI shelf, and BN31K-0E for external third port connection |  |  |  |  |  |  |  |  | 2 |  | 2 |  |  |  |  | KZPAC-CA |  |  |
| Same as KZPAC-CA with 8-MB cache memory |  |  |  |  |  |  |  |  | 2 |  | 2 |  |  |  |  | KZPAC-CB |  |  |
| Cache battery backup for KZPAC controller |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | KZPSC-UB |  |  |
| Factory configured RAID of KZPAC with StorageWorks shelves Note: Factory configured RAID sets for disks in single-bus StorageWorks shelves connected to KZPAC require completion of questionnaire located at http://www.digital.com/alphaserver/fcr |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YS-ASDRD-BA |  |  |
| VHDCI male-to-VHDCI male UltraSCSI cable; xx = 02, 2E [2.5], 03, 05, 10, 20, 25 m Note: Manufacturing may substitute BN37A-02 cable for specific cabinet configurations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN37A-xx |  |  |
| SCSI cable/bulkhead assembly kit, supports one third-port connection from KZPAC-CA/CB, allows external connection of one third port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BN31K-0E |  |  |
| SCSI cable/bulkhead assembly kit, supports two third-port connections from KZPAC-CA/CB, allows external connection of two third ports |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | KZPAC-SB |  |  |
| Removable Media Devices for External StorageWorks Shelves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - For maximum performance, tape libraries must be configured with Fast Wide Differential (FWD) controllers for tape backup |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12/24-GB 4 mm DAT narrow SCSI tape drive in StorageWorks carrier |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-TLZ10-VA |  |  |  |
| 35/70-GB DLT single-ended tape drive in StorageWorks carrier |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-TZ89N-VW |  |  |  |
| 35/70-GB AIT tape drive in StorageWorks carrier |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-AIT35-VW |  |  |  |
| 600-MB 32X CD-ROM drive in StorageWorks carrier |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RRD47-VA |  |  |  |
| BA370 Rackmount Enclosure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Mandatory rear cabinet extender ordered for systems installed in M-series cabinets (H9A10 and H9A15) allows two BA370 enclosures to be installed front to back in cabinet. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Host adapter, RAID Array controller, cache battery, platform specific Solutions Software Kit, and disk drives must be ordered separately. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BA370 front mount rackmount enclosure with five 180W power supplies, includes RETMA rackmount kit; first BA370 enclosure must be front mounted in cabinet, 20U height |  |  |  |  |  |  |  |  |  |  |  |  |  | 2T-BA370-AS |  |  |  |  |
| BA370 rear mount rackmount enclosure with five 180W power supplies, includes RETMA rackmount kit; second BA370 enclosure must be rear mounted in cabinet, 20U height |  |  |  |  |  |  |  |  |  |  |  |  |  | 2T-BA370-AR |  |  |  |  |
| Factory configured RAID of rackmounted BA370/HSZ70 |  |  |  |  |  |  |  |  |  |  |  |  |  | YS-ASCRD-AA |  |  |  |  |
| Note: Factory configured RAID sets for disks in rackmounted BA370 StorageWorks shelf controlled by one or two HSZ70s require completion of questionnaire located at http://www.digital.com/alphaserver/fcr |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| StorageWorks Shelf  Requires selection of UltraSCSI adapter and SCSI cable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| StorageWorks UltraSCSI single-bus shelf kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BA36R-RC |  |  |  |
| StorageWorks UltraSCSI split-bus shelf kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BA36R-RD |  |  |  |
| External Disk Drive (Supported in StorageWorks Shelf) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1-GB 7200 RPM UltraSCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1DA-VW |  |  |  |  |
| 18.2-GB 7200 RPM UltraSCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1EA-VW |  |  |  |  |
| 36.4-GB 7200 RPM UltraSCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1FB-VW |  |  |  |  |
| 9.1-GB 10000 RPM UltraSCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1DD-VW |  |  |  |  |
| 18.2-GB 10000 RPM UltraSCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1ED-VW |  |  |  |  |
|  | 36.4-GB 10,000 rpm UltraSCSI drive |  |  |  |  |  |  |  |  |  |  |  |  |  | DS-RZ1FC-VW |  |  |  |
| Note- A maximum of (6) 18.2-GB and 36.4 GB 10000 RPM drives can be configured in a Storageworks shelf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step 9 UltraSCSI RAID Array Controllers and UltraSCSI RAID Packaged Solutions Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAID Array 3000 Rackmount Controller Shelf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Selection of RA3000 Solutions Software Kit is required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Supported SCSI adapter: KZPBA-CB PCI 1-port UltraSCSI differential adapter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RA3000 rackmount controller shelf, blue, with one controller, 110V Includes: UltraSCSI BA356 controller shelf, one HSZ22 2-channel controller with 16-MB cache, integrated Environmental Monitoring Unit (EMU), two 180W power supplies, two dual-port host differential UltraSCSI I/O SBBs, dual-channel UltraSCSI single-ended personality modules, one 120V 1000VA rackmount UPS, 5-m host SCSI cable (BN37A-05), BN38E-0B adapter, UPS serial cable, PC connection serial cable (DB-9), two 68-pin HD tri-link adapters (H8861-AA), two North American power cords, and RETMA mounting bracket (disk shelves and disks not included) Requires: Solutions Software Kit for platform, host adapter, RA3000 disk shelf, and disks Options: Second HSZ22 controller, up to four additional RA3000 disk shelves, cache memory upgrade |  |  |  |  |  |  |  |  |  |  |  |  | DS-SWXRA-GH |  |  |  |  |  |
| RA3000 rackmount controller shelf, blue, with one controller, 230V Includes: UltraSCSI BA356 controller shelf, one HSZ22 2-channel controller with 16-MB cache, integrated Environmental Monitoring Unit (EMU), two 180W power supplies, two dual-port host differential UltraSCSI I/O SBBs, dual-channel UltraSCSI single-ended personality modules, one 230V 1000VA rackmount UPS, 5-m host SCSI cable (BN37A-05), BN38E-0B adapter, UPS serial cable, PC connection serial cable (DB-9), two 68-pin HD tri-link adapters (H8861-AA), RETMA mounting bracket (disk shelves and disks not included) Requires: Solutions Software Kit for platform, two country-specific power cords, host adapter, RA3000 disk shelf, and disks Options: Second HSZ22 controller, up to four additional RA3000 disk shelves, cache memory upgrade |  |  |  |  |  |  |  |  |  |  |  |  | DS-SWXRA-GK |  |  |  |  |  |
| RAID Array 3000 Rackmount Disk Shelf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Select one to four disk shelves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Third and fourth DS-SWXRA-GN disk shelf ordered will be installed in rear of M-series cabinet; rear mounted shelves require that a 2-m BN37A-02 SCSI cable be ordered |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RA3000 rackmount disk shelf, blue, includes UltraSCSI BA356 disk shelf, two 180 W power supplies, single-ended UltraSCSI personality card, one half meter SCSI cable (BN37A-0E), two North American power cords, and RETMA mounting bracket (disks not included); order two country-specific power cords for non-North American use |  |  |  |  |  |  |  |  |  |  |  |  | DS-SWXRA-GN |  |  |  |  |  |
| HSZ22 Controller |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Note that second HSZ22 controller ordered is field installed only, not factory installed; requires BN37A-0E SCSI jumper cable be ordered separately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Second HSZ22 controller with 16-MB cache and two 16-MB SIMMs for mirror cache, one BN37A-05 5-m UltraSCSI cable, one BN38E-0B cable adapter  field installable only; note that second HSZ22 controller does not require additional Solution Software Kit |  |  |  |  |  |  |  |  |  |  |  |  | DS-HSZ22-AA |  |  |  |  |  |
| Second HSZ22 controller with 32-MB cache and two 32-MB SIMMs for mirror cache, one BN37A-05 5-m UltraSCSI cable, one BN38E-0B cable adapter  field installed only; note that second HSZ22 controller does not require additional Solution Software Kit |  |  |  |  |  |  |  |  |  |  |  |  | DS-HSZ22-AB |  |  |  |  |  |
| RA3000 Solutions Software Kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - StorageWorks Command Console (SWCC) software |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Platform specific software |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Installation scripts and installation documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RA3000 Solutions Software Kit for Tru64 UNIX |  |  |  |  |  |  |  |  |  |  |  |  | QB-5TWAB-SA |  |  |  |  |  |
| RA3000 Solutions Software Kit for OpenVMS |  |  |  |  |  |  |  |  |  |  |  |  | QB-5TWAC-SA |  |  |  |  |  |
| HSG80/HSZ80 External Controllers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fibre Channel controller (DS-HSG80-BJ), includes 2-MB cache expandable to 128 MB; requires 128697-B21 HSG80 ACS V8.4F software kit or 128698-B21 V8.4P software kit for each HSG80 ordered; |  |  |  |  |  |  |  |  |  |  |  |  | 380672-B21 |  |  |  |  |  |
| Fibre Channel controller (DS-HSG80-BH), includes 64-MB cache expandable to 128 MB; requires 128697-B21 HSG80 ACS V8.4F software kit or 128698-B21 V8.4P software kit for each HSG80 ordered; |  |  |  |  |  |  |  |  |  |  |  |  | 380671-B21 |  |  |  |  |  |
| UltraSCSI controller (DS-HSZ80-AH), includes 64-MB cache expandable to 128 MB; requires 400569-001 for Tru64 UNIX or 400571-001 for OpenVMS |  |  |  |  |  |  |  |  |  |  |  |  | 400564-B21 |  |  |  |  |  |
| UltraSCSI controller (DS-HSZ80-AJ), includes 256-MB cache expandable to 512 MB; requires 400569-001 for Tru64 UNIX or 400571-001 for OpenVMS |  |  |  |  |  |  |  |  |  |  |  |  | 400565-B21 |  |  |  |  |  |
| ESA 10000 , RAID Array 7000, RAID Array 8000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Enterprise Storage Array 10000 and RAID Array 7000 (HSZ70) product set are supported on AlphaServer DS20E systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Supported SCSI adapter for HSZ80/HSZ70  KZPBA-CB PCI 1-port UltraSCSI differential adapter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Supported SCSI adapter for HSG80  KGPSA-BC PCI 1-port Fiber-Channel adapter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step 10 Graphics Adapters Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - RAID Configuration Utility is accessible via console port for Tru64 UNIX and OpenVMS systems; optional graphics capability available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Select graphics adapter, monitor, and country-specific keyboard if required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Maximum # Supported |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Tru64 UNIX |  | OpenVMS |  |  |  |  |  |  |
| ELSA Gloria synergy graphics with 8-MB SGRAM 1600 x 1200, 2D/3D graphics accelerator |  |  |  |  |  |  |  |  |  | 3 |  | 1 |  |  | SN-PBXGK-BB |  |  |  |
| PowerStorm 4D51T advanced 3D graphics accelerator |  |  |  |  |  |  |  |  |  | 4 |  | - |  |  | SN-PBXGI-AD |  |  |  |
| PowerStorm 300 3D multi-head graphics accelerator with 15-MB frame buffer memory and 16-MB texture memory |  |  |  |  |  |  |  |  |  | 3 |  | - |  |  | SN-PBXGD-AD |  |  |  |
| PowerStorm 350 3D multi-head graphics accelerator with 30-MB frame buffer memory and 32-MB texture memory |  |  |  |  |  |  |  |  |  | 3 |  | - |  |  | SN-PBXGD-AE |  |  |  |
| Step 11 Monitors Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - RAID Configuration Utility is accessible via the console port for Tru64 UNIX and OpenVMS systems; optional graphics capability is available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Graphics monitors other than those listed can be used if compatible with SVGA graphics ordered with system |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Selection of a video extension cable and country-specific power cord is mandatory for all monitors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 Corporate Series auto-scan color monitor, VGA to 1024 x 768 @ 85 Hz; Northern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQV5-24 |  |  |  |  |
| Same as above except Southern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQV5-11 |  |  |  |  |
| 17 (16.0 viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 @75 Hz, TCO 99, MPR-II, Energy Star, attached 1.8-m video cable; Northern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQP7-24 |  |  |  |  |
| Same as above except Southern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQP7-23 |  |  |  |  |
| 21 (19.6 viewable image size) auto-scanning color monitor, Trinitron CRT, 0.28 mm aperture grill pitch, VGA to 1600 x 1200 @85 Hz NI, TCO 95, Energy Star, includes 1.8-m video cable; Northern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQP1-24 |  |  |  |  |
| Same as above except Southern Hemisphere without power cord |  |  |  |  |  |  |  |  |  |  |  |  |  | 3R-VRQP1-23 |  |  |  |  |

| Video Extension Cable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.8-m video extension cable—mandatory for each monitor ordered |  |  |  |  |  |  | BN39C-02 |  |  |  |  |  |  |  |
| Monitor Power Cords |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| North American, 120V, 75 |  |  |  |  |  |  | BN26J-1K |  |  |  |  |  |  |  |
| Australia, New Zealand, 2.5m |  |  |  |  |  |  | BN19H-2E |  |  |  |  |  |  |  |
| Central Europe, 2.5m |  |  |  |  |  |  | BN19C-2E |  |  |  |  |  |  |  |
| UK, Ireland, 2.5m |  |  |  |  |  |  | BN19A-2E |  |  |  |  |  |  |  |
| UK, Ireland, 2.5m |  |  |  |  |  |  | BN19A-2E |  |  |  |  |  |  |  |
| Switzerland, 2.5m |  |  |  |  |  |  | BN19E-2E |  |  |  |  |  |  |  |
| Denmark, 2.5m |  |  |  |  |  |  | BN19K-2E |  |  |  |  |  |  |  |
| Italy, 2.5m |  |  |  |  |  |  | BN19M-2E |  |  |  |  |  |  |  |
| Egypt, India, South Africa, 2.5m |  |  |  |  |  |  | BN19S-2E |  |  |  |  |  |  |  |
| Israel, 2.5m |  |  |  |  |  |  | BN18L-2E |  |  |  |  |  |  |  |
| Japan, 2.5m |  |  |  |  |  |  | 3X-BN46F-02 |  |  |  |  |  |  |  |
| Step 12 Networks and Communications  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Controllers/adapters require Tru64 UNIX V4.0F or OpenVMS V7.1-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - See Step 2 for mandatory supported Ethernet adapters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Maximum # Supported |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Tru64 UNIX |  | OpenVMS |  |  |  |  |  |  |  |  |  |
| FE FX Daughtercard for 3X-DE602-AA (3X-DE602-AA is a prerequisite) |  |  | 2 |  | 2 |  |  |  |  |  |  | 3X-DE602-FA |  |  |
| PCI Gigabit Ethernet adapter, SC connectors, MMF, no boot support |  |  | 2 |  | 1 |  |  |  |  |  |  | DEGPA-SA |  |  |
| SC-to-SC dual fiber optic cable |  |  |  |  |  |  |  |  |  |  |  | BN34B-xx |  |  |
| PCI FDDI adapter, single attachment station (SAS), multi-mode fiber (MMF), SC connector |  |  | 2 |  | 2 |  |  |  |  |  |  | 3X-DEFPA-AC |  |  |
| PCI FDDI adapter, DAS, MMF, SC connector |  |  | 2 |  | 2 |  |  |  |  |  |  | 3X-DEFPA-DC |  |  |
| MIC-SC dual fiber optic cable |  |  |  |  |  |  |  |  |  |  |  | BN34D-xx |  |  |
| SC-to-SC dual fiber optic cable |  |  |  |  |  |  |  |  |  |  |  | BN34B-xx |  |  |
| PCI FDDI adapter, SAS, TP-PMD, requires BN25H-03 cable |  |  | 2 |  | 2 |  |  |  |  |  |  | 3X-DEFPA-UC |  |  |
| PCI FDDI dual adapter, DAS, TP-PMD, requires BN25H-03 cable |  |  | 2 |  | 2 |  |  |  |  |  |  | 3X-DEFPA-MC |  |  |
| 3m (9.8") FDDI cross-over cable |  |  |  |  |  |  |  |  |  |  |  | BN25H-03 |  |  |
| PCI 4-port asynchronous controller |  |  | 2 |  | 2 |  |  |  |  |  |  | PBXDA-AA |  |  |
| PCI 8-port asynchronous controller, cable included |  |  | 2 |  | 2 |  |  |  |  |  |  | PBXDA-AB |  |  |
| PCI 16-port asynchronous controller, cable included |  |  | 2 |  | 2 |  |  |  |  |  |  | PBXDA-AC |  |  |
| PCI-to-ATM 155 Mb adapter - Fiber |  |  | 4 |  | - |  |  |  |  |  | 3X-DAPBA-FA |  |  |  |
| PCI-to-ATM 155 Mb adapter - UTP |  |  | 4 |  | - |  |  |  |  |  | 3X-DAPBA-UA |  |  |  |
| PCI-to-ATM 622 Mb adapter - Fiber |  |  | 2 |  | - |  |  |  |  |  | 3X-DAPCA-FA |  |  |  |
| Asynchronous multi-port MUX expander XEM |  |  |  |  |  |  |  |  |  |  |  | CXI01-AB |  |  |
| Step 13  MEMORY CHANNEL Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tru64 UNIX Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Requires a minimum of Tru64 UNIX V4.0F with TruCluster V1.6 or later |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Each system node in the cluster requires a TruCluster Production Server (QB-3RLAE-KA) or TruCluster MEMORY CHANNEL software license (QB-4ZCAE-AA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - TruCluster MEMORY CHANNEL license (QB-4ZCAG-AA), normally used for high performance technical computing applications, not required if systems have a TruCluster Production Server license (QB-3RLAE-KA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OpenVMS Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Requires OpenVMS V7.1-2 or later and OpenVMS Cluster license (QL-MUZAE-AA) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Configuring Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Up to two PCI to System Area Network adapters (CCMAB-AA) are supported in a single system |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Two-node clusters can be configured by ordering a CCMAB-AA for each system and one BN29B-04 or BN29B-10 cable; the cable connects directly to the CCMAB-AA in each system |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - For three or four system nodes, order one CCMAB-AA adapter, one BN29B-04 or BN29B-10 cable and one CCMHB-AA; make connections from the CCMAB-AA in a system to the CCMHB-AA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - CCMHB-AA includes four CCMLB-AA line cards and supports up to four nodes; expansion up to eight system nodes can be achieved by adding up to four additional CCMLB-AA line cards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - If two CCMAB-AA’s are configured in each system, a second CCMHB-AA is required for three or more nodes; in two-node clusters, the CCMAB-AA’s may be directly connected |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MEMORY CHANNEL Controller |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCI System Area Network Controller, maximum two |  |  |  |  |  |  |  |  |  | CCMAB-AA |  |  |  |  |
| System Area Network Hub with four line cards; includes BN19P-2E power cord for Canada, Japan, and US operationsfor other regions, order appropriate power cord from list below |  |  |  |  |  |  |  |  |  | CCMHB-AA |  |  |  |  |
| Expansion line card for CCMHB hub |  |  |  |  |  |  |  |  |  | CCMLB-AA |  |  |  |  |
| 4-m cable for CCMAB and CCMHB |  |  |  |  |  |  |  |  |  | BN39B-04 |  |  |  |  |
| 10-m cable for CCMAB and CCMHB |  |  |  |  |  |  |  |  |  | BN39B-10 |  |  |  |  |
| TruCluster Production Server Software for Tru64 UNIX |  |  |  |  |  |  |  |  |  | QB-3RLAE-AA |  |  |  |  |
| Tru64 UNIX Driver for MEMORY CHANNEL license |  |  |  |  |  |  |  |  |  | QB-4ZCAE-AA |  |  |  |  |
| OpenVMS Cluster license for Alpha systems |  |  |  |  |  |  |  |  |  | QL-MUZAE-AA |  |  |  |  |
| MEMORY CHANNEL Power Cords |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ireland, United Kingdom |  |  |  |  |  |  |  |  |  | BN19A-2E |  |  |  |  |
| Egypt, India |  |  |  |  |  |  |  |  |  | BN19S-2E |  |  |  |  |
| Central Europe |  |  |  |  |  |  |  |  |  | BN19C-2E |  |  |  |  |
| Israel |  |  |  |  |  |  |  |  |  | BN18L-2E |  |  |  |  |
| Switzerland |  |  |  |  |  |  |  |  |  | BN19E-2E |  |  |  |  |
| Italy |  |  |  |  |  |  |  |  |  | BN19M-2E |  |  |  |  |
| Denmark |  |  |  |  |  |  |  |  |  | BN19K-2E |  |  |  |  |
| Australia, New Zealand |  |  |  |  |  |  |  |  |  | BN19H-2E |  |  |  |  |
| Step 14 Keyboards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Keyboards are not included with system and must be ordered separately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Tru64 UNIX |  |  |  |  |  |  | OpenVMS |  |  |  |
| U.S./English keyboard |  |  |  | SN-LKQ47-AA |  |  |  |  |  |  | LK461-A2 |  |  |  |
| Belgian keyboard |  |  |  | SN-LKQ47-AB |  |  |  |  |  |  | LK461-AB |  |  |  |
| Canadian/French keyboard |  |  |  | SN-LKQ47-AC |  |  |  |  |  |  | LK461-AC |  |  |  |
| Danish keyboard |  |  |  | SN-LKQ47-AD |  |  |  |  |  |  | LK461-AD |  |  |  |
| UK keyboard |  |  |  | SN-LKQ47-AE |  |  |  |  |  |  | - |  |  |  |
| Finnish keyboard |  |  |  | SN-LKQ47-AF |  |  |  |  |  |  | LK461-AF |  |  |  |
| German keyboard |  |  |  | SN-LKQ47-AG |  |  |  |  |  |  | LK461-AG |  |  |  |
| Dutch keyboard |  |  |  | SN-LKQ47-AH |  |  |  |  |  |  | LK461-AH |  |  |  |
| Italian keyboard |  |  |  | SN-LKQ47-AI |  |  |  |  |  |  | LK461-AI |  |  |  |
| Swiss/French keyboard |  |  |  | SN-LKQ47-AK |  |  |  |  |  |  | LK461-AK |  |  |  |
| Swiss/German keyboard |  |  |  | - |  |  |  |  |  |  | LK461-AL |  |  |  |
| Swedish keyboard |  |  |  | SN-LKQ47-AM |  |  |  |  |  |  | LK461-AM |  |  |  |
| Norwegian keyboard |  |  |  | SN-LKQ47-AN |  |  |  |  |  |  | LK461-AN |  |  |  |
| French keyboard |  |  |  | SN-LKQ47-AP |  |  |  |  |  |  | LK461-AP |  |  |  |
| Canadian/English keyboard |  |  |  | - |  |  |  |  |  |  | LK461-AQ |  |  |  |
| Latin-American keyboard |  |  |  | SN-LKQ47-AR |  |  |  |  |  |  | - |  |  |  |
| Spanish keyboard |  |  |  | SN-LKQ47-AS |  |  |  |  |  |  | LK461-AS |  |  |  |
| Hebrew keyboard |  |  |  | SN-LKQ47-AT |  |  |  |  |  |  | LK461-AT |  |  |  |
| Portuguese keyboard |  |  |  | SN-LKQ47-AV |  |  |  |  |  |  | LK461-AV |  |  |  |
| BHCSY keyboard |  |  |  | SN-LKQ47-AX |  |  |  |  |  |  | - |  |  |  |
| International keyboard |  |  |  | SN-LKQ47-BA |  |  |  |  |  |  | - |  |  |  |
| Greek keyboard |  |  |  | SN-LKQ47-BH |  |  |  |  |  |  | LK461-BH |  |  |  |
| Taiwanese keyboard |  |  |  | SN-LKQ47-BI |  |  |  |  |  |  | - |  |  |  |
| Korean keyboard |  |  |  | SN-LKQ47-BK |  |  |  |  |  |  | - |  |  |  |
| Romanian keyboard |  |  |  | - |  |  |  |  |  |  | LK461-BL |  |  |  |
| Polish keyboard |  |  |  | SN-LKQ47-BP |  |  |  |  |  |  | LK461-BP |  |  |  |
| Hungarian keyboard |  |  |  | SN-LKQ47-BQ |  |  |  |  |  |  | LK461-BQ |  |  |  |
| Arabic keyboard |  |  |  | SN-LKQ47-BR |  |  |  |  |  |  | - |  |  |  |
| Cyrillic keyboard |  |  |  | SN-LKQ47-BT |  |  |  |  |  |  | LK461-BT |  |  |  |
| Turkish keyboard |  |  |  | SN-LKQ47-BU |  |  |  |  |  |  | LK461-BU |  |  |  |
| Czech keyboard |  |  |  | SN-LKQ47-BV |  |  |  |  |  |  | LK461-BV |  |  |  |
| Turkish/French keyboard |  |  |  | - |  |  |  |  |  |  | LK461-BW |  |  |  |
| Yugoslavian keyboard |  |  |  | - |  |  |  |  |  |  | LK461-BY |  |  |  |
| Thai keyboard |  |  |  | SN-LKQ47-CB |  |  |  |  |  |  | - |  |  |  |
| S. Chinese keyboard |  |  |  | SN-LKQ47-CV |  |  |  |  |  |  | - |  |  |  |
| Slovak keyboard |  |  |  | SN-LKQ47-CZ |  |  |  |  |  |  | LK461-CZ |  |  |  |
| Keyboard or Mouse Extension Cable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 ft keyboard or mouse extension cable; to extend both keyboard and mouse order 2 cables |  |  |  |  |  |  |  |  |  |  |  | 17-04016-01 |  |  |
| Step 15 Console Terminal  Optional |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Any terminal can be used as a console device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Third-party console devices require an EIA-232 25-pin DSUB connector |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VT510 video terminal; requires H857-x cable |  |  |  |  |  |  |  |  |  |  |  | VT510-xx |  |  |
| Console cable, female connector |  |  |  |  |  |  |  |  |  |  |  | H857-A |  |  |
| Console cable, male connector |  |  |  |  |  |  |  |  |  |  |  | H857-D |  |  |
| Console cable, male connector (IBM compatible) |  |  |  |  |  |  |  |  |  |  |  | H897-E |  |  |
| Step 16  Software  Required selection of media and documentation for first system on site |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Software Processor Code = E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tru64 UNIX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Tru64 UNIX systems include Tru64 UNIX Traditional Unlimited User license, Server Extension, Internet Access Software kit, and Netscape's Enterprise Server 3.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tru64 UNIX media and online documentation on CD-ROM |  |  |  |  |  |  |  | QA-MT4AA-H8 |  |  |  |  |  |  |
| Tru64 UNIX full hard copy documentation |  |  |  |  |  |  |  | QA-MT4AA-GZ |  |  |  |  |  |  |
| StorageWorks Software Package, includes licenses for Logical Storage Manager and AdvFS Utilities |  |  |  |  |  |  |  | QB-5RXAE-AA |  |  |  |  |  |  |
| TruCluster Available Server License |  |  |  |  |  |  |  | QL-05SAE-AA |  |  |  |  |  |  |
| Advanced Server for Tru64 UNIX, 25 client concurrent use license |  |  |  |  |  |  |  | QL-5U29M-3D |  |  |  |  |  |  |
| Advanced Server for Tru64 UNIX, 50 client concurrent use license |  |  |  |  |  |  |  | QL-5U29M-3E |  |  |  |  |  |  |
| Advanced Server for Tru64 UNIX, 100 client concurrent use license |  |  |  |  |  |  |  | QL-5U29M-3F |  |  |  |  |  |  |
| Advanced Server for Tru64 UNIX, 250 client concurrent use license |  |  |  |  |  |  |  | QL-5U29M-3G |  |  |  |  |  |  |
| Advanced Server for Tru64 UNIX, 500 client concurrent use license |  |  |  |  |  |  |  | QL-5U29M-3H |  |  |  |  |  |  |
| Layered products media and documentation for Tru64 UNIX on CD-ROM |  |  |  |  |  |  |  | QA-054AA-H8 |  |  |  |  |  |  |
| DECnet/OSI end-system license for Tru64 UNIX |  |  |  |  |  |  |  | QL-MTJAE-AA |  |  |  |  |  |  |
| DECnet/OSI extended function license for Tru64 UNIX |  |  |  |  |  |  |  | QL-MTKAE-AA |  |  |  |  |  |  |
| OpenVMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - OpenVMS systems include OpenVMS base license and Enterprise Integration Package V2.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - OpenVMS Concurrent Use Licenses are not specific to a single system and can be moved between systems at user discretion; OpenVMS Concurrent Use Licenses can also be shared in a mixed OpenVMS VAX and OpenVMS Alpha Cluster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Concurrent Use 1-user license |  |  |  |  |  |  |  |  | QL-MT3AA-3B |  |  |  |  |  |
| Concurrent Use 2-user license |  |  |  |  |  |  |  |  | QL-MT3AA-3C |  |  |  |  |  |
| Concurrent Use 4-user license |  |  |  |  |  |  |  |  | QL-MT3AA-3D |  |  |  |  |  |
| Concurrent Use 8-user license |  |  |  |  |  |  |  |  | QL-MT3AA-3E |  |  |  |  |  |
| Concurrent Use 16-user license |  |  |  |  |  |  |  |  | QL-MT3AA-3F |  |  |  |  |  |
| Concurrent Use 32-user license |  |  |  |  |  |  |  |  | QL-MT3AA-3G |  |  |  |  |  |
| Concurrent Use 64-user license |  |  |  |  |  |  |  |  | QL-MT3AA-3H |  |  |  |  |  |
| Concurrent Use 128-user license |  |  |  |  |  |  |  |  | QL-MT3AA-3J |  |  |  |  |  |
| Concurrent Use 256-user license |  |  |  |  |  |  |  |  | QL-MT3AA-3K |  |  |  |  |  |
| Traditional unlimited user license |  |  |  |  |  |  |  |  | QL-MT2AQ-AA |  |  |  |  |  |
| OpenVMS media and documentation on CD-ROM |  |  |  |  |  |  |  |  | QA-MT1AA-H8 |  |  |  |  |  |
| OpenVMS full hard copy documentation |  |  |  |  |  |  |  |  | QA-001AA-GZ |  |  |  |  |  |
| Layered products media and documentation for OpenVMS on CD-ROM, includes Enterprise Integration Server for OpenVMS media and documentation |  |  |  |  |  |  |  |  | QA-03XAA-H8 |  |  |  |  |  |
| DIGITAL Enterprise Integration Server for OpenVMS media and documentation |  |  |  |  |  |  |  |  | QA-5LVAA-H8 |  |  |  |  |  |
| DECnet extended function license for OpenVMS |  |  |  |  |  |  |  |  | QL-MTGAE -AA |  |  |  |  |  |
| DECnet end-system to extended function upgrade license for OpenVMS |  |  |  |  |  |  |  |  | QL-MTHAE-AA |  |  |  |  |  |
| OpenVMS Cluster license for Alpha systems |  |  |  |  |  |  |  |  | QL-MUZAE-AA |  |  |  |  |  |
| OpenVMS Volume Shadowing license |  |  |  |  |  |  |  |  | QL-2A1AE-AA |  |  |  |  |  |
| Step 17  Hardware and Software Supplemental Support Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Systems include 3-year hardware warranty, on-site, with 5 x 9, 24-hour response time; select optional Hardware Supplemental Support Services if required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Options ordered as part of "Initial System Sale" take on warranty of the enclosure; options added after "Initial System Sale" take on warranty of enclosure until enclosure warranty ends |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - All unique AlphaServer DS20E options have a minimum of 1-year Return-to-Compaq warranty |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hardware Supplemental Support Americas and Asia Pacific only |  | 1-GB memory or less |  |  |  | More than 1-GB memory |  |  |  |  |  |  |  |  |
| 5 x 9, 4-hour response time |  | FM-TC4HR-36 |  |  |  | FM-TF4HR-36 |  |  |  |  |  |  |  |  |
| 5 x 12, 4-hour response time |  | FM-TC512-36 |  |  |  | FM-TF512-36 |  |  |  |  |  |  |  |  |
| 6 x 16, 4-hour response time |  | FM-TC616-36 |  |  |  | FM-TF616-36 |  |  |  |  |  |  |  |  |
| 7 x 24, 4-hour response time |  | FM-TC724-36 |  |  |  | FM-TF724-36 |  |  |  |  |  |  |  |  |
| Years 1-5, next day, onsite |  | FM-TCXHW-60 |  |  |  | FM-TFXHW-60 |  |  |  |  |  |  |  |  |
| Years 1-5, 5 x 9, 4-hour response time |  | FM-TC4HR-60 |  |  |  | FM-TF4HR-60 |  |  |  |  |  |  |  |  |
| Years 1-5, 5 x 12, 4-hour response time |  | FM-TC512-60 |  |  |  | FM-TF512-60 |  |  |  |  |  |  |  |  |
| Years 1-5, 6 x 16, 4-hour response time |  | FM-TC616-60 |  |  |  | FM-TF616-60 |  |  |  |  |  |  |  |  |
| Years 1-5, 7 x 24, 4-hour response time |  | FM-TC724-60 |  |  |  | FM-TF724-60 |  |  |  |  |  |  |  |  |
| Installation Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hardware product installation for DS20E |  |  |  |  |  | FM-TC12X-IN |  |  |  |  |  |  |  |  |
| Add-on memory installation for DS20E |  |  |  |  |  | FM-TCMEM-IN |  |  |  |  |  |  |  |  |
| Add-on CPU installation for DS20E |  |  |  |  |  | FM-TCSMP-IN |  |  |  |  |  |  |  |  |
| Software Supplemental Support— Americas and Asia Pacific only |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Systems include 90-day Conformance to SPD and Telephone Advisory Support; select optional Software Supplemental Support Services if required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Software service upgrades for Tru64 UNIX include advisory and remedial software support with new version license rights for operating system for time period indicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Software service upgrades for OpenVMS include advisory and remedial software support with new version license rights for operating system and Enterprise Integration Package for time period indicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Software service upgrades for SMP upgrades running Tru64 UNIX or OpenVMS include new version license rights for Tru64 UNIX or OpenVMS SMP Upgrades for time period indicated; one required for each SMP upgrade |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Tru64 UNIX |  |  |  |  |  |  |  | OpenVMS |  |  |  |  |  |
| 5X9 12-month Software Supplemental Support | FM-D20U9-12 |  |  |  |  |  |  |  | FM-D20V9-12 |  |  |  |  |  |
| 5X9 36-month Software Supplemental Support | FM-D20U9–36 |  |  |  |  |  |  |  | FM-D20V9-36 |  |  |  |  |  |
| 5X9 60-month Software Supplemental Support | FM-D20U9-60 |  |  |  |  |  |  |  | FM- D20V9-60 |  |  |  |  |  |
| 7X24 12-month Software Supplemental Support | FM-D20US-12 |  |  |  |  |  |  |  | FM-D20VS-12 |  |  |  |  |  |
| 7X24 36-month Software Supplemental Support | FM-D20US–36 |  |  |  |  |  |  |  | FM- D20VS-36 |  |  |  |  |  |
| 7X24 60-month Software Supplemental Support | FM-D20US-60 |  |  |  |  |  |  |  | FM- D20VS-60 |  |  |  |  |  |
| 12-month Software Supplemental Support  DS20E SMP Upgrade | FM-D20UP-12 |  |  |  |  |  |  |  | FM-D20VP-12 |  |  |  |  |  |
| 36-month Software Supplemental Support  DS20E SMP Upgrade | FM-D20UP-36 |  |  |  |  |  |  |  | FM-D20VP-36 |  |  |  |  |  |
| 60-month Software Supplemental Support DS20E SMP Upgrade | FM-D20UP-60 |  |  |  |  |  |  |  | FM-D20VP-60 |  |  |  |  |  |

| Pedestal |  | Dimensions (HxWxD) | 18.5 x 8.85 x 27.5 in / 47.0 x 22.5 x 69.9 cm |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Shipping Dimensions | 24 x 26.25 x 40 in / 61.0 x 66.0 x 101.6 cm |  |  |  |  |
|  |  | Weight |  |  |  |  |  |
|  |  | Typical Configuration | 80 lbs/36 kg |  |  |  |  |
|  |  | Maximum Configuration | 88 lbs/ 40 kg |  |  |  |  |
|  |  | Shipping Weight |  |  |  |  |  |
|  |  | Nominal | 100 lbs/ 45 kg |  |  |  |  |
|  |  | Maximum | 110 lbs/ 50 kg |  |  |  |  |
|  |  | Clearances | Operating |  | Service |  |  |
|  |  | Front | 15 in/ 38.1 cm |  | 15 in/ 38.1 cm |  |  |
|  |  | Rear | 6 in/15 cm |  | 29.5 in/75 cm |  |  |
|  |  | Left Side | None |  | None |  |  |
|  |  | Right Side | None |  | None |  |  |
| Rackmount |  | Dimensions (HxWxD) | 8.75 x 17.5 x 26 in/22.2 x 44.5 x 66.0 cm (5U) |  |  |  |  |
|  |  | Shipping Dimensions | 24 x 26.25 x 40 in/61.0 x 66.0 x 101.6 cm |  |  |  |  |
|  |  | Weight |  |  |  |  |  |
|  |  | When lifting: |  |  |  |  |  |
|  |  | Nominal | 80 lbs/36 kg |  |  |  |  |
|  |  | Maximum | 86 lbs/39 kg |  |  |  |  |
|  |  | Total added to cabinet (brackets, slides, cables): |  |  |  |  |  |
|  |  | Nominal | 84 lbs/38 kg |  |  |  |  |
|  |  | Maximum | 88 lbs/40 kg |  |  |  |  |
|  |  | Shipping Weight |  |  |  |  |  |
|  |  | Nominal | 100 lbs/45 kg |  |  |  |  |
|  |  | Maximum | 110 lbs/50 kg |  |  |  |  |
|  |  | Clearances | Operating |  | Service |  |  |
|  |  |  | See specific cabinet requirements |  | Minimum 4 ft / 121.9 cm, 28 in / 71 cm withdrawal on rails |  |  |
| Cabinet |  |  | H9A10 M-Series |  | H9A15 M-Series |  |  |
|  |  | Dimensions (HxWxD) | 67 x 23.6 x 43.27 in/ 170 x 60 x 110 cm |  | 79 x 23.6 x 35.4 in 200 x 60 x 90 cm |  |  |
|  |  | Shipping Dimensions | 73 x 36 x 48 in / 185.5 x 91.5 x 122 cm |  | 85 x 36 x 48 in / 216 x 91.5 x 122 cm |  |  |
|  |  | Weight | Configuration dependent |  | 1000 lbs / 450 kg |  |  |
|  |  | Shipping Weight | Configuration dependent, maximum payload 1,000 lbs |  | 1056 lbs / 550 kg (normal) 1,408 lbs / 640 kg (maximum) |  |  |
| System Unit |  | Environmental |  |  |  |  |  |
|  |  | Temperature |  |  |  |  |  |
|  |  | Operating (Tower, Rackmount)1 | 50 to 95F/10 to 35C |  |  |  |  |
|  |  | Non-operating | Not tested |  |  |  |  |
|  |  | Storage (60 days) | –40 to –151F/–40 to –66C |  |  |  |  |
|  |  | Rate of change | 20F/hr/11C/hr |  |  |  |  |
|  |  | Relative Humidity |  |  |  |  |  |
|  |  | Operating | 20% to 80% |  |  |  |  |
|  |  | Non-operating | 20% to 80% |  |  |  |  |
|  |  | Storage (60 days) | 10% to 95% |  |  |  |  |
|  |  | Rate of change | 20% hr |  |  |  |  |
|  |  | Maximum Wet Bulb Temperature |  |  |  |  |  |
|  |  | Operating | 82F/28C |  |  |  |  |
|  |  | Storage (60 days) | 115F/46C |  |  |  |  |
|  |  | Maximum Dew Point Temperature |  |  |  |  |  |
|  |  | Operating | 36F/2C |  |  |  |  |
|  |  | Storage (60 days) | Not tested |  |  |  |  |
|  |  | Heat Dissipation | Nominal |  | Maximum |  |  |
|  |  | Tower | 550W, 1,878 BTU/hr |  | 780W, 2,664 BTU/hr |  |  |
|  |  | Pedestal |  |  |  |  |  |
|  |  | H9A10/H9A15 Cabinets | Configuration dependent |  |  |  |  |
|  |  | Air Flow and Quality |  |  |  |  |  |
|  |  | Intake location | Front |  |  |  |  |
|  |  | Exhaust location | Rear Tower, Pedestal, Rack; Rear/top H9A10/H9A15 |  |  |  |  |
|  |  | Particle size | N/A |  |  |  |  |
|  |  | Concentration | N/A |  |  |  |  |
|  |  | Altitude |  |  |  |  |  |
|  |  | Operating 2 | 10,000 f/3,037m |  |  |  |  |
|  |  | Non-operating | 40,000 ft/12,192m |  |  |  |  |
|  |  | Acoustics (Declared values per ISO 9296 and ISO 7779) |  |  |  |  |  |
|  |  | Acoustics 3 | LWAd3B |  |  | LnAm5dBA (bystander position) |  |
|  |  | Idle | 5.8 |  |  | 44 |  |
|  |  | Operating | 6.3 |  |  | 49 |  |
|  |  | Vibration |  |  |  |  |  |
|  |  | Operating | 10-500 Hz .1 G peak |  |  |  |  |
|  |  | Non-operating | 1.03 Grms 5-300 Hz |  |  |  |  |
| System Unit (continued) |  | Electrical (Power supplies are universal, PFC, auto ranging, 100/240 Vac) |  |  |  |  |  |
|  |  | Nominal Voltage (Vac) | 100 | 120 |  |  | 200-240 |
|  |  | Voltage Range (Vac) (temporary condition) | 90-100 | 110-128 |  |  | 180-250 |
|  |  | Power Source Phase | Single | Single |  |  | Single |
|  |  | Nominal Frequency (Hz) | 50/60 | 50/60 |  |  | 50/60 |
|  |  | Frequency Range (Hz) | 49 to 51/59 to 61 | 49 to 51/59 to 61 |  |  | 49 to 51/59 to 61 |
|  |  | RMS Current (maximum steady state) |  |  |  |  |  |
|  |  | Pedestal and Rackmount |  |  |  |  |  |
|  |  | Single power cord | 6.6A | 5.5A |  |  | 3.0A |
|  |  | Maximum VA | 780 | 765 |  |  | 730 |
|  |  | M-series Cabinet (configuration dependent) |  |  |  |  |  |
|  |  | Nominal voltage (Vac) | 100 | 120 |  |  | 220-240 |
|  |  | Each power cord | 24A | 24A |  |  | 16A |
|  |  | Power Cords |  |  |  |  |  |
|  |  | Pedestal | 1 (75 in / 190 cm) |  | IEC 320 C13 to NEMA 5-15 (N. America) or IEC 320 C13 to country-specific |  |  |
|  |  | Rackmount | 1 (14 ft 10 in / 452 cm) |  | IEC 320 C13 to NEMA 5-15 (N. America) or IEC 320 C13 to IEC 320 C14 (other countries) |  |  |
|  |  | Cabinet | 2 (10 ft 10 in / 330 cm) |  | 120V non-removable NEMA L5-30P or 200-240V non-removable IEC 309 |  |  |
|  |  | Regulatory |  |  |  |  |  |
|  |  | Agency Approvals | UL: Listed to UL1950 (3rd edition) |  |  |  |  |
|  |  |  | CSA: Certified to CAN/CSA-C22.2 No. 950-M95 |  |  |  |  |
|  |  |  | TUV: EN 60950/A4: 1997 VDE 0805 GS marked |  |  |  |  |
|  |  |  | FCC: Part 15.B Class A |  |  |  |  |
|  |  |  | CE: EN55022, EN50082 |  |  |  |  |
|  |  |  | VCCI Class II ITE |  |  |  |  |
|  |  |  | BCIQ: CISPR22, CNS13438 |  |  |  |  |
|  |  |  | C-Tick: CISPR22, AS/NZS 3548 |  |  |  |  |
|  |  | Reviewed to | AS/NZ 3260:1993 Australian/New Zealand Standard |  |  |  |  |
|  |  |  | EN 60950/A4: 1997 European Norm |  |  |  |  |
|  |  |  | IEC 950 (2nd edition, 3rd amend) |  |  |  |  |

- Maximum operating temperature at sea level; reduce by 1.8F (1C) for each 600m (2,000 ft) above sea level.

- Higher altitudes are possible if maximum operating temperature is reduced (see Temperature); other restrictions may apply such as maximum permissible altitude for hard drives.

- Current values for specific configurations are available from Compaq, note that 1 B = 10 dBA.