### DIGITAL Personal Workstations 
V2.6¾5 January 1998
DIGITAL Systems and Options Catalog

DIGITAL Personal Workstations feature

| i-Series (Windows NT) | i+-Series (Windows NT) | a-Series (Windows NT) | au-Series (UNIX and OVMS)) |
| --- | --- | --- | --- |
| -  Short-tower enclosure | -  Short-tower enclosure | -  Short-tower enclosure | -  Short-tower enclosure |
| -  Pentium II 266/300MHz with Intel 440FX PCIset | -  Pentium II 266/300MHz, with Intel 440LX AGPset | -  DIGITAL 21164 433/500/600MHz | -  DIGITAL 21164 433/500/600MHz |
| -  512KB cache | -  512KB cache | -  0MB or 2MB cache | -  0MB or 2MB cache |
| -  Optional graphics | -  On-board 3Dlabs Permedia P2 graphics on MLB, 4MB | -  Optional graphics | -  Optional graphics |
| -  5 slots (2 PCI, 3 PCI/ISA) | -  6 slots (3 PCI, 2 PCI/ISA, 1 AGP) | -  5 slots (2 PCI, 3 PCI/ISA) | -  5 slots (2 PCI, 3 PCI/ISA) |
| -  4 SIMM banks (8 slots) | -  4 DIMM banks (4 slots) | -  3 DIMM banks (6 slots) | -  3 DIMM banks (6 slots) |
| -  300W power supply | -  300W power supply | -  300W power supply | -  300W power supply |
| -  1.44MB floppy drive | -  1.44MB floppy drive | -  1.44MB floppy drive | -  1.44MB floppy drive |
| -  CD-ROM drive | -  CD-ROM drive | -  CD-ROM drive | -  CD-ROM drive |
| -  Adaptec UW SCSI controller | -  On-board Qlogic UW SCSI controller | -  Qlogic UW SCSI controller | -  Qlogic UW SCSI controller |
| -  On-board EIDE controller | -  On-board UltraDMA controller | -  On-board EIDE controller for CD ROM only | -  On-board EIDE controller for CD-ROM only |
| -  On-board 16-bit audio | -  On-board 16-bit audio | -  On-board 16-bit audio | -  On-board 16-bit audio |
| -  10/100 BaseT Ethernet twisted pair (thinwire—optional) | -  On-board 10/100 BaseT Ethernet twisted pair (thinwire—optional) | -  10/100 BaseT Ethernet twisted pair (thinwire—optional) | -  10/100 BaseT Ethernet twisted pair (thinwire—optional) |
| -  Windows NT 4.0 | -  Windows NT 4.0 | -  Windows NT 4.0 | -  DIGITAL UNIX 4.0C or OVMS 7.1-1H1 |

Step 1¾DIGITAL Personal Workstations

DIGITAL Personal Workstations are available through distribution partners.  A list of eligible partners is available on DIGITAL Business Link or through a local DIGITAL Sales Office.

DIGITAL Personal Workstations may be ordered as:

- System Building Blocks are designed for “a la carte” ordering and distributor integration.  System Building Blocks contain a system kernel, minimum of 64 MB memory (except for Intel Workstations), UltraSCSI disk controller (except for i-Series and i+ -Series EIDE SBB), 4.3 GB SCSI hard disk drive, and factory-installed Windows NT, DIGITAL UNIX or OpenVMS operating system.

- Packaged Systems are designed for ease of ordering and stocking.  Packaged Systems contain a system kernel, a minimum of 64 MB memory (except for Intel i+ package), UltraSCSI disk controller (except for Intel i+ package) , 4.3GB hard disk drive, graphics card, and factory-installed Windows NT operating system.

- OpenVMS customers must order QP-03FAE-AA, Digital Personal Workstations Value License Package.

DIGITAL Personal Workstations come standard with a three-year limited warranty.  Year one features on-site service; years two and three are return to DIGITAL service, parts only.  This warranty represents one of the most comprehensive in the industry and includes product lifetime toll-free technical assistance.

DIGITAL believes the information in this publication is accurate as of its publication date; such information is subject to change without notice. DIGITAL is not responsible for any inadvertent errors.

DIGITAL conducts its business in a manner that conserves the environment and protects the safety and health of its employees, customers, and the community.

DIGITAL, the DIGITAL logo are trademarks of Digital Equipment Corporation.

Printed in USA. Copyright 1997 Digital Equipment Corporation. All rights reserved.

Step 1¾DIGITAL Personal Workstations (continued)

System Building Blocks

- Selection of a System and Country Kit from Steps 1 and 2 are mandatory

- Additional options in Steps 3 through 12 are optional.

i-Series

| Order Number | CPU | Operating System | Memory / Cache | Graphics | Controller | Hard Disk Drive |
| --- | --- | --- | --- | --- | --- | --- |
| SN-B31WW-T4 | 266i | Windows NT 4.0 | 32 MB / 512 KB | ¾ | IDE+ | 4.3 GB EIDE disk |
| SN-B31WW-T3 | 266i | Windows NT 4.0 | 64 MB / 512 KB | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| SN-B31WW-T5 | 300i | Windows NT 4.0 | 64 MB / 512 KB | ¾ | UltraSCSI | 4.3 GB SCSI disk |

i+-Series

| SN-K3B5W-VA | 266i/LX | Windows NT 4.0 | 32 MB / 512 KB | Permedia P2 | UltraDMA | 4.3 GB EIDE disk |
| --- | --- | --- | --- | --- | --- | --- |
| SN-K3B6W-VC | 300i/LX | Windows NT 4.0 | 64 MB / 512 KB | Permedia P2 | UltraSCSI | 4.3 GB SCSI disk |

a-Series

| Order Number | CPU | Operating System | Memory / Cache | Graphics | Controller | Hard Disk Drive |
| --- | --- | --- | --- | --- | --- | --- |
| SN-B3AAP-SB | 433a | Windows NT 4.0 | 64 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3AAP-SH | 433a | Windows NT 4.0 | 512 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3DAP-SB | 500a | Windows NT 4.0 | 64 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3DAP-SH | 500a | Windows NT 4.0 | 512 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3EAP-SB | 600a | Windows NT 4.0 | 64 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3EAP-SH | 600a | Windows NT 4.0 | 512 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |

au-Series (UNIX)

| SN-B3AAU-SB | 433au | DIGITAL UNIX | 64 MB / 0 MB, | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| --- | --- | --- | --- | --- | --- | --- |
| SN-B3AAU-SH | 433au | DIGITAL UNIX | 512 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3AAU-WL | 433au | DIGITAL UNIX | 64 MB / 2 MB | ¾ | UltraSCSI | 2.1 GB SCSI disk |
| SN-B3DAU-SB | 500au | DIGITAL UNIX | 64 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3DAU-SH | 500au | DIGITAL UNIX | 512 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3EAU-SB | 600au | DIGITAL UNIX | 64 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3EAU-SH | 600au | DIGITAL UNIX | 512 MB / 0 MB | ¾ | UltraSCSI | 4.3 GB SCSI disk |

au-Series (OpenVMS)

| SN-B3AAV-SB | 433au | OpenVMS | 64 MB / O MB | ¾ | Ultra SCSI | 4.3 GB SCSI disk |
| --- | --- | --- | --- | --- | --- | --- |
| SN-B3AAV-SD | 433au | OpenVMS | 128 MB / O MB | ¾ | Ultra SCSI | 4.3 GB SCSI disk |
| SN-B3AAV-SH | 433au | OpenVMS | 512 MB / O MB | ¾ | Ultra SCSI | 4.3 GB SCSI disk |
| SN-B3DAV-SB | 500au | OpenVMS | 64 MB / O MB | ¾ | Ultra SCSI | 4.3 GB SCSI disk |
| SN-B3DAV-SD | 500au | OpenVMS | 128 MB / O MB | ¾ | Ultra SCSI | 4.3 GB SCSI disk |
| SN-B3DAV-SH | 500au | OpenVMS | 512 MB / O MB | ¾ | Ultra SCSI | 4.3 GB SCSI disk |
| SN-B3EAV-SB | 600au | OpenVMS | 64 MB / O MB | ¾ | Ultra SCSI | 4.3 GB SCSI disk |
| SN-B3EAV-SD | 600au | OpenVMS | 128 MB / O MB | ¾ | Ultra SCSI | 4.3 GB SCSI disk |
| SN-B3EAV-SH | 600au | OpenVMS | 512 MB / O MB | ¾ | Ultra SCSI | 4.3 GB SCSI disk |

Step 1¾DIGITAL Personal Workstations (continued)

Packaged Systems

- Selection of a System and Country Kit from Steps 1 and 2 are mandatory.

- Additional options in Steps 3 through 12 are optional.

- Bulk system orders must include Country Kits equal to the total number of systems ordered.

i-Series

| Order Number | CPU | Operating System | Memory / Cache | Graphics | Controller | Hard Disk Drive |
| --- | --- | --- | --- | --- | --- | --- |
| SN-B34WW-FL | 266i | Windows NT 4.0 | 64 MB / 512 KB | Millenium II | UltraSCSI | 4.3 GB SCSI disk |
| SN-B37WW-FL | 266i | Windows NT 4.0 | 64 MB / 512 KB | 4D30T | UltraSCSI | 4.3 GB SCSI disk |

i+-Series

| SN-K3B5W-VA | 266i/LX | Windows NT 4.0 | 32 MB / 512 KB | Permedia P2, 4MB SGRAM | UltraDMA | 4.3 GB EIDE disk |
| --- | --- | --- | --- | --- | --- | --- |
| SN-K3B6W-VC | 300i/LX | Windows NT 4.0 | 64 MB / 512 KB | Permedia P2, 4MB SGRAM | UltraSCSI | 4.3 GB SCSI disk |
| SN-K3B6W-VD | 300i/LX | Windows NT 4.0 | 64 MB / 512 KB | PowerStorm 4D30T, 4MB | UltraSCSI | 4.3 GB SCSI disk |

a-Series

| SN-B3AAP-AB | 433a | Windows NT 4.0 | 64 MB / 0 MB | Millenium II | UltraSCSI | 4.3 GB SCSI disk |
| --- | --- | --- | --- | --- | --- | --- |
| SN-B3AAP-FB | 433a | Windows NT 4.0 | 64 MB / 0 MB | PowerStorm 4D30T | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3DAP-AB | 500a | Windows NT 4.0 | 64 MB / 0 MB | Millenium II | UltraSCSI | 4.3 GB SCSI disk |
| SN-B3DAP-FB | 500a | Windows NT 4.0 | 64 MB / 0 MB | PowerStorm 4D30T | UltraSCSI | 4.3 GB SCSI disk |

Step 2¾Country Kit

- UNIX/OVMS universal country kit includes two country-specific power cords, user documentation, and mouse. UNIX license included in package system.

- OpenVMS customers must order QP-03FAE-AA, (Digital Personal Workstations Value License Package) separately.

- NT country kit includes two country-specific power cords, user documentation, mouse, operating system license, media and documentation.  Country kit ordered determines language of operating system factory installed.

- Country kit must be ordered for system to be operational.  Only one country kit allowed per system.

- Bulk system orders must include Country Kits equal to the total number of systems ordered.

- Keyboards must be ordered separately.

| Intel Windows NT 4.0 Country Kits i-Series | Intel Windows NT 4.0 Country Kits i+-Series | Alpha Windows NT 4.0 Country Kits a-Series | Windows NT 4.0 Language | DIGITAL UNIX 4.0C / OpenVMS 7.1-1H1 Country Kits au-Series | DIGITAL UNIX 4.0C / OpenVMS 7.1-1H1 H/W Users Doc au-Series | COUNTRY |
| --- | --- | --- | --- | --- | --- | --- |
| Americas |  |  |  |  |  |  |
| SN-PBB5P-AA | SN-PCB4N-AA | SN-PBB4P-AA | English | SN-PBB4U-AA | English | United States |
| SN-PBB5P-AC | SN-PCB4N-AC | SN-PBB4P-AC | French | SN-PBB4UAC | French | French Canada |
| SN-PBB5P-AR | SN-PCB4N-CS | SN-PBB4P-AR | Spanish | SN-PBB4U-AR | Spanish | LACR-Spanish |

Step 2¾Country Kit (continued)

| Intel Windows NT 4.0 Country Kits i-Series | Intel Windows NT 4.0 Country Kits i+-Series | Alpha Windows NT 4.0 Country Kits a-Series | Windows NT 4.0 Language | DIGITAL UNIX 4.0C / OpenVMS 7.1-1H1 Country Kits au-Series | DIGITAL UNIX 4.0C / OpenVMS 7.1-1H1 H/W Users Doc au-Series | COUNTRY |
| --- | --- | --- | --- | --- | --- | --- |
| Europe |  |  |  |  |  |  |
| SN-PBB5P-AB | SN-PCB4N-AB | SN-PBB4P-AB | English | SN-PBB4U-AB | French | Belgium |
| SN-PBB5P-AD | SN-PCB4N-AD | SN-PBB4P-AD | Danish | SN-PBB4U-AD | English | Denmark |
| SN-PBB5P-AE | SN-PCB4N-AE | SN-PBB4P-AE | English | SN-PBB4U-AE | English | United Kingdom |
| SN-PBB5P-AF | SN-PCB4N-AF | SN-PBB4P-AF | Finish | SN-PBB4U-AF | English | Finland |
| SN-PBB5P-AG | SN-PCB4N-AG | SN-PBB4P-AG | German | SN-PBB4U-AG | German | Germany |
| SN-PBB5P-AH | SN-PCB4N-AH | SN-PBB4P-AH | Dutch | SN-PBB4U-AH | English | Netherlands |
| SN-PBB5P-AI | SN-PCB4N-AI | SN-PBB4P-AI | Italian | SN-PBB4U-AI | Italian | Italy |
| SN-PBB5P-AK | SN-PCB4N-AK | SN-PBB4P-AK | French | SN-PBB4U-AK | French | Switzerland |
| SN-PBB5P-AL | SN-PCB4N-AL | SN-PBB4P-AL | German | SN-PBB4U-AL | English | Switzerland/ Germany |
| SN-PBB5P-AM | SN-PCB4N-AM | SN-PBB4P-AM | Swedish | SN-PBB4U-AM | English | Sweden |
| SN-PBB5P-AN | SN-PCB4N-AN | SN-PBB4P-AN | Norwegian | SN-PBB4U-AN | English | Norway |
| SN-PBB5P-AP | SN-PCB4N-AP | SN-PBB4P-AP | French | SN-PBB4U-AP | French | France |
| SN-PBB5P-AS | SN-PCB4N-AS | SN-PBB4P-AS | Spanish | SN-PBB4U-AS | Spanish | Spain |
| SN-PBB5P-AT | SN-PCB4N-AT | SN-PBB4P-AT | English | SN-PBB4U-AT | English | Israel |
| SN-PBB5P-AV | SN-PCB4N-AV | SN-PBB4P-AV | English | SN-PBB4U-AV | English | Portugal |
| SN-PBB5P-BH | SN-PCB4N-BH | SN-PBB4P-BH | English | SN-PBB4U-BH | English | Greece |
| SN-PBB5P-BP | SN-PCB4N-BP | SN-PBB4P-BP | Polish | SN-PBB4U-BP | English | Poland |
| SN-PBB5P-BQ | SN-PCB4N-BQ | SN-PBB4P-BQ | Hungarian | SN-PBB4U-BQ | English | Hungary |
| SN-PBB5P-BR | SN-PCB4N-BR | SN-PBB4P-BR | English | SN-PBB4U-BR | English | Arabic |
| SN-PBB5P-BT | SN-PCB4N-BT | SN-PBB4P-BT | Russian | SN-PBB4U-BT | English | Russia |
| SN-PBB5P-BU | SN-PCB4N-BU | SN-PBB4P-BU | English | SN-PBB4U-BU | English | Turkey |
| SN-PBB5P-BV | SN-PCB4N-BV | SN-PBB4P-BV | English | SN-PBB4U-BV | English | Slovak |
| SN-PBB5P-BY |  | SN-PBB4P-BY | English | SN-PBB4U-BY | English | Serbia |
| SN-PBB5P-CQ | SN-PCB4N-CQ | SN-PBB4P-CQ | English | SN-PBB4U-CQ | English | Iceland |
| SN-PBB5P-CZ | SN-PCB4N-CZ | SN-PBB4P-CZ | Czech | SN-PBB4U-CZ | English | Czech Republic |
| SN-PBB5P-EC | SN-PCB4N-EC | SN-PBB4P-EC | English | SN-PBB4U-EC | English | European |
| Asia Pacific |  |  |  |  |  |  |
| SN-PBB5P-AJ | SN-PCB4N-AJ | SN-PBB4P-AJ | Japanese | SN-PBB4U-AJ | Japanese | Japan |
|  | SN-PCB4N-AZ |  | English |  |  | Australia/ New Zealand |
| SN-PBB5P-BI | SN-PCB4N-BI | SN-PBB4P-BI | Trad. Chinese | SN-PBB4U-BI | English | Taiwan |
| SN-PBB5P-BK | SN-PCB4N-BK | SN-PBB4P-BK | Korean | SN-PBB4U-BK | English | South Korea |
| SN-PBB5P-CA | SN-PCB4N-CA | SN-PBB4P-CA | English | SN-PBB4U-CA | English | Far East |
| SN-PBB5P-CE | SN-PCB4N-CE | SN-PBB4P-CE | English | SN-PBB4U-CE | English | Philippines |
| SN-PBB5P-CG | SN-PCB4N-CG | SN-PBB4P-CG | English | SN-PBB4U-CG | English | Vietnam |
| SN-PBB5P-CP | SN-PCB4N-CP | SN-PBB4P-CP | English | SN-PBB4U-CP | English | Thailand |
| SN-PBB5P-CV | SN-PCB4N-CV | SN-PBB4P-CV | Simp. Chinese | SN-PBB4U-CV | English | China |

Step 3¾Keyboards (Optional)

Americas

DIGITAL VMS style keyboards used on Alpha VMS and UNIX products.

| Windows 95 Keyboard | Country | DIGITAL VMS Style Keyboard | Country |
| --- | --- | --- | --- |
| SN-LK97W-A2 | United States | LK46W-A2 | United States/United Kingdom |
| SN-LK97W-AC | French Canada | LK46W-AC | French Canada |
| SN-LK97W-AR | LACR-Spanish |  |  |

Europe

| Windows 95 Keyboard | Country | DIGITAL VMS Style Keyboard | Country |
| --- | --- | --- | --- |
| SN-LK97W-AB | Belgium | LK46W-AB | Belgium |
| SN-LK97W-AD | Denmark | LK46W-AD | Denmark |
| SN-LK97W-AE | United Kingdom |  |  |
| SN-LK97W-AF | Finland | LK46W-AF | Finland |
| SN-LK97W-AG | Germany | LK46W-AG | Germany |
| SN-LK97W-AH | Netherlands | LK46W-AH | Netherlands |
| SN-LK97W-AI | Italy | LK46W-AI | Italy |
| SN-LK97W-AK | Switzerland | LK46W-AK | Switzerland |
| SN-LK97W-AF | Sweden | LK46W-AM | Sweden |
| SN-LK97W-AN | Norway | LK46W-AN | Norway |
| SN-LK97W-AP | France | LK46W-AP | France |
| SN-LK97W-AS | Spain | LK46W-AS | Spain |
| SN-LK97W-AT | Israel | LK46W-AT | Israel |
| SN-LK97W-AV | Portugal | LK46W-AV | Portugal |
| SN-LK97W-BH | Greece | LK46W-BH | Greece |
| SN-LK97W-BP | Poland | LK46W-BP | Poland |
| SN-LK97W-BQ | Hungary | LK46W-BQ | Hungary |
| SN-LK97W-BR | Arabic |  |  |
| SN-LK97W-BT | Cyrillic/Russian | LK46W-BT | Russia |
| SN-LK97W-BU | Turkey | LK46W-BU | Turkey |
| SN-LK97W-BW | Turkey | LK46W-BW | Turkey |
| SN-LK97W-BY | Serbia | LK46W-BY | Serbia |
| SN-LK97W-CQ | Iceland |  |  |
| SN-LK97W-BV | Czech Republic | LK46W-BV | Czech Republic |
| SN-LK97W-CZ | Slovak | LK46W-CZ | Slovak |

Asia Pacific

| Windows 95 Keyboard | Country | DIGITAL VMS Style Keyboard | Country |
| --- | --- | --- | --- |
| SN-LK97W-AJ | Japan |  |  |
| SN-LK97W-BI | Taiwan |  |  |
| SN-LK97W-BK | South Korea |  |  |
| SN-LK97W-CB | Thailand |  |  |

NOTE:	Keyboards are not included with the system packaging, customer must order if required.

Step 4¾Processor Upgrades (Optional)

| SN-PBXBA-HB | 266 MHz Pentium II processor, must match speed of first processor, supported in MLB CPU socket |
| --- | --- |
| SN-D3B5W-5U | 266 MHz Pentium II processor, must match speed of first processor, for PWS 266i+, with 440LX |
| SN-D3B6W-5U | 300 MHz Pentium II processor, must match speed of first processor, for PWS 300i+, with 440LX |
| SN-PBXBA-DA | PWS 200i to 433a system upgrade, replaces system box |
| SN-PBXBA-GA | PWS 200i to 266i MLB and CPU upgrade, replaces MLB |
| SN-PBXBA-FA | PWS 266i to 600a MLB and CPU Upgrade, replaces MLB and memory |
| SN-PBXBA-IA | PWS 266i to 300i CPU Upgrade |
| SN-PBXBA-IB | PWS 300i single processor to 300i dual processor CPU Upgrade |
| SN-PBXBA-JA | PWS 433/500 a/au to 600 a/au CPU Upgrade |

Step 5¾Memory (Optional)

i-Series Memory

i-Series, with 440FX chipsets supports four memory options (4 SIMM banks with 2 slots each)

| SN-PCCAM-EB | 32 MB ECC EDO SIMMs (2 x 16 MB) |
| --- | --- |
| SN-PCCAM-EC | 64 MB ECC EDO SIMMs (2 x 32 MB) |
| SN-PCCAM-ED | 128 MB ECC EDO SIMMs (2 x 64 MB) |

i+-Series Memory

i+-Series, with 440LX chipsets supports four memory options (4 DIMM banks with 1 slot each)

| SN-MSP01-JB | 32 MB ECC SDRAM DIMMs (1 x 32 MB)) |
| --- | --- |
| SN-MSP01-JC | 64 MB ECC SDRAM DIMMs (1 x 64 MB)) |
| SN-MSP01-JD | 128 MB ECC SDRAM DIMMs (1 x 128 MB) |

a- and au-Series Memory

a- and au- series support three memory options (3 DIMM banks with 2 slots each)

| SN-MSP01-HC | 64 MB ECC SDRAM DIMMs (2 x 32 MB) |
| --- | --- |
| SN-MSP01-HD | 128 MB ECC SDRAM DIMMs (2 x 64 MB) |
| SN-MSP01-HE | 256 MB ECC SDRAM DIMMs (2 x 128 MB) |
| SN-MSP01-HF | 512 MB ECC SDRAM DIMMs (2 x 256 MB) |

a- and au-Series Cache Memory

Supported in MLB cache slot.  Note:  433au system (SN-B3AAU-WL) includes 2MB cache.

| SN-MSC01-AB | 2 MB Level 3 SRAM cache |
| --- | --- |

Step 6¾Storage (Optional)

SCSI disk drives

Maximum of four SCSI drives supported per system on UNIX and Windows NT systems.  A maximum of three SCSI drives supported per system on OpenVMS systems.  (See system diagram)

| SN-PBXRW-JD | 2.1 GB UltraSCSI wide disk drive 7200 RPM, 1” or 1.6” bay |
| --- | --- |
| SN-PBXRW-HC | 2.1 GB UltraSCSI wide disk drive 5400 RPM, 1” or 1.6” bay |
| SN-PBXRW-NC | 4.3 GB UltraSCSI wide disk drive 7200 RPM, 1” or 1.6” bay |
| SN-PBXRW-RA1 | 4.3 GB Ultr SCSI wide disk drive 10,000 RPM, 1.6" bay (includes fan kit) |
| SN-PBXRW-SA2 | 9.1 GB UltraSCSI wide disk drive 7200 RPM, 1.6” bay |

1.	Supported on a-Series only.

2.	Maximum two per system, supported in 5.25” removable media bays only.

EIDE Disk Drives

EIDE disk drives are supported on i-Series and i+-Series only, maximum of two per system.

| SN-PCXRA-AN | 2.1 GB EIDE disk drive, 4500 RPM, 1.6” bay |
| --- | --- |
| SN-PCXRA-AT | 4.3 GB Ultra DMA EIDE disk drive, 5400 RPM, 1.6” bay |
| SN-PCXRA-AU | 6.4 GB Ultra DMA EIDE disk drive, 5400 RPM, 1.6” bay |
| SN-PCXRA-AP | 3.2 GB EIDE disk drive, 4500 RPM, 1.6” bay |

Tape Drives

Maximum of two tape drives supported per system

| SN-TLZ10-LK | 12/24 GB 4mm DAT internal SCSI tape drive, 1.6” front accessible bay |
| --- | --- |
| SN-PBXKP-BA | Wide to narrow SCSI connection adapter |

Floppy Drives

| SN-PBXRX-BB | 100 MB Floppy zip drive |
| --- | --- |

Controllers

Each DPWS a and au system include a Q-logic UltraSCSI wide controller in the base system. No additional SCSI controllers are supported.

| Maximum controllers supported per system |  | i-Series | i+-Series | a-Series | au-Series |
| --- | --- | --- | --- | --- | --- |
| SN-PCTAZ-DE | Adaptec 2940 UltraSCSI wide controller, requires 1 PCI 32-bit slot | 3 | 3 | ¾ | ¾ |
| SN-KZPBA-CA1 | Q-Logic 1040B UltraSCSI wide controller, requires 1 PCI 32-bit slot | 3 | ¾ | ¾ | ¾ |
| SN-KZPAC-AA | 1-port PCI-based RAID controller, 4MB EDRAM cache, requires 1 PCI 32-bit slot | 1 | 1 | ¾ | ¾ |
| SN-KZPAC-CA | 3-port PCI-based RAID controller, 4MB EDRAM cache, requires 1 PCI 32-bit slot, third port requires one additional PCI bulkhead slot, and one BN31K-0E bulkhead assembly kit. | 1 | 1 | ¾ | ¾ |
| SN-KZPAC-CB | 3-port PCI-based RAID controller, 8MB EDRAM cache, requires 1 PCI 32-bit slot, third port requires one additional PCI bulkhead slot, and one BN31K-0E bulkhead assembly kit. | 1 | 1 | ¾ | ¾ |

1.	For i-Series only:  SN-KZPBA-CA can be used with DIGITAL StorageWorks DS-BA356-KF enclosure to run in UltraSCSI mode and requires BN38C-xx SCSI cable not to exceed 20 meters.

Step 7¾Graphics Options (Optional)

| Graphics Options require 1 PCI 64-bit slot¾Maximum supported per system |  | i-Series | i+-Series | a-Series | au-Series |
| --- | --- | --- | --- | --- | --- |
| SN-PB2GA-KB | Matrox Millennium II graphics with 4 MB WRAM, maximum 8 MB WRAM | 2 | 2 | 2 | ¾ |
| SN-PBXGB-AA | PowerStorm 3D30 graphics with 2 MB VRAM | ¾ | ¾ | ¾ | 2 |
| SN-PBXGK-AA | PowerStorm 4D10 graphics with 4 MB SGRAM | 1 | 1 | ¾ | ¾ |
| SN-PBXGK-AB | PowerStorm 4D10T graphics with 8 MB SGRAM | 1 | 1 | ¾ | ¾ |
| SN-PBXGB-CA | PowerStorm 4D20 graphics with 16 MB VRAM | ¾ | ¾ | ¾ | 2 |
| SN-PBXGD-AA | PowerStorm 4D30T 3D graphics with 4 MB texture memory, maximum 16 MB texture memory | 1 | 1 | 1 | ¾ |
| SN-PBXGD-AB | PowerStorm 4D30T 3D graphics with 16 MB texture memory, maximum 32 MB texture memory | 1 | 1 | 1 | ¾ |

| Graphics Options require 2 PCI 64-bit slots¾Maximum supported per system |  | i-Series | i+-Series | a-Series | au-Series |
| --- | --- | --- | --- | --- | --- |
| SN-PBXGI-AA | PowerStorm 4D40T 3D graphics with 16 MB VRAM, maximum 32 MB texture memory | 1 | ¾ | 1 | 11 |
| SN-PBXGI-AB | PowerStorm 4D50T 3D graphics with 16 MB VRAM, maximum 32 MB texture memory | ¾ | ¾ | 1 | 11 |
| SN-PBXGI-AC2 | PowerStorm 4D60T 3D graphics with 32 MB VRAM, plus 16MB texture memory, maximum 32 MB texture memory | ¾ | ¾ | 1 | 11 |

1.  Not supported on OpenVMS.

2.  Upgrading to 32 MB of texture memory requires replacing the existing 16 MB of texture memory.

Step 8¾Graphics Memory Options (Optional)

|  |  | i-Series | i+-Series | a-Series | au-Series |
| --- | --- | --- | --- | --- | --- |
| SN-PB2GA-KE | 4 MB WRAM Frame Buffer Upgrade: Millennium II supported in Millennium II memory slot | X | X | X | ¾ |
| SN-PBXGD-GB | 16 MB Texture memory for PowerStorm 4D30T supported in 4D30T memory slot | X | X | X | ¾ |
| SN-PBXGI-GA | 4 MB Texture memory for PowerStorm series supported in 4D40T, 4D50T, 4D60T memory slot | X | ¾ | X | X1 |
| SN-PBXGI-GB | 16 MB Texture memory for PowerStorm series supported in 4D40T, 4D50T, 4D60T memory slot | X | ¾ | X | X1 |
| SN-PBXGI-GC | 32 MB Texture memory for PowerStorm series supported in 4D40T, 4D50T, 4D60T memory slot | X | ¾ | X | X1 |
| SN-PBXGK-GA | 4 MB frame buffer memory for PowerStorm 4D10 supported in embedded Permedia P2 graphics on 440LX-based motherboard | ¾ | X | ¾ | ¾ |

1.  Not supported on OpenVMS.

Step 9¾Color Monitors (Optional)

|  |  | i-Series | i+-Series | a-Series | au-Series |
| --- | --- | --- | --- | --- | --- |
| SN-VRCX5-WA | 15” (13.9” viewable) Corporate Northern Hemisphere 120V | X | X | ¾ | ¾ |
| SN-VRCX7-WA | 17” (16.0” viewable) Corporate Northern Hemisphere 120V | X | X | X | X |
| SN-VRTX7-WA | 17” (16.0” viewable) Professional Northern Hemisphere 120V | X | X | X | X |
| SN-PCXAV-CZ | 19” (18.0” viewable) Professional Northern Hemisphere 120V | X | X | X | X |
| SN-VRCX1-WA | 21” (19.6” viewable) Professional Northern Hemisphere 120V | X | X | X | X |
| SN-PCXAV-AZ1 | 24” (22.5” viewable) Professional Northern Hemisphere 120V | ¾ | ¾ | X | X |

| SN-VRCX5-W3 | 15” (13.9” viewable) Corporate Northern Hemisphere 240V | X | X | ¾ | ¾ |
| --- | --- | --- | --- | --- | --- |
| SN-VRCX7-W3 | 17” (16.0” viewable) Corporate Northern Hemisphere 240V | X | X | ¾ | ¾ |
| SN-VRTX7-W3 | 17” (16.0” viewable) Professional Northern Hemisphere 240V | X | X | X | X |
| SN-VRCX1-W3 | 21” (19.6” viewable) Professional Northern Hemisphere 240V | X | X | X | X |
| SN-PCXAV-AZ1 | 24” (22.5” viewable) Professional Northern Hemisphere 240V | ¾ | ¾ | X | X |

| SN-VRCX5-W4 | 15” (13.9” viewable) Corporate Southern Hemisphere 240V | X | X | ¾ | ¾ |
| --- | --- | --- | --- | --- | --- |
| SN-VRCX7-W4 | 17” (16.0” viewable) Corporate Southern Hemisphere | X | X | ¾ | ¾ |
| SN-VRT17-W4 | 17” (16.0” viewable) Professional Southern Hemisphere | X | X | X | X |
| SN-PCXAV-CY | 19” (18.0” viewable) Professional Southern Hemisphere | X | X | X | X |
| SN-VRCX1-W4 | 21” (19.6” viewable) Professional Southern Hemisphere | X | X | X | X |

1.	Supported with PowerStorm 4D60T graphics options only.

Step 10¾Network Options (Optional)

i+-Series includes onboard 10/100 BaseT Ethernet controller

| PCI-based Network options require 1 PCI slot Maximum supported per system |  | i-Series | i+-Series | a-Series | au-Series |
| --- | --- | --- | --- | --- | --- |
| SN-DE450-CA | EtherWORKS PCI 10mbs (twisted pair/AUI/BNC) | 3 | ¾ | 1 | 1 |
| SN-DE500-BA | FastEthernet PCI 10/100mbs (twisted pair) | 1 | 1 | 1 | 1 |
| SN-PBXNP-AC | PCI Token Ring adapter | 1 | 1 | 1 | 1 |
| SN-DEFPA-AB | PCI - FDDI adapter SAS fiber | ¾ | ¾ | 1 | 1 |
| SN-DEFPA-DB | PCI - FDDI adapter DAS Fiber | ¾ | ¾ | 1 | 1 |
| SN-DEFPA-MB | PCI - FDDI adapter DAS UTP | ¾ | ¾ | 1 | 1 |
| SN-DEFPA-UB | PCI - FDDI adapter SAS UTP | ¾ | ¾ | 1 | 1 |
| PBXDA-AA | PCI Asynchronous 4-port adapter | ¾ | ¾ | 1 | 1 |
| PBXDA-AB | PCI Asynchronous 8-port adapter | ¾ | ¾ | 1 | 1 |
| SN-PBXIP-AA | PCI ISDN BRI S/T | ¾ | ¾ | 1 | 1 |
| SN-PBXIP-AB | PCI ISDN BRI U | ¾ | ¾ | 1 | 1 |
| SN-DGLPA-FA | PCI ATM adapter | ¾ | ¾ | ¾ | 11 |
| SN-DGLPA-UA | PCI ATM adapter | ¾ | ¾ | ¾ | 11 |

1.  Not supported on OpenVMS.

| ISA-based Network options require 1 ISA slot Maximum supported per system |  | i-Series | i+-Series | a-Series | au-Series |
| --- | --- | --- | --- | --- | --- |
| SN-PCXHF-AA | ISA Fax/modem 28.8Kbps | 2 | 2 | 1 | ¾ |
| PBXDF-BA | ISA Fax/modem 28.8Kbps | ¾ | ¾ | 1 | 1 |

| System Enclosure bulkhead connector |  | i-Series | i+-Series | a-Series | au-Series |
| --- | --- | --- | --- | --- | --- |
| SN-PCXAN-DB | 10BaseT/10Base 2 Ethernet Bulkhead Connector | 1 | ¾ | 1 | 1 |
| SN-PCXAN-DC | 10BaseT/10Base2 Ethernet bulkhead Connector, AUI | ¾ | 1 | ¾ | ¾ |
| SN-PCXAN-DD | 10BaseT/10Base2 Ethernet bulkhead Connector, BNC | ¾ | 1 | ¾ | ¾ |

Step 11¾Input Devices (Optional)

| PBXWA-AA | 2-button spaceball |
| --- | --- |
| PBXWB-AA | 3-button trackball |
| PBXWT-AA | 12x12 Digitizing tablet, 16-button cordless cursor 120V US wall mount power supply |
| PBXWT-AB | 12x12 Digitizing tablet, 16-button cordless cursor 220V European wall mount power supply |
| PBXWT-AC | 12x12 Digitizing tablet, 16-button cordless cursor 240V UK wall mount power supply |
| PBXWT-AD | 12x12 Digitizing tablet, 16-button cordless cursor 100V Japan wall mount power supply |
| PBXWT-BA | 2-button cordless click pen for all variants of PBXWT-Ax |

Step 12¾Hardware Supplemental Services (Optional)

| FM-W****-** | Hardware supplemental price depends on total price of the order, type of extension (e.g. Onsite Next Business Day, Onsite same business day, 7 x 24, and number of months). Refer to “Multivendor Customer Services Quick Quote Card” for part numbers (In Europe, refer to on-line Quoting System). First * after W could be A through P, depending on total list price of the hardware. Second through fourth * represent different levels (e.g. XHW for Onsite next business day service, 4 hour Onsite same business day service, and 7 x 24 service). Last two ** represent duration in months (e.g. 12, 36, 60) |
| --- | --- |
| FM-WSDTP-IN | Hardware Installation |

Step 12a¾Software Supplemental Services¾Windows NT

| FM-HD5**-** | 5 x 9 Helpdesk Support |
| --- | --- |
| FM-HD7**-** | 7 x 24 Helpdesk Support First two ** after 5 or 7 represent the number of calls, and could be 05, 15, or 25. Last two ** represent duration in months, and could be 12, 36, or 60 |

Step 12b¾Software Supplemental Services¾UNIX

| FM-3BUNS-12 | 12 Month FULL Software Supplemental Support Services |
| --- | --- |
| FM-3BUNS -36 | 36 Month FULL Software Supplemental Support Services |
| FM-3BUNS -60 | 60 Month FULL Software Supplemental Support Services |
| FM-3BUNN -12 | 12 Month NODE Software Supplemental Support Services |
| FM-3BUNN -36 | 36 Month NODE Software Supplemental Support Services |
| FM-3BUNN -60 | 60 Month NODE Software Supplemental Support Services |

Step 13¾Media and Documentation (optional)

Windows NT User Documentation

| EK-ALMIA-UI | DIGITAL Personal Workstation User’s Guide—English |
| --- | --- |
| EK-ALMIF-UI | DIGITAL Personal Workstation User’s Guide—French |
| EK-ALMID-UI | DIGITAL Personal Workstation User’s Guide—German |
| EK-ALMIJ-UI | DIGITAL Personal Workstation User’s Guide—Japanese |
| EK-ALMIS-UI | DIGITAL Personal Workstation User’s Guide—Spanish |
| EK-ALMII-UI | DIGITAL Personal Workstation User’s Guide—Italian |
| ER-B30WW-IM | DIGITAL Personal Workstation Quick Setup Guide—Multilingual |

Step 13¾Media and Documentation (optional) (continued)

DIGITAL UNIX User Documentation

| QA-MT4AA-GZ | DIGITAL UNIX 4.0C Full Documentation kit |
| --- | --- |
| QA-MT4AB-GZ1 | DIGITAL UNIX 4.0C End-User Documentation kit |

1.	Included in Full Documentation kit

DIGITAL UNIX Software Media and Optional Licenses

| QA-MT4AA-H8 | DIGITAL UNIX 4.0C Media & Documentation kit |
| --- | --- |
| QL-MT7AE-AA | DIGITAL UNIX 4.0C Unlimited User License |
| QL-MT6AE-AA | DIGITAL UNIX 4.0C Server Extensions License |
| QL-MT5AE-AA | DIGITAL UNIX 4.0C C Developer’s Extensions License |
| QL-MT7AM-3B | DIGITAL UNIX 4.0C 1-User Concurrent License |
| QL-MT7AM-3C | DIGITAL UNIX 4.0C 2-User Concurrent License |
| QL-MT7AM-3D | DIGITAL UNIX 4.0C 4-User Concurrent License |
| QL-MT7AM-3E | DIGITAL UNIX 4.0C 8-User Concurrent License |
| QL-MT7AM-3F | DIGITAL UNIX 4.0C 16-User Concurrent License |
| QL-MT7AM-3G | DIGITAL UNIX 4.0C 32-User Concurrent License |
| QL-MT7AM-3H | DIGITAL UNIX 4.0C 64-User Concurrent License |

NOTE: All DIGITAL UNIX systems include 2-user Base license

OpenVMS Media and Documentation

- OpenVMS Packaged and Base systems require operating system media and documentation for first system on site.

- Software Processor Code = E

| QA-MT1AD-H8 | OpenVMS V7.1-1H1 Media on CD-ROM |
| --- | --- |
| QA-MT1AA-H8 | OpenVMS V7.1 media and on-line doc on CD-ROM |
| QA-001AA-GZ | OpenVMS hardcopy documentation |

OpenVMS Optional Licenses

- OpenVMS Concurrent Use Licenses are not specific to a single system and can be moved between systems at user discretion. OpenVMS Concurrent Use Licenses can also be shared in a mixed OpenVMS VAX and OpenVMS Alpha Cluster.

| QL-MT3AA-3B | OpenVMS Concurrent Use 1-user license |
| --- | --- |
| QL-MT3AA-3C | OpenVMS Concurrent Use 2-user license |
| QL-MT3AA-3D | OpenVMS Concurrent Use 4-user license |
| QL-MT3AA-3E | OpenVMS Concurrent Use 8-user license |
| QL-MT3AA-3F | OpenVMS Concurrent Use 16-user license |
| QL-MT3AA-3G | OpenVMS Concurrent Use 32-user license |
| QL-MT3AA-3H | OpenVMS Concurrent Use 64-user license |
| QL-MT3AA-3J | OpenVMS Concurrent Use 128-user license |
| QL-MT3AA-3K | OpenVMS Concurrent Use 256-user license |
| QL-MT2AE-AA | OpenVMS traditional unlimited user license |

Specifications

| Height | 40.64 cm (16 inches) |
| --- | --- |
| Width | 21.59 cm (8.5 inches) |
| Depth | 44.45 cm (17.5 inches) |

DIGITAL Personal Workstations System Diagram (i-Series, a-Series, au-Series)

1.	Front door assembly

2.	5.25” CD-ROM drive bay

3.	3.5” floppy diskette drive bay

4.	Two 5.25” removable media bays

5.	Main Logic Board retainer bracket

6.	Main Logic Board

7.	Power supply

8.	Ethernet connector card (MAU or MII)

9.	Audio connector card

10.	Two 3.5” x 1” hard disk drive bays

11.	64-bit (or 32-bit) PCI option installed on riser card

12.	ISA card installed in PCI/ISA combination slot

13.	Riser card

1.	AC power connector

2.	Voltage selector switch

3.	Audio line In

4.	Speaker Out

5.	MIDI adapter or joystick connector

6.	Ethernet Twisted pair

7.	ThinWire Ethernet (optional)

8.	Communication ports 1 and 2

9.	Mouse port

10.	Keyboard port

11.	Parallel port

12.	Slot 1: 32-bit PCI (half size slot)

13.	Slot 2: 32-bit PCI/ISA (half size slot)

14.	Slot 3: 32-bit PCI/ISA (full size slot)

15.	Slot 4: 64-bit PCI/ISA (full size slot)

16.	Slot 5: 64-bit PCI (full size slot)

DIGITAL Personal Workstations System Diagram (i+-Series)

A.	Door assembly

B.	Front access 5¼-inch

C.	3½-inch diskette drive

D.	Main Logic Board

E.	Pentium II processor module

F.	Power supply

G.	Internal 3½-inch drive bays (supports IDE, and wide SCSI hard disk)

H.	CD-ROM drive

A.	Optional AGP port connector

B.	Video port connector

C.	Keyboard connector

D.	Mouse connector

E.	USB ports

F.	Serial port connector 1

G.	Serial port connector 2

H.	Parallel port connector

I.	10 Base-T port connector (RJ45)

J.	Expansion board slots

K.	MIDI/joystick port connector

L.	Chassis lockdown

M.	Speaker out

N.	Line in (microphone)

O.	ac line connector

P.	Voltage select switch