### AlphaServer 800 Pedestal–Rackmount
V2.4¾20 May 1998
DIGITAL Systems and Options Catalog

Product Description 	The AlphaServer 800 is an Alpha microprocessor server, available in 333 MHz, 400 MHz, and 500 MHz versions with 2MB ECC cache. All are available in Pedestal and Rackmount systems with a choice of three popular operating systems: DIGITAL UNIX, OpenVMS, and Microsoft Windows NT Server . The AlphaServer 800 offers the following high-reliability features: hot swap drives, Remote Management Console, and ECC memory. Integrated on system motherboard is an UltraSCSI-2 controller, SVGA controller (TRIO-S3, 1MB RAM), Remote Management Console, diskette controller, two serial ports, one parallel port, and keyboard and mouse interfaces.

The AlphaServer 800 Pedestal and Rackmount Packaged Systems offer up to 8 internal storage devices, including a floppy diskette drive, a CD-ROM, two additional 5.25-inch removable media bays for optional removable media drives, and four hot swap SCA-2 Ultra disk drives. The system supports up to 36 GB of internal storage (with four 9 GB disks) and 2GB memory.

The AlphaServer 800 integrated Remote Management Console (RMC) features significantly enhances the ability to remotely manage and diagnose system conditions. The RMC allows a remote operator to monitor server voltages, temperatures, and fan, as well as manipulate the server (Power On/Off, reset, halt).  An auxiliary regulator powers the RMC in the main power system that allows systems to be remotely interrogated even if systems are turned off or shut down.  In the event of an alert condition, the RMC can be set up to automatically dial a user-definable pager phone or another system to make a remote operator aware of the alert condition.

ServerWORKS Manager provides advanced server and network management capabilities and is supplied with all AlphaServer systems. Simple Network Management Protocol (SNMP) enables information to pass from the managed system to the console for DIGITAL UNIX, Windows NT, and OpenVMS systems. Using ServerWORKS, the system manager may build and view topological maps of the network. Detailed server information is viewable, including system, network, storage, and environmental information. Alarms may be set against critical system parameters and events. All AlphaServer systems are supplied with management tools to complement ServerWORKS Manager. For more information, see the StorageWorks Software section of this catalog.

DIGITAL believes the information in this publication is accurate as of its publication date; such information is subject to change without notice. Digital is not responsible for any inadvertent errors.

DIGITAL conducts its business in a manner that conserves the environment and protects the safety and health of its employees, customers, and the community.

DIGITAL, and  the DIGITAL logo are trademarks of Digital Equipment Corporation.

Printed in USA. Copyright 1998 Digital Equipment Corporation. All rights reserved.

Step 1—AlphaServer 800 Pedestal and Rackmount Package Systems

- Windows NT packaged systems include Factory Installed Software (FIS) Windows NT Server 4.0 on the hard disk drive.

- North American variants include the Windows NT license and media kit (CD-ROM) in North American English.

- Non-North American variants include FIS Windows NT 4.0 in English.

- Selection of language specific Windows NT license and media kit (CD-ROM) is mandatory for all non-North American variants.  See Step 9.

- DIGITAL UNIX and OpenVMS packaged systems include FIS software on the hard disk drive.

- All DIGITAL UNIX system ship with UNIX V4.0D.

- All DIGITAL OpenVMS systems ship with OpenVMS V7.1-1H1.

- Uninterruptable Power Supplies (UPS) are available.  See Uninterruptable Power Supplies.

- Options ordered FIS will be factory installed unless specified as spares.

Pedestal and Rackmount Systems include

- Alpha microprocessor 21164 with

- 333MHz CPU or
400MHz CPU or
500MHz CPU
each CPU includes 2 MB ECC onboard cache

- Pedestal enclosure with

- Six expansion slots: three PCI and one PCI/EISA combo and two EISA

- Integral UltraSCSI-2 (internal-only SCSI bus) supports removable media (CD-ROMs and tapes) and internal drive shelf

- One memory kit

- Eight storage slots
One diskette drive slot with 1.44 MB floppy drive
One CD-ROM drive slot with 600 MB CD-ROM
Two additional half height (or one full height) removable media slots
Four SCA-2 Fast Wide/Ultra hard drive slots

- 350 W switchable (120/240 V, 60/50 Hz) power supply

- Two serial ports: One serial port with full duplex asynchronous modem control; one RMC dedicated serial port with full duplex asynchronous modem control.

- One bi-directional enhanced parallel port

- PS/2 style keyboard port and mouse port

- 3-button mouse.

- 120 V power cord.  Mandatory selection of country-specific power cord for all non-North American variants.  See Step 7.

- PCI-based Fast 100 Ethernet (Twisted Pair)—uses one PCI slot.

- One UltraSCSI 4.3 GB hard disk drive.

- Windows NT US variants include a 101-key, PC-style US English keyboard. Windows NT non-US variants must order the appropriate keyboard.  See Step 7.

- OpenVMS variants include a 108-key, VT-style US English keyboard. Other keyboard variants must be ordered. See Step 7.

- DIGITAL UNIX variants include a 104/105-key PC-style US English keyboard. Other keyboard variants must be ordered. See Step 7.

- US English documentation.  Certain language selections are available.  See Step 9.

- Integrated Advanced Server Management features, including ServerWORKS Manager kit.

- Windows NT Server 4.0 plus 10-client access license, media kit (CD-ROM) or2

- DIGITAL UNIX Unlimited User license, Server Extension license, Internet AlphaServer System software kit, or

- OpenVMS base license with System Manager license, Enterprise Integrated Package

- Hardware warranty: three-year on-site.1

- Software Warranty: 1

- 90-day SPD conformance with advisory telephone support for DIGITAL UNIX and OpenVMS.

- Warranty on Microsoft's Windows NT Server 4.0 is conformance to the written material accompanying the software for a period of ninety days.

1.	Service upgrades are available.  See Step 10.

2.	Windows NT language.

Note: 	Rackmount packaged systems must be installed in a cabinet.  System can be installed in an existing cabinet or select a cabinet from Step 8.

Step 1a—Windows NT Pedestal and Rackmount Packaged Systems

- Digital’s OEM license agreement for Windows NT mandates compliance with specific configuration rules:

- Windows NT Language Configuration

- North American variants include pre-installed Windows NT Server 4.0 plus 10-client access license/North American English media (CD-ROM) kit.

- For all non-North American variants, selection of language-specific Windows NT Server 4.0 license/media (CD-ROM) kit is mandatory.  See Step 9. All non-North American variants include factory installed software (FIS) Windows NT 4.0 English.

- Video monitor is required for system management; order separately if not available on site.

- Windows NT US variants include a 101-key, PC-style US English keyboard.  Windows NT non-U.S. variants must order country-specific keyboard.  See Step 7.

Note:	Rackmount packaged systems must be installed in a cabinet.  System can be installed in an existing cabinet or select a cabinet from Step 8.

Windows NT Pedestal Packaged Systems

| Order Number | O/S Media | CPU | Power cord | Keyboard | Memory | Disk | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PB80B-AE | Included | 5/333 | 120 V | Included | 64 MB | 4.3 GB | Included | Required |
| PB80B-AF | Mandatory | 5/333 | Mandatory | Required | 64 MB | 4.3 GB | Included | Required |
| PB80B-AG | Included | 5/333 | 120 V | Included | 128 MB | 4.3 GB | Included | Required |
| PB80B-AH | Mandatory | 5/333 | Mandatory | Required | 128 MB | 4.3 GB | Included | Required |
| PB80B-AL | Included | 5/333 | 120 V | Included | 512 MB | 4.3 GB | Included | Required |
| PB80B-AM | Mandatory | 5/333 | Mandatory | Required | 512 MB | 4.3 GB | Included | Required |

| PB81B-AL | Included | 5/400 | 120 V | Included | 64 MB | 4.3 GB | Included | Required |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PB81B-AM | Mandatory | 5/400 | Mandatory | Required | 64 MB | 4.3 GB | Included | Required |
| PB81B-AN | Included | 5/400 | 120 V | Included | 128 MB | 4.3 GB | Included | Required |
| PB81B-AP | Mandatory | 5/400 | Mandatory | Required | 128 MB | 4.3 GB | Included | Required |
| PB81B-AT | Included | 5/400 | 120 V | Included | 512 MB | 4.3 GB | Included | Required |
| PB81B-AU | Mandatory | 5/400 | Mandatory | Required | 512 MB | 4.3 GB | Included | Required |

| PB82B-AJ | Included | 5/500 | 120 V | Included | 64 MB | 4.3 GB | Included | Required |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PB82B-AK | Mandatory | 5/500 | Mandatory | Required | 64 MB | 4.3 GB | Included | Required |
| PB82B-AA | Included | 5/500 | 120 V | Included | 128 MB | 4.3 GB | Included | Required |
| PB82B-AB | Mandatory | 5/500 | Mandatory | Required | 128 MB | 4.3 GB | Included | Required |
| PB82B-AE | Included | 5/500 | 120 V | Included | 512 MB | 4.3 GB | Included | Required |
| PB82B-AF | Mandatory | 5/500 | Mandatory | Required | 512 MB | 4.3 GB | Included | Required |

Key:	Mandatory items must be on purchase order at initial order acceptance.
Required items are essential for full system operation.
Recommended items enhance system functionality.

Step 1a—Windows NT Pedestal and Rackmount Packaged Systems (continued)

Windows NT Rackmount Packaged Systems

| Order Number | O/S Media | CPU | Power cord | Keyboard | Memory | Disk | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PB80P-AE | Included | 5/333 | 120 V | Included | 64 MB | 4.3 GB | Included | Required |
| PB80P-AF | Mandatory | 5/333 | Mandatory | Required | 64 MB | 4.3 GB | Included | Required |
| PB80P-AG | Included | 5/333 | 120 V | Included | 128 MB | 4.3 GB | Included | Required |
| PB80P-AH | Mandatory | 5/333 | Mandatory | Required | 128 MB | 4.3 GB | Included | Required |
| PB80P-AL | Included | 5/333 | 120 V | Included | 512 MB | 4.3 GB | Included | Required |
| PB80P-AM | Mandatory | 5/333 | Mandatory | Required | 512 MB | 4.3 GB | Included | Required |

| PB81P-AL | Included | 5/400 | 120 V | Included | 64 MB | 4.3 GB | Included | Required |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PB81P-AM | Mandatory | 5/400 | Mandatory | Required | 64 MB | 4.3 GB | Included | Required |
| PB81P-AN | Included | 5/400 | 120 V | Included | 128 MB | 4.3 GB | Included | Required |
| PB81P-AP | Mandatory | 5/400 | Mandatory | Required | 128 MB | 4.3 GB | Included | Required |
| PB81P-AT | Included | 5/400 | 120 V | Included | 512 MB | 4.3 GB | Included | Required |
| PB81P-AU | Mandatory | 5/400 | Mandatory | Required | 512 MB | 4.3 GB | Included | Required |

| PB82P-AJ | Included | 5/500 | 120 V | Included | 64 MB | 4.3 GB | Included | Required |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PB82P-AK | Mandatory | 5/500 | Mandatory | Required | 64 MB | 4.3 GB | Included | Required |
| PB82P-AA | Included | 5/500 | 120 V | Included | 128 MB | 4.3 GB | Included | Required |
| PB82P-AB | Mandatory | 5/500 | Mandatory | Required | 128 MB | 4.3 GB | Included | Required |
| PB82P-AE | Included | 5/500 | 120 V | Included | 512 MB | 4.3 GB | Included | Required |
| PB82P-AF | Mandatory | 5/500 | Mandatory | Required | 512 MB | 4.3 GB | Included | Required |

Key:	Mandatory items must be on purchase order at initial order acceptance.
Required items are essential for full system operation.
Recommended items enhance system functionality.

Step 1a—DIGITAL UNIX Pedestal and Rackmount Packaged Systems

- All DIGITAL UNIX Pedestal and Rackmount packaged systems include pre-installed DIGITAL UNIX, Base license, Unlimited User license, Server Extension license, and Internet Access Server Software (IASS) media and license.

- Order DIGITAL UNIX media and documentation separately.  See Step 9.

- DIGITAL UNIX packaged systems include factory installed software (FIS) DIGITAL UNIX 4.0D on the hard disk drive.

- DIGITAL UNIX variants ordered in North America include a 104/105-key PC-style US English keyboard.

- Selection of a keyboard and power cord are mandatory for all non-North American variants.  See Step 7.

- A minimum of 64 MB memory is recommended for DIGITAL UNIX.

Note:	Rackmount packaged systems must be installed in a cabinet.  System can be installed in an existing cabinet or select a cabinet from Step 8.

Step 1a—DIGITAL UNIX Pedestal and Rackmount Packaged Systems (continued)

DIGITAL UNIX Pedestal Packaged Systems

| Order Number | Operating System | CPU | Memory | Disk | Keyboard1 | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB80B-FK | DIGITAL UNIX | 333 | 32 MB | 1.05 GB | Mandatory | Included | Recommended |
| PB80B-FE | DIGITAL UNIX | 333 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80B-FF | DIGITAL UNIX | 333 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80B-FG | DIGITAL UNIX | 333 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80B-FH | DIGITAL UNIX | 333 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80B-FJ | DIGITAL UNIX | 333 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

| PB81B-FL | DIGITAL UNIX | 400 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB81B-FM | DIGITAL UNIX | 400 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81B-FN | DIGITAL UNIX | 400 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81B-FP | DIGITAL UNIX | 400 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81B-FR | DIGITAL UNIX | 400 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

| PB82B-FE | DIGITAL UNIX | 500 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB82B-FA | DIGITAL UNIX | 500 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82B-FB | DIGITAL UNIX | 500 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82B-FC | DIGITAL UNIX | 500 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82B-FD | DIGITAL UNIX | 500 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

DIGITAL UNIX Rackmount Packaged Systems

| Order Number | Operating System | CPU | Memory | Disk | Keyboard1 | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB80P-FK | DIGITAL UNIX | 333 | 32 MB | 1.05 GB | Mandatory | Included | Recommended |
| PB80P-FE | DIGITAL UNIX | 333 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80P-FF | DIGITAL UNIX | 333 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80P-FG | DIGITAL UNIX | 333 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80P-FH | DIGITAL UNIX | 333 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80P-FJ | DIGITAL UNIX | 333 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

| PB81P-FL | DIGITAL UNIX | 400 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB81P-FM | DIGITAL UNIX | 400 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81P-FN | DIGITAL UNIX | 400 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81P-FP | DIGITAL UNIX | 400 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81P-FR | DIGITAL UNIX | 400 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

| PB82P-FE | DIGITAL UNIX | 500 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB82P-FA | DIGITAL UNIX | 500 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82P-FB | DIGITAL UNIX | 500 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82P-FC | DIGITAL UNIX | 500 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82P-FD | DIGITAL UNIX | 500 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

1.	North American packaged systems include US keyboard. Non-North American keyboards must be ordered.

Key:	Mandatory items must be on purchase order at initial order acceptance.
Recommended items enhance system functionality.

Step 1a—OpenVMS Pedestal and Rackmount Packaged Systems

- All packaged systems include pre-installed OpenVMS, Base license, and DIGITAL Enterprise Integration Package. See Step 9 for contents.

- DIGITAL OpenVMS packaged systems include Factory Installed Software (FIS) DIGITAL OpenVMS V7.1-1H1 on the hard disk.

- OpenVMS variants ordered in North America include a 108-key, VT-style US English keyboard.

- Selection of a keyboard and power cord are mandatory for all non-North American variants.  See Step 7.

- Order OpenVMS media and documentation separately.  See Step 9.

Note: 	Rackmount packaged systems must be installed in a cabinet.  System can be installed in an existing cabinet or select a cabinet from Step 8.

DIGITAL OpenVMS Pedestal Packaged Systems

| Order Number | Operating System | CPU | Memory | Disk | Keyboard1 | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB80B-MK | OpenVMS | 333 | 32 MB | 1.05 GB | Mandatory | Included | Recommended |
| PB80B-ME | OpenVMS | 333 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80B-MF | OpenVMS | 333 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80B-MG | OpenVMS | 333 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80B-MH | OpenVMS | 333 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80B-MJ | OpenVMS | 333 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

| PB81B-ML | OpenVMS | 400 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB81B-MM | OpenVMS | 400 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81B-MN | OpenVMS | 400 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81B-MP | OpenVMS | 400 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81B-MR | OpenVMS | 400 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

| PB82B-ME | OpenVMS | 500 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB82B-MA | OpenVMS | 500 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82B-MB | OpenVMS | 500 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82B-MC | OpenVMS | 500 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82B-MD | OpenVMS | 500 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

DIGITAL OpenVMS Rackmount Packaged Systems

| Order Number | Operating System | CPU | Memory | Disk | Keyboard1 | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB80P-MK | OpenVMS | 333 | 32 MB | 1.05 GB | Mandatory | Included | Recommended |
| PB80P-ME | OpenVMS | 333 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80P-MF | OpenVMS | 333 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80P-MG | OpenVMS | 333 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80P-MH | OpenVMS | 333 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB80P-MJ | OpenVMS | 333 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

| PB81P-ML | OpenVMS | 400 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB81P-MM | OpenVMS | 400 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81P-MN | OpenVMS | 400 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81P-MP | OpenVMS | 400 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB81P-MR | OpenVMS | 400 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

1.  North American package systems include US keyboard.  Non-North American keyboards must be ordered.

Key:	Mandatory items must be on purchase order at initial order acceptance.
Recommended items enhance system functionality.

Step 1a—OpenVMS Pedestal and Rackmount Packaged Systems (continued)

DIGITAL OpenVMS Rackmount Packaged Systems

| Order Number | Operating System | CPU | Memory | Disk | Keyboard1 | Graphics | Monitor |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB82P-ME | OpenVMS | 500 | 64 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82P-MA | OpenVMS | 500 | 128 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82P-MB | OpenVMS | 500 | 256 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82P-MC | OpenVMS | 500 | 512 MB | 4.3 GB | Mandatory | Included | Recommended |
| PB82P-MD | OpenVMS | 500 | 1 GB | 4.3 GB | Mandatory | Included | Recommended |

1.  North American package systems include US keyboard.  Non-North American keyboards must be ordered.

Key:	Mandatory items must be on purchase order at initial order acceptance.
Recommended items enhance system functionality.

Step 2—Memory

- Packaged systems include one memory kit.

- System supports up to two memory kits. (Each kit includes four industry-standard DIMMs with ECC support).

- System supports a maximum of 2 GB of memory.

| PB8MA-AB | 32 MB (4 x 8 MB DIMMs) memory kit |
| --- | --- |
| PB8MA-AC | 64 MB (4 x 16 MB DIMMs) memory kit |
| PB8MA-AD | 128 MB (4 x 32 MB DIMMs) memory kit |
| PB8MA-AE | 256 MB (4 x 64 MB DIMMs) memory kit |
| PB8MA-AF | 512 MB (4 x 128 MB DIMMs) memory kit |
| PB8MA-AG | 1 GB (4 x 256 MB DIMMs) memory kit |

Step 3—Storage

Select storage devices as required. See Storage Devices section for more information.

Step 3a—Internal Storage

- Internal Storage included

- One 600 MB 12X CD-ROM drive

- One 4.3 GB UltraSCSI SCA-2 hard drive

- One 1.44 MB diskette drive in dedicated slot

- Additional Internal Storage supported

- Three hard disk drive bays

- Two removable 5.25-inch half-height removable media bays, or

- One removable 5.25-inch full-height removable media bay

- An integral UltraSCSI-2 controller is included in each system, which supports a maximum of seven internal devices, four SCA-2 devices plus three removable devices.

Storage devices in the Pedestal and Internal Disk Drives

- All Pedestal and Rackmount packaged systems include one 4.3 GB SCSI SCA-2 hard disk drive (RZ1CB-SB).

- Up to three additional hard disk drives may be added internally.

| RZ26N-SB | 1.05 GB SCSI SCA-2 5400 RPM 3.5 x 1-inch hard disk drive |
| --- | --- |
| RZ1BB-SB | 2.1 GB UltraSCSI SCA-2 7200 RPM 3.5 x 1-inch hard disk drive |
| RZ1CB-SB | 4.3 GB UltraSCSI SCA-2 7200 RPM 3.5 x 1-inch hard disk drive |
| RZ1DB-SB | 9.0 GB UltraSCSI SCA-2 7200 RPM 3.5 x 1.6" hard disk drive |

Step 3a—Internal Storage (continued)

Internal Removable Media Devices

- All Pedestal and Rackmount packaged systems include one 600 MB CD-ROM drive.

- System supports two additional 5.25-inch half-height removable media devices or one additional 5.25-inch full-height removable media device.

| RRD46-AB | 600 MB 5.25 inch half-height 12X CD-ROM drive |
| --- | --- |
| PBXTZ-AA | 2.0 GB 5.25 inch half-height SCSI QIC tape drive (TZK11) |
| TLZ09-LK | 8.0 GB 5.25 inch x 1.6-inch SCSI 4 mm DAT drive, embedded |
| TLZ10-LK | 12/24 GB 4MM DAT SCSI Tape Drive, embedded |

Internal SCSI Cable Kits

| PB8HA-DA | Internal 68-pin HD SCSI cable. Connects 16-bit wide SCSI controller to internal storage assembly. Example—connects the internal port of the KZPAC-xx or KZPBA-CA PCI RAID controller to internal AlphaServer 800 storage assembly, maximum one per system. |
| --- | --- |
| BC25V-1H | Internal UltraSCSI cable breakout assembly kit. Connects the internal second and third ports of the KZPAC-AC to one of the two expansion breakout slots on back of AlphaServer 800. |

Step 3b—Storage Controllers

- All packaged systems include an integral UltraSCSI adapter that supports seven internal devices (four SCA-2 hard disk drives, plus three removable media devices).

- The PB8HA-DA cable is required to connect optional PCI RAID based controllers to the internal disk drives.

- If the internal drives are connected to a PCI based RAID controller (KZPAC-xx) instead of the integral Ultra SCSI controller, Fault Management must be disabled when the RCU (Raid Configuration Utility) is executed.

- SCSI cables are not included and must be ordered separately.

- External DSSI cables are not included and must be ordered separately.

- Maximum combination number of KZPAC PCI-based SCSI controllers supported is four.

- Charts indicate minimum level of operating system support.
Note: Minimum operating system level support for DIGITAL UNIX with MEMORY CHANNEL is V4.0B.

- Refer to the Supported Options List for O/S support (URL=http://www.digital.com/alphaserver/tech_docs/alphasrv800/docs/supported_options.html).

- The following adapters and controllers are supported.

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPBA-CA | PCI one-port Single-Ended UltraSCSI adapter internal 68-pin and 50-pin HD connectors and external 68-pin HD SCSI connector. | 2 | 2 | 2 |
| BN38C-02 | 2 meter Single-ended UltraSCSI cable for external connections. 68-pin HD connector to 68-pin VHDCI connector. Required for each UltraSCSI StorageWorks enclosure attached. |  |  |  |
| PB8HA-DA | Internal 68-pin HD SCSI cable. Connects 16-bit wide SCSI controller to internal storage assembly. Example—connects internal port of KZPAC-xx or KZPBA-CA PCI RAID controller to internal AlphaServer 800 storage assembly, maximum one per system. |  |  |  |

Step 3b—Storage Controllers (continued)

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPBA-CB1 | PCI-based one port UltraSCSI Differential adapter, 68-pin HD connector | 2 | 2 | 2 |
| BN38C-02 | 2 m VHDCI male to HD 68-pin male SCSI cable, required for each StorageWorks shelf attached to KZPBA adapter. |  |  |  |

1.	DIGITAL UNIX and OpenVMS operating systems currently support the KZPBA-CB adapter via direct attachment in existing environments.  Multi-host shared SCSI bus configurations using the KZPBA-CB adapter are supported on OpenVMS systems.  Multi-host configurations are not supported on DIGITAL UNIX systems at this time.  For a complete list of restrictions, refer to Supported Options List at: http://www.digital.com/alphaserver/tech_docs/alphasrv800/docs/supported_options.html.

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPSA-BB | PCI one-port Fast Wide Differential (FWD) SCSI controller with 68-pin HD external connector. Supports Single-Ended storage devices via the DWZZA or DWZZB signal converters. Internal hard disk drives are not supported on FWD controller. | 4 | 4 | 4 |
| BN21K-02 | 2 meter Single-Ended SCSI cable 68-pin HD straight male connector with thumb latches and one 68-pin HD right angle male connector with screw fasteners. |  |  |  |
| BN31G-xx | SCSI Cable 68-pin HD straight connector at each end; -xx denotes length of cable |  |  |  |
| BN21W-0B | SCSI Cable 68-pin HD SCSI "Y" Cable |  |  |  |
| DWZZA-xx | SCSI bus signal converter. 16-bit differential to 8-bit Single-Ended. |  |  |  |
| DWZZB-xx | SCSI bus signal converter. 16-bit differential to 16-bit Single-Ended |  |  |  |

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPAA-AA | PCI one-port Fast Narrow Single-Ended 8-bit SCSI controller with 50-pin external connector | 4 | 4 | 4 |
| BN21N-02 | 2 meter 8-bit to 16-bit SCSI cable, connects KZPAA to 16-bit Fast10 StorageWorks shelf. |  |  |  |

Step 3b—Storage Controllers (continued)

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPAC-AA | PCI one-port UltraSCSI RAID controller with 4 MB cache memory, includes RAID Array 230/Plus Subsystem software and documentation kit for OpenVMS, DIGITAL UNIX and Windows NT. 68-pin HD connector on module for internal connection and 68-pin VHDCI connector on module’s standard bulkhead for external connections, tape drives are not supported. KZPAC-xx installed in a system requires PB8HA-DA SCSI cable kit for connection to internal storage assembly. If the internal drives are connected to the KZPAC Fault Management must be disabled when RCU is executed. | 4 | 4 | 4 |
| KZPSC-UB | Cache battery-backup for KZPAC controller |  |  |  |
| PB8HA-DA | SCSI cable kit for connection to internal storage assembly |  |  |  |
| BN37A-02 | 02 meter UltraSCSI cable with two 68-pin VHDCI connectors. Required for each UltraSCSI StorageWorks shelf attached to KZPAC controller |  |  |  |

Step 3b—Storage Controllers (continued)

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KZPAC-CA | PCI three-port UltraSCSI RAID controller with 4 MB cache memory, includes RAID Array 230/Plus Subsystem Software and documentation kit for OpenVMS, DIGITAL UNIX and Windows NT. Tape drives are not supported. SCSI cable are required for each active port. BN31K-0E or KZPAC-SB or BC25V-1H must be ordered for connection of third port to AlphaServer 800 bulkhead breakout. KZPAC-xx installed in a system requires PB8HA-DA SCSI cable kit for connection to internal storage assembly. If the internal drives are connected to KZPAC, Fault Management must be disabled when RCU is executed. | 2 | 2 | 2 |
| KZPAC-CB | Same as KZPAC-CA with 8 MB cache memory |  |  |  |
| KZPSC-UB | Cache battery-backup for KZPAC controller |  |  |  |
| MS100-BB | 8 MB EDRAM cache memory replacement, upgrades KZPAC-CA to KZPAC-CB |  |  |  |
| PB8HA-DA | Internal 68-pin HD SCSI cable. Connects 16-bit wide SCSI controller to internal storage assembly. Example—connects internal port of KZPAC-xx or KZPBA-CA PCI RAID controller to internal AlphaServer 800 storage assembly, maximum one per system. |  |  |  |
| BC25V-1H | Internal UltraSCSI cable breakout assembly kit. Connects internal second and third ports of KZPAC-AC to one of the two expansion breakout slots on back of AlphaServer 800 |  |  |  |
| BN37A-xx | UltraSCSI cable with two 68-pin VHDCI connectors. Required for each UltraSCSI StorageWorks shelf/enclosure attached to KZPAC controller. |  |  |  |
| BN31S-1E | 1.5 meter external SCSI cable, required for each F10 StorageWorks shelf attached to KZPAC controller. 68-pin VHDCI to 68-pin HD. |  |  |  |
| BN31K-0E | SCSI cable/bulkhead assembly kit for connection of third port to AlphaServer 800 PCI/EISA bulkhead slot. Uses one PCI/EISA bulkhead slot. |  |  |  |
| KZPAC-SB | SCSI bulkhead assembly kit with two ports, allows connection of 2 third-port outputs using one PCI bulkhead slot |  |  |  |

Step 3b—Storage Controllers (continued)

DSSI Controllers

|  |  | Maximum # Supported |  |  |
| --- | --- | --- | --- | --- |
| Order Number | Description | Windows NT 4.0 | DIGITAL UNIX V4.0B | OpenVMS V7.1-1H1 |
| KFPSA-AA | PCI-DSSI controller. PCI-based single DSSI controller. (OpenVMS systems only). | — | — | 4 |
| BC29S-xx | For KFPSA PCI based DSSI controller. KFPSA to any external “Micro-Ribbon” DSSI right-angle connection (all other DSSI systems and storage devices requiring right-angle connection) requires BC29S-xx DSSI cable. |  |  |  |
| BC29S-09 | For KFPSA PCI based DSSI controller. Order BC29S-09 DSSI cable for HSD10 in BA36R-Ax shelves |  |  |  |

Step 3c—External Tape Expansion

- External tape drives are also supported on optional PCI-based SCSI controllers.

- KZPAA-AA—maximum bus length including cable and tape device cannot exceed 3.0 meters.

- KZPSA-BB—maximum bus length including cable and tape device cannot exceed 25.0 meters.

- External tape drives are not supported on one-and three-port (KZPAC-xx) Fast UltraSCSI-2 controllers on systems running DIGITAL UNIX or OpenVMS.

External Tapes

| Operating System Supported |  | OpenVMS | DIGITAL UNIX | Windows NT |
| --- | --- | --- | --- | --- |
| TLZ09-DB | 4.0/8.0 GB 4 mm DAT tabletop tape drive with 50-pin HD SCSI port. Includes BN23G-01 cable. | V7.1-1H1 | V4.0B | V4.0 SP2 |
| TLZ9L-DB1,2 | 32/64 GB 4 mm DAT tabletop autoloader with 50-pin HD SCSI port. Includes BN23G-01 cable. | V7.1-1H1 | V4.0B | V4.0 SP2 |
| DS-TLZ10-DB | 12/24 4mm DAT SCSI tape drive, tabletop. Requires SCSI cable BN23G-01 or BN31W-01 (68-pin HD host) or BC09D-03 (50-pin LD host). | V7.1-1H1 | V4.0B |  |
| TSZ07-CA/FA | 40/140 MB,1600 b/in, tabletop tape drive (SE/DIFF). Requires country kit which includes cables (TSZK7-xx; see StorageWorks tape device section. | V7.1-1H1 | V4.0B | V4.0 SP2 |
| TSZ07-AA/AD | 40/140 MB, 1600 b/in tape drive rackmountable. Requires country kit which includes cables (TSZK7-xx; see StorageWorks tape device section. | V7.1-1H1 | V4.0B | V4.0 SP2 |
| TSZ07-BA/EB | 40/140 MB, 1600 b/in, tape in a cabinet (H9642/H9A10). Includes SCSI cable (see StorageWorks tape device section). | V7.1-1H1 | V4.0B | V4.0 SP2 |
| DS-TZ89N-TA | 35/70 5.25-inch differential DLT tabletop tape drive with 68-pin HD female connector. SCSI cable not included. Requires BN21K-xx. | V7.1-1H1 | V4.0B | V4.0 SP2 |
| TZK11-DA | 2.0 GB 5.25-inch tabletop QIC streaming tape drive. Includes 3 foot SCSI cable and terminator. Includes 3 ft SCSI cable. | V7.1-1H1 | V4.0B | V4.0 SP2 |
| DS-TZS20-DB | 25/50GB 5.25-inch AIT (8 mm) tabletop tape drive; #277 blue. SCSI cable not included. Requires BN21N-01 (50- pin HD host) or BN31G-01(68-pin HD host) SCSI cable | V7.1-1H1 | V4.0B | V4.0 SP2 |

1.	Includes four cartridge loader. Larger magazines are supported.

2.	Base operating systems support sequential back-up mode only; additional software is required for random access backup. See Storage Devices section for details.

Step 3c—External Tape Expansion (continued)

SCSI Cables for External Tapes

| BN23G-01 | 3-foot molded SCSI cable. 50-pin Single-Ended SCSI cable (one 50-pin HD straight male connector with thumb latches and one 50-pin low density straight male connector with bale locks.) |
| --- | --- |
| BN23G-0E | 3 foot molded SCSI cable. 68-pin Single-Ended SCSI cable. (one 68-pin HD straight male connector with thumb latches and one 68-pin HD right angle male connector with screw fasteners.) |
| BN21K-xx | Single-Ended SCSI cable 68-pin HD straight male connector with thumb latches and one 68-pin HD right angle male connector with screw fasteners. |

Step 3d—External Disk Expansion

See Storage Devices section for StorageWorks products and configuration rules.

Step 4—Monitors

- Windows NT systems require a graphics monitor to run all functions.

- Graphics monitors other than those listed below can be used if compatible with SVGA graphics included with system.

Note:	Higher resolution available with optional EISA or PCI graphics adapters.  See Step 5.

| SN-VRCX5-WA/W3/W4 | 15" (13.9" viewable image size) Corporate Series auto-scan color monitor, flat square invar CRT, 0.28 mm dot pitch, VGA to 1024 x 768 @ 85 Hz, TCO 92, MPRII, Energy Star, attached video cable. -WA = Northern Hemisphere with 120 V power cord. -W3 = Northern Hemisphere without power cord. -W4 = Southern Hemisphere without power cord. Select country specific power cords for -W3 and -W4 variants. |
| --- | --- |

| SN-VRTX7-WA | 17” (16.0” viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 at 75 Hz, TCO 92, MPR-II, Energy Star, HD15 male to male video cable. Northern Hemisphere with 120V power cord. |
| --- | --- |

| SN-VRCX1-WA/W3/W4 | 21" (19.7" viewable image size) auto-scanning color monitor, 0.28 Dot Pitch, VGA to 1600 x 1200 at 75 Hz NI, TCO 92, Energy Star, includes video cable. -WA = Northern Hemisphere with 120 V power cord. -W3 = Northern Hemisphere without power cord. -W4 = Southern Hemisphere without power cord. Select country specific power cord for -W3 and -W4 variants. |
| --- | --- |

Step 5—Graphics Adapters

- Select only if additional graphics card is required.

- All systems include onboard SVGA with 1 MB video RAM. On-board SVGA must be disabled when using multi-headed graphics options.

- Note:  Only 1 GB maximum system memory is supported by PBXGB-AA/CA.

- Maximum of one PB2GA-JD supported per system. Onboard SVGA must be disabled.

- Maximum of four PBXGB-AA/CA supported on DIGITAL UNIX systems. Maximum of one PBXGB-AA/CA supported on OpenVMS and Windows NT systems. Onboard SVGA must be disabled.

| PB2GA-JC | PCI-based 1 MB DRAM graphics adapter |
| --- | --- |
| PB2GA-JD | PCI-based 2-MB DRAM graphics adapter |
| PBXGB-AA | PCI-based 8-plane 1280 x 1024 graphics adapter |
| PBXGB-CA | PCI-based 24-plane 1280 x 1024 x 1024 graphics adapter |

Step 6—Networks and Communications

- Packaged systems include PCI-based Fast EtherWorks (Twisted Pair) controller (DE500-AA)—uses one PCI slot.

- Select networking cable for Ethernet controller

- BN25G-03/04/07 or BN24Q-xx for 10BaseT (Twisted Pair)

- Refer to the Supported Options List for O/S support, restrictions, and maximum number of options supported per system (URL=http://www.digital.com/alphaserver/tech_docs/alphasrv800/docs/supported_options.html).

| CXI01-AA | ISA asynchronous MUX adapter, 16 lines – expandable to 64 lines; DIGITAL UNIX, Windows NT—not supported in bottom EISA/ISA slot |
| --- | --- |
| CXI01-AD | ISA asynchronous MUX adapter, 16 lines - expandable to 224 lines; DIGITAL UNIX, Windows NT— not supported in bottom EISA/ISA slot |
| CXI01-AB | Asynchronous 16-port XEM expansion for CXI01-AA/AD |
| CXI01-AC | Asynchronous MUX cable converter (RJ45-DB25) |
| CXI01-AE | Asynchronous 16-port EPC expansion for CXI01-AD; maximum supported is four |
| CXI01-AF | Asynchronous MUX adapter (RJ45-MJ11), 8-pack |
| DE500-AA | PCI-based Fast Ethernet network interface card |
| DE500-FA | PCI-based EtherWorks PCI 10/100 32-bit PCI 10/100 Mb/s MMF Adapter |
| DE450-CA | PCI-based DIGITAL Etherworks 32-bit network interface card (Twisted Pair, Thickwire, ThinWire) |
| DEFPA-AB | PCI-based DEC FDDIcontroller, single attachment; maximum supported is four, OpenVMS, Windows NT, DIGITAL UNIX |
| DEFPA-DB | PCI-based DEC FDDIcontroller, dual attachment, maximum supported is four, OpenVMS, Windows NT, DIGITAL UNIX |
| DEFPA-UB | PCI-based DEC FDDIcontroller (UTP); maximum supported is four, OpenVMS, Windows NT, DIGITAL UNIX |
| DGLPA-FA | ATMworks 351 ATM PCI bus adapter, MMF |
| DGLPA-UA | ATMworks 351 ATM PCI bus adapter, UTP |
| PBXDA-AA | PCI-based 4-port asynchronous communications controller—not supported in PCI/EISA combo slot |
| PBXDA-AB | PCI-based 8-port asynchronous communications controller—not supported in PCI/EISA combo slot |
| PBXDP-AA | PCI-based 2-port synchronous communications controller, maximum four, OpenVMS only |
| PBXDP-AB | PCI-based 4-port synchronous communications controller, maximum four, OpenVMS only |
| PBXDP-AC | PCI-based 8-port synchronous communications controller, maximum four, OpenVMS only |
| SN-PBXNP-AC | PCI Token Ring Adapter, maximum two, OpenVMS, Windows NT, DIGITAL UNIX |

Step 7—Power Cords, Keyboards, and Miscellaneous

Keyboards

- Windows NT US variants include a 101-key, PC-style US English keyboard. Windows NT non-US variants must order the appropriate keyboard.

- OpenVMS variants include a 108-key, VT-style US English keyboard. Other keyboard variants must be ordered.

- UNIX variants include a 104/105 key PC-style US English keyboard. Other keyboard variants must be ordered.

- English language documentation. Certain language selections are available.  See Step 10.

| 104/105 key PC style keyboard |  | Language | 108 key VT style keyboard |  |
| --- | --- | --- | --- | --- |
| Windows NT | DIGITAL UNIX |  | OpenVMS | DIGITAL UNIX |
| LK97W-A2 | LK97W-A2 | U.S./English | LK46W-A2 | LK46W-A2 |
| LK97W-AB | LK97W-AB | Belgian | LK46W-AB | LK46W-AB |
| LK97W-AC | LK97W-AC | Canadian/French | LK46W-AC | LK46W-AC |
| LK97W-AD | LK97W-AD | Danish | LK46W-AD | LK46W-AD |

Step 7—Power Cords, Keyboards, and Miscellaneous (continued)

| 104/105 key PC style keyboard |  | Language | 108 key VT style keyboard |  |
| --- | --- | --- | --- | --- |
| Windows NT | DIGITAL UNIX |  | OpenVMS | DIGITAL UNIX |
| LK97W-AE | LK97W-AE | United Kingdom | LK46W-A2 | LK46W-A2 |
| LK97W-AF | LK97W-AF | Finnish | LK46W-AF | LK46W-AF |
| LK97W-AG | LK97W-AG | German | LK46W-AG | LK46W-AG |
| LK97W-AH | LK97W-AH | Dutch | LK46W-AH | LK46W-AH |
| LK97W-AI | LK97W-AI | Italian | LK46W-AI | LK46W-AI |
| LK97W-AF | LK97W-AF | Swedish | LK46W-AM | LK46W-AM |
| LK97W-AN | LK97W-AN | Norwegian | LK46W-AN | LK46W-AN |
| LK97W-AP | LK97W-AP | French | LK46W-AP | LK46W-AP |
|  |  | Canadian/English |  | LK46W-AQ |
| LK97W-AR | LK97W-AR | Lat. America |  |  |
| LK97W-AS | LK97W-AS | Spanish | LK46W-AS | LK46W-AS |
| LK97W-AV | LK97W-AV | Portuguese | LK46W-AV | LK46W-AV |
| LK97W-BH | LK97W-BH | Greek | LK46W-BH | LK46W-BH |
| LK97W-AT | LK97W-AT | Hebrew | LK46W-AT | LK46W-AT |
| LK97W-BI | LK97W-BI | Taiwanese |  |  |
| LK97W-BP | LK97W-BP | Polish | LK46W-BP | LK46W-BP |
| LK97W-BQ | LK97W-BQ | Hungarian | LK46W-BQ | LK46W-BQ |
| LK97W-CZ | LK97W-CZ | Slovak | LK46W-CZ | LK46W-CZ |
| LK97W-BY | LK97W-BY | Serbian |  |  |
| LK97W-CB | LK97W-CB | Thai |  |  |
| LK97W-CQ | LK97W-CQ | Icelandic |  |  |
| LK97W-BV | LK97W-BV | Czech | LK46W-BV | LK46W-BV |
| LK97W-BR | LK97W-BR | Arabic |  |  |
| LK97W-BU | LK97W-BU | Turkish Q |  |  |
| LK97W-BW | LK97W-BW | Turkish F | LK46W-BW | LK46W-BW |
| LK97W-BT | LK97W-BT | Cyrillic |  |  |

Pedestal and Monitor Power Cords

- North American variants include (North American, Japan) power cord. If another power cord is selected, both power cords ship with system.

- For non-North American system variants, selection of a power cord is mandatory.

| BN19H-2E | Australia, New Zealand, 2.5 meters long |
| --- | --- |
| BN19C-2E | Central Europe, 2.5 meters long |
| BN19A-2E | UK, Ireland, 2.5 meters long |
| BN19E-2E | Switzerland, 2.5 meters long |
| BN19K-2E | Denmark, 2.5 meters long |
| BN19Z-2E | Italy, 2.5 meters long |
| BN19S-2E | Egypt, India, South Africa, 2.5 meters long |
| BN18L-2E | Israel, 2.5 meters long |

Step 7—Power Cords, Keyboards, and Miscellaneous (continued)

Rackmount Power Cords

- North American variants include BN20Z-4E 120 V (North American, Japan) power cord.  If another power cord is selected, both power cords ship with system.

- For non-North American system variants, selection of a power cord is mandatory.

Note:	AlphaServer 800 Pedestal system power cords are less then 15-feet long and are not supported in cabinet enclosures.

| BN20Q-4E | 15-feet 240 V, North American, 50 Hz power cord connects system to power distribution unit located inside cabinet |
| --- | --- |
| BN20Z-4E | 15-foot (4.5-meter) 120 V, 60 Hz power cord connects system to power distribution unit located inside cabinet |

Pedestal to Rackmount Conversion Kit

| PB8HA-BA | AlphaServer 800 Pedestal to Rackmount conversion kit. Includes bezel, slide rails and mounting hardware necessary for mounting system in any cabinet equipped with industry-standard RETMA or METRIC rails. Rackmount power cords must be ordered separately. |
| --- | --- |

Mouse and Extension Cable Kit

| PBXWS-WA | 3-button mouse (included with all systems, order as spare or replacement) |
| --- | --- |
| 2T-450KM-AA | Extension cable kit for VGA, PC style keyboard, and mouse, for use with Rackmount systems. |

Step 8—Cabinet Enclosure

Rackmount packaged systems may be placed in an existing cabinet. If required, select cabinet enclosure for packaged AlphaServer 800 Rackmount systems. The color of cabinets listed is DIGITAL 277 Blue-Gray.

Note:	For custom configuration and integration for an AlphaServer 800 Rackmount system into a DIGITAL cabinet, call Computer Special Systems 1-800-DIGITAL

- H9A10 19-inch EIA cabinet enclosure dimensions

- Outside: 66.9-inches high, 23.62-inches wide, 33.8-inches deep.

- Internal usable rackmountable space: 56 inches high, 19 inches wide, 30.8 inches deep.

- An H9A10 cabinet with two H7600 power controllers can support a maximum of six AlphaServer 800 Rackmount systems.

- H9A15 19-inch EIA cabinet enclosure dimensions

- Outside: 78.7 inches high, 23.62 inches wide, 33.4 inches deep.

- Internal usable rackmountable space: 68.25 inches high, 19 inches wide, 29.8 inches deep.

- An H9A15 cabinet with two H7600 power controllers can support a maximum of six AlphaServer 800 Rackmount systems.

- H9A11 19-inch EIA Cabinet Enclosure Dimensions

- Outside: 43.3-inches high, 33.8-inches wide, 30.8-inches deep

- Internal usable rackmountable space: 35-inches high, 19-inches wide, 30.8 inches

- An H9A11 cabinet with one H7600 power controller can support a maximum of three AlphaServer 800 Rackmount systems.

- Cabinet power plugs

- 120 V cabinets have one or two L5-30P plugs

- 240 V cabinets have one or two IEC-pin-and-sleeve 316 P6 plugs

| H9A10-RL | 120V EIA Cabinet assembly, with two power controller, rear swing door |
| --- | --- |
| H9A10-RT | 240V EIA Cabinet assembly, with two power controller, rear swing door |
| H9A10-EJ | 120V EIA Cabinet assembly, with two power controller, front and rear swing door |
| H9A10-EH | 240V EIA Cabinet assembly, with two power controller, front and rear swing door |

| H9A15-RA | 120V EIA Cabinet assembly, with two power controllers, rear swing door |
| --- | --- |
| H9A15-RG | 240V EIA Cabinet assembly, with two power controllers, rear swing door |
| H9A15-RE | EIA Cabinet assembly, no power, rear swing door |

| H9A11-RA | 120V EIA Cabinet assembly, with one power controller, lift-off rear door |
| --- | --- |
| H9A11-RG | 240V EIA Cabinet assembly, with one power controller, lift-off rear door |

Step 9—Software

Windows NT

- North American variants of Windows NT packaged systems include Windows NT license, media kits (CD-ROM).

- Selection of language specific Windows NT media kit is mandatory for non North American variants.

- Windows NT Server kits are only orderable with packaged systems. Not available separately.

Windows NT Server plus 10-client access license, media kits (CD-ROM)

| QB-23CAA-SB | Windows NT server license, media kit North American English |
| --- | --- |
| QB-23C8A-SB | Windows NT Server license, media kit International English |
| QB-23CPA-SB | Windows NT server license, media kit French |
| QB-23CGA-SB | Windows NT server license, media kit German |
| QB-23CSA-SB | Windows NT server license, media kit Spanish |
| QB-23CUA-SB | Windows NT server license, media kit Italian |
| QB-23CJA-SB | Windows NT server license, media kit Japanese |
| QB-23CMA-SB | Windows NT server license, media kit Swedish |
| QB-23CHA-SB | Windows NT server license, media kit Dutch |
| QB-23CVA-SB | Windows NT server license, media kit Brazilian/Portuguese |
| QB-23C4A-SB | Windows NT server license, media kit Korean |
| QB-23C3A-SB | Windows NT server license, media kit Taiwanese |
| QB-23C2A-SB | Windows NT server license, media kit PRC Chinese |

Windows NT Server Optional software and documentation

| QB-53V9A-SA | Windows NT Server Cluster Kit |
| --- | --- |

DIGITAL UNIX

Software Processor Code = E

- DIGITAL UNIX packaged systems include Traditional Unlimited User license.

- DIGITAL UNIX packaged systems require operating system media and documentation for first system on site.

DIGITAL UNIX Media and Documentation—required for first system on site

| QA-MT4AA-H8 | DIGITAL UNIX media and on-line documentation on CD-ROM |
| --- | --- |
| QA-MT4AA-GZ | DIGITAL UNIX full hardcopy documentation |

StorageWorks Software Packages for DIGITAL UNIX

| QB-5RXAE-AA | StorageWorks Software Package (includes licenses for Logical Storage Manager and AdvFS Utilities). |
| --- | --- |
| QB-5RYAE-AA | StorageWorks Software Package (includes licenses for Logical Storage Manager and AdvFS Utilities plus Networker Save and Restore). |

TruCluster Available Server License

| QL-05SAE-AA | TruCluster Available Server License |
| --- | --- |

DIGITAL UNIX Software Layered Products Library CD-ROM

| QA-054AA-H8 | Layered products media and documentation for DIGITAL UNIX on CD-ROM |
| --- | --- |

DECnet for DIGITAL UNIX

| QL-MTJAE-AA | DECnet/OSI end-system license for DIGITAL UNIX |
| --- | --- |
| QL-MTKAE-AA | DECnet/OSI extended function license for DIGITAL UNIX |

Step 9—Software

OpenVMS

- Software Processor Code = E.

- OpenVMS Concurrent Use Licenses are not specific to a single system and can be moved between systems at user discretion.

- OpenVMS Concurrent Use Licenses can also be shared in a mixed OpenVMS VAX and OpenVMS Alpha Cluster.

OpenVMS Concurrent Use Licenses

| QL-MT3AA-3B | OpenVMS Concurrent Use 1-user license |
| --- | --- |
| QL-MT3AA-3C | OpenVMS Concurrent Use 2-user license |
| QL-MT3AA-3D | OpenVMS Concurrent Use 4-user license |
| QL-MT3AA-3E | OpenVMS Concurrent Use 8-user license |
| QL-MT3AA-3F | OpenVMS Concurrent Use 16-user license |
| QL-MT3AA-3G | OpenVMS Concurrent Use 32-user license |
| QL-MT3AA-3H | OpenVMS Concurrent Use 64-user license |
| QL-MT3AA-3J | OpenVMS Concurrent Use 128-user license |
| QL-MT3AA-3K | OpenVMS Concurrent Use 256-user license |
| QL-MT2AE-AA | OpenVMS Traditional unlimited user license |

OpenVMS layered products

| QL-MUZAE-AA | OpenVMS Cluster License |
| --- | --- |
| QL-2A1AE-AA | Volume Shadowing License |

OpenVMS media and documentation—required for first system on site

| QA-MT1AA-H8 | OpenVMS media and on-line documentation CD-ROM (V7.1) |
| --- | --- |
| QA-MT1AG-H8 | OpenVMS media and on-line documentation CD-ROM (V6.2-1H3))2 |
| QA-MT1AD-H8 | OpenVMS media and on-line documentation CD-ROM (V7.1-1H1)1 |
| QA-001AA-GZ | OpenVMS hardcopy documentation |

1.	Required to  run Ultra SCSI.

2.	Ultra SCSI is not yet supported on OpenVMS V6.2-1H3.  OpenVMS V6.2-1H3 will run in Fast-10 Mode.

OpenVMS Layered Products CD-ROM

| QA-03XAA-H81 | Layered products media and documentation for OpenVMS on CD-ROM |
| --- | --- |

1.	Includes DIGITAL Enterprise Integration Package for OpenVMS media and documentation.

DIGITAL Enterprise Integration Package

| QA-5LVAA-H8 | DIGITAL Enterprise Integration Package for OpenVMS media and documentation |
| --- | --- |

DIGITAL Enterprise Integration Package

DIGITAL Enterprise Integration Package for OpenVMS includes: OpenVMS Management Station, DECamds, Archive Backup System for OMT, Archive Backup Agent for NT, Datametrics ViewPoint Data Collector, Datametrics ViewPoint Management Console, eXcursion, ISG/Navigator ODBC & OLE DB Server, ISG/Navigator ODBC & OLE DB Client, Internet Product Suite, Pathworks V5.0 LAN Manager, Pathworks32, Team Links Mail Windows, MAPI Driver for
All-In-1, DECwindows Motif, DECprint Supervisor, DEC TCP/IP, DECnet.

DECnet for OpenVMS

| QL-MTGAE-AA | DECnet extended function license for OpenVMS |
| --- | --- |
| QL-MTHAE-AA | DECnet end-system to extended function upgrade license for OpenVMS |

Step 9—Software (continued)

DSSI Information

| EK-410AB-MG | DSSI VMScluster Installation Guide |
| --- | --- |
| EK-D4AXP-TS | DSSI VMScluster Troubleshooting Guide |

Step 10—Hardware and Software Supplemental Support Services

Hardware—Americas and Asia Pacific only

- Systems include three-year hardware warranty, on-site with 5 x 9, 24-hour response time.

- Select optional Hardware Supplemental Support Services if required.

- Select optional Hardware Installation.

| FM-CH4HR-36 | 5 x 9, 4-hour response time |
| --- | --- |
| FM-CH512-36 | 5 x 12, 4-hour response time |
| FM-CH616-36 | 6 x 16, 4-hour response time |
| FM-CH724-36 | 7 x 24, 4-hour response time |
| FM-CHXHW-60 | Years 1-5, next day, Onsite |
| FM-CH4HR-60 | Years 1-5, 5x9, 4-hour response time |
| FM-CH512-60 | Years 1-5, 5x12, 4-hour response time |
| FM-CH616-60 | Years 1-5, 6x16, 4-hour response time |
| FM-CH724-60 | Years 1-5, 7x24, 4-hour response time |
| FM-24INT-IN | Hardware Installation |

Software—Americas and Asia Pacific only

- DIGITAL UNIX and OpenVMS systems include 90-day Conformance to SPD and Telephone Advisory Support. Select optional Software Supplemental Support Services, if required.

- Warranty on Microsoft’s Windows NT Server 4.0 is conformance to the written material accompanying the software for a period of 90 days.

- Software service upgrades for Windows NT include advisory and remedial software support for the time period indicated.

| FM-WNTO3-12 | 12-month Software Supplemental Support for Windows NT AlphaServer 800 systems |
| --- | --- |
| FM-WNTO3-36 | 36-month Software Supplemental Support for Windows NT AlphaServer 800 systems |
| FM-WNTO3-60 | 60-month Software Supplemental Support for Windows NT AlphaServer 800 systems |
| FM-80BUS-12 | 12-month Full Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BUS-36 | 36-month Full Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BUS-60 | 60-month Full Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BUN-12 | 12-month Node Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BUN-36 | 36-month Node Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BUN-60 | 60-month Node Software Supplemental Support for DIGITAL UNIX AlphaServer 800 systems |
| FM-80BVS-12 | 12-month Software Supplemental Support for OpenVMS AlphaServer 800 systems |
| FM-80BVS-36 | 36-month Software Supplemental Support for OpenVMS AlphaServer 800 systems |
| FM-80BVS-60 | 60-month Software Supplemental Support for OpenVMS AlphaServer 800 systems |
| FM-80BVN-12 | 12-month Node Software Supplemental Support for OpenVMS AlphaServer 800 systems |
| FM-80BVN-36 | 36-month Node Software Supplemental Support for OpenVMS AlphaServer 800 systems |
| FM-80BVN-60 | 60-month Node Software Supplemental Support for OpenVMS AlphaServer 800 systems |

Step 10a—Hardware and Software Supplemental Support Services (Europe only)

Europe does not have specific part numbers for Hardware and Software Supplemental Support Services. Prices can be quoted using the Excelerator tool; contact MCS Sales in your country for information on Hardware and Software Supplemental Support Services.

Step 11—CPU Upgrades

| Order Number | From | To | Includes |
| --- | --- | --- | --- |
| PB81U-AZ | AlphaServer 800 5/333 | AlphaServer 800 5/400 | 400 MHz CPU Note: Mandatory return of 333 MHz CPU |
| PB82U-AZ | AlphaServer 800 5/333 | AlphaServer 800 5/500 | 500 MHz CPU Note: Mandatory return of 333 MHz CPU |
| PB82U-FZ | AlphaServer 800 5/400 | AlphaServer 800 5/500 | 500 MHz CPU Note: Mandatory return of 400 MHz CPU |

AlphaServer 800 System Diagram–Side View

1.	Removable side cover of system enclosure

2.	Floppy diskette drive

3.	Removable media drives

4.	Lower and upper doors

5.	Control panel

6.	Hard disk drives

7.	System board

8.	CPU daughter card

9.	Power supply

AlphaServer 800 System Diagram – Rear of Enclosure

1.	Three 32-bit PCI slots

2.	One 64-bit PCI/EISA slot

3.	Two EISA slots

4.	Parallel port

5.	Serial port

6.	Mouse port

7.	VGA port

8.	Keyboard port

9.	Remote console modem port

10.	Power inlet

11.	Serial port/terminal port (MMJ with data leads only)

12.	SCSI breakouts

AlphaServer 800 Pedestal System Specifications

| Shipping Dimension |  |  |  |
| --- | --- | --- | --- |
| Height Width Depth Weight |  | 47 cm (18.5 in.) 60 cm (23.6 in) 77.5 cm (30.5 in) 33 kg (73 LB) typical 37 kg (82 LB) maximum |  |
| Installed Dimensions |  |  |  |
| Height Width Depth Weight |  | 45 cm (17.1 in.) 22.6 cm (8.9 in.) 65.8 cm (25.9 in.) 24.5 kg (54 LB) typical 28 kg (62 LB) maximum |  |
| Clearances | Operating |  | Service |
| Front Rear Left side Right side | 75 cm (29.5 in.) 15 cm (5.9 in.) None None |  | 75 cm (29.5 in.) 75 cm (29.5 in.) 75 cm (29.5 in.) 5 cm (2 in.) |

AlphaServer 800 Pedestal System Specifications (continued)

| Environmental |  |  |  |
| --- | --- | --- | --- |
| Temperature | Operating1 Nonoperating Storage (60 days) Rate of change |  | 10-40° C (50-104° F) Not tested -40-66° C (-40-151° F) 11° C/hr (20° F/hr) |
| Relative humidity | Operating Nonoperating Storage (60 days) Rate of change |  | 20–80% 20–80% 10–95% 20%/hr |
| Maximum wet bulb temperature | Operating Storage (60 days) |  | 28° C (82° F) 46° C (115°F) |
| Minimum dew point temperature | Operating Storage (60 days) |  | 2° C (36° F) Not tested |
| Maximum heat dissipation | 380 Watt 1297 Btu/hr |  |  |
| Air flow and quality | Intake location Exhaust location Particle size Concentration |  | Front Rear N/A N/A |
| Altitude | Operating2 Nonoperating |  | 3097 m (10,000 ft) 12190 m (40,000 ft) |
| Mechanical shock | Operating Nonoperating |  | 7.5 G 10 ms 20 G peak 30 ms |
| Vibration | Operating |  | 10-500 Hz .1 G peak |
| Acoustics | Operating |  | LNPEc (BELs) 6.0 MAX per ISO 7779 |
| Electrical—Power Supplies are universal 120/240 Vac |  |  |  |
| Nominal ac voltage Operating Voltage range Power source phase Nominal frequency Frequency range RMS current at nominal voltage (steady state) Maximum Power Consumption (Watts) | 100-120 V ac 90-128 V ac Single 50/60 Hz 49/51/59-61 Hz 7.0 Amps 380 Watts | 220-240 Vac 180-256 Vac Single 50/60 Hz 49-51/59-61 Hz 3.0 Amps |  |
| Power cord | Type Length US plug | IEC 320 C13 190 cm (75 in.) NEMA 5-15 |  |
| Regulatory |  |  |  |
| Agency approvals | UL Listed to UL1950 (3rd edition) CSA Certified to CAN/CSA-C22.2 No. 950-M95 TUV EN 60950 GS marked CB Certification pending FCC (Class B) Part 15, Subpart B (CFR 54,1995) EMC Directive 89/336/EEC EN 55022:1994 Class B EN 50082-1 1992 Light Industrial CE Class B VCCI Class II ITE AS/NZS 3548:1992 (Class B) |  |  |
| Reviewed to | AS 3260:1993 Australian/New Zealand Standard EN 60 950: 1995 European Norm IEC 950 |  |  |

1.	Maximum operating temperature at Sea Level. Reduce by 1 C (1.8 F) for each 600 m (2000 ft) above Sea Level.

2.	Higher altitudes are possible if maximum operating temperature is reduced (see Temperature, above); other restrictions may apply, such as maximum permissible altitude for hard drives.

AlphaServer 800 Rackmount System Specifications

| Shipping Dimension |  |  |  |  |
| --- | --- | --- | --- | --- |
| Same as Pedestal |  |  |  |  |
| Installed Dimensions |  |  |  |  |
| Height Width Depth Weight |  | 22 cm (8.6 in.) fits 8.75 in. [5U] standard EIA 310D standard EIA 310D (RETMA) 63.8 cm (25.1 in.) 24.5 kg (54 LB) typical 28 kg (62LB) maximum |  |  |
| Clearances | Service |  |  |  |
| Front Rear Sides | See requirements of specific cabinet Slides forward 68.6 cm (27 in.) then accessible from the top |  |  |  |
| Environmental |  |  |  |  |
| Temperature Operating Nonoperating Storage (60 days) | 10-35° C (50-95° F) Not tested -40-66° C (-40-151° F) 11 c/hr (20 F/HR) |  |  |  |
| Relative humidity Operating Nonoperating Storage (60 days) Rate of change | 20–80% 20–80% 10–95% 20%/hr |  |  |  |
| Air flow and quality Intake location Exhaust location | Front Rear |  |  |  |
| Electrical—Power Supplies are universal 120/240 V ac |  |  |  |  |
| Same as Pedestal |  |  |  |  |
| Regulatory |  |  |  |  |
| Same as Pedestal |  |  |  |  |

Uninterruptable Power Supplies

Prestige 1250EXT

UPS offerings include EIA232 port for local or network monitoring and plug-in battery extension provisions to increase run time up to two hours.  For complete protection, UPS  products should be used with data line surge protectors and UPS monitoring software.

Solutions for Pedestal Systems

| 4N-AEABF-AA | Prestige 1250EXT, 1250 VA/900W, 1 phase, 50/60 Hz, 120V 6-foot cord with 5-15P plug, (4) 5-15R receptacle, 9 minutes battery at full UPS load. |
| --- | --- |
| 4N-AEABF-BF | Prestige 1250EXT, 1250 VA/900W, 1 phase, 50/60 Hz, 200-240V in/out selectable. Uses system power cord for detachable IEC320 input connection at UPS. Includes (3) IEC320 10A output receptacles, (2) output jumpers with IEC320 connectors to system. |
| 4N-AEABF-BB | Extension pack for 120V 60 Hz models, up to four packs at 30 minutes per pack supported |
| 4N-AEABF-BD | Extension pack for 240V 50 Hz models, up to four packs at 30 minutes per pack supported |
| 4N-AEAEO-PA | Hot-swap Power-Pass Module for 60 Hz UPS with (7) 5-15R Surge Protected outlets. |
| 4N-AEAEO-PB | Hot-swap Power-Pass Module for 50 Hz UPS with (6) IEC320, 10A surge protected outlets. |
| 4N-AEWAR-G1 | Prestige 5 year on-site exchange warranty upgrade (US only). |

Rackmount UPS

| UPS Model | Application | Type | Receptacle module |
| --- | --- | --- | --- |
| 4N-AEAAH-FA 60Hz | Rackmount UPS, supports up to 5 systems | 3kVA Custom RM1 | Included2 |
| 4N-AEAAH-FB 50Hz | Rackmount UPS, supports up to 5 systems | 3kVA Custom RM1 | Included |
| 4N-AEAAJ-CL3 60Hz | Out of Cabinet UPS | 6kVA Prestige4 | Included |
| 4N-AEAAJ-CU 50Hz | Out of Cabinet UPS | 6kVA Prestige4 | Hardwired |

1.	Factory installation must be specified at time of order.  Up to two UPS systems, one UPS for up to 5 fully configured systems. Recommended UPS distribution option combines multiple UPS input plugs into a single L6-30P plug-in connection to simplify future UPS add-on.  Order 4NAEAAH-FF 60 Hz; 4N-AEAAH-FG 50 Hz).

2.	Includes six IEC receptacle strips for connection to IEC plugs in cabinet.

3.	208-120/208V shown; order 4N-AEAAJ-CT for 240-120/240V models

4.	Optional mobile stacker with seismic mounting provision for 6kVA Prestige (4N-AEACH-HD)

| 4N-AEAAJ-CL | Prestige 6 kVA (4 kW), single phase, 60Hz, 208-120/208V, 6 ft. cord with L6-30P and (2) L6-30R, (8) 5-15R receptacle. Modular hot-swap design with 7 minute battery at full UPS load, extendible plug and play batteries and receptacle provisions. Substitute -CT for 240V 240/120V operation. |
| --- | --- |
| 4N-AEAAJ-CU | Prestige 6 kVA (4 kW) 50Hz package, single phase, 50Hz, 200-240V in/out selectable, hardwired input/output. |
| 4N-AEWAR-G3/G4 | Prestige 5 year on-site exchange warranty upgrade for 4N-AEAAH-CM/CT. G4 provides on-site remedial and start-up service. |

UPS Monitoring and Unattended Shutdown Software (for above UPS systems only) is included in ServerWORKS Kits shipping with all AlphaServers.  Order Cable Kit separately.

| 4N-ONLIN-NT | Cable Kit for Windows NT and DIGITAL UNIX |
| --- | --- |
| 4N-ONLIS-FE | Cable Kit for OpenVMS |

Network Management or multi system shutdown (via SNMP or ServerWORKS Manager) for DIGITAL UNIX and OpenVMS requires Network Adapter option.

| 60 Hz Applications | 50 Hz Application | Network Adapter Option |
| --- | --- | --- |
| 4N-AEAEO-DB/DD | 4N-AEAEO-DB/DD | Prestige 3 kVA RM, DB=Twisted pair; DD=ThinWire |
| 4N-AEAEO-DA/DC | 4N-AEAEO-DB/DD | Prestige 6 kVA and PUPS Plus UPS, DA/DB=Twisted pair; DC=ThinWire |

Note:	Multiple system shutdown via OpenVMS requires multi-port hardware option (RN-JMIU4-AB).  Ports can be daisy chained.