### AlphaServer 400
V96-2.5¾22 Apr 1997
DIGITAL Systems and Options Catalog

Product Description 	The AlphaServer 400 Mini Tower System is an Alpha microprocessor  233 MHz CPU system.  This highly reliable product offers many server specific features, including management and security, with remote management possible via a simple serial link. For investment protection, customers get a choice of three popular operating systems: DIGITAL UNIX, OpenVMS, and Microsoft Windows NT Server.  The product also has been designed to allow for future, seamless upgrades to faster Alpha processors.

The server supports up to 384 MB of industry-standard SIMM memory and has an integrated Fast Narrow SCSI-2 controller. The system enclosure supports five storage devices including a floppy diskette drive, CD-ROM, and hard drives. Six industry standard  I/O expansion slots (two PCI, three ISA, and one PCI/ISA slot provide for options such as high-performance graphics, networking and SCSI adapters.

Advanced server management features are provided with all AlphaServer 400 shipments via the bundled ServerWORKS Manager kit.  The kit  provides remote management capability through Simple Network Management  Protocol agents (SNMP) for Windows NT, DIGITAL UNIX, and OpenVMS.  The  management console software runs on any Windows NT or Windows 95 PC.  The SNMP agents required to manage the server ship with the TCP/IP services  of the operating system (with the exception of Windows NT which ships with  the ServerWORKS kit).  The SNMP agents collect critical server information  including system status, I/O information, storage and disk information and  network status information.  Thresholds can be set to monitor key server events and alarms are forwarded to the ServerWORKS console.  ServerWORKS  Manager V2.0 includes remote pager support for alarm notification.  In addition to ServerWORKS, DIGITAL includes key third party software applications including DIGITAL Power Management Software for Alpha provided  by EXIDE, and application / database management software through BMC software's PATROL family of products.

All AlphaServer 400 systems come with a three year on-site warranty. DIGITAL’s ADVANTAGE-UPGRADE Program provides a cost-effective upgrade path for investment protection.

DIGITAL believes the information in this publication is accurate as of its publication date; such information is subject to change without notice. DIGITAL is not responsible for any inadvertent errors.

DIGITAL conducts its business in a manner that conserves the environment and protects the safety and health of its employees, customers, and the community.

DIGITAL, the DIGITAL logo are trademarks of Digital Equipment Corporation.

Printed in USA. Copyright 1997 Digital Equipment Corporation. All rights reserved.

Step 1—Systems

- DIGITAL UNIX and OpenVMS Packaged and Base systems ordered with a minimum of a 1.05 GB disk include factory installed software (FIS).

- Windows NT Packaged and Base systems include Windows NT Server 4.0 plus 10-client access license, media (CD-ROM) kit.

- Options orderedwill be factory installed unless specified as spares.

AlphaServer 400 Systems include

- Alpha microprocessor 21064A 233-MHz CPU with 	512-Kbyte onboard secondary cache

- Mini tower enclosure with:

- Six expansion slots: 2 PCI, 3 ISA, and 1 PCI/ISA combination

- 6 SIMM memory slots support 3 memory options

- Five storage slots:
One dedicated diskette drive slot,
Three 5.25-inch removable media or hard disk drives slots
One internal 3.5-inch hard disk drive slot

- Integrated PCI-based Fast Narrow SCSI-2 controller with DMA and external SCSI-2 connector

- 300-Watt power supply

- Two serial ports, support full duplex asynchronous modem control

- One bi-directional enhanced parallel port

- PS/2 style keyboard port and mouse port

- 1.44 MB diskette drive in dedicated slot

- High-performance Ethernet adapter—uses one PCI slot (included in Packaged systems only)

- 3-button mouse

- Hardware documentation (Americas and AP orders only)

- Hardware Warranty: Three-year on-site*

- Software Warranty:*

- 90-day SPD conformance with advisory telephone support for DIGITAL UNIX and OpenVMS

- Warranty on Microsoft's Windows NT Server 4.0 is conformance to the written material accompanying the software for a period of ninety days

- Windows NT Server 4.0 plus 10-client access license, media (CD-ROM) kit or

- DIGITAL UNIX unlimited user license and Server Extension License, or

- OpenVMS base license with System Manager license Enterprise Integrated Package (EIP)

*	Service upgrades are available; see Step 11, Hardware and Software Supplemental Services.

Packaged Systems

- Packaged systems ordered in the Americas and Asia Pacific (AP) include 120 V power cord packaged with keyboard, and English hardware documentation.

- Select country-specific power cord, keyboard and hardware documentation for all Packaged systems ordered in Europe.

- All Packaged systems include high-performance PCI-based Ethernet adapter—uses one PCI slot

- Windows NT Packaged systems include entry level graphics (PB2GA) 1280 x 1024 x 256 colors, 72 Hz—uses one PCI slot

| Order Number | Operating System | Memory | Graphics | Monitor | 1.05 GB Hard Drive | 600 MB CD-ROM | Ethernet |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB523-AC/AD | DIGITAL UNIX | 32 MB | Recommended | Recommended | Included | Included | Included |
| PB524-AC/AD | OpenVMS | 32 MB | Recommended | Recommended | Included | Included | Included |
| PB525-AA/AB | Windows NT | 32 MB | Included | Required | Included | Included | Included |

Note:	xA = 120V and xB = 240V

Step 1—Systems (continued)

Base Systems

- DIGITAL UNIX and OpenVMS Base systems require:

- Minimum of 32 MB memory

- One hard disk drive

- Country-specific power cord

- Graphics option, monitor, and keyboard for DIGITAL UNIX and OpenVMS base systems are available as options if required

- Windows NT base systems require:

- Minimum of 32 MB memory

- One hard 1.05 GB or 2.1 GB disk drive

- One CD-ROM Drive

- Graphics Option

- Monitor and keyboard

- Country-specific power cord

Note:	Mandatory items must be on purchase order at initial order acceptance.

| Order Number | Operating System | Memory | Graphics | Monitor | Hard Drive1 | 600 MB CD-ROM | Ethernet |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB51C-AB | DIGITAL UNIX | Mandatory | Recommended | Recommended | Mandatory | Recommended | Recommended |
| PB51C-BB | OpenVMS | Mandatory | Recommended | Recommended | Mandatory | Recommended | Optional |
| PB51C-CA | Windows NT | Mandatory | Required | Required | Mandatory | Mandatory | Optional |

1.	Base systems ordered with one hard disk include factory installed software (FIS).  Note:  4.3 GB hard disk (PBXRZ-NA) is not supported as a system disk on Windows NT servers

Step 2—Memory

- Six SIMM slots support 3 memory options for maximum of 384 MB.  Memory pairs can be mixed.

- Packaged systems include 32 MB memory, one MSP01-AC.  Base systems require minimum of 32 MB

- System maximum of 384 MB can be obtained by selecting a Base system and 3 MSP01-AE 128 MB memory options.

| MSP01-AA | 8 MB (2 x 4 MB 70ns SIMMs) |
| --- | --- |
| MSP01-AB | 16 MB (2 x 8 MB 70ns SIMMs) |
| MSP01-AC | 32 MB (2 x 16 MB 70ns SIMMs) |
| MSP01-AD | 64 MB (2 x 32 MB 70ns SIMMs) |
| MSP01-AE | 128 MB (2 x 64 MB 70ns SIMMs) |
| DJ-ML200-AA | PCI-based 2-MB PrestoServe I/O performance enhancement option for DIGITAL UNIX systems only, maximum 1 per system |

Step 3—Monitors

- Windows NT Packaged systems include video adapter that support 1024 x 768 and 1280 x 1024 resolution, 72 Hz monitors.  Systems require a graphics monitor to run all functions.

| VRC15-KA | 15” (13.9” viewable image size) high-resolution color monitor with Light Gray enclosure. Flat-square CRT with 0.28 mm dot pitch and anti-reflection, anti-glare, anti static coating. Auto-scanning from VGA to 1024 x 768 at 75Hz NI refresh rates. MPR-II, Energy Star, DPMS and NUTEK compliant. 120/240V universal power supply. Includes 1.4 meter HD15 male to male video cable. Select -KA for Northern Hemisphere or , if purchased in North America, -KA includes 120V power cord, otherwise power cord must be ordered separately. |
| --- | --- |
| VRT17-PA | 17” (16.0” viewable image size) high-resolution color monitor with Light Gray enclosure. Trinitron aperture grille CRT with 0.26mm stripe pitch and anti-reflection, anti-glare, anti static coating. Auto-scanning from VGA to 1280 x 1024 at 75Hz NI refresh rates. MPR-II, Energy Star, DPMS and NUTEK compliant. 120/240V universal power supply. Includes 3.0 meter HD15 male to BNC video cable. Select -PA for Northern Hemisphere, if purchased in North American, -PA includes 120V power cord, otherwise power cord must be ordered separately. |
| VRC21-LA | 21” (19.6” viewable image size) ultra high-resolution color monitor with Light Gray enclosure. Diamondtron aperture grille CRT with 0.30 mm stripe pitch and anti-reflection, anti-glare, anti static coating. Auto-scanning from VGA to 1600 x 1200 at 75Hz NI refresh rates. On Screen display (OSD). Stereo viewing compatible. MPR-II, Energy Star, DPMS and NUTEK compliant. 120/240V universal power supply. Includes 3.0 meter HD15 male to BNC video cable. Select -LA for Northern Hemisphere, if purchased in North America, -LA includes 120V power cord, otherwise power cord must be ordered separately. |

Step 4—Storage

- Integrated Fast Narrow SCSI-2 controller supports maximum of seven narrow devices, four of which may be internal.

- Maximum Integrated SCSI bus length cannot exceed 3.0 meters.

Step 4a—Internal Storage

- System enclosure supports five narrow SCSI storage devices: One dedicated diskette drive slot, three front accessible 5.25-inch slots for removable media (tapes or CD-ROM) or hard drives, and one internal 3.5-inch hard drive slot. See system diagram.

- Packaged systems include: one 1.44 MB diskette drive, one 1.05 GB narrow SCSI hard drive, and one CD-ROM.

- Base systems include one 1.44 MB diskette drive.

- A hard disk drive is mandatory for base systems. Note:  4.3 GB hard disk (PBXRZ-NA) is not supported as a system disk on Windows NT servers

- CD-ROM drive is mandatory for Windows NT Base systems; software distribution is only available on CD-ROM.

Removable Media Devices

| PBXRD-DA | 600 MB 5.25-inch half-height 12X CD-ROM drive (RRD46) |
| --- | --- |
| PBXTZ-AA | 2.0 MB 5.25-inch half-height QIC tape drive (TZK11) |
| PBXTL-DA | 8.0 MB 5.25-inch half-height 4-mm DAT drive (TLZ09) |

Internally Supported Hard Drives

| PBXRZ-EB | 1.05 GB 8-bit narrow 5400 RPM 3.5 x 1” SCSI hard disk drive (RZ26N-E) |
| --- | --- |
| PBXRZ-HB | 2.1 GB 8-bit narrow 5400 RPM 3.5 x 1” SCSI hard disk drive (RZ28M-E) |
| PBXRZ-JB | 2.1 GB 8-bit narrow 7200 RPM 3.5 x 1” SCSI disk drive (RZ28D-E) |
| PBXRZ-NA* | 4.3 GB 8-bit narrow 7200 RPM 3.5 x 1.6” SCSI hard disk drive (RZ29B-E) |

*	Not supported as a system disk on Windows NT servers.

Sote:	Wide drives installed in system enclosure require a wide SCSI controller and BC25V-1A cable.

| PBXRW-EB | 1.05 GB 16-bit wide 5400 RPM 3.5 x 1” half-height disk drive (RZ26N-W) |
| --- | --- |
| PBXRW-HB | 2.1 GB 16-bit wide 5400 RPM 3.5 x 1” SCSI disk drive (RZ28M-W) |
| PBXRW-JB | 2.1 GB 16-bit wide 7200 RPM 3.5 x 1” SCSI disk drive (RZ28D-W) |
| PBXRW-NA | 4.3 GB 16-bit wide 7200 RPM 3.5 X 1.6” SCSI disk drive (RZ29B-W |

Step 4b—External Expansion

- Systems support three external devices on integrated PCI-based Fast Narrow SCSI-2 controller when four devices are installed in system enclosure.

- Seven external narrow SCSI devices are supported on integrated Fast Narrow SCSI-2 controller if storage devices are not installed in system enclosure.

- Select additional PCI-based wide or narrow SCSI-2 controllers if additional external storage is required.

- System supports maximum of three PCI-based SCSI-2 controllers (Note: Packaged systems have two PCI slots available due to installed PCI-based Ethernet controller.)

- PCI-based SCSI-2 controllers supports 7 additional external devices.

Controllers

| KZPAA-AA | PCI-based one-port high-performance Fast Narrow Single Ended (FNSE) SCSI-2 controller |
| --- | --- |
| BN21H-xx | Connects from KZPAA-AA to BA353 narrow storage enclosure |
| KZPSA-BB | PCI-based one-port high-performance Fast Wide Differential (FWD) SCSI controller. Supports external disks only. |
| BN21K-01 | Connects from KZPSA to DWZZA-VA in narrow BA353, or DWZZB-VW in wide BA356/BA346 storage enclosure |

Step 4b—External Expansion (continued)

Controllers

| KZPSC-AA* | One-port PCI backplane RAID Fast Wide Single Ended (FWSE) controller; includes StorageWorks RAID Array 230 Subsystem family software and documentation kit. Supports external disks only. |
| --- | --- |
| BN31S-1E | Connects from KZPSC-AA to BA346/BA356 wide storage enclosure |
| BN31L-1E | Connects from KZPSC-AA to BA353 narrow storage enclosure |
| KZPSC-BA* | Three-port PCI backplane RAID (FWSE) controller; includes StorageWorks RAID Array 230 Subsystem family software and documentation kit. Supports external disks only. |
| BN31K-0E | SCSI cable/bulkhead assembly kit for KZPSC-BA; required for connection to third port; uses one PCI bulkhead slot |
| BN31S-1E | Connects from KZPSC-BA to BA346/BA356 wide storage enclosures |
| BN31L-1E | Connects from KZPSC-BA to BA353 narrow storage enclosure |
| KFPSA-AA | PCI-based single-DSSI controller (OpenVMS systems only). See Step 4d DSSI cables. |

*	See Storage Devices for additional information on StorageWorks RAID Array 230 systems.

External Tabletop Tape Expansion

- Tabletop expansion units include North American power cord; order country-specific power cord for 240 V use.

| TLZ09-DB* | 8.0 GB 5.25-inch half-height 4 mm DAT tape drive, includes SCSI cable |
| --- | --- |

*	Supported on DIGITAL UNIX, OpenVMS, and Windows NT 4.0.  Systems running Windows NT 3.51 require upgrade kit, part number QC-00LAC-UC, which includes driver and installation instructions. Driver and instructions are also available at Web Site http://www.storage.digital.com under "Technomania" section.

External Disk Expansion

- BA353 expansion units are supported on wide and narrow SCSI controllers.  Devices operate in narrow mode when BA353 is connected to a Fast Wide SCSI controller.

- BA356 and BA346 expansion units are supported on Fast Wide SCSI controllers.

StorageWorks Modular Storage Options

| BA353-AA | StorageWorks 8-bit Narrow Desktop expansion unit, supports up to three 3.5” narrow hard disk drives, includes 120 V power cord. Not supported with RAID controllers. |
| --- | --- |
| BA356-KC | StorageWorks 16-bit Wide Pedestal expansion unit includes BA356 basic shelf, BA35X-HF 150-W universal ac power supply, pedestal mounting kit, and 120 V power cord. Supports up to seven 3.5” wide or narrow drives; narrow drives operate in narrow mode. |
| BA346-KB | StorageWorks 16-bit Wide Pedestal expansion unit includes BA356 basic shelf, BA35X-HA universal ac power supply, pedestal mounting kit, and 120 V power cord. Supports up to nine devices, two 5.25” narrow and seven 3.5” wide or narrow drives; narrow drives operate in narrow mode. |
| DWZZA-VA | Bi-directional SCSI signal converter and bus extender kit. 16 bit Fast Wide Differential SCSI-2 on one end and 8 bit Fast Narrow Single-Ended SCSI-2 on other end. |
| DWZZB-VW | Bi-directional SCSI signal converter and bus extender kit. 16 bit Fast Wide Differential SCSI-2 on one end and 8 or 16 bit Fast Wide or Fast Narrow Single-Ended SCSI-2 on other end. |

Externally supported SCSI devices

| RZ26N-VA | 1.05 GB 8-bit narrow 5400 RPM 3.5 x 1” SCSI hard disk drive |
| --- | --- |
| RZ28M-VA | 2.1 GB 8-bit narrow 5400 RPM 3.5 x 1” SCSI hard disk drive |
| RZ28D-VA | 2.1 GB 8-bit narrow 7200 RPM 3.5 x 1” SCSI disk drive |
| RZ29B-VA | 4.3 GB 8-bit narrow 7200 RPM 3.5 x 1.6” SCSI hard disk drive |

Step 4b—External Expansion (continued)

Externally supported SCSI devices

| DS-RZ26N-VZ | 1.05 GB 16-bit wide 5400 RPM 3.5 x 1” half-height disk drive |
| --- | --- |
| DS-RZ28M-VZ | 2.1 GB 16-bit wide 5400 RPM 3.5 x 1” SCSI disk drive |
| RZ28D-VW | 2.1 GB 16-bit wide 7200 RPM 3.5 x 1” SCSI disk drive |
| RZ29B-VW | 4.3 GB 16-bit wide 7200 RPM 3.5 X 1.6” SCSI disk drive |

Step 5—Graphics Adapters and Multimedia Options

Windows NT systems include entry-level (PB2GA) graphics adapter.

| Order Number | Description | Supported Operating System | Bus Slots Required/ Recommended Slot |
| --- | --- | --- | --- |
| PBXGA-AA1 | ZLXp-E1 8-plane, PCI graphics accelerator, 2 Mpixels, 1280 x 1024 x 256 colors, 72 Hz | DIGITAL UNIX OpenVMS | One PCI/1, 2, 3a |
| PBXGA-BA1 | ZLXp-E2 24-plane, PCI graphics accelerator. 2 Mpixels, 1280 x 1024 x 256 colors, 72 Hz | DIGITAL UNIX OpenVMS | One PCI/1, 2, 3a |
| PBXGA-CA1 | ZLXp-E3 24-plane, PCI graphics accelerator, 24-bit Z-buffer, 4 Mpixels, 1280 x 1024 x 256 colors, 72 Hz | DIGITAL UNIX OpenVMS | One PCI/1, 2, 3a |
| PBXGA-AN2 | ZLXp-E1 8-plane, PCI graphics accelerator, 2 Mpixels, 1280 x 1024 x 256 colors, 72 Hz | Windows NT | One PCI/1, 2, 3a |
| PBXGA-BN2 | ZLXp-E2 24-plane, PCI graphics accelerator, 2 Mpixels, 1280 x 1024 x 256 colors, 72 Hz | Windows NT | One PCI/1, 2, 3a |
| PBXGA-CN2 | ZLXp-E3 24-plane, PCI graphics accelerator, 24-bit Z-buffer, 4 Mpixels, 1280 x 1024 x 256 colors, 72 Hz | Windows NT | One PCI/1, 2, 3a |

1	Multi-screen graphics options for DIGITAL UNIX and OpenVMS systems. Options require DIGITAL Open3D media which is included in factory installed software (FIS)

2	Graphics options include Graphics Support Services Software for Windows NT license, media, and documentation.

Multimedia Options

| AVA01-AA | Window sound system; compatible sound card, headset and microphone | DIGITAL UNIX OpenVMS Windows NT | One PCI/1, 2, 3a |
| --- | --- | --- | --- |
| AVH01-AA | Headset and microphone for AVA01 sound card | DIGITAL UNIX OpenVMS Windows NT | One PCI/1, 2, 3a |
| AV301-AA | FullVideo Supreme for all systems | DIGITAL UNIX OpenVMS Windows NT |  |
| AV321-AA | FullVideo Supreme JPEG for all systems | DIGITAL UNIX OpenVMS Windows NT |  |
| AVC00-AA | Toshiba camera, North America | DIGITAL UNIX OpenVMS Windows NT |  |

Step 7—Communications and Miscellaneous Adapters

- Packaged systems include high-performance Ethernet—Thick wire, Twisted Pair, ThinWire)

| Order Number | Description |  | Supported Operating Systems |  | Bus Slots Required / Recommended Slot |  |
| --- | --- | --- | --- | --- | --- | --- |
| DE205-AC | ISA-based EtherWORKS 3 Network Interface Card |  | DIGITAL UNIX OpenVMS Windows NT |  | One ISA/3a, 4, 5, 6 |  |
| DE435-AA |  | PCI based High-performance Ethernet. Select AUI (Thick wire), 10BaseT (Twisted Pair) or ThinWire networking cable BNE4G-02 for AUI BN26K-xx for 10BaseT (twisted pair) BC16M-xx for ThinWire |  | DIGITAL UNIX OpenVMS Windows NT |  | One PCI/1, 2, 3a |
| DE500-XA |  | PCI-based Fast Ethernet network interface card |  | DIGITAL UNIX OpenVMS Windows NT |  | One PCI/1, 2, 3a |
| DEFPA-AB1 | PCI-based DEC FDDI controller, Single Attachment |  | DIGITAL UNIX OpenVMS Windows NT |  | One PCI/1, 2, 3a |  |
| DEFPA-DB1 | PCI-based DEC FDDI controller, Dual Attachment |  | DIGITAL UNIX OpenVMS Windows NT |  | One PCI/1, 2, 3a |  |
| DEFPA-UB1 | PCI-based DEC FDDI (UTP) controller |  | DIGITAL UNIX OpenVMS Windows NT |  | One PCI/1, 2, 3a |  |
| DGLPB-AB |  | PCI-based ATMworks 350 adapter |  | DIGITAL UNIX V3.2D-1 Windows NT 3.51/4.0 |  | One PCI/1, 2, 3a |
| PBXNP-AA1 |  | PCI-based Token Ring adapter, no boot support. |  | DIGITAL UNIX OpenVMS |  | One PCI/1, 2, 3a |
| CXI01-AA | ISA Asynchronous MUX Adapter, 16 lines. Expandable to 64 lines. |  | DIGITAL UNIX Windows NT |  | One ISA/3b, 4, 5, 6 |  |
| CXI01-AD | ISA Asynchronous MUX Adapter, 16 lines. Expandable to 224 lines. |  | DIGITAL UNIX Windows NT |  | One ISA/3b, 4, 5, 6 |  |
| PBXDI-AA |  | ISA-based Two Port Synchronous Communications controller with interface support for EIA-232/V.24/V.28, |  | Windows NT |  | One ISA/3b, 4, 5, 6 |
| PBXDI-AB |  | ISA-based Two Port Synchronous Communications controller with interface support for V.35 |  | Windows NT |  | One ISA/3b, 4, 5, 6 |
| PBXDI-AC |  | ISA-based Two Port Synchronous Communications controller with interface support for X.21 and EIA-530 |  | Windows NT |  | One ISA/3b, 4, 5, 6 |
| DIIAA-AA |  | ISA-based ISDN terminal adapter (U.S.) |  | Windows NT |  | One ISA/3b, 4, 5, 6 |
| DIIAA-AB |  | ISA-based ISDN terminal adapter (non-U.S.) |  | Windows NT |  | One ISA/3b, 4, 5, 6 |
| PBXDF-AA | Modem 14.4K |  | Windows NT |  | One ISA/3b, 4, 5, 6 |  |

1	DIGITAL UNIX and OpenVMS systems require driver floppy.

Step 8—Printers

The AlphaServer 400 includes two serial ports and one bi-directional parallel port on rear of system enclosure. Either port may be used as a printer port depending on printer type.

Step 9—Software

Windows NT Servers

Windows NT  Packaged and Base systems include Windows NT Server 4.0 plus 10-client access license, media (CD-ROM) kit.

DIGITAL UNIX Concurrent Use Licenses

Software Processor Code = E

Select user licenses and additional software as required. Media and documentation is recommended for first system on site.

DIGITAL UNIX Concurrent Use licenses are not specific to a single system and can be moved from one system to another at user discretion.

| QL-MT7AM-3B | DIGITAL UNIX Concurrent Use 1-user license |
| --- | --- |
| QL-MT7AM-3C | DIGITAL UNIX Concurrent Use 2-user license |
| QL-MT7AM-3D | DIGITAL UNIX Concurrent Use 4-user license |
| QL-MT7AM-3E | DIGITAL UNIX Concurrent Use 8-user license |
| QL-MT7AM-3F | DIGITAL UNIX Concurrent Use 16-user license |
| QL-MT7AE-AA | DIGITAL UNIX Traditional unlimited user license |
| QL-MT6AE-AA | DIGITAL UNIX server extension license |
| QL-MT5AE-AA | DIGITAL UNIX C developer’s extension license |

DIGITAL UNIX Media and Documentation

| QA-MT4AA-H8 | DIGITAL UNIX media and documentation (end user, developer, server) on CD-ROM |
| --- | --- |
| QA-MT4AA-GZ | DIGITAL UNIX full hardcopy documentation |
| QA-MT4AB-GZ | DIGITAL UNIX end user documentation |
| QA-MT5AA-GZ | DIGITAL UNIX developer’s documentation |
| QA-MT6AA-GZ | DIGITAL UNIX server extension documentation |

DIGITAL UNIX Layered Products (CD-ROM)

DIGITAL UNIX systems include Multimedia Services License, order media and documentation separately.  Media and documentation for Multimedia Services is included in DIGITAL UNIX Layered Products CD-ROM.

| QA-054AA-H8 | Layered products media and documentation for DIGITAL UNIX on CD-ROM |
| --- | --- |

Internet for AlphaServers

| QB-4GQAA-KA | Internet AlphaServer Software (USA and Canada Only) |
| --- | --- |
| QB-4GQAA-KB | Internet AlphaServer Software (International) |

DIGITAL UNIX Logical Storage Manger

| QL-2GVAE-AA | DIGITAL UNIX Logical Storage Manager License |
| --- | --- |
| QT-2GVAE-AA | DIGITAL UNIX Logical Storage Manger Product Support Services |

POLYCENTER NetWorker Save and Restore Licenses

| QL-2ALAE-AA | POLYCENTER NetWorker Save and Restore Server License |
| --- | --- |
| QL-3P2AE-AA | POLYCENTER NetWorker Save and Restore Archive Server Add-on License. Note: NetWorker Server License is a requirement for Archive License. |

Step 9—Software (continued)

DECsafe Available Server for DIGITAL UNIX

| QL-05SAE-AA | DECsafe ASE U/A Traditional license |
| --- | --- |
| QB-05SAE-AA | DECsafe ASE U/A Traditional license and configuration guide |

Server Extension License

| QL-MT6AE-AA | DIGITAL UNIX server extension license |
| --- | --- |
| QL-MT5AE-AA | DIGITAL UNIX C developer’s extension license |

DIGITAL NAS Base Server 200 for DIGITAL UNIX

| QL-36MAE-RA | DIGITAL NAS Base Server 200 for DIGITAL UNIX Update license |
| --- | --- |
| QT-36MAE-L9 | DIGITAL NAS Base Server 200 for DIGITAL UNIX Layered Product Support Service |

DECnet for DIGITAL UNIX

| QL-MTJAE-AA | DECnet/OSI end-system license for DIGITAL UNIX |
| --- | --- |
| QL-MTKAE-AA | DECnet/OSI extended function license for DIGITAL UNIX |

OpenVMS Concurrent Use Licenses

OpenVMS Concurrent Use license provide the right to interactively use the operating system by the specified number of concurrent users on a designated OpenVMS system. OpenVMS Concurrent Use licenses can be moved from one system to another at user discretion and can be shared in a mixed OpenVMS VAX and OpenVMS Alpha cluster.

| QL-MT3AA-3B | OpenVMS Concurrent Use 1-user license |
| --- | --- |
| QL-MT3AA-3C | OpenVMS Concurrent Use 2-user license |
| QL-MT3AA-3D | OpenVMS Concurrent Use 4-user license |
| QL-MT3AA-3E | OpenVMS Concurrent Use 8-user license |
| QL-MT3AA-3F | OpenVMS Concurrent Use 16-user license |
| QL-MT3AA-3G | OpenVMS Concurrent Use 32-user license |
| QL-MT3AA-3H | OpenVMS Concurrent Use 64-user license |
| QL-MT3AA-3J | OpenVMS Concurrent Use 128-user license |
| QL-MT3AA-3K | OpenVMS Concurrent Use 256-user license |
| QL-MT2AE-AA | OpenVMS Traditional unlimited user license |

OpenVMS Media and Documentation—required for first system on site

| QA-MT1AA-H8 | OpenVMS V6.2 media and documentation on CD-ROM |
| --- | --- |
| QA-MT1AP-H8 | OpenVMS V6.1-1H2 media and documentation on CD-ROM |
| QA-MT1AH-GZ | OpenVMS hardcopy documentation |

OpenVMS Layered Products (CD-ROM)

| QA-03XAA-H8 | Layered products media and documentation for OpenVMS on CD-ROM |
| --- | --- |

DIGITAL NAS Base Server 200 for OpenVMS

| QL-23EAE-RA | DIGITAL NAS Base Server 200 for OpenVMS upgrade license |
| --- | --- |
| QT-23EAE-L9 | DIGITAL NAS Base Server 200 for OpenVMS Layered Product Support Service |

DECnet for OpenVMS

| QL-MTGAE-AA | DECnet extended function license for OpenVMS |
| --- | --- |
| QL-MTHAE-AA | DECnet end-system to extended function upgrade license for OpenVMS |

Step 9—Software (continued)

Support Services Software for Microsoft Windows NT

ZLXp-Ex graphics options for Windows NT servers include the following license, media and documentation.

| QM-356AA-AA | License for ZLXp-Ex Support Services Software for Microsoft Windows NT |
| --- | --- |
| QB-356AA-SA | ZLXp-Ex Support Services Software for Microsoft Windows NT¾license, media (diskette) and documentation |
| QA-356AA-GZ | ZLXp-Ex Support Services Software for Microsoft Windows NT (documentation only) |

Step 10—Power Cords, Keyboards, and Documentation

- Packaged systems ordered in the Americas and Asia Pacific (AP) include 120 V U.S. power cord packaged with keyboard, and English hardware documentation. Select country-specific power cord, keyboard and documentation for all Packaged systems ordered in Europe.

- Base systems require country-specific power cord

- Windows NT Base systems require a keyboard.  Selection of keyboard is optional for DIGITAL UNIX and OpenVMS Base systems.

- Packaged and Base systems include mouse.

- LK471-xx are 101/102 key PC style keyboards.  LK461-xx are 108 key VT style keyboards.

| Power Cord | PC style keyboards | VT style keyboards | Country |
| --- | --- | --- | --- |
| BN19P-1K | LK471-A2 | LK461-A2 | North America, Japan (English) |
| BN19C-2E | LK471-AB | LK461-AB | Belgium (French) |
| BN19K-2E | LK471-AD | LK461-AD | Denmark |
| BN19C-2E | LK471-AP | LK461-AP | France |
| BN19C-2E | LK471-AF | LK461-AM | Sweden (Finnish) |
| BN19C-2E | LK471-AG | LK461-AG | Germany |
| BN19Z-2E | LK471-AI | LK461-AI | Italy |
| BN19C-2E | LK471-AN | LK461-AN | Norway |
| BN19C-2E | LK471-AV | LK461-AV | Portugal |
| BN19C-2E | LK471-AS | LK461-AS | Spain |
| BN19E-2E | LK471-AK | LK461-AK | Switzerland (French) |
| BN19A-2E | LK471-AE | LK461-AA | UK, Ireland |
| BN18L-2E | LK471-AT | LK461-AT | Israel (Hebrew) |
| BN19S-2E | LK471-AA | LK461-AA | Africa, India (English) |
| BN19H-2E | LK471-AA | LK461-AA | Australia, New Zealand (English) |

Mouse—included in all Packaged and Base systems

| PCXWS-AA | PS/2 compatible 3-button Mouse |
| --- | --- |

User Documentation—select for 240 V systems

| User Documentation |  |
| --- | --- |
| EK-PCSVA-UI | User Documentation—English |
| EK-PCSVJ-UI | User Documentation—Japanese |

Step 11—Hardware and Software Supplemental Support Services

Hardware—Americas and Asia Pacific only

- Systems include three-year, on-site, 24 hr. response hardware warranty.

- Select optional Hardware Supplemental Support Services if required.

| FM-CH4HR-36 | Years 1-3, 5 x 9, 4 hour response |
| --- | --- |
| FM-CH4HR-48 | Years 1-4, 5 x 9, 4 hour response |
| FM-CH4HR-60 | Years 1-5, 5 x 9, 4 hour response |

| FM-CH512-36 | Years 1-3, 5 x 12, 4 hour response |
| --- | --- |
| FM-CH512-48 | Years 1-4, 5 x 12, 4 hour response |
| FM-CH512-60 | Years 1-5, 5 x 12, 4 hour response |

| FM-CH616-36 | Years 1-3, 6 x 16, 4 hour response |
| --- | --- |
| FM-CH616-48 | Years 1-4, 6 x 16, 4 hour response |
| FM-CH616-60 | Years 1-5, 6 x 16, 4 hour response |

| FM-CH724-36 | Years 1-3, 7 x 24, 4 hour response |
| --- | --- |
| FM-CH724-48 | Years 1-4, 7 x 24, 4 hour response |
| FM-CH724-60 | Years 1-5, 7 x 24, 4 hour response |

| FM-CHXHW-48 | Year 4, 5 x 9, next day response, on-site |
| --- | --- |
| FM-CHXHW-60 | Years 4&5, 5 x 9 next day response, on-site |

Software—Americas and Asia Pacific only

- Software Warranty:

- 90-day SPD conformance with advisory telephone support for DIGITAL UNIX and OpenVMS

- Warranty on Microsoft's Windows NT Server 4.0 is conformance to the written material accompanying the software for a period of ninety days

- Software service upgrades for DIGITAL UNIX and OpenVMS include advisory and remedial software support with new version license rights for operating system and DIGITAL NAS 200 for the time period indicated.

- Software Supplemental Support Service options upgrade 90-day service to time period indicated below.

| FM-MKOSF-12 | 1 year Software Supplemental Support Services for DIGITAL UNIX systems |
| --- | --- |
| FM-MKOSF-36 | 3 year Software Supplemental Support Services for DIGITAL UNIX systems |
| FM-MKOSF-60 | 5 year Software Supplemental Support Services for DIGITAL UNIX systems |
| FM-MKVMS-12 | 1 year Software Supplemental Support Services for OpenVMS systems |
| FM-MKVMS-36 | 3 year Software Supplemental Support Services for OpenVMS systems |
| FM-MKVMS-60 | 5 year Software Supplemental Support Services for OpenVMS systems |
| FM-WNTN3-12 | 1 year Software Supplemental Support Services for Windows NT systems |
| FM-WNTN3-36 | 3 year Software Supplemental Support Services for Windows NT systems |
| FM-WNTN3-60 | 5 year Software Supplemental Support Services for Windows NT systems |

Step 11a—Hardware and Software Supplemental Support Services (Europe only)

Europe does not have specific part numbers for Hardware and Software Supplemental Support Services.  Prices can be quoted using the Excelerator tool; contact MCS Sales in your country for information on Hardware and Software Supplemental Support Services.

AlphaServer 400 System Diagram

Specifications

| PCI | 132 MB/second |
| --- | --- |
| ISA | 16 MB/second |
| Fast SCSI-2 bus | 10 MB/s transfer rate |
| Ethernet | DE435-xx 10-Mbit/s Twisted Pair/Thin Wire and AUI thick wire |
| Power Requirements |  |
| Line voltage | 120/240 V |
| Voltage tolerance | 88-132/176-264 V |
| Frequency single phase | 50/60 Hz |
| Frequency tolerance | 47-63 Hz |
| Maximum running current | 8.0A/4.0A |
| Maximum power consumption | 300 W |
| Operating Environment |  |
| Operating temperature | 10° to 40° C (50° to 104° F) |
| Operating humidity —Maximum wet bulb | 20% to 80% relative humidity 40° C (104° F) |
| Storage temperature | -20° C to 65° C (-4° F to 149° F) |
| Storage Humidity —Maximum wet bulb | 10% to 90% relative humidity 65° C (149° F) |
| Maximum altitude —Operating —Nonoperating | 2,438 m (8,000 ft) maximum 4,876 m (16,000 ft) maximum |
| Nonoperating shock | 30G, 25 ms halfsine |
| Physical Characteristics |  |
| Height | 43 cm (17 inches) |
| Width | 18 cm (7 inches) |
| Depth | 43 cm (17 inches) |
| Weight | 12+ kg (28+ lb) |

Prestige Model 800 “On-line” UPS

For complete protection, UPS  products should be used with data line surge protectors.

| 4N-AEABD-AF | For 120 Vac, 50 or 60 Hz systems Includes detachable 6-foot input power cord with 5-15P plug and four NEMA 5-15R output receptacles. |
| --- | --- |
| 4N-AEABD-BF | For 240 Vac, 50 or 60 Hz systems Selectable 220, 230 or 240V ac output. Uses system power cord for detachable IEC 320 input connection at UPS. Unit has 3 IEC 320 receptacles and includes two output jumpers with IEC 320 connectors to connect to system. |
| 4N-GA249-AB | Surge Protectors for 2 wire modem |
| 4N-GA249-CA | Surge Protectors for 10BaseT |
| 4N-GA510-BF | Surge Protectors for ThinWire |
| 4N-AEAEO-PA/PB | Optional hot-swap power pass with built in surge protected outlets including one extra off-line for laser printer. -PA for 120 V, -PB for 220 V models. |
| 4N-AEWAR-G1 | 5 year vendor on-site exchange warranty upgrade (available in Intercontinental U.S. only at time of purchase). |

Monitoring and Unattended Shutdown Software for above UPS systems only

- Include cables, media and documentation.

- SNMP Network connectivity adapters (4N-AEAEO-DA/DC) Twisted Pair/ThinWire are available.

| 4N-AEAES-AA/AB | Windows NT for Alpha and Intel x86 |
| --- | --- |
| 4N-AEAES-AK | DIGITAL UNIX |
| 4N-AEAES-EM | OpenVMS |