### DIGITAL Personal Workstation 433a and 500a for Windows NT
V96-2.5—19 Feb 1997
DIGITAL Systems and Options Catalog

Product Description	The DIGITAL Personal Workstations 433a and 500a for Windows NT are designed to provide price and performance leadership for low to midrange workstation users in the targeted MCAD, ECAD, GIS and Science and Research markets.  They are among the next generation of Alpha XL products to be introduced to existing users of DIGITAL UNIX and the expanding market of Windows NT desktop computing.

The DIGITAL EV56 microprocessor accommodates demanding integer applications and runs 3D graphics options to significantly improve productivity for high end 2D and the spectrum of 3D floating point intensive, graphics applications.

DIGITAL Personal Workstations 433a and 500a have leveraged their commonality with the DIGITAL Personal Workstation 180I and 200i family to achieve unprecedented cost benefits.

The DIGITAL Personal Workstation for Windows NT system chassis supports 
5 PCI slots (two PCI slots, three PCI/ISA combination slots) for industry standard options.  All systems also include an on-board E-IDE disk controller, a dedicated floppy diskette drive, and 600 MB EIDE CD-ROM drive.

Storage supported in system enclosure includes up to three 5.25-inch half height removable media bays, one dedicated floppy disk drive bay, and two internal 3.5” hard disk drives bays. System ports include one bi-directional enhanced parallel port, two serial ports and external SCSI connectors.

DIGITAL Personal Workstation for Windows NT is  available in three models:

- DIGITAL Personal Workstation 433a and 500a Advantage Configurations

- DIGITAL Personal Workstation 433a Entry Level Packaged Configurations

- DIGITAL Personal Workstation 433a and 500a Base Configurations

All U.S. (120 V) variants include Windows NT 4.0 Country kit with American English Language documentation. Worldwide (240 V) system users must select a Country Kit from the option list.  The Country Kit determines the language variant of user documentation.

DIGITAL Personal Workstations come standard with a three-year limited warranty.  Year one features on-site service; years two and three are return to DIGITAL service.  This warranty represents one of the most comprehensive in the industry and includes product lifetime toll-free technical assistance.

DIGITAL believes the information in this publication is accurate as of its publication date; such information is subject to change without notice. DIGITAL is not responsible for any inadvertent errors.

DIGITAL conducts its business in a manner that conserves the environment and protects the safety and health of its employees, customers, and the community.

DIGITAL, the DIGITAL logo are trademarks of Digital Equipment Corporation.

Printed in USA. Copyright 1997 Digital Equipment Corporation. All rights reserved.

### DIGITAL Personal Workstation 433a and 500a Advantage Configurations

Y=in base configuration; M=mandatory option; O=option

| DIGITAL PWS 433a | Resources Used |  | 3D CAD | 3D+ CAD | 3D+ CAD |
| --- | --- | --- | --- | --- | --- |
| 120 V North American (a) |  |  | SN-B3AAP-FS | SN-B3AAP-FW |  |
| 240 V Worldwide (b) |  |  | SN-B3AWW-FS | SN-B3AWW-FW |  |
| CPU Alpha microprocessor EV56 | 433 MHz |  | Y | Y |  |

| DIGITAL PWS 500a | Resources Used |  | 3D CAD | 3D+ CAD | 3D+ CAD |
| --- | --- | --- | --- | --- | --- |
| 120 V North American (a) |  |  |  | SN-B3DAP-FW | SN-B3DAP-LX |
| 240 V Worldwide (b) |  |  |  | SN-B3DWW-FW | SN-B3DWW-LX |
| CPU Alpha microprocessor EV56 | 500 MHz |  |  | Y | Y |

| Memory | 1 bank | Y | 64 MB ECC | 64 MB ECC | 128 MB ECC |
| --- | --- | --- | --- | --- | --- |
| 2 MB external CPU Cache memory (c) |  |  | O | Y-500Mhz only | Y |
| 1.44-Mbyte Floppy disk drive | 3.5” x 1” bay | Y | Y | Y | Y |
| Adaptec UW SCSI controller (d) | 1 PCI (1, 2, or 3) | Y | Y | Y | Y |
| On-board EIDE controller (e) |  | Y | Y | Y | Y |
| Internal storage | Two 3.5 x 1.0” bays | Y | 2 x 2.1 GB SCSI | 2 x 2.1 GB SCSI | 2 x 2.1 GB SCSI |
| 600-Mbyte 12X EIDE CD-ROM | 5.25” x 1.6” bay | Y | Y | Y | Y |
| On-board Ethernet 10BaseT/10Base2 |  | Y | Y | Y | Y |
| On-board 16-bit Soundblaster Audio |  | Y | Y | Y | Y |
| 10/100 BaseTx Fast Ethernet (f) | Accessory kit | Y | Y | Y | Y |
| Telex Voice Commander Microphone | Microphone port | Y | Y | Y | Y |
| Koss earphone | Earphone port | Y | Y | Y | Y |
| Accelgraphics 2500TX 3D graphics | 1 PCI slot (4 or 5) | Y | 2500TX |  |  |
| PowerStorm 4D40T Graphics | 1 PCI slot and 1 PCI/ISA slot (4 & 5) | Y |  | 4D40T |  |
| PowerStorm 4D60T Graphics | 1 PCI slot and 1 PCI/ISA slot (4 & 5) | Y |  |  | 4D60T |
| 300 Watt power supply |  | Y | Y | Y | Y |
| Remaining available resources |  |  |  |  |  |
| I/O slots PCI slots PCI/ISA slots |  |  | 0 3 | 0 2 | 0 2 |
| Memory banks |  |  | 2 | 2 | 2 |
| 5.25” x 1.6” | User accessible bays |  | 2 | 2 | 2 |
| 3.5” x 1.0” | Internal bays |  | 0 | 0 | 0 |
| SCSI Device connections |  |  | 13 | 13 | 13 |
| EIDE disk connections |  |  | 3 | 3 | 3 |
| 1. Windows NT Country Kit |  |  |  |  |  |
| 120 V North American | Included | Y | Y | Y | Y |
| 240 V Worldwide | See Options List | M | SN-PBB3P-xx | SN-PBB3P-xx | SN-PBB3P-xx |
| 2. Color Monitors | Select |  | 1 | 1 | 1 |
| 17” (16.0” viewable) 1280 x 1024 | Northern Hemisphere | O | SN-VRT17-WA | SN-VRT17-WA | SN-VRT17-WA |
| 17” (16.0” viewable) 1280 x 1024 | Southern Hemisphere | O | SN-VRT17-W4 | SN-VRT17-W4 | SN-VRT17-W4 |
| 17” (16.0” viewable) 1600 x 1200 | Northern Hemisphere | O | SN-VRTX7-W3 | SN-VRTX7-W3 | SN-VRTX7-W3 |
| 21” (19.7” viewable) 1600 x 1200 | Northern Hemisphere | O | SN-VRC21-WA | SN-VRC21-WA | SN-VRC21-WA |
| 21” (19.7” viewable) 1600 x 1200 | Southern Hemisphere | O | SN-VRC21-W4 | SN-VRC21-W4 | SN-VRC21-W4 |
| 21” (19.7” viewable) 1600 x 1200 | Northern Hemisphere | O | SN-VRCX1-W3 | SN-VRCX1-W3 | SN-VRCX1-W3 |

(a)	120 V U.S. variants include Windows NT 4.0 Country kit

(b)	240 V Worldwide variants require selection of Windows NT 4.0 Country Kit.

(c)	Only 500 MHz systems (SN-B3D**-FW variants) include 
2 MB Cache Module Card.

(d)	Adaptec UW SCSI controller supports up to sixteen internal and external SCSI devices (controller uses one SCSI device connection). For more than fifteen SCSI devices, a second SCSI controller is required.

(e)	On-board EIDE controller supports up to four internal IDE devices.

(f)	10/100 BaseTx Fast Ethernet with cable assembly is included in Accessory Kit.

Notes:

- Windows NT 4.0 Country Kit ordered determines language variant of user documentation.

- Windows NT 4.0 Country Kit include system documentation, PS/2 mouse, power cord and PS/2 style keyboard, Windows NT media and Windows NT documentation.

- Systems do not include a monitor, order separately.  Monitors include video cable and 120V power cord, order country specific power cord for 240V use.

### DIGITAL Personal Workstation 433a Entry Level Packaged Configurations

Y=in base configuration; M=mandatory option; O=option

| DIGITAL PWS 433a | Resources Used |  | Entry Level with Graphics | Entry Level |
| --- | --- | --- | --- | --- |
| 120 V North American (a) |  |  | SN-B3AAP-EB | SN-B3AAP-FQ |
| 240 V Worldwide (b) |  |  | SN-B3AWW-EB | SN-B3AWW-FQ |
| CPU Alpha microprocessor EV56 | 433 MHz |  | Y | Y |
| Memory | 1 bank | Y | 32 MB ECC | 64 MB ECC |
| 1.44-Mbyte Floppy disk drive | 3.5” x 1” bay | Y | Y | Y |
| Adaptec UW SCSI controller (c) | 1 PCI slot (1, 2, or 3) |  | O | Y |
| On-board EIDE controller (d) |  | Y | Y | Y |
| Internal storage | 3.5” x 1” bay | Y | 2.1 GB EIDE | 2.1 GB SCSI |
| 600-Mbyte 12X EIDE CD-ROM | 5.25” x 1.6” bay | Y | Y | Y |
| On-board Ethernet 10BaseT/10Base2 |  | Y | Y | Y |
| On-board 16-bit Soundblaster Audio |  | Y | Y | Y |
| 10/100 BaseTx Fast Ethernet (e) | Accessory kit |  | O | Y |
| Telex Voice Commander Microphone | Microphone port | Y | Y | Y |
| Koss earphone | Earphone port | Y | Y | Y |
| Matrox Millennium (f) | 1 PCI slot (4 or 5) |  | Y | (f) |
| 300 Watt power supply |  | Y | Y | Y |
| Remaining available resources |  |  |  |  |
| I/O slots PCI slots PCI/ISA slots |  |  | 1 3 | 1 3 |
| Memory banks |  |  | 2 | 2 |
| 5.25” x 1.6” | User accessible bays |  | 2 | 2 |
| 3.5” x 1.0” | Internal bays |  | 1 | 1 |
| SCSI Device connections |  |  | 0 | 14 |
| EIDE disk connections |  |  | 2 | 3 |
| 1. Windows NT Country Kit |  |  |  |  |
| 120 V North American | Included | Y | Y | Y |
| 240 V Worldwide | See Options List | M | SN-PBB3P-xx | SN-PBB3P-xx |
| 2. Color Monitors | Select |  | 1 | 1 |
| 17” (16.0” viewable) 1280 x 1024 | Northern Hemisphere | O | SN-VRT17-WA | SN-VRT17-WA |
| 17” (16.0” viewable) 1280 x 1024 | Southern Hemisphere | O | SN-VRT17-W4 | SN-VRT17-W4 |
| 17” (16.0” viewable) 1600 x 1200 | Northern Hemisphere | O | SN-VRTX7-W3 | SN-VRTX7-W3 |
| 21” (19.7” viewable) 1600 x 1200 | Northern Hemisphere | O | SN-VRC21-WA | SN-VRC21-WA |
| 21” (19.7” viewable) 1600 x 1200 | Southern Hemisphere | O | SN-VRC21-W4 | SN-VRC21-W4 |
| 21” (19.7” viewable) 1600 x 1200 | Northern Hemisphere | O | SN-VRCX1-W3 | SN-VRCX1-W3 |

(a)	120 V U.S. variants include Windows NT 4.0 Country kit

(b)	240 V Worldwide variants require selection of Windows NT 4.0 Country Kit.

(c)	Adaptec UW SCSI controller (included in SN-B3A**-FQ variants) supports up to sixteen internal and external SCSI devices (controller uses one SCSI device connection). For more than fifteen SCSI devices, a second SCSI controller is required.

(d)	On-board EIDE controller supports up to four internal IDE devices.

(e)	10/100 BaseTx Fast Ethernet with cable assembly is included in Accessory Kit.

(f)	Selection of a graphics options is recommended for 
SN-B3A**-FQ variants.

Notes:

- Windows NT 4.0 Country Kit ordered determines language variant of user documentation.

- Windows NT 4.0 Country Kit include system documentation, PS/2 mouse, power cord and PS/2 style keyboard, Windows NT media and Windows NT documentation.

- Systems do not include a monitor, order separately.  Monitors include video cable and 120V power cord, order country specific power cord for 240V use.

### DIGITAL Personal Workstation 433a and 500a Base Configurations

Y=in base configuration; M=mandatory option; O=option

| DIGITAL PWS 433a | Resources Used |  | Base Configurations | Base Configurations |
| --- | --- | --- | --- | --- |
| 120 V North American (a) |  |  | SN-B3AAP-F1 | SN-B3AAP-L1 |
| 240 V Worldwide (b) |  |  | SN-B3AWW-F1 | SN-B3AWW-L1 |
| CPU Alpha microprocessor EV56 | 433 MHz |  | Y | Y |
| DIGITAL PWS 500a | Resources Used |  | Base Configurations | Base Configurations |
| 120 V North American (a) |  |  | SN-B3DAP-F1 | SN-B3DAP-L1 |
| 240 V Worldwide (b) |  |  | SN-B3DWW-F1 | SN-B3DWW-L1 |
| CPU Alpha microprocessor EV56 | 500 MHz |  | Y | Y |

| Memory | 1 bank | Y | 64 MB ECC | 128 MB ECC |
| --- | --- | --- | --- | --- |
| 1.44-Mbyte Floppy disk drive | 3.5” x 1” bay | Y | Y | Y |
| On-board EIDE controller (c) |  | Y | Y | Y |
| Internal storage (d) | 3.5” x 1” bay | M | M | M |
| 600-Mbyte 12X EIDE CD-ROM | 5.25” x 1.6” bay | Y | Y | Y |
| On-board Ethernet 10BaseT/10Base2 |  | Y | Y | Y |
| On-board 16-bit Soundblaster Audio |  | Y | Y | Y |
| Telex Voice Commander Microphone | Microphone port | O | O | O |
| Koss earphone | Earphone port | O | O | O |
| Graphics option (e) |  | O | O | O |
| 300 Watt power supply |  | Y | Y | Y |
| Remaining available resources |  |  |  |  |
| I/O slots PCI slots PCI/ISA slots |  |  | 2 3 | 2 3 |
| Memory banks |  |  | 2 | 2 |
| 5.25” x 1.6” | User accessible bays |  | 2 | 2 |
| 3.5” x 1.0” | Internal bays |  | 2 | 2 |
| SCSI Device connections |  |  | 15 | 15 |
| EIDE disk connections |  |  | 3 | 3 |
| 1. Windows NT Country Kit |  |  |  |  |
| 120 V North American | Included | Y | Y | Y |
| 240 V Worldwide | See Options List | M | SN-PBB3P-xx | SN-PBB3P-xx |
| 2. Color Monitors | Select |  | 1 | 1 |
| 17” (16.0” viewable) 1280 x 1024 | Northern Hemisphere | O | SN-VRT17-WA | SN-VRT17-WA |
| 17” (16.0” viewable) 1280 x 1024 | Southern Hemisphere | O | SN-VRT17-W4 | SN-VRT17-W4 |
| 17” (16.0” viewable) 1600 x 1200 | Northern Hemisphere | O | SN-VRTX7-W3 | SN-VRTX7-W3 |
| 21” (19.7” viewable) 1600 x 1200 | Northern Hemisphere | O | SN-VRC21-WA | SN-VRC21-WA |
| 21” (19.7” viewable) 1600 x 1200 | Southern Hemisphere | O | SN-VRC21-W4 | SN-VRC21-W4 |
| 21” (19.7” viewable) 1600 x 1200 | Northern Hemisphere | O | SN-VRCX1-W3 | SN-VRCX1-W3 |

(a)	120 V U.S. variants include Windows NT 4.0 Country kit

(b)	240 V Worldwide variants require selection of Windows NT 4.0 Country Kit.

(c)	On-board EIDE controller supports up to four internal IDE devices.

(d)	Selection of one hard disk drive is mandatory for Base systems.  Note: Base systems do not include a SCSI controller; select a SCSI controller if SCSI storage devices are selected.

(e)	Selection of a graphics options is recommended for Base systems, see Step 5.

Notes:

- Windows NT 4.0 Country Kit ordered determines language variant of user documentation.

- Windows NT 4.0 Country Kit include system documentation, PS/2 mouse, power cord and PS/2 style keyboard, Windows NT media and Windows NT documentation.

- Systems do not include a monitor, order separately.  Monitors include video cable and 120V power cord, order country specific power cord for 240V use.

### DIGITAL Personal Workstation 433a and 500a Base Configurations (continued)

Y=in base configuration; M=mandatory option; O=option

| DIGITAL PWS 433a | Resources Used |  | Base Configurations | Base Configurations |
| --- | --- | --- | --- | --- |
| 120 V North American (a) |  |  | SN-B3AAP-F1 | SN-B3AAP-L1 |
| 240 V Worldwide (b) |  |  | SN-B3AWW-F1 | SN-B3AWW-L1 |
| CPU Alpha microprocessor EV56 | 433 MHz |  | Y | Y |
| DIGITAL PWS 500a | Resources Used |  | Base Configurations | Base Configurations |
| 120 V North American (a) |  |  | SN-B3DAP-F1 | SN-B3DAP-L1 |
| 240 V Worldwide (b) |  |  | SN-B3DWW-F1 | SN-B3DWW-L1 |
| CPU Alpha microprocessor EV56 | 500 MHz |  | Y | Y |

| 3. Hard Disks (c) | Select |  | 1 or 2 | 1 or 2 |
| --- | --- | --- | --- | --- |
| 2.1 GB EIDE 4500 RPM disk | 3.5 x 1 or 1.6” bay | M | SN-PCXRA-AN | SN-PCXRA-AN |
| 3.2 GB EIDE 4500 RPM disk | 3.5 x 1 or 1.6” bay | M | SN-PCXRA-AP | SN-PCXRA-AP |
| 2.1 GB UltraWide SCSI 5400 RPM disk | 3.5 x 1 or 1.6” bay | M | SN-PBXRW-HC | SN-PBXRW-HC |
| 2.1 GB Wide SCSI 7200 RPM disk | 3.5 x 1 or 1.6” bay | M | SN-PCXWR-AY | SN-PCXWR-AY |
| 4.3 GB Wide SCSI 7200 RPM disk* | 1.6” bay | M | SN-PCXWR-AZ | SN-PCXWR-AZ |
| * 3.5” x 1.6” hard disk drive is supported in 5.25” removable media bay, brackets are included with system |  |  |  |  |
| 4. Memory | Select |  | 0, 1, or 2 | 0, 1, or 2 |
| –64 M-byte ECC DIMM Pair (2 x 32) | 1 bank | O | SN-MSP01-HC | SN-MSP01-HC |
| 128 M-byte ECC DIMM Pair (2 x 64) | 1 bank | O | SN-MSP01-HD | SN-MSP01-HD |
| 4a. Cache memory | Select |  | 0 or 1 | 0 or 1 |
| 2 MB External CPU SRAM cache | Cache socket | O | SN-MSC01-AA | SN-MSC01-AA |
| 5. Graphics |  |  |  |  |
| Matrox Millenium 3D graphics (d) | 1 PCI slot (4 or 5) | O | SN-PCXAG-AD | SN-PCXAG-AD |
| 2 MB WRAM for Matrox Millennium (d) | Matrox WRAM slot | O | SN-PCCAM-CA | SN-PCCAM-CA |
| 6 MB WRAM for Matrox Millennium (d) | Matrox WRAM slot | O | SN-PCCAM-CB | SN-PCCAM-CB |
| Accelgraphics 2500TX graphics (d) | 1 PCI slot (4 or 5) | O | SN-PCXAG-AW | SN-PCXAG-AW |
| PowerStorm 4D40T (d) | 1 PCI slot and 1 PCI/ISA slot (4 & 5) | O | SN-PBXGI-AA | SN-PBXGI-AA |
| PowerStorm 4D50T (d) | 1 PCI slot and 1 PCI/ISA slot (4 & 5) | O | SN-PBXGI-AB | SN-PBXGI-AB |
| PowerStorm 4D60T (d) | 1 PCI slot and 1 PCI/ISA slot (4 & 5) | O | SN-PBXGI-AC | SN-PBXGI-AC |
| 5a. PowerStorm Texture Memory Upgrades—Supported in graphics card memory slot |  |  |  |  |
| –4 MB Texture memory module |  | O | SN-PBXGI-GA | SN-PBXGI-GA |
| 16 MB Texture memory module |  | O | SN-PBXGI-GB | SN-PBXGI-GB |
| 32 MB Texture memory module |  | O | SN-PBXGI-GC | SN-PBXGI-GC |
| 6. Controllers and Adapters | Select |  | 1 or 2 | 1 or 2 |
| Adaptec UltraWide SCSI controller | 1 PCI slot (1, 2, or 3) | O | SN-PCTAZ-DE | SN-PCTAZ-DE |
| Fast Narrow Single Ended SCSI controller | 1 PCI slot (1, 2, or 3) | O | KZPAA-AA | KZPAA-AA |
| Wide to Narrow SCSI Adapter |  | O | SN-PBXKP-BA | SN-PBXKP-BA |
| 7. Removable media | Select |  | 0, 1 or 2 | 0, 1 or 2 |
| 600-Mbyte 12X EIDE CD-ROM | 5.25 x 1.6” bay | O | SN-PBXRD-DB | SN-PBXRD-DB |
| 8. Multimedia |  |  |  |  |
| Supreme Video JPEG PCI Card | 1 PCI Slot (1-5) |  | AV321-AA | AV321-AA |
| Microphone and Headset | External Ports |  | AVH01-AA | AVH01-AA |

(a)	120 V U.S. variants include Windows NT 4.0 Country kit

(b)	240 V Worldwide variants require selection of Windows NT 4.0 Country Kit.

(c)	Selection of one hard disk drive is mandatory for Base systems.  Note: Base systems do not include a SCSI controller; select a SCSI controller if SCSI storage devices are selected.

(d)	Selection of a graphics options is recommended for Base systems, see Step 5.

Notes:

- Windows NT 4.0 Country Kit ordered determines language variant of user documentation.

- Windows NT 4.0 Country Kit include system documentation, PS/2 mouse, power cord and PS/2 style keyboard, Windows NT media and Windows NT documentation.

- Systems do not include a monitor, order separately.  Monitors include video cable and 120V power cord, order country specific power cord for 240V use.

### DIGITAL Personal Workstation 433a and 500a Base Configurations (continued)

Y=in base configuration; M=mandatory option; O=option

| DIGITAL PWS 433a | Resources Used |  | Base Configurations | Base Configurations |
| --- | --- | --- | --- | --- |
| 120 V North American (a) |  |  | SN-B3AAP-F1 | SN-B3AAP-L1 |
| 240 V Worldwide (b) |  |  | SN-B3AWW-F1 | SN-B3AWW-L1 |
| CPU Alpha microprocessor EV56 | 433 MHz |  | Y | Y |
| DIGITAL PWS 500a | Resources Used |  | Base Configurations | Base Configurations |
| 120 V North American (a) |  |  | SN-B3DAP-F1 | SN-B3DAP-L1 |
| 240 V Worldwide (b) |  |  | SN-B3DWW-F1 | SN-B3DWW-L1 |
| CPU Alpha microprocessor EV56 | 500 MHz |  | Y | Y |

| 9. Network |  |  |  |  |
| --- | --- | --- | --- | --- |
| PCI 10/100 Mbs Fast Ethernet (e) | 1 PCI slot (1-5) | O | SN-DE500-AA | SN-DE500-AA |
| 10BaseT to 10/100 Mbs Ethernet Upgrade | Bulkhead Connector | O | SN-PCXAN-DA | SN-PCXAN-DA |
| PCI to FDDI Adapter | 1 PCI slot (1-5) | O | SN-DEFPA-AA | SN-DEFPA-AA |
| PCI EtherWorks card (TW, TP, AUI) | 1 PCI slot (1-5) | O | SN-DE450-CA | SN-DE450-CA |
| Additional Options: See DIGITAL Personal Workstation Options for a comprehensive list of qualified options |  |  |  |  |

(e)	Fast Ethernet (SN-DE500-AA) option ordered in Europe is not factory integrated, option ships as spare.

### DIGITAL Personal Workstation 433a and 500a Options

| Color Monitors | Description |
| --- | --- |
| SN-VRT17-WA | 17” (16.9” viewable) Color Monitor, MPRII, E-Star, 1280x1024, Northern Hemisphere, United States |
| SN-VRT17-W4 | 17” (16” viewable) Color Monitor , MPRII, E-Star, 1280x1024, Southern Hemisphere |
| SN-VRTX7-W3 | 17” (16.0” viewable) Color Monitor, TCO, MPRII, E-Star, 1280x1024, Northern Hemisphere |
| SN-VRC21-WA | 21" (19.6” viewable) Color Monitor, E-Star, 1600x1200, Northern Hemisphere, United States |
| SN-VRC21-W4 | 21"(19.7" viewable) Color Monitor, E-Star, 1600x1200, Southern Hemisphere |
| SN-VRCX1-W3 | 21” (19.7” viewable) Color Monitor, TC092, MPRII, E-Star, 1600x1200, Northern Hemisphere |
| User Documentation |  |
| EK-ALMIA-UI | DIGITAL Personal Workstation User’s Guide—English |
| EK-ALMIF-UI | DIGITAL Personal Workstation User’s Guide—French |
| EK-ALMID-UI | DIGITAL Personal Workstation User’s Guide—German |
| EK-ALMIJ-UI | DIGITAL Personal Workstation User’s Guide—Japanese |
| EK-ALMIS-UI | DIGITAL Personal Workstation User’s Guide—Spanish |
| EK-ALMII-UI | DIGITAL Personal Workstation User’s Guide—Italian |
| ER-B30WW-IM | DIGITAL Personal Workstation Quick Setup Guide—Multilingual |
| Hardware Supplemental Services |  |
| FM-PCXHW-36 | Years 2 and 3, 5x9, Next day response time |
| FM-PC4HR-12 | Year 1, 5x9, 4 Hour Response time |
| FM-PC4HR-36 | Years 1 to 3, 5x9, 4 Hour Response time |
| FM-PC724-12 | Year 1, 7x24, 4 Hour Response time |
| FM-PC724-36 | Years 1 to 3, 7x24, 4 Hour Response time |
| Software Supplemental Services |  |
| FM-PCXSW-12 | 12 Month Software Supplemental Support Services |
| FM-PCXSW-36 | 36 Month Software Supplemental Support Services |
| FM-PCXSW-60 | 60 Month Software Supplemental Support Services |

Specifications

| Height | 40.64 cm (16 inches) |
| --- | --- |
| Width | 21.59 cm (8.5 inches) |
| Depth | 44.45 cm (17.5 inches) |

### Windows NT 4.0 Country Kits

- 120 V U.S. North America Packaged and Base systems include Windows NT Country kit

- Country Kit must be ordered for 240V systems to be operational

- Windows NT Country Kit includes:

- Windows NT operating system license and media, country keyboard, power cord, User Documentation, and 
Logitech S35 PS/2 style 3-button mouse (SN-PCXLN-AD)

- Availability of local language is dependent upon Microsoft’s release schedule.

| Order Number | Area | Keyboard | Keyboard Language | Power Cord (2 in Each Kit) | User Documentation | Windows NT 4.0 Language |
| --- | --- | --- | --- | --- | --- | --- |
| Americas |  |  |  |  |  |  |
| SN-PBB3P-AA | United States | PCXLA-KA or -NA | English | 17-04357-02 | English | English |
| SN-PBB3P-AR | LACR-Spanish | PCXLA-KR or -NR | S. Amer. Spanish | 17-04357-02 | Spanish | Spanish |
| SN-PBB3P-AX | United States-GOVT | PCXLA-KA or -NA | English | 17-04357-02 | English | English |
| SN-PBB3P-AC | French Canadian | PCXLA-KC or -NC | French Canadian | 17-04357-02 | French | French |
| Europe |  |  |  |  |  |  |
| SN-PBB3P-AB | Belgium | PCXLA-KB or -NB | Belgian | 17-04364-01 | French | English |
| SN-PBB3P-AD | Denmark | PCXLA-KD or -ND | Danish | 17-04364-06 | English | Danish |
| SN-PBB3P-AE | UK | PCXLA-KE or -NE | English | 17-04364-02 | English | English |
| SN-PBB3P-AF | Finland | PCXLA-KF or -NF | Finland/Suomi | 17-04364-01 | English | Finish |
| SN-PBB3P-AG | Germany | PCXLA-KG or -NG | German | 17-04364-01 | German | German |
| SN-PBB3P-AH | Netherlands | PCXLA-KH or -NH | N. American | 17-04364-01 | English | Dutch |
| SN-PBB3P-AI | Italy | PCXLA-KI or -NI | Italian | 17-04364-01 | Italian | Italian |
| SN-PBB3P-AK | Switzerland | PCXLA-KK or -NK | Swiss | 17-04364-05 | French | French |
| SN-PBB3P-AL | Switzerland/Germany | PCXLA-KK or -NK | Swiss/French/German | 17-04364-05 | German | German |
| SN-PBB3P-AM | Sweden | PCXLA-KF or -NF | Finland/Suomi | 17-04364-01 | English | Swedish |
| SN-PBB3P-AN | Norway | PCXLA-KN or -NN | Norwegian | 17-04364-01 | English | Norwegian |
| SN-PBB3P-AP | France | PCXLA-KP or -NP | French | 17-04357-02 | French | French |
| SN-PBB3P-AS | Spain | PCXLA-KS or -NS | Spanish | 17-04357-02 | Spanish | Spanish |
| SN-PBB3P-AT | Israel | PCXLA-KT or -NT | Hebrew | 17-04364-07 | English | English |
| SN-PBB3P-AV | Portugal | PCXLA-KV or -NV | Portuguese | 17-04357-02 | English | English |
| SN-PBB3P-BH | Greece | PCXLA-LH or -PH | Greek | 17-04357-02 | English | English |
| SN-PBB3P-BP | Poland | PCXLA-KA or -NA | N. American | 17-04364-02 | Polish | Polish |
| SN-PBB3P-BQ | Hungary | PCXLA-LQ or -PQ | Hungarian | 17-04364-01 | English | English |
| SN-PBB3P-BR | Arabic | PCXLA-LR or -PR | Arabic | 17-04364-01 | English | English |
| SN-PBB3P-BT | Russia | PCXLA-LT | Russian | 17-04364-01 | English | Russian |
| SN-PBB3P-BU | Turkey | PCXLA-LU or -PU | Turkish | 17-04364-01 | English | English |
| SN-PBB3P-BY | Serbia | PCXLA-LY | Serbic | 17-04364-01 | English | English |
| SN-PBB3P-CQ | Iceland | PCXLA-MQ or -QQ | Icelandic | 17-04364-01 | English | English |
| SN-PBB3P-CZ | Czech Republic | PCXLA-MZ or -QZ | Czech | 17-04364-01 | English | English |
| SN-PBB3P-EC | European | PCXLA-KA or -NA | N. American | 17-04364-01 | English | English |
| APA |  |  |  |  |  |  |
| SN-PBB3P-BI | Taiwan | PCXLA-LI or -PI | Traditional Chinese | 17-04357-02 | English | Trad. Chinese |
| SN-PBB3P-BK | South Korea | PCXLA-LK or -PK | Korean | 17-04364-01 | English | English |
| SN-PBB3P-CA | Far East | PCXLA-KA or -NA | N. American | 17-04364-02 | English | English |
| SN-PBB3P-CE | Philippines | PCXLA-KA or -NA | N. American | 17-04357-02 | English | English |
| SN-PBB3P-CG | Vietnam | PCXLA-KA or -NA | N. American | 17-04364-01 | English | English |
| SN-PBB3P-CP | Thailand | PCXLA-KA or -NA | N. American | 17-04364-02 | English | English |
| SN-PBB3P-CV | China | PCXLA-KA or -NA | N. American | 17-04364-02 | English | English |
| SN-PBB3P-AJ | Japan | PCXLA-KY | Japanese | 17-04357-02 2J-6510A-01 | Japanese | Japanese |