### AlphaServer DS20 Linux Ready
V1.0—29 March 1999
Systems and Options Catalog

Product Description 	The Compaq AlphaServer DS20 is a 500MHz EV6 based Alpha microprocessor server with 4 MB level two cache.  The AlphaServer DS20 supports two CPUs, and is available in Pedestal configurations without an operating system to allow the installation of Linux on these systems.

The AlphaServer DS20 system’s high-reliability features include: hot swap disk drives, redundant power supplies, ECC memory, heat sensor, fan failure sensor, and power supply sensor. Integrated on the system motherboard is a diskette controller, two serial ports, one parallel port, and keyboard and mouse controller.

AlphaServer DS20 Pedestal System supports up to 10 internal storage devices including one floppy diskette drive, one CD-ROM, one additional 5.25-inch removable media device, and seven hot swap StorageWorks disk drives.  The system supports up to 128 GB of internal storage with seven 18.2 GB disk drives.

System Ordering Rules

- AlphaServer DS20 systems require the mandatory selection of:  a system disk, two SCSI ports, and a network adapter.  Users can select 4.3 GB, 
9.1 GB, or 18.2 GB disk drive as the system disk.

- To order a complete system select:  (1) a system, (2) a system disk, (3) two SCSI ports, and (4) an Ethernet Adapter.

Compaq and the names of Compaq products referenced herein are either trademarks and/or service marks or registered trademarks and/or service marks of Compaq.

DIGITAL is a Trademark of Compaq Computer Corporation.

Microsoft, Windows, Windows NT, SQL Server, Office and BackOffice are either trademarks or registered trademarks of Microsoft Corporation.

Intel and Pentium are registered trademarks of Intel Corporation.  Linux is a registered trademark of Linus Torvalds in several countries.

Other product and company names mentioned herein may be trademarks and/or service marks of their respective owners.

- Step 1AlphaServer DS20 Pedestal Systems

Configuration menus require the mandatory selection of:

- LINUX system from Step 1a

- Mandatory components:

- One system disk drive

- Two SCSI ports1

- One Ethernet adapter

1.	Two SCSI ports are required for internal removable media devices, and internal StorageWorks disk shelf operating in single-bus mode.  Note:  One additional SCSI port and an H8253-AA UltraSCSI bus splitter kit are required to operate StorageWorks disk shelf in split-bus mode.

Pedestal Systems include

- Alpha microprocessor 21264 500 MHz CPU with 4 MB ECC onboard cache.

- Pedestal enclosure with:

- 6 expansion slots 
Five 64 bit PCI slots and one 64-bit PCI/ISA combination slot

- 16 DIMM memory slots support 3 memory options

- 10 storage slots
One diskette drive slot (1.44MB diskette drive included)
One 600 MB 32X CD-ROM drive slot (CD-ROM included)
One additional removable media slot
Seven StorageWorks hard drive slots.

- Two serial ports, support full duplex asynchronous modem control

- One bi-directional enhanced parallel port

- PS/2 style keyboard port and mouse port.

- 675 Watt self-switching (120/240V, 60/50Hz) power supply1.

- 1.44 MB diskette drive in dedicated slot.

- 600 MB 32X CD-ROM—uses one removable media slot.

- 128 MB, 256 MB, or 512 MB.

- 3-button mouse.

- U.S. English keyboard and 120V power cord with North America orders.

- Selection of a country-specific keyboard and power cord is mandatory for all non-North American orders.

- Hardware Warranty: Three-year onsite.

1.	Included power supply provides power for normal system operation.  System supports an additional optional power supply for N+1 redundancy.

Step 1aCompaq AlphaServer DS20 OS-less Pedestal System

- Systems ordered in North America include U.S. English keyboard and 120 V power cord.  Selection of a country-specific keyboard and power cord is mandatory for all non-North American orders.

- System has been tested with Red Hat 5.2  and requires kernel 2.2.x.

Compaq 6/500 MHz CPU Pedestal System without Operating System Software

Note:	Pedestal systems have six available PCI slots.  Installation of mandatory options and recommended graphics reduces available PCI slots.

| Order Number | Memory | 2 SCSI Ports | Hard Disk Drive | Network Adapter | Graphics1 | Monitor |
| --- | --- | --- | --- | --- | --- | --- |
| DJ-55NJA-CA | 128 MB | Mandatory | Mandatory | Mandatory | Recommended | Recommended |
| DJ-55NJA-DA | 256 MB | Mandatory | Mandatory | Mandatory | Recommended | Recommended |
| DJ-55NJA-EA | 512 MB | Mandatory | Mandatory | Mandatory | Recommended | Recommended |

- Step 1bMandatory Options

- Selection of a minimum of two SCSI ports, a system disk drive, and a Network Adapter is mandatory.

- Additional options are supported, select as required.

Select one system disk driveMandatory

| Order Number | System Disk |
| --- | --- |
| DS-RZ1CF-VW | 4.3 GB 7200 RPM UltraSCSI disk |
| DS-RZ1DF-VW | 9.1 GB 7200 RPM UltraSCSI disk |
| DS-RZ1EF-VW1 | 18.2 GB 7200 RPM UltraSCSI disk |

Select two SCSI portsMandatory

- Two SCSI ports are mandatory to support internal storage:

- Internal removable media devices are supported on KZPCM-DA and KZPBA-CA adapters only.   Internal StorageWorks seven disk shelf requires one SCSI port when configured in single-bus mode1

- System includes SCSI cable(s) to connect SCSI adapters/controllers to internal storage.

1.	To configure StorageWorks disk shelf in split-bus mode one addition SCSI port is required.

- The requirement of two SCSI ports can be fulfilled by ordering:

- One KZPCM-DA two port UltraSCSI single-ended adapter, or

- Two KZPBA-CA one port UltraSCSI single-ended adapter

| Order Number | SCSI Adapter |
| --- | --- |
| KZPCM-DA | PCI-based two port UltraSCSI Single-ended adapter and 10/100 Mbit Ethernet Combo adapter |
| KZPBA-CA | PCI-based one port UltraSCSI Single-ended adapter |

Select one Ethernet AdapterMandatory

- Note:	Ethernet adapter is not required if KZPCM is selected as the SCSI adapter.

| Order Number | Network Adapter and Minimum O/S version supported |
| --- | --- |
| DE500-BA | PCI-based 10/100 Mbit Fast Ethernet adapter UTP |
| DE504-BA | PCI-based 10/100 Mbit Quad Channel Ethernet adapter |

Step 2Additional CPU Modules (SMP Add-ons and Upgrades)

AlphaServer DS20 system supports one additional CPU.

| 3X-KN310-AD | SMP upgrade includes one 6/500 CPU. Linux SMP support must be obtained separately. Required Linux Kernel 2.2.x |
| --- | --- |

Step 3Memory

- System supports up to four memory kits (each kit includes 4 industry-standard ECC DIMMs).

- System includes one memory kit.

- System supports three additional memory kits for a system maximum of 2 GB memory

| MS340-BA | 128 MB (4 x 32 MB DIMMs) memory kit |
| --- | --- |
| MS340-CA | 256 MB (4 x 64 MB DIMMs) memory kit |
| MS340-DA | 512 MB (4 x 128 MB DIMMs) memory kit |

Step 4Internal Storage

Pedestal Enclosure supports a total of ten storage devices:

- One dedicated diskette drive slot (1.44 MB diskette drive included).

- One 5.25” removable media slot (600 MB 32X CD-ROM included).

- One 5.25” additional removable media slot.

- Seven 3.5-inch internal hard disk drive slots.

Internal Storage Disk Drives

- Selection of one System Disk Drive is mandatory.

- Up to six additional hard disk drives are supported in internal storage disk shelf.

Note:	Internal storage disk shelf is configured in single-bus mode.  To configure internal storage disk shelf in split-bus mode one additional SCSI port and an H8253-AA UltraSCSI bus splitter kit are required.

| DS-RZ1CF-VW | 4.3 GB 7200 RPM UltraSCSI disk drive |
| --- | --- |
| DS-RZ1DF-VW | 9.1 GB 7200 RPM UltraSCSI disk drive |
| DS-RZ1EF-VW | 18.2 GB 7200 RPM UltraSCSI disk drive |

Internal UltraSCSI Cable Kit

| H8253-AA | UltraSCSI bus splitter kit includes SCSI bus repeater card, two data cables, Y power cable harness, and SCSI active terminator assembly, and installation guide. Required to configure internal storage shelf in split bus mode. |
| --- | --- |

Internal Removable Media Devices

- Systems include CD-ROM drive in 5.25” half-height removable media slot.

- Systems support one additional 5.25” half-height removable media device.

| RRD47-VD | 600 MB 32X CD-ROM drive (RRD47) 5.25” Top Gun Blue |
| --- | --- |

Step 5External Storage

UltraSCSI Configuration Rules

- UltraSCSI configurations require UltraSCSI components (controllers, adapters, shelves, disks, and cables).

- UltraSCSI adapters support UltraSCSI disks at UltraSCSI speeds in UltraSCSI Top Gun Blue shelves (DS-BA356-xx).

- Fast10 BA356-xx SCSI shelves can not be retrofitted. Replace Fast10 SCSI shelf with DS-BA356-xx shelf when adding an UltraSCSI disk and when connecting to UltraSCSI adapter.

- UltraSCSI and Fast10 drives can be mixed in DS-BA356 Top Gun Blue UltraSCSI shelf.  UltraSCSI adapter negotiates maximum transfer speed of drives.

UltraSCSI Connectors and Cables

- KZPBA-CA PCI one port UltraSCSI Single-ended adapter, 68HD connector, requires BN38C cables (2 to 20 m) to connect to UltraSCSI shelf.

- KZPBA-CB PCI one port UltraSCSI Differential adapter, 68HD connector, requires BN38C cables (2 to 25 m) to connect to UltraSCSI shelf.

- KZPCM-DA PCI two port UltraSCSI Single-ended adapter and 10/100M-bit Ethernet Combo adapter. Internal 68-pin and 50-pin HD connectors and two external 68-pin VHDCI connectors

BN38C VHDCI to 68HD CablesKZPBA UltraSCSI adapter to UltraSCSI shelf

| BN38C-xx | VHDCI male to 68HD male. xx=2, 3, 5, 10, and 20 meter lengths |
| --- | --- |

- Step 5External Storage (continued)

BN37A VHDCI to VHDCI Cables KZPCM UltraSCSI adapter to UltraSCSI shelf

| BN37A-0C | 0.3 m VHDCI male to VHDCI male |
| --- | --- |
| BN37A-0E | 0.5 m VHDCI male to VHDCI male |
| BN37A-01 | 1.0 m VHDCI male to VHDCI male |
| BN37A-1E | 1.5 m VHDCI male to VHDCI male |
| BN37A-02 | 2.0 m VHDCI male to VHDCI male |
| BN37A-2E | 2.5 m VHDCI male to VHDCI male |
| BN37A-xx | 3.0 m VHDCI male to VHDCI male. xx=3, 5, 10, 15, 20, and 25 meter lengths |

UltraSCSI StorageWorks Pedestals and Shelves

UltraSCSI Top Gun Blue shelf provides support for UltraSCSI disk drives.  Shelf is available in single-bus or split-bus configurations.  Shelf includes a personality module and a 180 W power supply.

UltraSCSI shelf supports:

- Seven 3.5-inch hard disk drives, or

- One 5.25-inch device and four 3.5-inch hard disk drives, or

- Two 5.25-inch devices and one 3.5-inch hard disk drive.

Note:	Installation of redundant power supply reduces number of 3.5” drive slots by one.

Pedestal Enclosure

| DS-BA356-KF | - UltraSCSI single-ended pedestal, single-bus personality module, 180 W power supply |
| --- | --- |
| DS-BA356-KG | - UltraSCSI single-ended pedestal, split-bus personality module, 180 W power supply |
| DS-BA356-KH | - UltraSCSI pedestal, differential personality module, 180 W power supply |
| DS-BA35X-HH | 180 W power supply |

- Step 5aStorage Controllers

- Selection of two SCSI ports is mandatory for internal storage, see Step 1b.

| Order Number | Description |  |
| --- | --- | --- |
| KZPCM-DA | PCI-based two port UltraSCSI Single-ended adapter and 10/100Mbit Ethernet Combo adapter, uses 1 PCI slot. Internal 68-pin and 50-pin HD connectors and two external 68-pin VHDCI SCSI connectors. |  |
| KZPBA-CB | - PCI one port UltraSCSI Differential adapter, 68HD connector, requires BN38C cables (2 to 25 m) to connect to UltraSCSI shelf. |  |
| KZPBA-CA | PCI one port UltraSCSI Single-ended adapter, 68HD connector, requires BN38C cables (2 to 20 m) to connect to UltraSCSI shelf |  |
| BN37A-xx | VHDCI male to VHDCI male UltraSCSI cable; required for each external UltraSCSI shelf attached to KZPCM. |  |

- Step 6Monitors

Graphics monitors other than those listed below can be used if compatible with SVGA graphics ordered with a system.

Note:	Selection of a video extension cable and a country specific power cord is mandatory for all monitor variants.
See Step 6a for video extension cable
See Step 6b for country specific power cords

| SN-VRQV5-24/23 | 15" (13.8" viewable image size) Corporate Series auto-scan color monitor, flat square Invar CRT, 0.28 mm dot pitch, VGA to 1024 x 768 @ 75 Hz, TCO 95, MPRII, Energy Star, attached 1.8 meter video cable. Requires mandatory selection of video extension cable and country specific power cord for all variants. –24 = Northern Hemisphere without power cord –23 = Southern Hemisphere without power cord |
| --- | --- |

- Step 6Monitors (continued)

| SN-VRQP7-24/23 | 17” (16.0” viewable image size) professional series auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1280 x 1024 at 75 Hz, TCO 95, MPR-II, Energy Star, attached 1.8 meter video cable. Requires mandatory selection of video extension cable and country specific power cord for all variants. –24 = Northern Hemisphere without power cord –23 = Southern Hemisphere without power cord |
| --- | --- |

| SN-VRQP1-24/23 | 21" (19.6" viewable image size) auto-scanning color monitor, Trinitron CRT, 0.25 mm aperture grill pitch, VGA to 1600 x 1200 at 75 Hz NI, TCO 95, Energy Star, includes a 1.8 meter video cable. Requires mandatory selection of video extension cable and country specific power cord for all variants. –24 = Northern Hemisphere without power cord –23 = Southern Hemisphere without power cord |
| --- | --- |

Step 6aVideo Extension Cable

| BN39C-02 | 1.8 meter video extension cable—mandatory for each monitor ordered |
| --- | --- |

Step 6bMonitor Power Cords

| BN26J-1K | North American, Japan, 120 V, 75 inches long |
| --- | --- |
| BN19H-2E | Australia, New Zealand, 2.5m long |
| BN19C-2E | Central Europe, 2.5 m long |
| BN19A-2E | UK, Ireland, 2.5 m long |
| BN19E-2E | Switzerland, 2.5 m long |
| BN19K-2E | Denmark, 2.5 m long |
| BN19M-2E | Italy, 2.5 m long |
| BN19S-2E | Egypt, India, South Africa, 2.5 m long |
| BN18L-2E | Israel, 2.5 m long |

- Sep 7Graphics Adapters

- Select graphics adapter, monitor, and country specific keyboard for Linux if required.

- System supports maximum of one graphics adapter.

| SN-PBXGK-AA | Powerstorm 4D10T 4MB graphics adapter |
| --- | --- |

- Step 8Networks and Communications

| Order Number | Description |
| --- | --- |
| DE500-BA | PCI-based 10/100-Mbit 1 port Fast Ethernet controller (Twisted Pair) |
| DE504-BA | PCI-based 10/100-Mbit 4 Port Fast Ethernet controller (Twisted Pair) |
| BN25G-xx | Cable for 10BaseT Twisted Pair connection |

Step 9Additional Power Supply

- One additional power supply may be added to AlphaServer DS20 Pedestal system for N+1 redundancy.

- Redundant Power Supply requires a country-specific power cord.

| H7899-AA | 675 W Redundant Power Supply Option, requires country-specific power cord |
| --- | --- |

- Step 10Keyboards, Power Cords, and Documentation

Keyboards

Systems ordered in North America include U.S. English keyboard and 120V power cord.  Selection of a country-specific keyboard and power cord is mandatory for all non-North American orders.

| Compaq Keyboard | Country |
| --- | --- |
| SN-LKQ47-AA | US/English |
| SN-LKQ47-AB | Belgian |
| SN-LKQ47-AC | Canadian/French |
| SN-LKQ47-AD | Danish |
| SN-LKQ47-AE | UK |
| SN-LKQ47-AF | Finnish |
| SN-LKQ47-AG | German |
| SN-LKQ47-AI | Italian |
| SN-LKQ47-AK | Swiss/French |
| SN-LKQ47-AM | Swedish |
| SN-LKQ47-AN | Norwegian |
| SN-LKQ47-AP | French |
| SN-LKQ47-AR | Latin-American |
| SN-LKQ47-AS | Spanish |
| SN-LKQ47-AV | Portuguese |
| SN-LKQ47-AX | Bhcsy |
| SN-LKQ47-BA | International |
| SN-LKQ47-BH | Greek |
| SN-LKQ47-BI | Taiwanese |
| SN-LKQ47-BK | Korean |
| SN-LKQ47-BP | Polish |
| SN-LKQ47-BQ | Hungarian |
| SN-LKQ47-BR | Arabic |
| SN-LKQ47-BT | Cyrillic |
| SN-LKQ47-BU | Turkish |
| SN-LKQ47-BV | Czech |
| SN-LKQ47-CB | Thai |
| SN-LKQ47-CZ | Slovak |

Pedestal Power Cords

Systems ordered in North America include one BN26J-1K 120 V North American power cords.  Order one country specific power cords for all systems ordered in Europe and AP.

| BN26J-1K | North American, Japan, 120 V, 75 inches long |
| --- | --- |
| BN19H-2E | Australia, New Zealand, 2.5m long |
| BN19C-2E | Central Europe, 2.5 m long |
| BN19A-2E | UK, Ireland, 2.5 m long |
| BN19E-2E | Switzerland, 2.5 m long |
| BN19K-2E | Denmark, 2.5 m long |
| BN19M-2E | Italy, 2.5 m long |
| BN19S-2E | Egypt, India, South Africa, 2.5 m long |
| BN18L-2E | Israel, 2.5 m long |

- Step 10Keyboards, Power Cords, and Documentation (continued)

Documentation

| QZ-014AA-GZ | Additional AlphaServer DS20 User Guide (print and CD), Installation Card, and Release Notes |
| --- | --- |

Step 11Printers and Remote Console

Systems include two full duplex asynchronous serial ports and one bi-directional enhanced parallel port.

Asynchronous serial ports have 9-pin D-subminiature connectors. A 9-pin to MMJ adapter (H8571-J) is required for each connection.  See printer section of Systems and Options Catalog for serial and parallel printer information.

- Step 12Software

Operating system software is not supplied with this system.

See the AlphaServer DS20 Linux-Ready web site for more details:  http://www.digital.com/alphaserver/linux/index.html

- Step 13Hardware Support Services

Hardware—Americas and Asia Pacific only

- Systems include three-year hardware warranty, onsite with 5 x 9, 24-hour response time.

- Option order as part of the Initial System Sale take on the Warranty of the enclosure

- Option added after the Initial System Sale take on the Warranty of the enclosure until the enclosure warranty ends.

- All unique AlphaServer DS20 options have a minimum of 1 Year Return to Compaq Warranty.

- Select optional Hardware Supplemental Support Services if required.

| FM-TC4HR-36 | 5 x 9, 4-hour response time |
| --- | --- |
| FM-TC512-36 | 5 x 12, 4-hour response time |
| FM-TC616-36 | 6 x 16, 4-hour response time |
| FM-TC724-36 | 7 x 24, 4-hour response time |
| FM-TCXHW-60 | Years 1-5, next day, Onsite |
| FM-TC4HR-60 | Years 1-5, 5x9, 4-hour response time |
| FM-TC512-60 | Years 1-5, 5x12, 4-hour response time |
| FM-TC616-60 | Years 1-5, 6x16, 4-hour response time |
| FM-TC724-60 | Years 1-5, 7x24, 4-hour response time |

- Step 13aHardware and Software Supplemental Support Services (Europe only)

Europe does not have specific part numbers for Hardware. Prices are quoted based on worldwide service reference pricing, contact MCS Sales in your country for information on Hardware Support Services.

AlphaServer DS20 System Diagram Front View

AlphaServer DS20 System Diagram Side and Back View

System Specifications

| Shipping Dimension |  |  |
| --- | --- | --- |
| Height Width Depth Weight | 60 cm (23.8 in.) 43 cm (16.9 in) 65 cm (25.6 in) 43 kg (95 lb) typical 71 kg (156 lb) maximum |  |
| Installed Dimensions |  |  |
| Height Width Depth Weight | 46 cm (18.1 in.) 35.8 cm (14.1 in.) 58.6 cm (23.1 in.) 39 kg (86 lb) typical 51 kg (113 lb) maximum |  |
| Clearances | Operating | Service |
| Front Rear Sides | 75 cm (29.5 in.) 15 cm (6 in.) None | 75 cm (29.5 in.) 75 cm (29.5 in.) 75 cm (29.5 in.) |
| Environmental |  |  |
| Temperature | Operating* Rate of change Nonoperating Storage (60 days) Rate of change | 10-40° C (50-104° F) 11° C/hr (20° F/hr) -40-66° C (-40-151° F) -40-66° C (-40-151° F) 20° C/hr (36F/hr) |
| Relative humidity | Operating Nonoperating Storage (60 days) Rate of change | 10–90% 10–95% 10–95% 20%/hr |
| Maximum wet bulb temperature | Operating Storage (60 days) | 28° C (82° F) 46° C (115°F) |
| Minimum dew point temperature | Operating Storage (60 days) | 2° C (36° F) Not tested |
| Maximum heat dissipation Dual supply | Current 4097 Btu/hr |  |
| Air flow and quality | Intake location Exhaust location Particle size Concentration | Front Rear N/A N/A |
| Altitude | Operating† Nonoperating | 3048 m (10,000 ft) 12192 m (40,000 ft) |
| Mechanical shock | Operating Nonoperating | 10 G 10 ms 20 G peak 30 ms |
| Vibration | Operating Nonoperating | 10-500 Hz .1 G peak 1.03Grms 5-300Hz |
| Acoustics | Operating Idle | Average Declared 5.7 LwA, B 38 LpAm, dba 5.6 LwA, B 37 LpAm, dba |
| Electrical—Power Supplies are universal, PFC, auto Ranging, 100/240 Vac |  |  |
| Nominal ac voltage Operating Voltage range Power source phase Nominal frequency Frequency range Maximum inrush current per line core RMS current (maximum) Single power supply (Single cord set) Dual power supply (Multiple cord set) RMS current (typical) | 100-120 Vac 90-128 Vac Single 60 Hz 59-61 Hz 11 Amps 12 Amps 7.4 Amps each supply 11.7 Amps @ 100Vac 9.3 Amps @ 120Vac | 220-240 Vac 180-256 Vac Single 50 Hz 49-51 Hz 22 Amps 5 Amps 3.7 Amps each supply 4.9 Amps @ 220Vac 4.4 Amps @ 240Vac |
| Power cord | Type Length U.S. plug | IEC 320 C13 190 cm (75 in.) NEMA 5-15 |

System Specifications (continued)

| Regulatory |  |
| --- | --- |
| Agency approvals | UL: Listed to UL1950 (3rd edition) CSA: Certified to CAN/CSA-C22.2 No. 950-M95 TUV: EN 60950 VDE 0805 GS marked FCC: Part 15.B Class A CE: EN55022, EN50082 VCCI Class II ITE BCIQ: CISPR22, CNS13438 C-Tick: CISPR22, AS/NZS 3548 |
| Reviewed to | AS/NZ 3260:1993 Australian/New Zealand Standard EN 60950/A4: 1997 European Norm IEC 950 (2nd edition, 3rd amend) |

*	Maximum operating temperature at Sea Level. Reduce by 1 C (1.8 F) for each 600 m (2000 ft) above Sea Level.

†	Higher altitudes are possible if maximum operating temperature is reduced (see Temperature, above); other restrictions may apply, such as maximum permissible altitude for hard drives.