Explorer 21

Entirely dedicated to asset and personnel tracking, the Explorer 21 combines GSM/GPRS and GPS functions in a single robust waterproof (IP 65) case.

Autonomous and discreet, the Explorer 21 integrates GSM and GPS antennas as well as new technology rechargeable batteries associating optimised energy management. The GPS information may be transmitted by SMS, GSM or GPRS data communication.

The Explorer 21 is Quad-Band (850/900/1800/1900 MHz) GSM/GPRS Class 10. The high sensitivity GPS solution ensures the reception of GPS data in difficult environmental conditions.

The connectivity IP protocols (TCP UDR FTR SMTP, P0P3) integrated in the embedded application (locally or remotely configurable), allow the rapid development of onboard high value-added telematic solutions.

Explorer 21

•  GPS TRACKER

•  GSM / GPRS COMMUNICATION

•  WATERPROOF (IP 65)

•  3 AXIS ACCELEROMETER

•  2 BUTTONS, 3 LEDS

•  BATTERY 1.5 AH or 3 AH

•  INTELLIGENT ENERGY

MANAGEMENT

GSM / GPRS FEATURES

- E-GSM Quad-band 850/900/1800/1900 MHz

- ETSI GSM Phase 2+

- Class 4 (2W @ 850 / 900 MHz)

- Class 1 (1W @ 1800 / 1900 MHz)

- SIM Toolkit Release 99

- GPRS Class 10 (Up to 4Rx / 2Tx)

- Supports PBCCH, coding schemes: CS1 to CS4

- Library TCP/IP (PPP RFC, TCP Socket,

UDP Socket, SMTP, POP3, FTP)

- Asynchronous data circuit, transparent and non-transparent up to 14400 bits/s

- SMS point to point MT/MO and SMS CB

GPS FEATURES

- 16 channel μblox receiver

- Precision:

- 2,5 meters CEP

- DGPS 2m CEP

- Compatible A-GPS

- Compatible GALILEO

-	Acquisition time :

- Hot start: < 3,5 sec

- Cold start: 36 sec

- Re-acquisition time after loss of signal < 1 s

- High Sensitivity: -158 dBm when Tracking

- Protocols:

•	NMEA-0183 Input/output

FUNCTIONS

- Profiled energy and event management

- 3 axis Accelerometer (±2g / ±8g):

- Movement

- Shock

- Verticality

- Memory capacity: 6000 positions

- Zone management (Geo-fencing)

- AT Commands

- Watchdog

INTERFACES

- Internal GSM / GPS antennas

- Internal rechargeable battery - LiPolymer 3.7 Vdc

- Version Explorer 21: 1,5 Ah

- Version Explorer 23: 3 Ah

-	Multi-function connector (base charger):

- Intelligent charging

- Configuration (via USB)

- SIM reader (SIM 1,8 / 3V)

- 2 programmable buttons

- 3 programmable LEDs (GSM, GPS, ...)

AVERAGE AUTONOMY

-	Version Explorer 21:

30 days (1 GPS position transmission per day)

-	Version Explorer 23:

60 days (1 GPS position transmission per day)

-	Idle mode active: 6 months

ENVIRONMENTAL & MECHANICAL

- Operating temperature: -20°C to +55°C

- Storage temperature: -25°C to +60°C

- Relative humidity: 80% to 100%

- Waterproof case (IP65)

Height 106 mm x Width 48,5 mm x Depth 26 mm

-	Weight :

- Version Explorer 21: 90 g

- Version Explorer 23: 115 g

APPROVALS

- R&TTE (Radio & Telecom. Terminal Equipment)

- Certification European (CE)

OPTIONS

- Base charger: 100 to 240 Vac with USB cable

- Strap

- Belt clip

- Magnetic fixing