The Missile Application Condensed Measurements record (MACM)

## Summary: This application note describes the MACM message.  This message was produced to optimize data output and data transmission in missile applications requiring high speed data output or communications bandwidth limitations.

## Missile Applications present data output challenges:

## 1. High Speed Data Output

Several of Magellan’s precision OEM receivers are capable of extremely rapid data output rates.  In several applications raw data messages are required for on-the-fly tracking or data downlink and storage for later use in processors.  HDMA customers (High Dynamics for Missile Applications) often find that they are tracking vehicles that move at many times the speed of sound and these high output rates are required.  Often, over the course of one second, many things can change with respect to the location device.

## 2. Bandwidth Limitations

Often more general uses of GPS receivers for accurate positioning have the problem of band width limitations.  These receivers are capable of outputting data at very high rates but the actual utility of these features can be limited because the radio modem or other data transmission devices used can only convey the messages as a lower rate.   In some cases the data radio may have other information to transmit along with the GPS derived location information.  This can further reduce the amount of data that can be transmitted in some situations.

## 3. Multiple Messages

Often the user is forced to request the receiver to output several different messages only to use bits and pieces and ignore the unused portions.  This can quickly overburden a data transmission device and reduce the efficiency of the GPS receiver by a great deal.

## Customer Defined Message Widely Used

In the case of missile trajectory data transmissions for scoring and tracking high speed vehicles, these issues are very important.  On the recommendation of the Joint Advanced Missile Instrumentation group and Waypoint Consulting, Magellan has incorporated a special message into its G12-HDMA receiver to increase the efficiency of the trajectory module in which the receiver is found.  This message has been adopted by several other of Magellan’s receivers and can be used for other applications as well.

All versions of G12 Firmware from versions GF08 have been given this new message named the Missile Application Condensed Measurements record (MACM):

Missile Application Condensed Measurements record (MACM):

Listed below is the format for a one epoch measurement.  The syntax for each parameter listed in brackets is “ [parameter name : number of bytes for this parameter]”  To the right of each line listed below is an identifier and a byte count.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

[MACM:4] [COUNT:2] [RCVTIME:4] [NAVT: 4] 											Header: 14

[PRN:1][WRN:1][POL:1][CN0:1][PHASE:8][RANGE:4][DPL:4][LCK_TIME:4]				Prn Data: 24

[PRN:1][WRN:1][POL:1][CN0:1][PHASE:8][RANGE:4][DPL:4][LCK_TIME:4] 				Prn Data: 24

[PRN:1][WRN:1][POL:1][CN0:1][PHASE:8][RANGE:4][DPL:4][LCK_TIME:4] 				Prn Data: 24

[PRN:1][WRN:1][POL:1][CN0:1] PHASE:8][RANGE:4][DPL:4][LCK_TIME:4] 				Prn Data: 24

[PRN:1][WRN:1][POL:1][CN0:1][PHASE:8][RANGE:4][DPL:4][LCK_TIME:4] 				Prn Data: 24

[PRN:1][WRN:1][POL:1][CN0:1][PHASE:8][RANGE:4][DPL:4][LCK_TIME:4] 				Prn Data: 24

[PRN:1][WRN:1][POL:1][CN0:1][PHASE:8][RANGE:4][DPL:4][LCK_TIME:4] 				Prn Data: 24

[PRN:1][WRN:1][POL:1][CN0:1][PHASE:8][RANGE:4][DPL:4][LCK_TIME:4] 				Prn Data: 24

[CHECKSUM:1] 																		Checksum:  1

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Total message length for 8 SV measurements: 207 bytes.

| Byte# | Name | Type | Size | Content | Origin |
| --- | --- | --- | --- | --- | --- |
| 1 | MACM | char | 4 | Name of Message (“MACM”) | sync_word(ASCII"MACM") |
| 5 | COUNT | char | 2 | Number of remaining structures to be sent for the current epoch. | MBN record, count |
| 7 | RCVTIME | long | 4 | Signal received in milliseconds of week GPS system time. This is the time tag for all measurements and position data. | PBN record, rcvtime |
| 11 | NAVT | float | 4 | Receiver clock offset in meters. | PBN record, navt |
| 24*j-9 | PRN | unsigned char | 1 | Satellite PRN number | MCA record, svprn |
| 24*j-8 | WRN | unsigned char | 1 | Warning flag, where: Bit 1 set = see note below Bit 2 set = see note below Bit 3 set = carrier phase questionable Bit 4 set = code phase questionable Bit 5 set = code phase integration questionable Bit 6 set = not used Bit 7 set = possible loss of lock Bit 8 set = loss of lock counter reset The interpretation of bits 1 and 2 is as follows: Bit 1 Bit 2 0 0 Same as 22 in goodbad flag (see next field) 1 0 Same as 23 in goodbad flag 0 1 Same as 24 in goodbad flag Note that more than one bit may be set at the same time, e.g., if bits 1, 3, and 6 are set at the same time, the warning flag is 37 (1 + 4 + 32) | MCA record, warning |
| 24*j-7 | POL | unsigned char | 1 | This number is either 0 or 5, 0 meaning satellite is just locked, and 5 meaning the beginning of the first frame has been found. | MCA record, polarity_known |
| 24*j-6 | CN0 | unsigned char | 1 | Signal-to-noise of satellite observation | MCA record, ireg |
| 24*j-5 | PHASE | double | 8 | Full carrier phase measurements in cycles. Not available unless carrier phase option is installed. | MCA record, full_phase |
| 24*j+3 | RANGE | unsigned long | 4 | Pseudo-range in seconds, sf=3.0e10 | G-8 ITA record, raw_range |
| 24*j+7 | DPL | long | 4 | Doppler (10-4 Hz) | MCA record, doppler |
| 24*j+11 | LCK_TIME | unsigned long | 4 | Continuous counts since satellite is locked. This number is incremented about 500 times per second | MBN record, lost_lock_ctr |
| 24*N+15 | checksum | unsigned char | 1 |  | MCA record |

Note:         j = 1,2, ..., N        where N = the order # of the PRN record in message

The message is variable in length, which is defined by the COUNT field in the header.

The message begins with the 4 byte sync word [4D 41 43 4D]

(ASCII  "MACM") and ends with the checksum byte. There are no CR, LF pairs

associated with the message.