Firmware Release Notice

Date:		March 20, 1999

Product:  	G8 and G8 Development Kit

Subject: 	HE00 Firmware Release

Number: 	G8_9901

Introduction:  HE00 firmware is available on G8 boards shipped after March 31, 1999 (P/N 701067-02 Rev. B).  HE00 is GPS week rollover and Y2K compliant. Dealers and Direct Thales Navigation customers that desire upgrades will be emailed a HE00 Hex file at no cost so they may copy the file onto PROMs for use on the G8.  The direct customer must supply the PROM; Thales Navigation recommends PROM ATMEL AT27C2048-90 JI or equivalent.  As an alternative, Thales Navigation will provide programmed PROMs at a cost of $15 each.

NOTE:  Only G8 boards with PROM sockets can be upgraded.  These boards can be identified by Thales Navigation part number 701067 (any Revision level) and 701067-01 (any Revision level).

Fixes/Enhancements:

- Improved hot start performance when the receiver has been turned off for more than 1.5 hours.  Receiver now supports 2D/3D in hot start and warm start situation.

- Improved satellite data by avoiding data collection when the receiver does not have a good phase lock and by detecting  errors in decoded data (where possible).

- Revised checksum calculation routine to obtain correct values for binary messages.

- Corrected problem in elevation calculation that occasionally produced wrong elevation angle calculation.

- Added SNR mask command for discarding low SNR satellites to improve performance in extremely high multipath environments.  The command is <$PASHS,SNM,f> where f is the SNR Mask threshold in dB-HZ, with a range from 0.0 to 60.0.  Query <$PASHQ,SNM> will return <$PASHR,SNM,f> with current setting.  Default setting is f = 37.

- Modified GPALM message to output 10 bit week number

- Modified GGA message to always output HDOP.

- Addressed minor Y2K compliance issues present in HD00 firmware.

Comments and Known Problems:

- There are no known problems.

Known Incompatibilities:

- There are no known incompatibilities.