Firmware Release Notice						Survey and Mapping

Date:		December 10, 1999

Product:  	L/M/P/DN-12/Ranger

Subject: 		7G firmware release

Introduction:

7G is the final release of firmware to support the older obsolete products that used 6A through 7C. 7G is a compilation of 7E and 7F which may be obtained via Ashtech’s FTP server for receivers equipped with a FLASH memory board. For units that have only EPROMs, obtaining the firmware update requires the unit to be serviced by Ashtech or an authorized servicing dealer.

7C to 7E Change Description

- GPS 1023/1024 week rollover related functions.

  - Fixes GPS 1023/1024 week rollover. GPS week will roll to 1024, 1025, etc. rather than rolling back to 0.

  - Messages such as almanac (NMEA, DAL, ALM and binary SAL), ephemeris SNV now have 4-digit GPS week number.

  - Screen 8 will show 4-digit GPS week number.

  - GPS day in the session programming sub-menu will display the correct value.

- RTCM age display on screen 5 for RTCM type 9.

  - The RTCM age for the RTCM type 9 is now the average age.

- Binary almanac output via serial ports.

  - The SAL message is now output for all available satellites instead of only one.

## 7E to 7F Change Description

- The antenna height upper limit is increased from 6.4 to 64.0 meters.

  - Screen 9 accepts and displays the antenna height up to 64.0 metes. The maximum antenna height value sent to the HOSE.EXE and REMOTE.EXE programs is increased to 64.0 meters.

- Screen 10 display.

  - Screen 10 now displays correct satellites.

## 7F to 7G Change Description

- Screen 10 now has the orbit path display.

- Screen 11 now shows correct almanac information.