Firmware Release Notice			Survey and Mapping

Date:		September 5, 2000

Product:  	SCA-12, SCA-12S

Subject: 	1J00 Firmware Release Notice

Number: 	SCA0001

Introduction:

1J00 is a no cost update for the SCA12.  Firmware can be downloaded from the Ashtech FTP site ftp://ftp.ashtech.com/pub/firmware/sca.   To load firmware on receivers with 1i00 the Universal Firmware Loader program or plsca.exe may be used.   The Universal Firmware Loading program and instructions are available on the Ashtech ftp site in the software/Universal Firmware Loader folder.  To load 1J00 firmware over 1H00 or earlier, plsca with the –old flag must be used as described in the upload.txt file that is included with the firmware.

1J00 Firmware Part Numbers:

690840 (Ashtech)

690841 (Topcon)

New Features:

- The antenna height range has been changed from 0 – 6.4 meters to 0.000 – 64.0 meters.  The resolution has changed from 0.0001 to 0.001 meters.  The ANH and INF set commands have been modified to accept antenna height values between 0.000 and 64.0 meters.  The RAW, INF and GIS response to query commands have been modified to display antenna height values between 0.0000 and 64.0 meters.

- Added PTT message to be output via the serial port selected to display 1PPS timing information.  1PPS option (L) option must be installed for this feature.

Command:  $PASHS,NME,PTT,x,c <CR><LF>

Enable/disable 1PPS timing information message on port x, and c is ON or OFF.

Command:  $PASHQ,PTT,x <CR><LF>

This is the associated query command where x is the optional output port.

Format:  $PASHR,PTT,dd,hh:mm:ss.sssssss*cc

Header			$PASHR,PTT,

Day of the week		dd,

Pulse time tag		hh:mm:ss.sssssss

- Checksum			*cc

Known Problems:

- The <$PASHS,CLM> command (clear external memory) clears both external and internal memory.

- Running receiver in point positioning mode will lock up the receiver after approximately fifteen to twenty hours in this mode.

Known Incompatibilities:

- 1.  When using HOSE to download the receiver, version 5.5.00 must be used.. If a previous version of hose is used, the data will transfer properly, but the incorrect antenna height information will be displayed in the S-file.  HOSE.EXE ver. 5.5.00 is available in the software/utils folder on the Ashtech ftp site.