Firmware Release Notice

Date:		April 1, 1999

Product:  	Z-12, DNS-12

Subject: 	1K00 firmware release

Number:  Z12_9901

Introduction:

1K00 is a no cost firmware upgrade for the Z-12 and DNS-12.  It can be downloaded from Ashtech FTP site ftp://csguest@ftp.ashtech.com.  Self-extracting ZIP file Z12_1K00.exe in the Z12 folder contains the 1K00 firmware and loading instructions.

Fixes/Enhancements:

- Y2K and GPS weeks 999 and  1023 rollover compliance has been added.

- Swedish NAVY Datum, RT90, has been added.

- MSK setup option has been added to the Differential menu.

- MSK status is now available on page 2 of screen 5.

- Daisy chain operation is now available for all four ports.

- External frequency command and status query added.

- $PASHS,EXT,c,d where c = the frequency, and d = A for Automatic Switch or N to stay in external frequency.

- $PASHQ,EXT,x, where x = the port.

- $PASHS command is now available to set up PPS and $PASHQ,PPS,port will display PPS setup status.

- Firmware now supports Survey Control software.

- An occasional problem that would allow position jumps while in DGPS remove mode has been corrected.

- Signs in NMEA messages comply with NMEA standards.  A positive field will not have a sign and a negative field will have a “-“ sign.  <$PASHS,TST,200> will turn on “+” signs and <$PASHS,TST,201> will turn off  “+” signs.

- ZDA NMEA message is now available from the NMEA sub screen..

- Proprietary POS message is now available from the NMEA sub screen.

- Antenna heights in range of 0 to 64 meters can now be entered through $PASHS,ANH,xxx or entered into screen nine.

Comments and Known Problems:

- When maximum age is exceeded in the remote DGPS mode, the age of corrections jumps to 999.

- There is an intermittent problem loading waypoint files via HOSE.  Sometimes the file is not accepted and sometimes the range and bearing are incorrect.

- In DGPS remote mode, the QAFREQ parameter on screen five does not provide an accurate radio link reliability.  Reliability is better than depicted.

Known Incompatibilities:

- Remote 5.0 is not compatible with the Z12.  Version 5.5 can be downloaded from the FTP site.