Firmware Release Notice			Survey and Mapping

Date:		June 26, 1999

Product	Z-12, DNS 12

Subject: 	Version 1L00 Firmware Release

Number:

Introduction:

1L00 is a no cost firmware upgrade for the Z-12 and DNS-12. It can be downloaded from Ashtech FTP site ftp://ftp.ashtech.com. Self-extracting ZIP file z_1l00.exe in the z12 folder contains the 1L00 firmware and loading instructions.

Fixes/Enhancements:

- In RTCM remote, the MSK setup menu will setup the Starlink Beacon receiver.  The setup will go out the RTCM port when the enter key is pushed, and the Status update rate message will be transmitted to the beacon receiver at the periodicity selected in the Status update rate.  Default is 000 in 1L00 so by default, nothing is transmitted and cause a transceiver to key.  NOTE:  This is the only change between 1K00 and 1L00.

Although not designed to setup the beacon transmitter, if the Z12 is set to Base and the MSK is set to on, the SPEED will effect the output of the Z12 like the SPD setting under RTCM Options. MSK should be set to OFF.

Comments and Known Problems:

- When maximum age is exceeded in the remote DGPS mode, the age of corrections jumps to 999.

- There is an intermittent problem loading waypoint files via HOSE. Sometimes the file is not accepted and sometimes the range and bearing are incorrect.

- In DGPS remote mode, the QAFREQ parameter on screen five does not provide an accurate radio link reliability. Reliability is better than depicted.

- Download MUST be done with RCS 3.2 or HOSE 5.4.0 to ensure antenna heights are recorded correctly.

Known Incompatibilities:

- Remote 5.0 is not compatible with the Z12. Version 5.5 can be downloaded from the FTP site.