Software Release Notice			Survey and Mapping

Date:		October 11, 2000

Product:  	Ashtech Solutions

Subject: 	Version 1.2 Software Release Notice

Number: 	Ashtech Solutions 2000-01

Introduction:

Ashtech Solutions, version 1.2, is a free upgrade to all registered Ashtech Solutions, version 1.0 users.  Users who have not sent in the warranty registration will not receive the automatic upgrade until Ashtech receives the warranty registration.   The upgrade CD will start shipping 10/12/00.  This upgrade is available by mail on CD only.

New Features:

1. GEOID99 support has been added.

2. Weighted adjustment capability has been added.  Use of calculated sigma values in addition to the a-priori values is now supported.  The selection between the two can be made in the Control Tab in the Site Properties Dialog.

3. The parameters for predefined coordinate systems have been updated.  Naming conventions and numeric input values have been normalized between the handheld software products and Solutions.  See item #7 in the General issues section below.

Known Issues:

Importing Files:

1. When adding raw data files to a project from a CD ROM, copy the files to the computer hard drive and then add them to the project.  Failure to use this procedure may generate a General Protection Fault.

2. When importing data contained on a PC card, transfer the data from the PC card using the Download module in Ashtech Solutions.  Failure to follow this procedure will require the use of Download to convert raw data into a usable format.

General:

1. When working with very large datasets on systems using Windows 95/98, display problems may occur in Map View and Time View.  The internal data is not corrupted. To remedy the problem, saving the project, exit Ashtech Solutions, and start again with the saved project.

2. In single frequency kinematic data-collection mode, it is very important to try and retain lock to all satellites throughout the survey.  In cases of loss of lock due to obstructions, it is possible that the accuracy of processed results will be degraded.

3. Blunders in the naming of sites, antenna heights can cause processing problems, which have effects throughout a project.

4. Currently the adjustment module is restricted to a maximum of 500 points.  If the project contains more than this number of points, it should split it into smaller projects and adjust each separately.

5. Ashtech Solutions requires carrier phase data in order to provide a post-processed solution.  It will not process data containing only code phase measurements.

6. A problem has been encountered on some computers when using the Ashtech Solutions hardware sentinel key in conjunction with other keys.  Keys connected after the Ashtech Solutions key will sometimes not be detected.

7. Pre-defined Datum transformation values and Grid Zone definitions are based on a variety of sources including government agencies such as NIMA, NGS, and the Ordnance Survey.  Users of these systems should verify that the values are consistent with their local sources and if not, consider defining a new system using the User defined Datum, User defined Ellipsoid, and User defined Grid System options that are available from the Coordinate System settings tab.

Installation:

- Obtain product ID from old CD or from the HELP pull down menu in current  Ashtech Solutions installation.  This is the eight-digit number before the hyphen.

- Uninstall current solutions using the uninstall feature.

- Install Ashtech Solutions version 1.2.

- Use same product ID number