PRODUCT RELEASE NOTES

# PRODUCT:  Ashtech Solutions

# Version:  2.50

# Date:  08/25/01

# Previous Version:  2.0, 2.4

##### Introduction

Ashtech Solutions, version 2.5, is an upgrade to all registered users of Ashtech Solutions 2.4*.  This upgrade will be available via a patch that can be downloaded from the Ashtech FTP site. The version replaces version 2.4 and will be shipped from the Ashtech factory with all new products effective from 8/25/01.  Registered Locus users who want dual-frequency capability will need to contact Ashtech sales and purchase an upgrade which requires a hardware key.

*Some restrictions may apply

New Features vs. Ashtech Solutions 2.0:

Capability of downloading and processing ProMark2 data was added.

Fixes vs. Ashtech Solutions 2.0 and LOCUS Processor

- Fixed error in version 2.0 computation of the elevation factor at the origin point of ground coordinate systems.  The elevation factor was calculated improperly when linear units other than meters were being used. Ground systems that were defined in version 2.0 projects must be redefined or the origin  point must be selected again if “US feet” or “International feet” were used as project linear units.

2. 	Fixed an error in version 2.0 computation whereby the Northings and

Eastings of a point in a ground  coordinate system were transposed. i.e. the

Northing value that was shown was actually the Easting value for the point.

- In version 2.0 implemented proper handling of receiver clock corrections in that may be inserted by CORS sites or other reference stations.

4.	For users upgrading from LOCUS Processor, any known problems indicated in previous LOCUS processor release notes can be considered repaired if these known problems are not listed in these Solutions 2.40 release notes.

Known Problems:

- The installation of Ashtech Solutions v2.4 over earlier versions of Ashtech Solutions will completely overwrite the coordinate system files. Preserve your coordinate system definitions by copying these files prior to installing Ashtech Solutions v2.4.  See installation notes in this document for instructions on how to manage coordinate system files through the new installation.

- On some Win32 platforms [Windows ME], Install Shield will begin to load, then disappear without continuing the installation.  This occurs if the user does not have write permissions on the local drive[s].

Workaround: Once permissions have been properly set, the software will install normally.

- When loading raw data files without D-files into Solutions 2.4, the software will try to automatically interpret segments of the data as either kinematic or static.  The segments will be displayed in the Time View Window for that data file.

Workaround: Use a D-file created by Locus handheld or Survey Control II software.  You can also edit the data manually by using the following procedure. Toggle the data in the segment to the desired type (kinematic or static) and rename the segment with the same Site ID as the segment immediately before it. Repeat this process for any other unintended segments that occur for that particular data file. Make sure each segment has the same Site ID and antenna height.  Then, process the data as you normally would.

- Because ProMark 2 does not use serial commands ($PASHQ, $PASHS) the receiver firmware version cannot be displayed in the RECEIVER functions in Download.

- For the same reasons as item 2 (above), Download cannot set the recording interval on ProMark 2.

Known Incompatibilities

1.	Solutions 2.x includes some of the same utility programs that are also a part of Ashtech RCS software.  When using both packages, use the utilities in Solutions when available.  Choose to exclude the RINEX converter and the receiver download modules when installing the RCS program.

General

- When working with very large datasets on systems using Windows 95/98, display problems may occur in Map View and Time View.  The internal data is not corrupted. To remedy the problem, save the project, exit Ashtech Solutions, and start again with the saved project.

- In single frequency kinematic data-collection mode, it is very important to retain continuous lock to at least 5 satellites throughout the survey.  In cases of loss of lock due to obstructions, it is possible that the accuracy of processed results will be degraded if re-initialization is not performed in the field. Therefore, re-initialization in the field after a loss of lock is critical to maintaining accuracy.

- Blunders in the naming of sites, antenna heights can cause processing problems, which have effects throughout a project.

- Ashtech Solutions requires carrier phase data in order to provide a post-processed solution.  It will not process data containing only code phase measurements.

- A problem has been encountered on some computers when using the Ashtech Solutions hardware sentinel key in conjunction with other keys.  Keys connected after the Ashtech Solutions key will sometimes not be detected.

- Pre-defined Datum transformation values and Grid Zone definitions are based on a variety of sources including government agencies such as NIMA, NGS, and the Ordnance Survey.  Users of these systems should verify that the values are consistent with their local sources and if not, consider defining a new system using the User defined Datum, User defined Ellipsoid, and User defined Grid System options that are available from the Coordinate System settings tab.

- When adjusting projects with a large number of vectors, there may be an extended period between selection of 'Adjustment' and the appearance of the 'Performing Adjustment...' dialog box.  Be patient.  The adjustment will execute.

- Download new and updated Sentinel Pro Plus drivers if you encounter problems accessing the hardware sentinel.  The sentinel drivers are available from Rainbow Technologies' download site at: http://www.rainbow.com/tech/sysdrivers.htm Click on the “Click here for current Sentinel system Drivers” link.

- Ashtech Solutions was not tested with Windows XP operating system.

Installation Notes for 2.5 patch :

Download patch from ftp://ftp.ashtech.com/software/AS2.5patch .  Doulble click the installer and install into the same directory that Ashtech Solutions is installed into.  The patch will not install into Solutions or Locus processor versions earlier than 2.4.

Installation Notes for 2.4 CD:

Ashtech Solutions v2.4 requires a new product ID if your present Ashtech Solutions version is less than 2.0.  The product ID from Solutions versions prior to v2.0 will not work.  The Product ID from LOCUS Processor will not work. The new product ID can be found on the label enclosed with the CD.  This is the eight-digit number before the hyphen.

The installation of Ashtech Solutions v2.4 over earlier versions of Ashtech Solutions will overwrite the coordinate system files and any updates or changes made to the coordinate system will be lost.  Preserve your coordinate system updates by copying these files prior to installing Ashtech Solutions v2.4.

- Open the Ashtech Solutions\Bin folder, select all *.DB files, and copy them to another folder.

- Install Ashtech Solutions v2.4 over the previous version.

3. Replace the *.DB files installed with version 2.4 with the files that you just saved..

Note that the updates to the pre-defined coordinates systems available in Ashtech Solutions v2.4 will not be present in your v1.2 based coordinate system files.

It is not recommended to perform this procedure when updating from Ashtech Solutions v1.0 to Ashtech Solutions v2.4 as the coordinate system file formats are different.

Installation Procedure:

- Install Ashtech Solutions version 2.4 by placing the CD in the CD ROM drive. The installation program should auto run and appear on your screen.

- Choose “Install Ashtech Solutions”. Follow all on-screen instructions.

- When prompted to overwrite the current version of Solutions, say yes. Users upgrading from Locus Processor will not be warned about overwriting, since the LOCUS program files are in a different directory path.  The old version of Locus Processor does not have to be removed.

- Use the product ID number enclosed with the CD.