Two modes of radio initialization are available.

1. If the radio has been programmed to accept a soft break sequence (BREAK=ON), FSRADIO will be able to initialize the radio without a radio modem power down sequence.

2. If the radio has not been programmed to accept a soft break sequence (BREAK=OFF), FSRADIO will NOT be able to initialize the radio without a radio modem power down sequence.

Run RFMCONF.EXE (supplied with the radio) and verify the setting.

***************************************

FSRADIO Instructions

***************************************

1. Type FSRADIO from the DOS prompt.

2. Select the GPS receiver port connected to the radio modem. 3. Press (P) to connect to the Pacific Crest radio modem.

************************************************************ Note: If the radio is not programmed to accept a soft break sequence (BREAK=OFF), cycle the radio power while FSRADIO is displaying the "Trying..." message.

************************************************************

FSRADIO will display

"Trying...

Reading. Please wait..."

FSRADIO displays two options:

(C) to change the settings

(Q) to return to the previous screen

5. Press (C) to call the CHANGE PARAMETERS SCREEN

The CHANGE PARAMETERS SCREEN displays five options: (S) Sensitivity (current setting)

(C) Channel (current setting)

(L) Link speed (current setting)

(A) Accept

(Q) Back

6. Press (S) to change the sensitivity

7. Press (H), (M), (L), or (O) to select the sensitivity.

8. Press (C) to change the channel

9. Press (0) - (15) to select the channel.

10. Press (L) to change the link speed.

11. Press (9) to select 9600 baud.

12. Press (A) to accept the current settings and program the radio.

FSRADIO will display

"Programming..."

13. Press (Q) until exited from the program.

14. Power cycle the radio modem.