NAV10.EXE - V1.0

To get started immediately please see GETTING STARTED at the end of the document.

INTRODUCTION

The NAV10.EXE software is designed to operate in conjunction with the AIV-10 - 10 channel OEM

receiver from Magellan Systems Corp.  The NAV10 allows monitoring of the input/output data and the

control of all AIV-10 functions.

SCREENS

There are six data screens in the NAV10 program.  Screens can be swapped in one of two ways.  The

F9 key will successively progress through the data screens, wrapping around to screen 1.  Hot keys,

numeric '1' through '6' will go directly to the selected screen, 1 through 6.

FUNCTION KEYS

The primary functions in each screen are provided through the function keys.  A short cryptic name

of the function they perform is displayed across the bottom of the display.  Some of the functions are

the same across all screens, some are different.  The functions that have TOGGLE capability indicate

their status on the top line in the screen.

COMMON FUNCTION KEYS

Common function keys across all screens are the F1, F9 and F10 keys.

F1/Ctrl

The F1 key provides the following menu choices:

1 - MESSAGES REQUEST

2 - INITIALIZE POS

3 - INITIALIZE TIME

4 - SEND ALTITUDE

5 - SEND SETUP

6 - COLD START

In order to fully understand the functions performed it is necessary to become familiar with the

Firmware exchange protocol used to communicate with the board.  More information on this can be

found in the Software Interface Specification for the AIV-10.

1 - MESSAGES REQUEST

This selection prompts for a INDEX and SUB-INDEX of the message being requested.  The INDEX =

81 through 86, and the subindex is specific to the message being requested.  If a request for almanac

(86 - 00) is made and the almanac is received the almanac message will automaticly be logged to a

file called '1032.alm'.  This file name can not be changed within the program.

2 - INITIALIZE POS

This uses the type 03 message to initialize position or position and altitude selecting, the initial position

and format from the NAV10 defaults set in screen 4.

3 - INITIALIZE TIME

This selection uses the internal computer time, adjusted by the local offset set in screen 4, to initialize

UTC time in the AIV-10.  The successful initialization of time depends on the PC vomputer time being

correctly set, and the correct UTC offset in screen 4.

4 - SEND ALTITUDE

Prompts the user for altitude and units then uses the type 05 message with a SUB-INDEX of 00 for

meters and a SUB-INDEX of 01 for feet to send altitude to the AIV-10.  For units a M(F) or m(f) can be

used, but not both (Mm Ff).  See the AIV-10 Software Interface Specification for conditions under which

the AIV-10 will accept altitude aiding.

5 - SEND SETUP

Uses the type 01 message to send setups to RAM or NVRAM.  Page 4 contains all setup data

necessary and can be modified to any users setup requirements.  The NAV10 uses the RAM/NVRAM

selection on screen 4 to determine whether the SUB-INDEX is 00 or 01.

6 - RESTART

The restart selection sends the type 06 data message to the receiver.  A sub-type of 00 causes a warm

start on the receiver.  A sub-type of 01 clears RAM and does a restart off of NVRAM values.  A sub-

type of 02 clears RAM and NVRAM and does a restart off of NVRAM defaults.

7 - COLD START

Sends the type 03, type 04 and type 01 message to the receiver with a single key stroke for quick

initialization.

F9/Swap

This selection allows paging successively through all data screens. After passing the last screen (6) it

wraps around to the first screen.

F10/Exit

Exits the program.  The program can be exited from all screens.  If file logs are open it will close them.

The command to exit will be ignored if the F2/Pause is active on screen 1 or screen 3.

SCREEN 1

Screen 1 is a general data display screen.  It displays most common information.  To adequately

function, messages 81, 82, 83 and 84 need to be turned on.  If only selected messages are turned on

(say just 81) then all other flags from the 82, 83 and 84 messages will be displayed as if they were set

to 0.  Messages and rates are commanded with the 01 (setup) message and the defaults are stored

in screen 4.

F2/Pause

The F2/Pause function key freezes the input data as displayed.  Data continues to be accumulated in

the receive buffer but is not processed until the pause key is toggled back.  The lower line of the

display shows only one key choice available, F2/Resume.  When normal operation is resumed the

program looks to see if the receive buffer is full.  If so, it dumps all of the receive buffer.  The receive

buffer holds 4096 bytes.

F3/Clr

The F3/Clr key clears all displayed data.

F5/Dsk

The F5/Data-Log key allows logging of selected ASCII data to disk.  The data to be logged is set in the

CONFIGURE DATA LOG screen (6).  The log name is set in the SETUP VALUES screen (4) and can

be changed if desired.  If the file exists the user will be promped to overwrite, append or ESC to abort.

If the system experiences a disk error either by attempting to open a log or during logging, the NAV10

will close the log and report a DISK ERROR.

F6/Raw-Log

The F6/Raw-Log key allows logging of selected binary messagrs to disk.  The raw messages to be

logged are set in the CONFIGURE DATA LOG screen (6).  The log name is set in the SETUP VALUES

screen (4) and can be changed if desired.  If the file exists the user will be promped to overwrite,

append or ESC to abort.  If the system experiences a disk error either by attempting to open a log or

during logging, the NAV10 will close the log and report a DISK ERROR.

F7/Cfg

The F7/Cfg menu offers the following choices.

1 - COMM PORT 1

2 - COMM PORT 2

3 -    COLOR

4 - MONOCHROME

5 - SAVE CONFG

6 - BAUD RATE

7 -    EXIT

When a menu change is made it is reflected in this menu immediately but not in the program until the

menu is exited normally.  This means selecting '7' or CR on selection '7'.  ESC aborts all changes.  If

the changes are not saved then they will only be active until the program is exited.  If the changes are

saved and the menu is left by ESC, the changes will become the new default but will not become active

in the current program.  The color/monochrome selection is not true monochrome.  This program is

used extensively on laptop computers which usually use CGA or VGA emulation of monochrome.  To

avoid washouts of the pale colors, the monochrome mode is actually color using only black and white.

This gives the most readable displays on laptops with poor backlight.  Although the AIV-10 firmware

protocol is specified with a fixed baud rate of 9600 the NAV10 allows it's baud rate to be changed from

1200, 2400, 4800 and 9600.

SCREEN 2

Screen 2 displays valid almanac data, satellite visibility schedule and healthy satellites.  The the almanac

data comes from the 1032.alm file that is created when a message request for almanac (86-00) is

made.  The satellite visibility schedule is generated from the same 1032.alm file, all calculations being

done within the NAV10 PC software.  The healthy satellite information is extracted from the type 83 data

message.  If the 1032.alm is available in the current directory it is read into the NAV10 on start up.

F2/Page

Successive pressing of the F2 key allows advancing through all valid almanac pages, wrapping around

to the first valid page when the last is reached.

F3/Satsched

This key clears the satellite visibiliity window and computes a new table based on the local date (from

the PC) with the mask angle that has been set in screen 4.

F8/Disk

The F8 key writes a listing of all valid almanac pages to disk. They appear in the same format as the

single almanac page on screen 2.  The default name of almanac.dat is used and cannot be changed.

SCREEN 3

Screen 3 is a full screen display of the incoming serial data.  Each two byte value is the hexidecmal

ASCII representation of each binary data byte.  If the data string is to long for a single line display it

is wrapped around to the next line.  With the exception of the F7/Cfg, all function keys that operate on

screen 1 also operate on screen 3.

SCREEN 4

Screen four contains all of the NAV10 defaults.  These values are used for the 01, 02 and 04

initialization messages.  In addition the names of the raw log and data log are entered on screen 4.

The defaults can be changed, saved to disk and retrieved for later use.  The file has a default name

of setup.dat and cannot be changed.  If the setup.dat file is in the current directory it is read into the

NAV10 on start up.  If you enter L/L position in the SETUP VALUES it is automatically converted to

ECEF and UTM and displayed.  Conversion does not take place from UTM or ECEF to L/L.

The top and bottom lines in screens 4, 5 and 6 give real time information on the input and output

messages.  The top line tracks all of the 8X-XX data messages and the bottom line tracks the 0X-XX

data messages.  If a message is sent from the AIV-10 it is posted as 'S' for sent.  When it is received

by the AIV-10 and echoed back the 'S' is replaced by an 'R'.  If a message 02 is sent requesting a 8X-

XX message the 0200 is posted on the bottom line and the message requested (field seven and eight

of the 02 message) is posted on the top line as 'S' to indicate which 8X-XX message has been

requested and then replaced with an 'R' when received.

F2/Edit

Pressing F2 begins editing the default values.  The NAV10 expects that numeric entries be entered in

full.  Arrow keys can not be used to pass over existing data.  The up-down arrows move through the

data fields one at a time.  The right-left arrow keys move to the top of the next colum.  Entered data

is not accepted as valid unless the ENTER (RETURN) key is pressed.  Flag data is toggled with the

page-up/page-down key.  The ENTER key need not be pressed to accept toggled data.  The ESC key

ends the edit mode.  If the new values are not saved the disk defaults are read in the next time the

program starts.

F3/Save

The setups are saved in the file setup.dat and the file name can not be changed.

F4/Retrieve

Reads in the setup.dat log.

F8/Advance

The F8 key is used to change the log file names without actually having to edit the setup screen.  When

F8 is pressed NAV10 looks at the log names that are entered in screen 4.  If the log has a name of

raw.log or rawX.log, and oem.log or oemX.log (X = 1 through 99), it then looks at the state of the log

(open or closed).  If the log is open it closes it, advances the log name (oem.log becomes oem1.log

or oem77.log becomes oem78.log, etc.) and opens it again.  No data is lost.  This occurs for both oem

and raw.  If the log is closed the log name on the screen is advanced one and this is all that occurs.

If the setups are saved the current name is saved as default.  If one log is open and the other closed

both names are still advanced and the log that was open is re-opened.

SCREEN 5

Screen 5 displays the default RAM and NVRAM setup values.  When screen 5 is entered NAV10 asks

for a 85-00 and 85-01 message and the values are displayed.

F2/Update

The F2 key clears both displays and requests the 85-00 and 85-01 message.

SCREEN 6

Screen 6 controls the content of the raw and oem logs.  There are thirty individual types of ASCII data

that can be logged (Data-Log) and six raw data messages (Raw-Log) that can be logged.  The ASCII

data is all individually identified and the raw data is the 8X-XX series data messages as they are

received.  Each time the data is writen to disk the logging function looks to see which data is 'marked'

to log and it logs that data only.  The rate at which the ASCII data is logged is the rate of the most

frequent data message.  For example, if the 81-XX message is turned on every 5 seconds and all other

messages at 10 seconds, all marked data is logged every 5 seconds.  The raw messages are logged

they are received.

F2/Edit

The F2 key allows editing of the data to log.  The up-down arrow keys move through the fields, the

right-left arrow keys move directly across colums.  The page-up/page-down keys toggle ON/OFF of

the selected data.  If the changes are not saved they are aborted when the program is exited.

F3/Save

The logs setup is saved in the file called logs.dat and the logs.dat file name can not be changed.  The

defaults in the logs.dat file are read in as defaults on next NAV10 start up.

F4/Retreive

Reads in the logs.dat and conditions the data selections accordingly.

GETTING STARTED

When you first start up the NAV10 program, there are several things you should do to prepare it for

your operating environment.

1.   Set up your system configuration using screen 1, F7.

2.   Create a new 1032.alm file by pressing F1, ENTER (RETURN), 86, 00.  This should not

be done, however, until the almanac age on screen 1 is = 0, indicating that the receiver

has a current almanac.

NOTE:     When the receiver is powered on but does not have internal time (cold power up)

the almanac age will be displayed as = 0.  It does have an almanac in ROM (or

E-Squared ROM) that can be numbers of weeks old but until it has time it does

not know how old the almanac is.  After a satellite is tracked or time is initialized

with the 04 data message then the accurate almanac age will be output.

3.   Enter your defaults on screen 4 such as L/L (ECEF, UTM) and all other receiver and

program setup parameters and save your setups.

4.   Configure your data and raw logs and save your log setups.

5.   F1, selection 6 should initialize the AIV-10.

ASSOCIATED FILES

NAV10.EXE       Program file

NAV10.CFG       Configuration file - created/modified by F7/Cfg

SETUPS.DAT     File storing the setups from screen 4

LOGS.DAT       File storing setups for the log selection on screen 6

ALMANAC.DAT    Output of the F8 key on screen 2

1032.ALM       Almanac file on disk.  Exactly the same as the 86-00 message