| Writer : | Isabelle Tocquer |
| --- | --- |
| Date creation : | 2002-02-11 |
| Date modification: |  |
| Number of pages : | 22 |
| Version | 3.0 |

| Subject : | Explanation of the R&TTE directive |
| --- | --- |

The purpose of this document is to explain why this directive was established and what a GSM manufacturer must do to be in accordance with this directive.

# Aim of the R&TTE

The R&TTE directive was created by the European Community.

This Directive establishes a regulatory framework for the placing on the market, free movement and putting into service in the Community of Radio Equipment and Telecommunications Terminal Equipment (R&TTE).

With this directive, the control before selling the product is reduced as before with the old TTE directive and less tests are mandatory to do before selling a product.

Nevertheless, the use and the installation of radio equipments must still be in accordance with the radio electric rules of each European countries.

A product sold in the EC (European Community) must be in accordance with 3 requirements :

- The protection of the health and the safety of the user (paragraph 3.1.a of the directive 1999/5/CE)

- The protection requirements with respect to electromagnetic compatibility (paragraph 3.1.b of the directive 1999/5/CE)

- The efficient use of the spectrum allocated to terrestrial/space radio communication and orbital resources so as to avoid harmful interference (paragraph 3.2 of the directive 1999/5/CE)

Summary :

- Directive for the radio and telecommunications equipments sold in the European Community

- Reduce of the control on the products

- Mandatory since the 8th of April 2001

- Harmonisation of the procedures in Europe

- Elimination of all national approval systems

# Steps to follow

Before selling a product in Europe, these questions must be asked and solved.

- Which directives are applicable to my product?

  - R&TTE

  - Environment

  - Automotive

- What are the harmonised standards applicable? (cf. Official journal of the European Community)

- Are the frequencies of my product harmonised ? In GSM Yes. That means that specifications exist to define the use of this way of communicate by using the 900 and 1800MHz frequencies.

- Is the radio interface harmonised? In Europe, for the GSM yes. That means that the GSM/DCS frequency bands allocated to each customer by a EC member are identical in all the European Community (EC).

- Preparation of the user manual :  see the description of the annex chosen

  - Intended use

  - Using countries

  - DoC (Declaration of Conformity)

- Which label uses? CE or CE + N°NB ?  see the choice of the annex

- Do the tests to demonstrate that the product is in conformity with the essential requirements defined in the directive.  see the applicable standards

- Use a NB if the standards are not harmonised.

- Do the notification of the product either by using a NB or directly by sending a letter to each government of each country where the product will be sold.  For GSM products, no notification process is requested. Collect each documents defined in the Annex III into a specific file.

These steps summarize the conformity assessment process before selling a product in Europe. The next paragraph will describe the process for a GSM application.

# Conformity assessment procedure for a GSM product

## Choice of the annex

The CE label will change depending on the evaluation procedure chosen by the manufacturer.

There are 4 evaluation procedures, which are described is the article 10 of the directive.

Here, we will only discuss about the one related to the GSM products.

Text of the Directive

- Article 10 (4): Where a manufacturer has applied the harmonised standards referred to in Article 5 (1), radio equipment not within the scope of paragraph 3 shall be subject to the procedures described in any one of Annexes III, IV or V at the choice of the manufacturer.

- Article 5 (1): Where apparatus meets the relevant harmonised standards or parts thereof whose reference numbers have been published in the Official Journal of the European Communities, Member States shall presume compliance with those of the essential requirements referred to in Article 3 as are covered by the said harmonised standards or parts thereof.

Explanation: If a manufacturer has applied the harmonised standards published in the Official Journal of the EC, its radio equipment if it is using the spectrum allocated to terrestrial/space radio communications, shall be subject to the procedures described in any one of Annexes III, IV or V at the choice of the manufacturer.

In case of GSM equipments, the standards used to check that a product is in accordance with the essential requirements defined in the directive, are harmonised.

Then, the applicable annex for the certification process will be the Annex III, the Annex IV or the Annex V.

For GSM products only, I recommend to use the annex III which is easier and faster to implement.

## Tests part

To know which standards apply to the GSM ME, please referred to the Annex B.

In all the cases, radio, EMC, safety and SAR tests must be performed on the ME and passed to be allowed to be sold in the EC.

All these tests must be performed in an accredited laboratory.

## Administrative part

### TCF

If a manufacturer wants to sell a GSM product in the EC, he must carry out these documents in a file called TCF (Technical Construction File):

- Electrical schematics of the ME

- PCB layout of the ME

- BOM of the ME

- Mechanicals schematics of the ME

- Specifications of the ME

- User manual of the ME

- Tests reports performed on the ME (harmonised standards)

- Declaration of conformity (DoC) of the ME. This declaration is made by the manufacturer. (cf. annex E for an example of the DoC)

(Cf. Annex III for more information).

The manufacturer or his authorised representative established within the Community or the person responsible for placing the apparatus on the market must keep the file for a period ending at least 10 years after the last apparatus has been manufactured at the disposal of the relevant national authorities of any Member States for inspection.

The European governments can ask to have access to the TCF in case of inspection after a complaint.

### Notification

The manufacturer doesn’t have to use a NB to perform the notification procedure of his product.

Notification procedure: A manufacturer must notify a Member State that an apparatus will be sold in his country, only if the frequency bands are not harmonised. This is not the case for GSM or DCS apparatus. So a manufacturer is not obliged to notify a Member State that his GSM terminal equipment will be placed on its national market.

### IMEI number

To have an IMEI number, a manufacturer can use:

- either a NB and pay for that

- or via BABT and it is free

Please find below some NB’s contacts:

Dieter Griep

Electronic Technology Systems Dr Genz GMBH (also called ETS)

Storkower strasse, 38c

15526 Reichenwalde

Germany

Direct line: +49 33 631 888 22

Direct fax: +49 33 631 888 72

E-mail: griep@ets-bzt.com

IMEI Co-ordinator

BABT

Claremont House

34 Molesey Road

Walton-on Thames

Surrey KT12 4RQ

United Kingdom

Tel: +44 1932 251251

Fax: +44 1932 251252

E-mail:	imei@babt.com

To ask an IMEI number, a document has to be sent to one of this contact. This document is called :

- IMEI request form for ETS (please contact ITO for that)

- Request for allocation of IMEI for use on GSM 900/1800 networks for BABT + a copy of the DoC of the application  (cf. IMEI BABT.doc file)

### Certification

With the R&TTE directive no certificate is issued. The “certificate” is now the DoC and the EC label on the apparatus.

This DoC must be accessible to everyone, and then must be :

- in the user manual

- or on the website of the manufacturer

Advise : keep 10 original copies of the DoC in the TCF, in case of demand from an operator, a customer or the EC relevant authorities.

Remark : in the case of using the annex IV for the certification process, the certificate issued by the NB will say that the TCF is complete and it will list the pieces of the TCF presented. This NB’s certificate is not the R&TTE Certificate, it is only an expert opinion on the documents of the TCF.

With the R&TTE directive, the manufacturer is the only responsible of the apparatus placed on the EC market.

### EC label

Which EC label is applicable?

- For telecommunication terminal equipments not containing any radio transmitter and for radio receivers

| Conformity assessment | Label |
| --- | --- |
| Annexe II (Internal control of the manufacture) |  |
| Annexe IV (TCF) or Annexe V (Full assurance quality) |  |

- For radio transmitters (i.e. for GSM products)

| Conformity assessment | Use of the frequency band |  |
| --- | --- | --- |
|  | Is harmonised | Is not harmonised |
| Annexe III : 2 cases | Label | Label |
| i. radio tests described (or defined) in the harmonised standards (for products with a Wavecom module) |  |  |
| ii. Radio tests not described (or non defined) in the harmonised standards |  |  |
| Annexe IV (TCF) or Annexe V (Full assurance quality) |  |  |

ON = Organisme notifié = Notified body = NB.

# The future

The harmonised standards applicable to apparatus always change. These changes are updated when the Official Journal concerning the Directive is published.

Many websites talk about the R&TTE directive. These are the most important:

http://europa.eu.int/comm/enterprise/rtte/harstand.htm

http://www.rtte.net

# Conclusion

This directive is easier to understand than the former called TTE directive. It is enhanced the certification of the product by the manufacturer himself, i.e. no need to use a NB to certify a ME.

Annexe A: Description of the annexes

Annexe II (Basis)

This annex is used for :

- radio equipment whose frequencies used to communicate are not harmonised

- for telecommunication terminal equipments which are not containing any radio transmitter

- radio receivers

Process:

- The manufacturer must establish the technical documentation described at point 2 and he or his authorised representative established within the Community must keep it for a period ending at least 10 years after the last product has been manufactured at the disposal of the relevant national authorities of any Member State for inspection purposes.

- The technical documentation is :

- a general description of the product,

- the conceptual design and manufacturing drawings and schemes of components, sub-assemblies, circuits, etc….

- the descriptions and explanations necessary for the understanding of said drawings and schemes and the operation of the product,

- the explanations to know how the product operates,

- a list of the standards referred to in Article 5, applied in full or in part, and descriptions and explanations of the solutions adopted to meet the essential requirements of the Directive where such standards referred to in Article 5 have not been applied or do not exist,

- the results of design calculations made, examinations carried out, etc.,

- The test reports

- The tests could be performed in an accredited laboratory or by the manufacturer. The tests could be chosen by the accredited laboratory or by the manufacturer depending of the frequency used and based on the experience of each parts.

- The manufacturer or his authorized representative must keep a copy of the declaration of conformity (DoC) with the technical documentation.

- The manufacturer must take all measures necessary in order that the manufacturing process ensures compliance of the manufactured products with the technical documentation referred to in point 2 and with the requirements of this Directive that apply to them.

- The manufacturer must then affix on his product the label CE alone.

Annexe III (Mandatory for GSM products)

This annex is used when:

- the ME are radio transmitters

- the frequencies used by a product are harmonised

- the applicable standards are harmonised.

To summarize, this certification procedure is based on an internal control of the manufacturing of the product and on the achievement of specific radio tests on the product.

The difference between the annexes II and III is that the manufacturer has to do all the essentials radio tests applicable to his ME because the frequencies bands and the applicable standards are harmonised.

Process :

- The manufacturer must establish the technical documentation described at point 2 and he or his authorised representative established within the Community must keep it for a period ending at least 10 years after the last product has been manufactured at the disposal of the relevant national authorities of any Member State for inspection purposes.

- Technical documentation :

- A general description of the product,

- The user manual of the ME

- The conceptual design and manufacturing drawings and schemes of components, sub-assemblies, circuits, etc…. (Electrical schematics, Mechanicals schematics, PCB layout and BOM of the ME)

- The descriptions and explanations necessary for the understanding of said drawings and schemes and the operation of the product,

- The specifications of the ME

- The explanations to know how the product operates,

- A list of the standards referred to in Article 5, applied in full or in part, and descriptions and explanations of the solutions adopted to meet the essential requirements of the Directive where such standards referred to in Article 5 have not been applied or do not exist,

- The results of design calculations made, examinations carried out, etc.,

- The test reports performed

- The Declaration of conformity (DoC) of the ME

- All essential radio test suites must be carried out by the manufacturer in an accredited laboratory

- To know the essential radio tests, the manufacturer has two choices :

    - There are defined in harmonised standards

    - If the harmonised standard used does not give a definitive answer about the essential radio tests, a notified body has to be called in to determine them. The manufacturer uses a notified body (NB) to know which tests have to be performed.

- The manufacturer or his authorised representative established within the Community or the person responsible for placing the apparatus on the market must declare that these tests have been carried out and that the apparatus complies with the essential requirements. This declaration made by the manufacturer is called the Declaration of Conformity (DoC).

- The manufacturer must then affix on his product the label CE alone if he doesn’t use a NB during all the process. Otherwise, he must affix the label CE + the NB’s EC identifier.

Annexe IV (optional for GSM products)

We used this annexe when:

- the ME are telecommunication terminal equipments not containing any radio transmitter or radio receivers or radio transmitters

- the standards applicable to a product are not harmonised

- the frequencies bands are harmonised

- a manufacturer need an external approval of the tests performed on his application and of the TCF

The differences between this annex and the annex III for the GSM products is that all the TCF are presented to a NB for a review to be sure that no tests or documents are missing.

Using this annex is useful when a manufacturer has modified something in his product and need the approval of the NB on the tests he has performed to validate these changes.

In that case, the manufacturer can use a NB who will define the tests to do on the ME to check that it is compliant with the essential requirements of the Directive.

Process :

- The manufacturer must establish the technical documentation described at point 2 and he or his authorised representative established within the Community must keep it for a period ending at least 10 years after the last product has been manufactured at the disposal of the relevant national authorities of any Member State for inspection purposes.

- Technical file called Technical construction File (TCF) made of :

- A general description of the product,

- The user manual of the ME

- The conceptual design and manufacturing drawings and schemes of components, sub-assemblies, circuits, etc…. (Electrical schematics, Mechanicals schematics, PCB layout and BOM of the ME)

- The descriptions and explanations necessary for the understanding of said drawings and schemes and the operation of the product,

- The specifications of the ME

- The explanations to know how the product operates,

- A list of the standards referred to in Article 5, applied in full or in part, and descriptions and explanations of the solutions adopted to meet the essential requirements of the Directive where such standards referred to in Article 5 have not been applied or do not exist,

- The results of design calculations made, examinations carried out, etc.,

- The test reports performed

- The Declaration of conformity (DoC) of the ME

- The manufacturer, his authorised representative established within the Community or the person responsible for placing the apparatus on the market, must present the file to one or more notified bodies, each of the notified bodies must be informed of others who have received the file.

- The notified body must review the file and if it is considered that it has not been properly demonstrated that the requirements of the Directive have been met, the notified body may issue an opinion to the manufacturer, his representative or the person responsible for placing the apparatus on the market and must inform the other notified bodies who have received the file accordingly.

- Such an opinion must be given within four weeks of receipt of the file by the notified body. On receipt of this opinion, or after the end of the four-week period, the apparatus may be placed on the market, without prejudice to Articles 6(4) and 9(5).

- The manufacturer must then affix on his product the label CE + the NB’s EC identifier.

Annexe V

We used this annex for all the ME (telecommunication terminal equipments not containing any radio transmitter or radio receivers or radio transmitters).

This certification procedure is also called Full Quality Assurance System.

This process is very restricting for the manufacturer.

To use this procedure:

- The manufacturer must install an approved quality system for design, manufacture and final product inspection and testing in accordance to the criteria defined  in the directive in annexe V paragraph 3.

- This quality system is subject to a EC surveillance under the responsibility of a NB.

The EC mark will be : CE + NB’ EC identifier.

Annexe B: Essentials requirements for a GSM product

A product must be in accordance with 3 requirements :

- The protection of the health and the safety of the user (paragraph 3.1.a of the directive 1999/5/CE)

- The protection requirements with respect to electromagnetic compatibility (paragraph 3.1.b of the directive 1999/5/CE)

- The efficient use of the spectrum allocated to terrestrial/space radio communication and orbital resources so as to avoid harmful interference (paragraph 3.2 of the directive 1999/5/CE)

  - Protection of the health and the safety of the user

For the safety of the user, the applicable standard for the GSM products is in the Official Journal of the European Community. This is the EN60950:2000 (safety of information technology equipment).

The applicable standard for the protection of the health of the user is the EN 50360:2001 (product standard to demonstrate the compliance of mobile phones with the basic restrictions related to human exposure to electromagnetic fields (300MHz – 3GHz)). This standard is mandatory since the 26th of July 2001. The methods used for the measurements are explained in the standard EN 50361 (Basic standard for the measurement of Specific Absorption Rate (SAR) related to human exposure to electromagnetic fields from mobiles phones (300MHz – 3GHz)).

  - Electromagnetic Compatibility

The applicable standards for the GSM products are EN 301 489-1 v1.2.1 (07-2000) and the EN 301 489-7 (09-2000).

The previous standard was ETS 300 342-1 which is still validated until the 31srt of March  2002.

  - Efficiently use of the spectrum

The applicable standards since the 26th of July 2001 are the EN 301 419-1 v4.0.1 (12-1999) and the EN 301 511 v7.0.1 (12-2000) for the GSM products.

There are around 518 tests :

- radio tests in conducted and radiated mode

- SIM tests

- Audio tests

- MMI tests (SMS and AOC tests)

- Protocol tests

- Resource radio tests

- Mobility management tests

- Call control tests

Annexe C : Example with the Wismo2D

Description of the apparatus : Reference design

The application validated will be:

- A Wismo 2D E-GSM/DCS on a starter kit V2.x

- A dual band antenna

- A handset

A document called “Reference design for the approval of the Wismo 2D” gives a better description and the schematics of the reference design.

Tests done

In Europe the applicable directive is the R&TTE for telecommunication terminal equipments. Wavecom will do the harmonised standards applicable for this kind of GSM equipment in Europe, and will also prepare further certifications in countries outside the European market.

That’s why all the tests of the GCF and of the R&TTE will be performed on the module.

To begin, only the GSM part of the 2D will be tested, and after the GPRS part.

EMC and Safety tests are planned to be done on the module for the EC mark.

The R&TTE certification and the TCF

All the documents for the TCF, regarding the 2D will be kept at Wavecom, and everyone at Wavecom can have access to it.

This TCF will change when anything regarding the module will change. This TCF will contain all the hardware or software evolutions on the 2D, all the internal test documents made on it which prove that no more tests are necessary to do in an accredited laboratory to validate our changes.

For the certification of the 2D, Wavecom will choose the Annex III. With this annex, there is no need to use a NB, and then the schematics of the 2D will remain confidential.

The Wismo2D will be CE marked.

For next customer’s applications

All the further applications which will used a 2D module in GSM/DCS bands, don’t need to perform a lot of tests to be EC marked.

The basis design is the reference design validated during the FTA of the 2D.

Each modifications must be validated by tests in an accredited laboratory.

How to know which tests must be done?

- If the antenna has changed, the radiated tests has to be retested. Nevertheless, we recommend to the customer to retest radiated spurious emissions tests in order to be sure that the components around the module are OK.

- If the application has its own MMI, all the MMI tests has to be retested (FDN, STK, SMS, AOC).

- When designing the power supply circuit, the manufacturer must be careful with the hardware specifications of the 2D. If the power supply of the application delivers a Vbatt signal which is compliant with our specifications, no radio tests have to be performed again.

- If the SIM connection is different from the one on the reference design, electrical SIM tests have to be done.

- If the audio circuit and/or the handset have changed, audio tests have to be retested.

All the difference between the reference design and the final application have to be carefully studied.

Impact of the 2D evolutions

All the modifications of the 2D and the reasons are in the TCF.

If the hardware or software modifications are too much, GSM or GPRS tests have to be chosen and made again in an accredited laboratory.

All the tests due to hardware or software evolutions only have to be performed on the reference design. No customers’ products would be concerned by that.

All the 2D modules sold to a customer would be approved (internal or external tests).

Annexe D: example of the 2C-2 module

Description of the apparatus: reference design

The 2C-2 module E-GSM/DCS was shown as an evolution of the 2C module E-GSM/DCS.

The 2C-2 module, BOM 400 version 4.0, was tested in an integra modem WMOi3-2 (reference design for the approval of the 2C-2), whereas the 2C module, BOM 300 version 3.0 was tested in an external modem WMOD2B (this is the reference design for the approval of the 2C).

Tests performed

On the 2C V3.0, all the tests of the GSM 11.10 standard (version GT01 V4.2.1) were performed.

On the 2C-2 V4.0, only a FTA stage 2 was performed.

The R&TTE directive and the TCF

We have carried out all the documents for the TCF of the 2C-2 module E-GSM/DCS.

This TCF will be available in: G:\projet\fta\fta\Certification GSM et CE\WAVECOM\Infos evolution 2C et 2C2.

For customers’ applications

It is very difficult for the moment to apply completely the directive for customers’ products, because the TCF for the 2C-2 is not clear and because we don’t prove that the module is compliant with the EN 301 419-1 standard. Indeed, all the evolutions of the 2C and the 2C-2 module has to be check in order to be sure that all GSM tests have been performed to validate the evolutions.

When the TCF will be complete, we have to check if all the tests have been performed on the 2C-2 reference design to validate all the hardware and software evolutions.

If not, we have to see if we want to update our TCF and be compliant with the European laws.

If many tests are missing, the customer can integrate the 2C-2 module in its application, and in case of control, the responsible person will be Wavecom.

Until the TCF of the 2C and 2C-2 module is not complete, all the customers’ applications will be tested at ETS, and the certification will be in accordance to the annex IV. This will avoid any problems for the customer, because no EC declaration of conformity for the 2C-2 module is given to him.

Evolution of the 2C-2

Between the 2C V3.0 and the 2C-2 V4.0, many major hardware and software modifications were done, and only FTA stage 2 were performed to validate these evolutions.

These evolutions are available in this document: G:\projet\fta\fta\Certification GSM et CE\WAVECOM\Infos evolution 2C et 2C2

The 2C-2 module is nearly at the end of its life, then no major investment will be done on it to update the TCF with additional tests.

Annexe E: Declaration of Conformity

Please have a look on this enclosed document.

Each manufacturer must put on this document the stamp of the company before distributing it.

Annex F: Countries where the Directive is applicable

European Community :

- Belgium

- Denmark

- France

- Greece

- United Kingdom

- Spain

- Portugal

- Italy

- Germany

- Ireland

- Luxembourg

- Netherlands

- Austria

- Finland

- Sweden

Other European countries:

- Switzerland

- Norway

- Iceland

- Hungary

- Czech Republic

- Slovenia

- Estonia

- Latvia

- Slovakia

- Malta

- Bulgaria

Annex G: External Europe

America :

- USA : RF and EMC : FCC part 15 (emission only). European standards are a good basis. ES : contact UL or MET Lab (2 laboratories) on a voluntary basis

- Canada: RF and EMC : comparable to the US. ES : CSA requirements

- Mexico : RF and EMC : FCC part 15 is a good basis, but still particular requirements. ES : ?

- Other countries: RF and EMC : European standards are a good basis, but still complementary testing. ES: ?

Africa:

- South Africa: RF and EMC : tendency to align on European Standards

- Israel Jordan : RF : tendency to align on European Standards

- Others countries : Wet finger and contacts persons – projects

Asia:

- Hong-Kong: is aligned to Europe

- Japan:

  - RF : a complicated matter (specifications are under development), but more and more based on European standards.

  - EMC : a great part of European standards are ‘acceptable’

  - ES : ?

- Korea:

  - RF and EMC : take the way of the European standards

  - ES: ?

- Taiwan:

  - RF and EMC : good consideration for European standards, but strange EMC legislation

  - ES : ?

- China : MII is applicable (a part of EMC, RF, SAR and ES European standards)

- Singapore:

  - RF and EMC : European standards are a good basis, but still particular test requirements

  - ES: ?

- Others : Wet finger and contacts persons – projects

Australia and New-Zealand:

- RF and EMC : European standards are applicable

- ES: ?