Understand the GCF (Global Certification Forum)

V2.0

2002-02-14

Contents

1	Aim of the GCF	4

2	Structure of the GCF	4

2.1	Members	4

2.2	Observers	4

2.3	Groups	5

2.3.1	The Steering Group	5

2.3.2	The Secretariat	5

2.3.3	The Working group	5

3	Structure of the GCF documents	7

3.1	Documents	7

3.2	Declarations	7

4	How to be GCF certified ?	8

5	GCF certification process	9

5.1	Become a member of the GCF	9

5.2	Quality qualified insurance	9

5.3	Compliance folder	9

5.4	Declarations	10

5.4.1	Quality qualify declaration	10

5.4.2	Assessment compliance declaration	10

5.4.3	Certification declaration	11

5.5	Certification	11

6	Modifications on the certified terminal	12

7	Conclusion	12

8	Annexe A : Conformity criteria of the GCF	13

8.1	Introduction	13

8.2	Definition of the applicable tests	13

8.2.1	Option table	13

8.2.2	Requirement table	13

8.2.3	Conformance assessment table	13

8.2.3.1	Full status	14

8.2.3.2	Provisional status	14

8.2.3.3	Other status	14

8.2.3.4	Example of conformance assessment table	15

9	Annexe B : Field trials	16

9.1	Description	16

9.2	Process	16

9.3	The field trial networks	16

9.4	Criteria to test	17

9.5	Compliance folder	18

9.6	New configuration of the terminal	18

9.7	The future	18

10	Annexe C : Quality Qualified Declaration	19

11	Annexe D : Assessment Compliant Declaration	20

12	Annexe E : Certification Declaration	21

13	Annex F : Additional information accompanying the Certification Declaration	22

14	Annexe G : Regional Regulatory Requirements	24

# Aim of the GCF

GCF = Global Certification Forum.

The GCF is a new certification process of GSM, GPRS and 3G mobile equipments (ME). This process is voluntary.

This new certification forum was developed in co-operation and through partnership between the GSM Association representing the GSM operator community and terminal manufacturers to ensure the interoperability of GSM terminals globally.

The Global Certification Forum provides for the verification of devices against the Certification Forum’s Technical requirements, with global recognition and acceptance of results, the avoidance of multiple testing and cost efficiencies.

The program includes all GSM frequency bands and multi-mode devices. It shall be complementary to all regulatory regimes with participation on a voluntary basis.

The Certification Forum and testing shall not cover commercial or quality aspects of a terminal and any such testing shall remain a part of the commercial agreement between the customer and the manufacturer.

# Structure of the GCF

## Members

The members of the GCF are voluntary and they are composed of 2 groups :

- The manufacturers of GSM terminal devices (like Nokia, Ericsson, Motorola …)

- The GSM network operators (D1, Orange, Vodafone ) who are members of the GSM Association

## Observers

Among the GCF, there are also the observers who don’t have the right to participate in voting. These observers are the notified bodies (like CETECOM, ETS, 7layers, BABT…) and others entities which have an active part into the design, development, manufacture and the test of GSM mobiles. The observers are neither operators nor manufacturers.

## Groups

### The Steering Group

All members of the Certification Forum are members of the Steering Group which is the decision making body of the Certification Forum. The duties of the Steering Group include:

- Financial, budget matters,

- Membership and observer matters

- All policy matters

- Approval of the technical requirements, criteria and tests

- Appointment and supervision of Secretariat

- Appointment of Working Groups for designated tasks

- All other tasks in the GCF-PD (Principles Document) designated to the Steering Group.

The Steering Group shall meet twice per year. Other Steering Group meetings may be called by the Chair or Vice Chair on an ad hoc basis. Invitations to all Members shall be sent by the Chair in writing at least two weeks in advance, referencing all decisions to be taken in the meeting.

### The Secretariat

The duties of the Secretariat shall include:

- Issuing instructions (within the scope of the GCF) to the Working Groups based upon policy set forth by the Steering Group

- Setting up and maintaining the Certification Web Site and links with the GSM Association and Declared Participants

- Managing the day-to-day operation and activities of the Certification Forum under the direction of the Chair / Vice Chair and in compliance with this Principles Document.

- Providing such assistance as is required by the Steering Group

### The Working group

Only one group exists for the moment : this is the Agreement Group.

The Agreement Group is opened to all Operator Members, Manufacturer Members and Observers who wish to participate.

The duties of the agreement group are:

- Development and maintenance of criteria and tests for existing and new services and features for GSM mobile stations (all GSM bands and the GSM part of multimode/multiband MS)

- Maintenance of relevant GCF documentation

- Liaison to other relevant organizations.

- Conduction Status Change review (SCR) on tests indicated as “validation completed” and “Commercially available” which is going to be proposed to move from Provisional to Full status indicated.

The Agreement Group shall meet as often as necessary to fulfil its duties. It is recommended that as many as possible of the routine activities of the Agreement Group are carried out by correspondence.

# Structure of the GCF documents

## Documents

| Description of the abbreviations and definitions used in the GCF document | GCF-AD (abbreviations and definitions) |
| --- | --- |
| Outline the processes for a Manufacturer to certify a GSM Terminal Device | GCF-AP (application procedure) |
| Description of the structure and working procedures of the GCF | GCF-OP (organization procedures) |
| Details of the approved operating budget of the Global Certification Forum for each year | GCF-OB (operating budget) |
| Summary of the Global Certification Forum Program | GCF-PD (principles documents) |
| Include the requirements and tests for GSM 900 and GSM 1800 that are in European Type Approval under the R&TTE Directive | GCF-CCR (Certification criteria – Part1, Regulatory) |
| Include requirements and tests added for the purposes of Terminal Certification. | GCF-CCV (Certification criteria – Part2, Additional voluntary tests) |
| Combine data previously contained in the documents GCF-CCR, GCF-CCV plus data from the GSM TAAB PRD GT.01 | GCF-CC (Certification Criteria) |

All these documents are available on the GCF website : http://gcf.gsm.org

## Declarations

| Name of Declaration | GCF-Document |
| --- | --- |
| Declaration of participation – Operator Participant | GCF-PD (Annex A) |
| Declaration of participation – Manufacturer Participant | GCF-PD (Annex A) |
| Declaration of participation – Observer Participant | GCF-PD (Annex A) |
| Field Trail Qualified Declaration | GCF-OP (Annex A) |
| Quality Qualified Declaration | GCF-AP (Annex A) |
| Assessment Compliant Declaration | GCF-AP (Annex A) |
| Certification Declaration | GCF-AP (Annex A) |

# How to be GCF certified ?

The complete process is described in the GCF-AP (Application Procedure) document.

These are the steps to follow to be GCF certified :

- To be a member GCF

- To have a quality qualified insurance (at least ISO 9000)

- Create a compliance folder file and keep it

- Address to the Secretariat, three declarations:

  - Quality qualified declaration

  - Assessment compliance declaration

  - Certification declaration

- The secretariat reviews the declarations

- The terminal certified appears on the website of the GCF

# GCF certification process

In this paragraph, all the points mentioned in the paragraph 3 are described.

## Become a member of the GCF

To become a member of the GCF, a manufacturer must address to the Secretariat of the GCF the Declaration of Participation – Manufacturer Participant filled. This document is available in the GCF-PD (Principles Document).

## Quality qualified insurance

The minimum insurance requested is the ISO9000 or equivalent. This quality program must be implemented for :

- The research and the development of the ME (design)

- The manufacture of the ME

This program must be periodically approved by an external organisation to be still validated.

## Compliance folder

This folder is composed of all the evidences which prove that:

- The ME is developed and manufactured through a quality qualified program

- The manufacturer has a quality insurance

- The manufacturer will use all the means of tests to conduct the Self Assessment on his ME (by himself or by using a third party organization)

- The ME has passed all the applicable certification criteria defined by the GCF in the GCF-CC document (cf. annex A)

- The ME has passed the field trials on at least 5 dual band networks (cf. annex B)

To summarize, all the evidences which prove the truthfulness of the three declarations addressed to the Secretariat, must be in the compliance folder.

The exact contains of the compliance folder is:

- Concerning the ME (request by Wavecom and not by the GCF)

- Electrical schematics

- PCB layout

- BOM

- Mechanical schematics

- Specifications

- User manual

- Concerning the quality insurance: all the evidences for that (certificate…)

- Concerning the tests of the GCF-CC

  - The test reports performed in an accredited laboratory

  - Or the test reports performed in the manufacturer’s in-house

- Concerning the field tests

  - The test reports performed on at least 5 dual band networks

  - The Manufacturer shall record what Field Trials have been performed, on what networks, with what Terminal Device configurations, against what Methods of Test, and what the results of these Field Trials were.

  - Where anomalous performance has been observed, this shall be recorded together with the action taken to resolve the anomaly - re-design of Terminal Device, acknowledged problem with network or SIM etc. i.e. improvements and regression tests shall be included in the Compliance Folder

- All the engineering changes information of the ME

The compliance folder must be updated with the changes of the terminal equipment certified.

No compliance folder is addressed to the GCF or to the GCF members. It must be kept by the manufacturer.

## Declarations

These declarations are sent to the GCF Secretariat. A copy could be kept by the manufacturer in his compliance folder.

### Quality qualify declaration

This declaration is available in the GCF document GCF-AP (annex A) and in this document in annex C.

Signing this declaration is declaring that the manufacturer has a quality insurance.

### Assessment compliance declaration

This declaration is available in the GCF document GCF-AP (annex B) and in this document in annex D.

Signing this declaration is declaring that the manufacturer will use all the means he has in order to conduct the self assessment on his ME.

The means can be:

- The means of tests of the manufacturer (which must be conformed with the requirements of Annex V of the R&TTE or an equivalent standard)

- Those of a third party organization conformed to the EN ISO / IEC 17025 series of standards, or an equivalent standard

### Certification declaration

This declaration is available in the GCF document GCF-AP (annex C) and in this document in annex E.

Signing this declaration is declaring that the ME complies with all the applicable certification criteria of the GCF, and that the manufacturer will takes all the measures in order to assure confidence and security of the IMEI as specified in the relevant GSM specifications.

With this declaration, the manufacturer must give:

- The option table filled of his terminal

- The version of each standards used (cf. annex F)

## Certification

When the Secretariat has received the three declarations (quality qualified declaration,  assessment compliance declaration and certification declaration), he checked them.

If there are complete, the secretariat sent to the manufacturer a letter (after 5 or 10 days) which said that the folder is complete.

If information is missing, the manufacturer is notified within 5 or 10 days by mail.

When all the process review of the declarations is complete, the GCF Secretariat shall publish the following details of the Declaration (of Certification) on the website accessible only to GCF operator members:

- Model Name

- Model Number;

- Hardware Version;

- Initial Software Version;

- Version of Standard Used (within the Certification Declaration)

- Date of Certification;

- tick box notation regarding certifications for certain technologies, e.g., WAP and Bluetooth; and

- All the information in the Options Table (cf. annex A and F)

When confirming the Declaration, the GCF Secretariat shall advise the Manufacturer of the information that will be published concerning the Declaration. The Manufacturer may request any corrections to this information that might be necessary.

The name of the terminal certified and of its manufacturer appears also on the website accessible to everyone.

# Modifications on the certified terminal

When a Certified Terminal Device is modified, the Manufacturer shall demonstrate continued compliance with the appropriate technical requirements of the Certification Forum.

Changes in the Terminal Device hardware and/or software configuration shall be recorded in the Compliance Folder together with the evidence of the continued compliance with the applicable Certification Criteria.

The applicable Certification Criteria are those which were applicable on the date the declaration of Terminal Certified Status was made. In particular, tests which were included in the GCF after the Terminal Device was Certified are not applicable, nor are changes to the tests made after the Terminal Device was Certified.

Any change which invalidates the Declaration made by the Manufacturer shall be notified to the GCF Secretariat within thirty days, accompanied by a request to withdraw a Terminal device from holding Certified Status. Any change that requires a modification to the information within the Declaration shall be notified to the GCF Secretariat within 30 days.

When a product is updated, if for any given feature group the number of Validated Tests have increased by 33% per feature group or more then all tests in scope at the time of update shall be declared against.  If the number of Validated Tests have increased by less than 33% per feature group then only those tests in scope at the time of the original declaration shall be in the scope of the revised declaration.

For example:

| Validated Tests at initial Certification | Number at Update | Number in scope |
| --- | --- | --- |
| 0 | 10 | 10 |
| 10 | 13 | 10 |
| 10 | 14 | 14 |
| 50 | 9 | 9 |

# Conclusion

The GCF certification is mainly composed by self assessment declarations. Nevertheless, the manufacturer must be able to provide any evidences which prove that this declarations are true in case of control by the GCF.

The GCF certification is gaining more and more importance for the operators who now require this certification to allow a mobile on their network.

# Annexe A : Conformity criteria of the GCF

## Introduction

There are defined in the GCF document GCF-CC (certification criteria) and there are based on the GERAN TS 51.010.

Be careful to always use the last version of the GCF-CC for the tests.

## Definition of the applicable tests

### Option table

With the pics-pixit file of the mobile, a manufacturer is able to know which tests are applicable or not. The GCF doesn’t use the name of pics-pixit file but the option table.

The option table describes all the features and services supported by the terminal. It can be found in the GCF-CC document.

### Requirement table

This table described the reasons why a test is mandatory or not and it also gives the reference of the standard where this test can be found in the 3GPP documents. It can be found in the GCF-CC document.

### Conformance assessment table

This table described all the test cases to do by a mobile in order to comply with the certification criteria of the GCF. It can be found in the GCF-CC document.

With each test cases, an indication of its status is available.

Three status exist:

- Full status

- Provisional status

- Other status

The status is the confidence indication of the GCF in the test.

#### Full status

Two categories : A and B

Class A : means that tests have been specified by an appropriate SDO (Standard Development Organization) and validated on commercially available test equipment.

Class B : means that tests have been specified by an appropriate SDO and validated on commercially available test equipment with exceptions.

Examples of exceptions :

- Means of Test where test purposes are not fully tested, because parts of the Means of Test are not complete and/or incorrect and therefore not valid for accredited testing.

- Means of Test where a "PASS" verdict is a valid verdict, but a "FAIL" verdict may not be valid.

- Means of Test not validated for a special Terminal Device configuration.

#### Provisional status

Class P : tests which have been specified by an appropriate SDO and  not validated on commercially available test equipment

This class is used for test purposes which are planned for future implementation within the Global Certification scheme.

#### Other status

Class N : test not applicable (for a particular band).

#### Example of conformance assessment table

Conformance Assessment Table

| GSM 11.10 / TS 51.010 Requirement | Description | Category |  |  | Effective Date |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | GSM 900 | GSM 1800 | GSM 1900 | GSM 900 | GSM 1800 | GSM 1900 |
| 12.1.1 | Conducted spurious emissions - MS allocated a channel | A | A | N |  |  |  |
| 12.1.2 | Conducted spurious emissions - MS in idle mode | A | A | N |  |  |  |
| 12.2.1 | Radiated spurious emissions - MS allocated a channel | A | A | N |  |  |  |
| 12.2.2 | Radiated spurious emissions - MS in idle mode | A | A | N |  |  |  |
| 13.1 | Transmitter – Frequency error and phase error | A | A | N |  |  |  |
| 13.2 | Transmitter – Frequency error under multipath and interference conditions | A | A | N |  |  |  |
| 13.3-1 | Transmitter output power and burst timing - MS with permanent antenna connector | A | A | N |  |  |  |
| 13.4 | Transmitter - Output RF spectrum | A | A | N |  |  |  |
| 13.6 | Transmitter – Frequency error and phase error in HSCSD multislot configuration | P | P | N |  |  |  |
| 13.7 | Transmitter output power and burst timing in HSCSD configurations | P | P | N |  |  |  |
| 13.8 | Transmitter, Output RF spectrum in HSCSD multislot configuration | P | P | N |  |  |  |
| 13.16.1 | Frequency error and phase error in GPRS multislot configuration | P | P | N |  |  |  |
| 13.16.2 | Transmitter output power in GPRS multislot configuration | P | P | N |  |  |  |
| 13.16.3 | Output RF spectrum in GPRS multislot configuration | P | P | N |  |  |  |
| 14.7.1 | Blocking and spurious response - speech channels | A | A | N |  |  |  |

# Annexe B : Field trials

## Description

This tests are performed on at least five dual band networks. It is mandatory to pass these field tests.

These tests have been included to ensure that there is confidence in the operation of Terminal Devices in the operational network environment.

## Process

These tests can be done:

- By the manufacturer

- By a third party organization

It is recommended that the manufacturer makes use of Field Trial Qualified networks (by the GCF) wherever practicable to carry out Field Trials of their Terminals.

In case of dual band mobiles, two ways of tests can be used:

- on five dual band 900/1800 networks in GSM and DCS bands

- on 2 GSM networks, on 2 DCS networks and on 3 dual band 900/1800 networks in GSM and DCS bands

## The field trial networks

The tested networks must be representative of the principle existing infrastructure in Europe.

The networks must be field trial qualified. On the Certification Website, there will be a list of all registered Field Trial Qualified networks. Associated with each of these registered networks will be the name and contact details of the network’s Field Trial Officer

It is recommended to have a non-disclosure agreement between the manufacturer and the operator whose network is used for the field trials.

The target of these tests is to verify inter working between the terminal device and the network. The Field Trial is beneficial to the development process since it is where compatibility issues with commercial networks can be discovered. That means that normally an operator must describe all the available services of his network and also all the new services which are still not available to all the user, in order that the ME can test them.

The operators should also give any information about the SIM allowed on their networks, but these SIM can’t be provide by the operators to the testers.

Regarding the tests to do for the field trials, no test cases list and no descriptions about how to do the tests are proposed by the GCF. To know which tests must be done, the manufacturer must chose them based on his experience.

The operators needn’t to provide a test list to manufacturer and the manufacturer must not accept the one proposed by an operator.

## Criteria to test

It is the Manufacturer’s responsibility to define their own Field Trial procedures based upon their experience. It is recommended that the following inter working functions of Terminal Device functionality should be addressed during Field Trials:

- Basic call handling, including set up, clear and in-call functions

- Cell selection and reselection

- Automatic & manual PLMN selection

- Handover

- Operation of each basic service, supplementary service and feature within the scope of GCF and which is supported by the mobile. e.g. For fax and data services this includes the ability to make and receive such calls where the mobile supports them, and that data transfer is successfully carried out. For speech, it includes a degree of subjective speech testing sufficient to satisfy the manufacturer's requirements.

- SMS (MO-PP, MT-PP and CB)

- Inter working with the SIM (using a different type of SIM card/profile for each corresponding network, where reasonably practicable)

- Any additional features and services supported by the MS.

Field Trials shall be performed by the Manufacturer or a third party under the quality control of the Manufacturer, at the sole discretion of the Manufacturer. Any anomaly in operation shall be investigated and appropriate action taken, and the results recorded in the Compliance Folder.

## Compliance folder

Results of the Field Trials performed shall be recorded in the Compliance Folder. The principle requirements for the Compliance Folder with respect to the Field Trials are:

- The Manufacturer shall record what Field Trials have been performed, on what networks, with what Terminal Device configurations, against what Methods of Test, and what the results of these Field Trials were.

- Where anomalous performance has been observed, this shall be recorded together with the action taken to resolve the anomaly - re-design of Terminal Device, acknowledged problem with network or SIM etc. i.e. improvements and regression tests shall be included in the Compliance Folder.

- The exact format and content of the Compliance Folder is a matter for the Manufacturer to define within its Quality Assurance Program Requirements.

## New configuration of the terminal

It is up to the manufacturer to know and to determine which tests must be re-tested depending of the changes on the terminal.

The manufacturer must update the Compliance Folder with any relevant new information.

## The future

A new document made by the Agreement Group will be soon available. In this document, each test to do for the field trials will be listed and explained.

# Annexe C : Quality Qualified Declaration

This declaration should be sent to the following address:

GCF Secretariat

GSM Association

Temple Road

Blackrock

Co Dublin

Ireland

Fax +353 1 269 59 58

Manufacturer’s Name:	_________________________________

Manufacturer’s Business Address:	_________________________________

_________________________________

_________________________________

Briefly describe the Quality Assurance Program utilized by the Manufacturer (attach additional pages as necessary):

When was the Quality Assurance Program implemented by the Manufacturer?

Who is the Certification Officer and Deputy responsible for the Quality Assurance Program on the part of the Manufacturer? (Provide names and contact information.)

State the name of the certifying organisation, the certification date, certificate number, and indicate the activities covered by the certificate. If more than one certifying organisation is involved, then all the organisations are listed. (Providing a copy of the certificate will normally fulfil this requirement.)

Declaration:

I,______________________________________, declare and certify that ______________________

_________________________________________________________________________________,

having it’s principal place of business as indicated above, has implemented and utilises an in-house developed, state approved, or a nationally recognised Quality Assurance Program, in its normal day to day business practices and that the Quality Assurance Program described above has been formally adopted by the Manufacturer and has as one of its objectives that the GSM terminals manufactured by the Manufacturer meet the requirements of the GCF Certification Forum.

The Manufacturer respectfully requests confirmation as achieving Quality Qualified status.

______________________________________		___________________________________

Certified By						Title

______________________________________		___________________________________

Name							Date

______________________________________		___________________________________

Tel							Email

# Annexe D : Assessment Compliant Declaration

This declaration should be sent to the following address:

GCF Secretariat

GSM Association

Temple Road

Blackrock

Co Dublin

Ireland

Fax +353 1 269 59 58

Manufacturer’s Name:					_________________________________

Manufacturer’s Business Address:			_________________________________

_________________________________

_________________________________

Briefly describe the Manufacturer’s in-house capabilities to conduct Self Assessment through the Means of Test and Methods of Test as specified (attach additional pages as necessary):

If the Manufacturer does not possess the in-house capabilities to conduct Self Assessment in respect of one or more Means of Test, and intends to subcontract one or more Third Party Organisation(s), list the Third Party Organisations (attach additional pages as necessary):

For  in-house capabilities, state the name of the certifying organisation, the certification date, certificate number, and indicate the activities covered by the certificate. (Providing a copy of the certificate will normally fulfil this requirement.)

Declaration:

I,______________________________________, declare and certify that _______________________

_________________________________________________________________________________,

having it’s principal place of business as indicated, possesses the skills and Means of Test (including any Third Party Organisations utilised) required to conduct Self Assessment according to the Means of Assessment specified in the GCF Certification Forum documentation, that a Quality Assurance Program is utilised on a daily basis to conduct such Self Assessment and that the Means of Test and Methods of Test are maintained in order to continue to conduct Self Assessment as the GCF Certification Forum develops.

The Manufacturer respectfully requests confirmation as Assessment Compliant.

______________________________________		___________________________________

Certified By						Title

______________________________________		___________________________________

Name							Date

______________________________________		___________________________________

Tel							Email

# Annexe E : Certification Declaration

This declaration should be sent to the following address:

GCF Secretariat

GSM Association

Temple Road

Blackrock

Co Dublin

Ireland

Fax +353 1 269 59 58

Manufacturer’s Name:				_________________________________

Manufacturer’s Business Address:		_________________________________

_________________________________

_________________________________

State the means by which the Terminal Device may be identified (e.g. a unique model name or model number, and minimum hardware/software version level as appropriate). Also, where different, state any names under which the Terminal Device will be sold or marketed.

Declaration:

I,______________________________________, declare that _______________________________

Terminal Device described above and submitted by________________________________________, having it’s principal place of business as indicated above, has conducted Self Assessment on the aforementioned Terminal Device, and has prepared a Compliance Folder which contains appropriate evidence that the Terminal Device has met all applicable Certification Criteria as specified by the Global Certification Forum.

I also declare that in the design of the Terminal Device I have taken steps that I consider reasonable to assure corresponding confidence and security level of IMEI as specified in relevant GSM core specifications.

I also declare that the Compliance Folder will be maintained by the Manufacturer for Compliance Folder Review purposes, and that any changes made to the Terminal Device shall be recorded in the Compliance Folder, along with appropriate evidence that the Terminal Device continues to meet all of the applicable Certification Criteria.

Relevant versions of GCF-CC:		GCF-CC:	Ver.	____________________

I request confirmation of Certified status for the Terminal Device submitted.

______________________________________		___________________________________

Certified By						Title

______________________________________		___________________________________

Name							Date

______________________________________		___________________________________

Tel							Email

# Annex F : Additional information accompanying the Certification Declaration

- Option table

Attach a copy of the Options Table from GCF-CC, with the blank entries completed to indicate the features supported by the Terminal Device. This shall form a part of the Declaration.

- Versions of standards used

In the following table, state the versions of the standards used as the basis for this declaration :

| Standard | Version | Standard | Version |
| --- | --- | --- | --- |
| GSM 02.07 |  | GSM 04.68 |  |
| GSM 02.09 |  | GSM 04.69 |  |
| GSM 02.11 |  | GSM 04.81 |  |
| GSM 02.16 |  | GSM 04.82 |  |
| GSM 02.22 |  | GSM 04.83 |  |
| GSM 02.24 |  | GSM 04.84 |  |
| GSM 02.30 |  | GSM 04.85 |  |
| GSM 02.40 |  | GSM 04.86 |  |
| GSM 03.13 |  | GSM 04.88 |  |
| GSM 03.14 |  | GSM 04.90 |  |
| GSM 03.22 |  | GSM 04.91 |  |
| GSM 03.26 |  | GSM 05.05 |  |
| GSM 03.40 |  | GSM 05.08 |  |
| GSM 03.41 |  | GSM 05.09 |  |
| GSM 03.45 |  | GSM 05.10 |  |
| GSM 03.50 |  | GSM 07.02 |  |
| GSM 04.06 |  | GSM 07.03 |  |
| GSM 04.08 |  | GSM 07.05 |  |
| GSM 04.10 |  | GSM 07.07 |  |
| GSM 04.11 |  | GSM 07.08 |  |
| GSM 04.12 |  | GSM 07.60 |  |
| GSM 04.13 |  | GSM 11.10-1 |  |
| GSM 04.21 |  | GSM 11.10-4 |  |
| GSM 04.22 |  | GSM 11.11 |  |
| GSM 05.60 |  | GSM 11.12 |  |
| GSM 04.63 |  | GSM 11.14 |  |
| GSM 04.64 |  | TS 51.010-1 |  |
| GSM 04.65 |  |  |  |
| GSM 04.67 |  |  |  |

Note: If a feature is implemented according to a 3GPP core specification not listed here, then the appropriate 3 GPP core specification can be given alternatively.

- Support  for other features:

Bluetooth:

Is Bluetooth enabled?    Y/N

Is the terminal Bluetooth Qualified?  Y/N

WAP:

Is WAP implemented? Y/N

Is the terminal WAP Certified?  Y/N

# Annexe G : Regional Regulatory Requirements

Details of tests which must be performed in order to comply with Regional Regulatory Requirements are given in the following table.

| GSM 11.10 / TS 51.010 Requirement | Description | Region / Regulatory Requirement |  |  |
| --- | --- | --- | --- | --- |
|  |  | EU / R&TTE Directive |  |  |
| 12.1.1 | Conducted spurious emissions - MS allocated a channel | Yes |  |  |
| 12.1.2 | Conducted spurious emissions - MS in idle mode | Yes |  |  |
| 12.2.1 | Radiated spurious emissions - MS allocated a channel | Yes |  |  |
| 12.2.2 | Radiated spurious emissions - MS in idle mode | Yes |  |  |
| 13.1 | Transmitter – Frequency error and phase error | Yes |  |  |
| 13.2 | Transmitter – Frequency error under multipath and interference conditions | Yes |  |  |
| 13.3-1 | Transmitter output power and burst timing - MS with permanent antenna connector | Yes |  |  |
| 13.4 | Transmitter - Output RF spectrum | Yes |  |  |
| 13.6 | Transmitter – Frequency error and phase error in HSCSD multislot configuration | Yes |  |  |
| 13.7 | Transmitter output power and burst timing in HSCSD configurations | Yes |  |  |
| 13.8 | Transmitter, Output RF spectrum in HSCSD multislot configuration | Yes |  |  |
| 13.16.1 | Frequency error and phase error in GPRS multislot configuration | Yes |  |  |
| 13.16.2 | Transmitter output power in GPRS multislot configuration | Yes |  |  |
| 13.16.3 | Output RF spectrum in GPRS multislot configuration | Yes |  |  |
| 14.7.1 | Blocking and spurious response - speech channels | Yes |  |  |